3GPP TSG RAN2 Meeting #131bis				DRAFT_R2-2507933

Prague, Czech Republic, October 13-17, 2025

Title:		         DRAFT LS on Early Alignment on Access Stratum security aspects

Response to:

Release:	Rel-20

Work Item:	Study on 6G Radio [New RAN1 led SI: FS_6G_Radio]

Source:	Vodafone [to be replaced by RAN2]

To:	SA3

Cc:	RAN 1, RAN 3, SA 2

Contact Person:

#### Name:	Alexey Kulakov

E-mail Address: 	Alexey.kulakov1@vodafone.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

RAN2 has started the study on control and user plane aspects for 6GR.

Security considerations are critical to ensure that protocol design aligns with security needs.

In 5G, the PDCP layer provides ciphering and integrity protection for both user data and RRC signaling. However, RAN2 mapped selected control information directly into the MAC PDU. I.e., the information elements do not pass through PDCP and RLC and are hence not subject to ciphering and integrity protection. RAN2’s ambition was typically to avoid the processing-, latency-, and/or signaling overhead.. For reference, the list of MAC CEs in 5G is defined in TS 38.321 chapter 6.1.3. It is possible that some of these information elements may also be for 6G.

RAN2 understands the need to ensure privacy and integrity  but considers it also important to take latency and signalling overhead into account. For example, the UE sends buffer status reports (BSR) frequently and it can determine their content only after deciding which user data to send in the same uplink slot. Hence, if the BSR was to be ciphered and integrity protected, it would likely have a noticeable impact on latency and signalling overhead.  While some information elements may have similar properties as the BSR, others may be less frequent and less time critical and it would hence be more affordable to cipher and integrity protect those. . RAN2 would appreciate to work jointly with SA3 to reach a common understanding on those trade-offs and to develop a feasible solution for the information elements that are deemed to require additional protection.

2. Actions:

To SA3:

ACTION:

RAN2 respectfully requests SA3 to provide information on the following aspects:

- Identification of only the critical lower layer control information that requires protection considering the above concerns from RAN2. RAN2 will be glad to assist with further information if needed.

- What type of protection (e.g., integrity protection and/or ciphering protection) is required for such critical information.

- What kind of overhead could the protection of the critical information per above incur

- Any other information that SA3 deems important for RAN2 to understand.

RAN2 would also greatly appreciate it if SA3 could provide input and/or follow-up questions timely so that RAN2 can take them into account during the 6G protocol stack design.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 	Meeting #132	17-21 November 2025	Dallas, USA

TSG-RAN2 	Meeting #133	 09-13 February   2025	Stor-Göteborg, Sweden