3GPP TSG RAN WG2 Meeting #131bis      	                                                 R2-250xxxx
Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	8.4.4

Source:	Huawei, HiSilicon (Rapporteur)

Title:	Updating the Rel-19 LPWUS capabilities CR and identifying open issues

WID/SID:	NR_LPWUS-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

This is to trigger the following email discussion.

- [Post131bis][211][LPWUS] CR for TS 38.306 (Huawei)

Intended outcome: Update the CR and identify any additional other open issues.

Deadline:  Long

Companies are invited to provide input by Wednesday October 29 18:00 UTC.

## Contact information:

| Company | Delegate Name | Email |
| --- | --- | --- |
| Ericsson | Martin van der Zee | martin.van.der.zee@ericsson.com |
| Huawei/HiSilicon | Rama Kumar Mopidevi | Rama.kumar@huawei.com |

# 2	Discussion

## 2.1 Aligning “minimumTimeGap-r19” capability with RAN1 FGs

This is to align “minimumTimeGap-r19” with RAN1 feature group list as pointed out by Lenovo during the main session.

R2-2507586 Corrections on Rel-19 RAN1/4 UE capability Xiaomi CR Rel-19 38.306 19.0.0 1370 - F NR_LPWUS, NR_MIMO_Ph5 Use a baseline for next revision R2-2507587 Corrections on Rel-19 RAN1/4 UE capability Xiaomi CR Rel-19 38.331 19.0.0 5547 - F NR_LPWUS, NR_AIML_air, NR_ENDC_RF_Ph4 - - Lenovo is concerned that this is not aligned with the feature list. Vivo confirms that it reflects RAN1 intention. Use a baseline for next revision

Following is the excerpt from “R1-2506627 Updated RAN1 UE features list for Rel-19 NR after RAN1 #122”:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 5. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring 6. Support of all M values {1, 2, 4} for FR1 for LP-WUS 7. Support of M value 1 for 120 kHz SCS FR2 for LP-WUS 8. Support of all M values {1, 2, 4} for LP-SS |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: {capability 1, capability 2, capability 3} Note: According to RAN2 agreement, UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading. How to capture this is up to RAN2 | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence 2a. The support of SSB-based RRM measurement 5. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring 6. “Support of all M values {1, 2, 4} for FR1 for LP-WUS 7. “Support of M value 1 for 120 kHz SCS FR2 for LP-WUS |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: {capability 1, capability 2, capability 3} Note: According to RAN2 agreement, UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading. How to capture this is up to RAN2 | Optional with capability signalling |

As shown in the above table, minimum time gap between LP-WUS reception and UE to start PDCCH monitoring has to be reported separately for UE supporting LP-WUS with OOK and OFDM. The endorsed UE capabilities CR (R2-2507586) has the following change:

- minimumTimeGap-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in below table: SSB periodicity (ms); cap1 (ms); cap2 (ms); cap3 (ms). 5/10/20; 70; 500; 900. 40; 130; 600; 1000. 80; 250; 800; 1200. 160; 490; 1200; 1600.

To align with RAN1 feature list, “minimumTimeGap-r19” has to be reported separate as shown below:

- minimumTimeGap-OOK-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring when UE indicates support of lpwus-OOK-r19. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in the Table of “values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity” below. - minimumTimeGap-OFDM-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring when UE indicates support of lpwus-OFDM-r19. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in the Table of “values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity” below. - Table of values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity: SSB periodicity (ms); cap1 (ms); cap2 (ms); cap3 (ms). 5/10/20; 70; 500; 900. 40; 130; 600; 1000. 80; 250; 800; 1200. 160; 490; 1200; 1600.

### Q1: do companies agree with the above understanding/change?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes, but also see comment | We should discuss how UE and NW use this information, and check if something needs to be clarified: - For discussion’s sake assume that to reach an OFDM WUR the gNB can send a “short” LP-WUS, while for OOK WUR a “long” LP-WUS is required. - Suppose the UE supports both OFDM and OOK. And for OOK needs an “early” wake-up, and for OFDM supports a “late” wake-up. - The gNB has to send “long” LP-WUS when there is one or more UEs monitoring the PO that only support OOK-based WUR. The gNB can send “short” LP-WUS when all the UEs monitoring the PO support OFDM-based WUR. - It is not clear why/when the UE would decide to wake-up “early”, i.e. the NW may decide to send a “short” LP-WUS to save power / resources. The UE signals the minimum gap, such that the gNB can determine whether the UE can monitor LP-WUS with the configured time offset(s). However there is a problem when the configured time offset is in between “early” and “late” wake-up of the UE. Furthermore, perhaps some clarification is needed what the gap between LO and PO is: There is a frame offset and firstPDCCH symbol offset. And there can be a 1:4 mapping of LO to POs over up to 4 Paging Frames (2 PFs depicted below): If you have a 1:4 mapping with 1/16th PFs, then UEs experience a very different time gap. With PEI there was a limit up to 2 PFs, but for LP-WUS there is no such restriction. We also wonder if the possible configuration options should be clarified. Perhaps that is implicitly defined in the formula in 38.213, but that formula is not easy to read. We have the following understanding for now: Some of the topics above are not related to the UE capability, but they need to be clear to understand how the “minimum time gap” capability is used by the NW. |

## 2.2 Terminology alignment for “RRM relaxation and offloading”

In R2-2507626, it was pointed out that different specs are using different terminology for “RRM relaxation and offloading” and proposed to align. Spec rapporteurs agreed to align the terminology during offline. Below shows the change in 38.306:

| Relaxation of serving cell and further relaxation of neighboring cell RRM measurements and offloading of serving cell RRM measurements | It is mandatory to support relaxation of serving cell and neighboring cell RRM measurements as specified in clause 5.2.4.12.1 in TS 38.304 [21] and offloading of serving cell RRM measurements as specified in clause 5.2.4.12.3 in TS 38.304 [21] if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21]. A UE supporting this feature shall also indicate the support at least one of lpwus-OOK-r19 and lpwus-OFDM-r19. |
| --- | --- |

### Q2: do companies agree with the above change?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |

Draft CRs for 38.306 and 38.331 with the changes are provided in Appendix.

### Q3: If there are there any other open issues in LP-WUS UE capabilities, please list them below:

| Company | Open Issue |
| --- | --- |
| Ericsson | RAN2 should confirm that no separate UE capability is needed to indicate that the UE supports LP-WUS with Paging Adaptation. When the UE supports LP-WUS and the UE supports Paging Adaptation, then there is a small delta to support LP-WUS with Paging Adaptation, i.e. no need for a separate capability. However for PEI and Paging Adaptation a separate capability was introduced with a separate bandlist: pagingAdaptionPEI-SupportBandList-r19 Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band for paging adaption. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs. A UE supporting this feature shall also indicate support of pagingAdaptation-r19.. So the UE can indicate to support Paging Adaptation and PEI in a certain band, but the UE does not support PEI and Paging Adaptation in that band (but in some other bands where the UE supports PEI). This seems overkill. In case it is agreed that for LP-WUS and Paging Adaption no new UE capability is needed, then some of the wording needs to be updated to clarify the difference, e.g.: pagingAdaptPEI-Config The PEI related configuration for paging adaptation. If the UE supports pagingAdaptionPEI-SupportBandList-r19 paging adaptation for PEI, it ignores pei-Config-r17, if configured.. lpwus-LoFrameOffsetListForPagingAdapt Indicates the frame-level offset between an LO and a reference PO/PF for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C) for paging adaptation. If the UE supports paging adaptation andfor LP-WUS, it ignores lpwus-LoFrameOffsetList, if configured. The entry number of offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay is equal to CEIL (Ns / lpwus-PoNumPerLo), where Ns is the number of paging occasions per paging frame configured in SIB1 for paging adaptation. At least one frame-level offset, i.e., offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay is provided.. |
| Huawei, HiSilicon | In the last meeting, RAN2 made the following agreement: - The UE capabilities for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message. Detailed signalling can be further discussed. We provided below 2 options in R2-2506965: - Option 1: only indicating the supported band list for LP-WUS regardless of the receiver type and other capabilities; - Option 2: indicating the supported band list for LP-WUS and the receiver type. Which option to go for can be further discussed. |

# 3	Conclusion

TBD.

# 4 Appendix 1: Draft CR to TS 38.306

3GPP TSG-RAN WG2 Meeting #131bis		R2-250xxxx

Prague, Czech Republic, Oct. 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to R19 LP-WUS UE Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2-131b: When describing the measurement relaxation related capabilities in TS 38.306, usually the description refers to other core specification that describes the corresponding measurement relaxation mechanism. Some examples: Relaxed measurement It is optional for UE to support relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21]. Rel-17 relaxed measurement for RRC_IDLE/RRC_INACTIVE It is optional for (e)RedCap UE to support Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21]. rrm-RelaxationRRC-ConnectedRedCap-r17 Indicates whether (e)RedCap UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331 [9]. RAN2-131b Post Meeting: - To align “” with RAN1 feature group list, define separate capabilities for “” - Align terminology on “RRM relaxation and offloading” among different specs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added reference for “Relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements” capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear what the “relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE” is. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS38.331 draft CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| airToGroundNetwork-r18 Indicates whether the UE supports air to ground network access. If the UE indicates this capability the UE shall support the following ATG essential features, e.g., acquiring ATG cell specific SIB22 and ATG cell specific P-Max. | UE | No | No | FR1 only |
| crossCarrierSchedulingConfigurationRelease-r17 Indicates whether the UE supports using crossCarrierSchedulingConfigRelease to release the configurations configured by crossCarrierSchedulingConfig. | UE | No | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| dl-DedicatedMessageSegmentation-r16 Indicates whether the UE supports reception of segmented DL RRC messages. | UE | No | No | No |
| drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| drx-PreferenceCellDTX-DRX-r19 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving and its preference on cell DTX/DRX related parameters for PCell in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| gNB-SideRTT-BasedPDC-r17 Indicates whether the UE supports gNB-side RTT-based PDC, as specified in TS 38.300 [28]. A UE supporting this feature shall also support rtt-BasedPDC-CSI-RS-ForTracking-r17 and/or rtt-BasedPDC-PRS-r17. | UE | No | No | No |
| hardSatelliteSwitchResyncNTN-r18 Indicates whether UE supports hard satellite switch with re-sync, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. When UE supports this feature and does not support softSatelliteSwitchResyncNTN-r18, this UE is able to perform hard satellite switch with re-sync in a network supporting soft satellite switch with re-sync, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_INACTIVE as specified in TS 38.331 [9]. This capability is not applicable to NCR-MT. | UE | Yes | No | No |
| inactiveStateNTN-r17 Indicates whether the UE supports RRC_INACTIVE in NTN as specified in TS 38.331 [9]. It is mandated if the UE indicates the support of nonTerrestrialNetwork-r17. | UE | CY | No | No |  |
| inactiveStatePO-Determination-r17 Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state. | UE | No | No | No |
| inDeviceCoexInd-r16 Indicates whether the UE supports reporting of affected NR carrier frequencies in IDC assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| inDeviceCoexIndAutonomousDenial-r18 Indicates whether the UE supports IDC autonomous denial as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndFDM-r18 Indicates whether the UE supports reporting of affected NR carrier frequency ranges in IDC assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndTDM-r18 Indicates whether the UE supports reporting of IDC TDM assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| lpwus-SupportedBandList-r19 Indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode for a list of frequency bands. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of LP-WUS operation for the frequency band. The UE supporting this feature shall also indicate the support of at least lpwus-OOK-r19 or lpwus-OFDM-r19. For each supported band, the capability signalling comprises of the following parameters: - supportedBandIndicator-r19 indicates the frequency bands where UE supports LP-WUS operation in IDLE/INACTIVE mode; - lpwus-OOK-r19 indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode based on OOK signal. The UE indicating this feature supports LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal, LP-SS based RRM measurement, all M values {1, 2, 4} for FR1 for LP-WUS, M value 1 for 120 kHz SCS FR2 for LP-WUS and all M values {1, 2, 4} for LP-SS; - lpwus-OFDM-r19 indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence. The UE indicating this feature supports LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence, SSB-based RRM measurement, all M values {1, 2, 4} for FR1 for LP-WUS, M value 1 for 120 kHz SCS FR2 for LP-WUS; - lpwus-LP-SS-r19 indicates whether the UE supports LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured. The UE indicating this feature also supports all M values {1,2,4} for LP-SS. A UE supporting lpwus-LP-SS-r19 shall also indicate support of lpwus-OFDM-r19; NOTE: If LP-SS overlaid sequence is configured, and if both SSB based and LP-SS based thresholds are configured for RRM measurement, it is up to UE implementation which threshold to use. - minimumTimeGap-OOK-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring when UE indicates support of lpwus-OOK-r19. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in the Table of “values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity” below. - minimumTimeGap-OFDM-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring when UE indicates support of lpwus-OFDM-r19. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in the Table of “values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity” below. - Table of values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity: SSB periodicity (ms); cap1 (ms); cap2 (ms); cap3 (ms). 5/10/20; 70; 500; 900. 40; 130; 600; 1000. 80; 250; 800; 1200. 160; 490; 1200; 1600. | UE | No | No | No |
| maxBW-Preference-r16, maxBW-Preference-r17 Indicates whether the UE supports providing its preference of a cell group on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes (Incl FR2-2 DIFF) |
| maxCC-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |

# 6	Conditionally mandatory features without UE radio access capability parameters

| Features | Condition |
| --- | --- |
| Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList | It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9]. |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |
| AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE | It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports" |
| ATG specific P-max | It is mandatory to support the ATG specific P-max configured by network for UEs supporting airToGroundNetwork-r18. |
| Downlink SDAP header | Either NAS reflective QoS or as-ReflectiveQoS is supported. |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |
| IMS emergency call | It is mandatory to support IMS emergency call over PLMN for UEs which are IMS voice capable in NR. It is mandatory to support IMS emergency call over SNPN for UEs that are SNPN capable and IMS voice capable over SNPNs. |
| Logged measurements suspension due to IDC interference | It is mandatory to support Logged measurements suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network and in-device coexistence indication as specified in TS 38.331 [9]. |
| MAC subheaders with LX field | It is mandatory to support MAC subheaders with LX field for UEs supporting MAC SDU(s) using the LCID value(s) as specified in Table 6.2.1-2c in TS 38.321 [8]. |
| MAC subheaders with one-octet eLCID field | It is mandatory to support MAC subheaders with one-octet eLCID field for UEs/IAB-MTs/NCR-MTs supporting MAC CEs using extended LCID values as specified in TS 38.321 [8]. |
| Paging cause in RAN paging message | It is mandatory for a UE to support paging cause in RAN paging if UE supports paging cause in CN paging. |
| Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | It is mandatory for a UE supporting NR sidelink in shared spectrum and when shared spectrum channel access must be used to support receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol and monitor a total up to the number reported in pscch-RxSidelink-r16 of PSCCHs in a slot in the 1st and 2nd starting symbols. A UE supporting this feature shall indicate support of sl-Reception-r16. |
| Receiving UE to UE COT sharing information | It is mandatory for a UE supporting NR SL in shared spectrum where shared spectrum channel access must be used to support monitoring SCI to read COT sharing information and transmitting NR SL based on COT sharing information subject to COT sharing conditions. A UE supporting this feature shall indicate support of sl-DynamicChannelAccess-r18. |
| SON report in PNI-NPN | It is mandatory for a UE to support a SON report in PNI-NPN if UE supports PNI-NPN and supports the SON report in PLMN. |
| Skipping UL configured grant if no data to transmit, as specified in release-15 version of TS 38.321 [8]. | Either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 or configuredUL-GrantType2 or configuredUL-GrantType2-v1650 is supported. |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 or uplinkTA-ReportingATG-r18 as specified in TS 38.321 [8]. |
| Inter-frequency configuration for less than 5MHz in SIB4 | It is mandatory to support configuration of dl-CarrierFreq-r18 and frequencyBandList-r18 as specified in TS 38.331 [9] for UEs supporting support5MHz-ChannelBW-20PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18 or support3MHz-ChannelBW-Asymmetric-r18. |
| Relaxation of serving cell and further relaxation of neighboring cell RRM measurements and offloading of serving cell RRM measurements | It is mandatory to support relaxation of serving cell and neighboring cell RRM measurements as specified in clause 5.2.4.12.1 in TS 38.304 [21] and offloading of serving cell RRM measurements as specified in clause 5.2.4.12.3 in TS 38.304 [21] if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21]. A UE supporting this feature shall also indicate the support at least one of lpwus-OOK-r19 and lpwus-OFDM-r19. |

END OF CHANGE

# 5 Appendix 2: Draft CR to TS 38.331

3GPP TSG-RAN WG2 Meeting #131bis	R2-250xxxx

# Prague, CZ, 13th – 17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-19 RAN1 UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Indicate separate capabilities for minimumTimeGap-r19 to align with RAN1 feature group list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define minimumTimeGap-OOK-r19 and minimumTimeGap-OOK-r19 in LPWUS-SupportedBandInfo-r19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capabilities for minimumTimeGap are not aligned with RAN1 feature group list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.306 draft CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Protocol data units, formats and parameters (ASN.1)

START OF CHANGE

#### –	UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

UE-RadioPagingInfo-r19 ::=            SEQUENCE {

lpwus-SupportedBandList-r19           SEQUENCE (SIZE (1..maxBands)) OF LPWUS-SupportedBandInfo-r19        OPTIONAL,

pagingAdaptation-r19                  ENUMERATED {supported}                                              OPTIONAL,

pagingAdaptionPEI-SupportBandList-r19 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR                OPTIONAL,

...

}

LPWUS-SupportedBandInfo-r19 ::=      SEQUENCE {

supportedBandIndicator-r19           FreqBandIndicatorNR,

-- R1 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal

lpwus-OOK-r19                        ENUMERATED {supported}                                               OPTIONAL,

-- R1 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence

lpwus-OFDM-r19                       ENUMERATED {supported}                                               OPTIONAL,

-- R1 62-1b: LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured

lpwus-LP-SS-r19                      ENUMERATED {supported}                                               OPTIONAL,

minimumTimeGap-OOK-r19

ENUMERATED {cap1, cap2, cap3}                                                                            OPTIONAL,

minimumTimeGap-OFDM-r19             ENUMERATED {cap1, cap2, cap3}                                         OPTIONAL

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP

END OF CHANGE