3GPP TSG-RAN WG2 Meeting #132	R2-250xxxx

Dallas, Texas, USA, November 17 - 21, 2025

Agenda item:	8.11.1

Source:	Samsung

Title:	Rel-19 SBFD MAC open issues for maintenance

Document for:	Discussion and Decision

Introduction

This offline discussion aims to identify and summarize any additional Rel-19 SBFD MAC open issues, as part of the following email discussion.

- [Post131bis][212][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Update the CR and identify any additional other open issues

Deadline:  Long

As the result of this discussion, the Rapporteur will provide a summary outlining:

- issues (if any) requiring further discussions with tdoc contributions in RAN2#132, and

- non-controversial/editorial issues (if any) that will be addressed by Rapp CR in RAN2#132, as well as the draft CR for review before submission deadline.

In this email discussion, we will collect the additional MAC open issues in Phase 1, followed by the discussions on the identified issues in Phase 2.

Input deadlines:

- Phase 1: Open Issue Identification, by Oct. 28, 10:00 UTC;

- Phase 2: Discussions on Identified Issues, by Oct. 31, 10:00 UTC.

Please provide your contact information in the table below when responding.

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |
| Nokia | Subin Narayanan (Nokia) | Subin.narayanan@nokia.com |
| LGE | Hanseul Hong | hanseul.hong@lge.com |
| Samsung | Weiping Sun | wp.son@samsung.com |

Phase 1: Open Issue Identification

Please share any additional MAC open issues, by explaining what the issue is, and sharing the suggested Way-Forward/TP, including editorial improvements.

| Company | Identified issues | Suggested TP | Rapp comment |
| --- | --- | --- | --- |
| Huawei, HiSilicon | As described in our contribution R2-2507003, when the conditions for RO type switching and for the Msg1 repetition number fallback are met at the same time (which would be quite possible considering the granularity of the configured threshold values), the further higher Msg1 repetition number would be chosen after the same or the next higher Msg1 repetition number is selected due to RO type switching. It is not the intended behavior as the RO type switching should have solved the transmission failure problem and it was never intended that we use both RO type switching and Msg1 repetition fallback at the same time for this scenario where the same Msg1 repetition number would never be used. It must be also noted that the resource selection would then be executed twice unnecessarily if not correction on this issue: 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; [Nokia]: We agree with the issue mentioned, and Ok with the proposed change. [Rapp] Kindly reformulate your input to the questionnaire of section 3.1 below. Thanks. | We are open to figure out the suitable change based on the below intention: 3> if the Random Access Preamble is transmitted with repetitions, and the RO type has not been changed from 1st-RO to 2nd-RO or vice versa, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: | An offline discussion (3.1) is triggered for this issue. |
| LGE | In TS 38.321, the SBFD-aware UE always selects the RO type between the first RO and second RO. In other words, if the SBFD RO is not configured (i.e., neither sbfd-RACH-SingleConfig nor sbfd-RACH-DualConfig is configured), the UE always selects first RO, regardless whether the cell is FDD cell or the TDD cell without tdd-UL-DL-ConfigurationCommon, or whether the selected carrier is SUL carrier. 1> else if neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure and either sbfd-RACH-SingleConfig or sbfd-RACH-DualConfig is configured by RRC for the Random Access procedure (as specified in TS 38.331 [5]): (…selection between 1st-RO and 2nd-RO based on conditions…) 1> else: 2> set the RO_TYPE to 1st-RO. 1> select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;. Based on the selected RO type, in clause 5.1.2 of TS 38.321, the UE selects the PRACH occasions 1> else if an SSB is selected above: (… omitted Msg1 repetition case) 2> else: 3> determine the next available PRACH occasion from the PRACH occasions of the selected RO type corresponding to the selected SSB permitted by ….. In summary, in following cases, MAC entity shall select the PRACH occasions of the first RO for CBRA cases, since there would be no SBFD RO: - Case 1): FDD cell or in SUL carrier; - Case 2) Unpaired spectrum, if a UE is not provided tdd-UL-DL-ConfigurationCommon On the other hand, in TS 38.213, the first RO is defined only for the case when tdd-UL-DL-ConfigurationCommon is configured: Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and considered as uplink for the random access procedure, or - second PRACH occasions, (…) The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions. Therefore, based on the current TS 38.321 and TS 38.213, when the tdd-UL-DL-ConfigurationCommon is not configured (e.g., in FDD cell and SUL carrier), the MAC entity selects the first RO (i.e., non-SBFD RO), but there is no definition of first RO when tdd-UL-DL-ConfigurationCommon is NOT configured, which makes inconsistency. | In order to resolve the inconsistency between the MAC specification and TS 38.213, followings can be considered: - Option 1) Update the MAC procedure to skip the selection of RO type, for the case of FDD cell, SUL carrier, or TDD cell without tdd-UL-DL-ConfigurationCommon - Option 2) Request RAN1 to update the definition of first RO, i.e., all ROs are defined as the first RO for the case of FDD cell, SUL carrier, or TDD cell without tdd-UL-DL-ConfigurationCommon In our understanding, Option 2 is much simpler, and it is aligned with the RA type selection procedure between 2-step RA and 4-step RA. | An offline discussion (3.2) is triggered for this issue. |

Rapp summary:

Two issues (identified by Huawei, and LG, respectively) worth further offline discussions, have been identified based on the inputs from companies in Phase 1. Consequently, the discussions seeking companies' views are triggered below, respectively, to determine whether tdoc contributions are required for these issues for the upcoming meeting.

Phase 2: Discussions on Issues Identified in Phase 1

[Huawei] Simultaneous Fallbacks of RO Type and Msg1 Repetition Number

Question 1: By taking into account the issue description provided by Huawei, do companies deem this issue worth addressing?

Question 2: If so, please indicate whether to support the TP from Huawei?

| Company | Need to address (Y/N) | Support TP from Huawei (Y/N) | Comment if any |
| --- | --- | --- | --- |
| Samsung | N with comment | N | We see no much negative impact with current operation. But, if the configuration granularity is the root cause of this issue, we can consider introducing more/finer values for the threshold configuration. |

[LG] RO Type When tdd-UL-DL-ConfigurationCommon is not applied

Question 1: By taking into account the issue description provided by LG, do companies deem this issue worth addressing?

Question 2: If so, please indicate the preferred way-forward to resolve this issue, between:

- Alt 1: Change MAC spec, to avoid introducing RO type selection and its impact for those cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured)

- Alt 2: Request RAN1 (e.g., LS) to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for those cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured)

| Company | Need to address (Y/N) | Alt 1 or 2 | Comment if any |
| --- | --- | --- | --- |
| Samsung | Y | Alt 2 | Prefer Alt 2 for spec conciseness and minimal spec change. |

Conclusions

To summarize:

Issues for further discussion with tdoc contributions in RAN2#132:

-

Issues that will be handled by Rapporteur CR in RAN2#132:

-