3GPP TSG RAN WG2 Meeting #132                	                	                          R2-250xxxx

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.0.1

Source:	Xiaomi

Title:	Summary of [POST131bis][004][UE caps] UE capability CR (Xiaomi)

Document for:	Discussion and Decision

# Introduction

- [POST132][005][UE Caps] Mega CR (Xiaomi)

Intended outcome: Update CR with further feature list update from Ran1/RAN4 and RAN2 endorsed CRs.

Deadline:  Short

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Seau Sian Lim | seau.sian.lim@huawei.com |

# Comment

| Comment Index | Capability IE/FG | Specification (306/331) | Comment | Proposed Change | Rapp resolution |
| --- | --- | --- | --- | --- | --- |
| H001 | R1 58-0-1 | 331 | We think this is not correctly implemented based on the following note: Note: Component 2 and 3 candidate values are signalled separately for each pool | It should be implemented e.g. as follow: aiml-CSI-ReportPerBC-r19 SEQUENCE { numCPUPool-r19 SEQUENCE (SIZE (1..2)) OF SEQUENCE {, maxNumCPUxPerCC-r19 INTEGER (1..8), maxNumCPUxAllCC-r19 INTEGER (1..32) } } OPTIONAL, |  |
| H002 | numCPUxPool-r19 | 331/306 | The ‘x’ can be removed since RAN1 has removed it in the field description and it is referring to the number of CPU pool which result in the presence of CPU.2 and CPU.3 pools. For maxNumCPUxPerCC-r19 and maxNumCPUxAllCC-r19 should be without the ‘x’ as it is referring to the CPU and to CPUx. | Removed ‘x ‘from numCPUxPool-r19. Likewise for maxNumCPUxPerCC-r19 maxNumCPUxAllCC-r19 |  |
| H003 | R1-58-1-2 resourceTypeSetB-CSI-RS-r19 inferenceReportType-r19 | 331 | For these 2 components in FG58-1-2, does it mean that UE can only support one of the report type for the BM case. Our understanding is that the UE can support e.g. periodic and semi-persistent at the same time. | If it is as our understanding, then it is probably more like the following: resourceTypeSetB-CSI-RS-r19 SEQUENCE { periodic-r19 Enumerated{supported}, aperiodic-r19 Enumerated{supported}, semiPersistent-r19 Enumerated{supported} }, |  |
| H004 | relaxationTimelineD-r19 relaxationTimelineD1-r19 | 331 | Typo. Some typo on ENUMERATED in SCS48 and SCS960 |  |  |

| H005 | R1 58-1-4 annotation | 331 | Typo. Missing ‘e’ below: -- R1 58-1-4: UE-side beam prediction for BM Case2 for inferenc |  |  |
| --- | --- | --- | --- | --- | --- |
| H006 | R1-58-1-4 resourceTypeSetB-CSI-RS-r19 inferenceReportType-r19 | 331 | For these 2 components in FG58-1-4, does it mean that UE can only support one of the report types for the BM case. Our understanding is that the UE can support e.g. periodic and semi-persistent at the same time. | If it is as our understanding, then it is probably more like the following: resourceTypeSetB-CSI-RS-r19 SEQUENCE { periodic-r19 Enumerated{supported}, aperiodic-r19 Enumerated{supported}, semiPersistent-r19 Enumerated{supported} }, |  |
| H007 | R1-58-1-4 | 306 | We are just wondering whether the following note is needed: NOTE 2: The UE should not report non-zero value for numberOfOccupiedCPUx-r19 if aiml-CSI-Report-r19 is not supported. This is the same as the note in R1 58-0-1 and this note is not in R1 58-1-2 for the same reason. | Either we do it in all and remove the one in 58-0-1 or remove the note in all dependency feature while keeping the one in 58-0-1. No strong view either way. |  |
| H008 | R1-58-3-1 | 331 | ‘}’ is missing in scs30kHz-r19 of relaxationTimelineT-r19 |  |  |
| H009 | R1 58-3-1 | 331 | For: inferenceReportType-r19 ENUMERATED {aperiodic, semiPersistent} Is it not possible that UE can support both reported types? | Similar to H006 or check with RAN1? |  |
| H010 | R1 59-2-1-9 | 306 | The following note was not added to per BC case. |  |  |
| H011 | R1 59-2-3-1, 59-2-3-2 | 306 | The added wording on the larger granularity sounds confusing: For component 1, maybe we just have to say ‘The cyclic prefix of ‘full’ is lower than ‘half’’ For component 2, maybe we just have to say ‘The larger report value is the lower capability’ |  |  |
| H012 | R1 59-2-3-5 | 306 | The added wording on the larger granularity sounds confusing: For component 1, maybe we just have to say ‘The cyclic prefix of ‘full’ is lower than ‘half’’ For component 2, maybe we just have to say ‘The larger report value is the lower capability’ For component 3, maybe ‘The ppmDot2 is of lower capability than ppmDot1’ For component 4, same as component 2. |  |  |
| H013 | R1 60-8 | 306 | Can’t find the field description in 306 related to this FG with field name l1-CLI-RSSI-MeasAndAperiodicReporting-r19 |  |  |
| H014 | R1 63-10 | 306 | Missing the following pre-requisite: 63-7 or 63-7a or 63-7c or 63-7d |  |  |
| H015 | R1 65-1-5 | 331 | It is being annotated as 65-1-4 – should be 65-1-5 |  |  |
| H016 | R1 66-7b | 331 | Not sure where the following is from: maxNumberPUCCH-Trans-r19 INTEGER (1..7) It is not in the FG description |  |  |
| H017 | R1 67-9 | 306 | The following is not implemented in the FRx-diff column: Note: This UE feature group is applicable only for FR1 TN | Set the FRx-diff column to FR1 only |  |
| H018 | R1 67-10 | 306 | The following is not implemented in the FRx-diff column: Note: This UE feature group is applicable only for FR1 TN | Set the FRx-diff column to FR1 only |  |
| H019 | Cover page | 331 | The latest RAN4 feature list is R4-2522332.zip |  |  |