3GPP TSG RAN WG2 Meeting #132                	                	                          R2-250xxxx

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.0.1

Source:	Xiaomi

Title:	Summary of [POST131bis][004][UE caps] UE capability CR (Xiaomi)

Document for:	Discussion and Decision

Introduction

- [POST132][005][UE Caps] Mega CR (Xiaomi)

Intended outcome: Update CR with further feature list update from Ran1/RAN4 and RAN2 endorsed CRs.

Deadline:  Short

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Seau Sian Lim | seau.sian.lim@huawei.com |
| Nokia | Andrew Lappalainen | andrew.lappalainen@nokia.com |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |

Comment

| Comment Index | Capability IE/FG | Specification (306/331) | Comment | Proposed Change | Rapp resolution |
| --- | --- | --- | --- | --- | --- |
| H001 | R1 58-0-1 | 331 | We think this is not correctly implemented based on the following note: Note: Component 2 and 3 candidate values are signalled separately for each pool | It should be implemented e.g. as follow: aiml-CSI-ReportPerBC-r19 SEQUENCE { numCPUPool-r19 SEQUENCE (SIZE (1..2)) OF SEQUENCE {, maxNumCPUxPerCC-r19 INTEGER (1..8), maxNumCPUxAllCC-r19 INTEGER (1..32) } } OPTIONAL, |  |
| H002 | numCPUxPool-r19 | 331/306 | The ‘x’ can be removed since RAN1 has removed it in the field description and it is referring to the number of CPU pool which result in the presence of CPU.2 and CPU.3 pools. For maxNumCPUxPerCC-r19 and maxNumCPUxAllCC-r19 should be without the ‘x’ as it is referring to the CPU and to CPUx. | Removed ‘x ‘from numCPUxPool-r19. Likewise for maxNumCPUxPerCC-r19 maxNumCPUxAllCC-r19 |  |
| H003 | R1-58-1-2 resourceTypeSetB-CSI-RS-r19 inferenceReportType-r19 | 331 | For these 2 components in FG58-1-2, does it mean that UE can only support one of the report type for the BM case. Our understanding is that the UE can support e.g. periodic and semi-persistent at the same time. | If it is as our understanding, then it is probably more like the following: resourceTypeSetB-CSI-RS-r19 SEQUENCE { periodic-r19 Enumerated{supported}, aperiodic-r19 Enumerated{supported}, semiPersistent-r19 Enumerated{supported} }, |  |
| H004 | relaxationTimelineD-r19 relaxationTimelineD1-r19 | 331 | Typo. Some typo on ENUMERATED in SCS48 and SCS960 |  |  |

| H005 | R1 58-1-4 annotation | 331 | Typo. Missing ‘e’ below: -- R1 58-1-4: UE-side beam prediction for BM Case2 for inferenc |  |  |
| --- | --- | --- | --- | --- | --- |
| H006 | R1-58-1-4 resourceTypeSetB-CSI-RS-r19 inferenceReportType-r19 | 331 | For these 2 components in FG58-1-4, does it mean that UE can only support one of the report types for the BM case. Our understanding is that the UE can support e.g. periodic and semi-persistent at the same time. | If it is as our understanding, then it is probably more like the following: resourceTypeSetB-CSI-RS-r19 SEQUENCE { periodic-r19 Enumerated{supported}, aperiodic-r19 Enumerated{supported}, semiPersistent-r19 Enumerated{supported} }, |  |
| H007 | R1-58-1-4 | 306 | We are just wondering whether the following note is needed: NOTE 2: The UE should not report non-zero value for numberOfOccupiedCPUx-r19 if aiml-CSI-Report-r19 is not supported. This is the same as the note in R1 58-0-1 and this note is not in R1 58-1-2 for the same reason. | Either we do it in all and remove the one in 58-0-1 or remove the note in all dependency feature while keeping the one in 58-0-1. No strong view either way. |  |
| H008 | R1-58-3-1 | 331 | ‘}’ is missing in scs30kHz-r19 of relaxationTimelineT-r19 |  |  |
| H009 | R1 58-3-1 | 331 | For: inferenceReportType-r19 ENUMERATED {aperiodic, semiPersistent} Is it not possible that UE can support both reported types? | Similar to H006 or check with RAN1? |  |
| H010 | R1 59-2-1-9 | 306 | The following note was not added to per BC case. |  |  |
| H011 | R1 59-2-3-1, 59-2-3-2 | 306 | The added wording on the larger granularity sounds confusing: For component 1, maybe we just have to say ‘The cyclic prefix of ‘full’ is lower than ‘half’’ For component 2, maybe we just have to say ‘The larger report value is the lower capability’ |  |  |
| H012 | R1 59-2-3-5 | 306 | The added wording on the larger granularity sounds confusing: For component 1, maybe we just have to say ‘The cyclic prefix of ‘full’ is lower than ‘half’’ For component 2, maybe we just have to say ‘The larger report value is the lower capability’ For component 3, maybe ‘The ppmDot2 is of lower capability than ppmDot1’ For component 4, same as component 2. |  |  |
| H013 | R1 60-8 | 306 | Can’t find the field description in 306 related to this FG with field name l1-CLI-RSSI-MeasAndAperiodicReporting-r19 |  |  |
| H014 | R1 63-10 | 306 | Missing the following pre-requisite: 63-7 or 63-7a or 63-7c or 63-7d |  |  |
| H015 | R1 65-1-5 | 331 | It is being annotated as 65-1-4 – should be 65-1-5 |  |  |
| H016 | R1 66-7b | 331 | Not sure where the following is from: maxNumberPUCCH-Trans-r19 INTEGER (1..7) It is not in the FG description |  |  |
| H017 | R1 67-9 | 306 | The following is not implemented in the FRx-diff column: Note: This UE feature group is applicable only for FR1 TN | Set the FRx-diff column to FR1 only |  |
| H018 | R1 67-10 | 306 | The following is not implemented in the FRx-diff column: Note: This UE feature group is applicable only for FR1 TN | Set the FRx-diff column to FR1 only |  |
| H019 | Cover page | 331 | The latest RAN4 feature list is R4-2522332.zip |  |  |
| S001 | R4 62-1/62-2 | 306 | We prefer to have per band and per BC for R4 62-1/62-2 to have a flexibility. However, if majority wants, we would accept per UE. One suggestion is to clarify pre-requisite because cjtc-DdReport-r19 and cjtc-DdReportPerBC-r19 are per band and per BC. The support of R4 62-1/62-2 is | A UE supporting this feature shall also indicate support at least one of cjtc-DdReport-r19 at least for one band and cjtc-DdReportPerBC-r19 at least for one BC. |  |
| N001 | R1 58-0-1 | 306 | In the RAN1 feature list, the component descriptions were edited to change all instances of “CPU,x” to just “CPU”. Suggest we do the same in the 306 descriptions since the meaning of “CPU,x” is not clear anyway. | In capability descriptions for aiml-CSI-Report-r19 and aiml-CSI-ReportPerBC-r19 change all instances of “CPU,x” to just “CPU”. |  |
| N002 | R1 58-1-4 | 331/306 | For component 8 (resourceTypeSetB-CSI-RS) RAN1 feature list only has periodic and semi-persistent, but in the 331/306 CRs it was defined including aperiodic as well. Do we expect that was a mistake by RAN1? | Double check whether aiml-BM-Case2-r19 > resourceTypeSetB-CSI-RS is supposed to include aperiodic as an option. |  |
| N003 | R1 58-3-1 | 331 | In aiml-CSI-PredictionPerBC-r19 and aiml-CSI-PredictionPerBC-r19, there is a missing missing ‘},’ in relaxationTimelineT after n224 in scs30kHz-r19 relaxationTimelineT-r19 SEQUENCE { scs15kHz-r19 ENUMERATED {n14,n28,n56,n112}, scs30kHz-r19 ENUMERATED {n28,n56,n112,n224 scs60kHz-r19 ENUMERATED {n56,n112,n224,n448}, scs120kHz-r19 ENUMERATED {n112,n224,n448}, scs480kHz-r19 ENUMEARTED {n448,n896,n1792}, scs960kHz-r19 ENUMEARTED {n896,n1792} }, | Add the missing ‘},’ to aiml-CSI-PredictionPerBC-r19 and aiml-CSI-PredictionPerBC-r19. relaxationTimelineT-r19 SEQUENCE { scs15kHz-r19 ENUMERATED {n14,n28,n56,n112}, scs30kHz-r19 ENUMERATED {n28,n56,n112,n224}, scs60kHz-r19 ENUMERATED {n56,n112,n224,n448}, scs120kHz-r19 ENUMERATED {n112,n224,n448}, scs480kHz-r19 ENUMEARTED {n448,n896,n1792}, scs960kHz-r19 ENUMEARTED {n896,n1792} }, |  |
| N004 | R1 58-3-1a-1 | 306 | The per-band capability under BandNR is misnamed aiml-CSI-PredictionUnitDurationDD-PerBC-r19 in 38.306 aiml-CSI-PredictionUnitDurationDD-PerBC-r19 Indicates whether the UE supports value of unitDurationDD-r18=1 for the DD unit duration when A-CSI-RS is configured for CMR vectorLengthDD-r18 >1 for UE side inference of CSI prediction. A UE supporting this feature shall also indicate support of aiml-CSI-Prediction-r19.; Band; No; N/A; N/A. | Rename the BandNR capability to aiml-CSI-PredictionUnitDurationDD-r19 |  |
| N005 | R1 58-3-1b | 331 | Parameter is misnamed CodebookParametersCSI-PreidctionDoppler-r19 aiml-CSI-PredictionDoppler-r19 CodebookParametersCSI-PreidctionDoppler-r19 OPTIONAL, aiml-CSI-PredictionDopplerPerBC-r19 CodebookParametersCSI-PreidctionDoppler-r19 OPTIONAL, CodebookParametersCSI-PreidctionDoppler-r19 SEQUENCE { … } | Change all instances to CodebookParametersCSI-PredictionDoppler-r19 |  |
| N006 | R1 58-3-1-7 | 331/306 | For codebookComboParameterMixedTypePrediction-r19, according to RAN1 feature list, the component 1 values correspond to {Type I SP, CSI prediction for UE-sided inference when N4=1 and R=1} {Type I SP, CSI prediction for UE-sided inference when N4>1 and R=1} {eType II R=1, CSI prediction for UE-sided inference when N4=1 and R=1} {eType II R=1, CSI prediction for UE-sided inference when N4>1 and R=1} Since it is eType II suggest to change the parameter value names to use eTypeII.. or eType2.. instead of eType1SP, eType1SP-NR. | Consider renaming following: type1SP-eType1SP-r19 type1SP-eType1SP-N4-r19 type1SP-N4-eType1SP-r19 type1SP-N4-eType1SP-N4-r19 eType1SP-eType1SP-N4-r19 |  |
| N007 | R1 59-2-1-3, R1 59-2-1-4 | 306 | For the perBC capabilities codebookParameterseType2ExtPerBC-r19 and codebookParametersfeType2ExtPerBC-r19 the new note about cap1 and cap2 is missing under the processing time of the 64-port capabilities. | Add following to descriptions of eType2-64PortExt-r19 > processingCapability-r19 and feType2-64PortExt-r19 > processingCapability-r19. ‘cap1’ is a higher capability than ‘cap2’. |  |
| N008 | R1 59-2-1-9 | 306 | For the perBC capability nes-SD-Type1-SP-PerBC-r19 the new note about cap1 and cap2 is missing under the description of timeline-r19. | Add following to description of nes-SD-Type1-SP-PerBC-r19 > timeline-r19. ‘cap1’ is a higher capability than ‘cap2’. |  |
| N009 | R1 59-2-3-5 | 306 | For the per BC capability (cjtc-DdFO-ReportPerBC-r19) the below notes about the lower capabilities were not captured. | Add the same notes to cjtc-DdFO-ReportPerBC-r19 |  |
| N010 | R1 59-4-4e | 306 | The text style is incorrect in the description for twoTA-IntraCellMultiTRP-r19 (using a combination of ‘Normal’ and ‘Arial’, but it should be ‘TAL’). | Change text style to TAL. |  |
| N011 | R1 60-8 | 306 | The capability name is incorrect in 306: 331 uses l1-CLI-RSSI-MeasAndAperiodicReporting-r19 306 uses l1-CLI-RSRI-MeasAndAperiodicReporting-r19 | Change capability name in 306 to l1-CLI-RSSI-MeasAndAperiodicReporting-r19 |  |
| N012 | R1 65-1-3 | 306 | Editorial: The description of pdcch-RepetitionTypeOthers-r19 refers to ‘RRC CONNECTED’ mode. Suggest to change it to ‘RRC_CONNECTED’. | Change ‘RRC CONNECTED’ to ‘RRC_CONNECTED’ in description of pdcch-RepetitionTypeOthers-r19 |  |
| N013 | R1 65-1-5 | 331 | This feature is mislabelled as ‘65-1-4’ in 38.331, but it is ’65-1-5’ in the RAN1 feature list | Change reference to ‘65-1-5’ in 38.331 |  |
| N014 | R1 66-5y | 306 | There are typos in the prerequisite: should be multiCell-PUSCH-DCI-0-3-DiffSCS-DiffCC-r19 (not multicell-PUSCH-DCI-1-3-DiffSCS-DiffCC-r19) | Change multicell-PUSCH-DCI-1-3-DiffSCS-DiffCC-r19 to multiCell-PUSCH-DCI-0-3-DiffSCS-DiffCC-r19 |  |
| N015 | R1 66-6a | 306 | There is a typo in the prerequisite: should be multiCell-PDSCH-DCI-1-3-DiffSCS-DiffCC-r19 (not multiCell-PUSCH-DCI-1-3-DiffSCS-DiffCC-r19) | Change multiCell-PUSCH-DCI-1-3-DiffSCS-DiffCC-r19 to multiCell-PDSCH-DCI-1-3-DiffSCS-DiffCC-r19 |  |
| N016 | R1 66-10 | 306 | Right now 306 says triggeredHARQ-CodebookRetxDCI-1-3-Diff-r19 Indicates whether the UE supports triggered HARQ-ACK codebook re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 and supports the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in simultaneous-2-1-HARQ-ACK-CB-r18) with different SCS and/or different carrier type. However, for component 2 the RAN1 feature list says "2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 66-8)". 66-8 is simultaneous-2-1-HARQ-ACK-CB-Diff-r19 while simultaneous-2-1-HARQ-ACK-CB-r18 is 49-6. | Change description as follows: triggeredHARQ-CodebookRetxDCI-1-3-Diff-r19 Indicates whether the UE supports triggered HARQ-ACK codebook re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 and supports the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in simultaneous-2-1-HARQ-ACK-CB-Diff-r19) with different SCS and/or different carrier type. |  |
| N017 | R1 66-14a | 306 | In the note for unifiedSeparateTCI-MultiMAC-CE-IntraCell-Diff-r19 it refers to the component maxActivatedDL-TCI-PerCC-r18 instead of maxActivatedDL-TCI-PerCC-r19 NOTE: For minBeamApplicationTime-r19, maxActivatedDL-TCI-PerCC-r18 and maxActivatedUL-TCI-PerCC-r19, if the UE also reports unifiedSeparateTCI-multiMAC-CE-r17, same values as for unifiedSeparateTCI-multiMAC-CE-r17 are reported. | Change maxActivatedDL-TCI-PerCC-r18 to maxActivatedDL-TCI-PerCC-r19 |  |
| N018 | R1 66-16 | 331 | This is written as -r18 capability in 331 (while 306 correctly uses -r19) | Change to bwp-SwitchingDCI-0-3-And-1-3-Diff-r19 in 38.331 |  |
| N019 | R1 67-9, R1 67-10 | 306 | According to the RAN1 feature list pdcch-RepetitionType0-TN-r19 and pdcch-RepetitionTypeOthersTN-r19 are only applicable to TN in FR1. This should at least be captured in the FR1/FR2 DIFF column, and probably also some additional clarification is needed that these capabilities apply to TN only. | Add ‘FR1 only’ under ‘FR1-FR2 DIFF’ column. Add some further restriction that: ‘This capability is applicable to TN only.’ |  |
| N020 | R4 62-1, R4 62-2 | 306 | Can simplify the description for both capabilities without the need for a note. | Suggest the following: cjtc-DdReportHighAccuracy-r19 Indicates whether the UE supports high accuracy reporting for CJTC delay offset reporting as defined in TS 38.133 [5] clause 10. A UE supporting this feature shall also indicate support at least one of cjtc-DdReport-r19 and cjtc-DdReportPerBC-r19. cjtc-FO-ReportHighAccuracy-r19 Indicates whether the UE supports high accuracy reporting for CJTC frequency offset reporting as defined in TS 38.133 [5] clause 10. A UE supporting this feature shall also indicate support at least one of cjtc-FO-Report-r19 and cjtc-FO-ReportPerBC-r19. |  |
| O001 | R1 67-4 | 331, 306 | Since R1 clarify the capability is to indicate simul-switching is feasible for a specific (target-) band pair, so that there seems no need for 3 loops as follows OPTIONAL, bandList-v1900 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1900 OPTIONAL, -- R1 67-4: Support of simultaneous SRS carrier switching srs-SwitchingSimultaneousList-r19 SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF SRS-SwitchingAffectedBandsNR-r17 OPTIONAL SRS-SwitchingAffectedBandsNR-r17 ::= BIT STRING (SIZE (1..maxSimultaneousBands)) | Suggest to remove one loop, bandList-v1900 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1900 OPTIONAL, -- R1 67-4: Support of simultaneous SRS carrier switching srs-SwitchingSimultaneousList-r19 SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF SRS-SwitchingAffectedBandsNR-r17 OPTIONAL SRS-SwitchingAffectedBandsNR-r17 ::= BIT STRING (SIZE (1..maxSimultaneousBands)) So that the first loop is to iterate for the first target band, and the second loop is to iterate for the second target band. And it seems reasonable to have limitation similar to srs-SwitchingTimesListNR - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on And the name of IE could be more comprehensive, From SRS-SwitchingAffectedBandsNR-r17 to SRS-SwitchingSimultaneousBandsNR-r17 |  |