3GPP TSG RAN WG2 Meeting #133                	                	                          R2-260xxxx

Gothenburg, Sweden, Feb. 9th-13th 2026

Agenda Item:	x.x.x

Source:	Xiaomi

Title:	Report of [POST132][007][UE caps] BC capability CRs (Xiaomi)

Document for:	Discussion and Decision

# Introduction

This report provides a summary for the following post-meeting email discussion:

- [POST132][007][UE caps] BC capability CRs (Xiaomi)

Intended outcome: Review CRs and agree on whether we start from R16

Deadline:  Long

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Xiaomi | Ziyi Li | liziyi5@xiaomi.com |

# Discussion

During RAN2 #132 meeting, the following agreement on ‘per band and per BC’ were agreed:

Agreements from RAN2 perspective

- No matter CA is configured or not, if the capability/component is not counted across CCs, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA).

- For the capability/component is counted across CCs, wait for RAN1 for the conclusion.

- When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability, the capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs.

- When the UE reports the per band capability but does not include the per BC capability for a certain BC, the UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs.

- If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’:

a. A UE supporting feature A per band shall also indicate support of feature B at the given band;

b. A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

6	For per-band per-BC capabilities for MIMO codebook capabilities:

-	If a UE supports such capabilities in a set of bands separately (per-band) then the UE must always support those capabilities in any CA combination composed of the respective bands (per-BC). No further enhancement is needed for this case.

8	Inform RAN1 about our agreements

7	Update at least Rel-19 CRs.  FFS whether we need Rel-16, 17, 18 CRs

Additionally, RAN1 further agreed the following agreements in RAN1 #123 meeting in R1-2509496 [1]:

Agreement: - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is not configured, then - gNB only considers per Band capability regardless of reported per BC capability (i.e., per BC capability can be ignored in this case) - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is configured, then - if all CCs over the CA are within the same band (intra-band CA), the CA is restricted by the per band capability/component corresponding to the CA - if the CCs of the CA are associated with a band combination (inter-band CA), the CA is restricted by the per BC capability/component corresponding to the CA

Based on the above, rapporteur summarizes the following general principle of ‘per band and per BC’ and its prerequisite:

When both per band and per BC capability are indicated: - if capability/component is not counted across CCs: - No matter CA is configured or not, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA); - if capability/component is counted across CCs: - if CA is not configured by network, only per band capability regardless of reported per BC capability; - if CA is configured by network, - if all CCs over the CA are within the same band (intra-band CA), the CA is restricted by the per band capability/component corresponding to the CA - if the CCs of the CA are associated with a band combination (inter-band CA), the CA is restricted by the per BC capability/component corresponding to the CA When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability: The capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs. When the UE reports the per band capability but does not include the per BC capability for a certain BC: The UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs. If a UE supports the per band capability in a set of bands separately, the UE should also support the corresponding per BC capabilities in any CA combination composed of the respective bands. If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’: a. A UE supporting feature A per band shall also indicate support of feature B at the given band; b. A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

During previous discussion, the discussion on ‘per band and per band combination’ was initiated from MIMO codebook capabilities, who are defined in both per band and per BC with the same feature components. However, there were some concerns on whether applying the above interpretation for ‘per band and per band combination’ capabilities to capabilities such as power class, etc. However, rapporteur observes that power class comprises different components with some components defined as per band (highlighted in green) while others defined as per BC (highlighted in yellow), which is not using ‘per band and per BC’ granularity (as for MIMO codebook capabilities) in RAN1/4 feature list [2].

| 2-9 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | Yes | gNB cannot know the UE power class | Type 1 and Type 3 (see remarks column) | No Need | No Need |  | Capability signalling - FR1 UE power class ( Type 1 (per band)) - FR2 UE power class ( Type 1 (per band)) - FR1 UE power class for EN-DC (Type 3 (per band combination)) - FR1 UE power class for NR CA (Type 3 (per band combination)) Note that default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. Note that the component 2) is also used as power class for intra-band contiguous NR-CA in FR2 | RAN4 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class. | Mandatory to report non-default power class if UE supports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Therefore, rapporteur suggests to consider define ‘per band and per band combination’ as a new granularity with the following definition:

Per band and per BC capability means that a UE capability parameter defined in both per band and per BC with the same feature components. The capability is signalled in per band and also signalled in per band combination.

#### Q1. Does company agree to define ‘per band and per band combination’ as a new granularity and its definition above?

| Company | Yes/No | Comment to definition |
| --- | --- | --- |

Following RAN2 #132 meeting agreement

7	Update at least Rel-19 CRs.  FFS whether we need Rel-16, 17, 18 CRs

Rapporteur provided Rel-19 draft CR, including the above ‘per band and per band combination’ definition and general principle,

#### Q2. Please companies provide comment to the draft CR in below table (to avoid changes over changes and so many bubble comment in draft CR):

| Company | Yes/No | Whether to capture ‘per band and per band combination’ definition in Section 3.1? (Yes/No) | Suggestion on changes in Section 4.2.1 |
| --- | --- | --- | --- |

In the end, considering capabilities with ‘per band and per band combination’ are widely captured in TS 38.306 since Rel-16 (almost all MIMO codebook capabilities), to keep consistent and avoid ambiguity, rapporteur proposes the above changes starting from Rel-16.

#### Q3. Does company agree to start to make the changes on ‘per band and per band combination’ (as in Rel-19, including changes in Section 3.1 and Section 4.2.1) from Rel-16 TS 38.306 (if agreed, rapporteur will provide the corresponding CR with same changes as in Rel-19 from the agreed release):

| Company | Yes/No | If not, which release to start the change? (Rel-17, Rel-18) |
| --- | --- | --- |

# Conclusion

<to be updated>

# Reference

[1] R1-2509496	LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#123 Wednesday

[2] R4-1907593 "RAN4 NR UE features", contribution to TSG-RAN WG4 meeting #XX.