3GPP TSG RAN WG2 Meeting #133                	                	                          R2-260xxxx

Gothenburg, Sweden, Feb. 9th-13th 2026

Agenda Item:	x.x.x

Source:	Xiaomi

Title:	Report of [POST132][008][6G] UE capability pain points (Xiaomi)

Document for:	Discussion and Decision

Introduction

This report provides a summary for the following post-meeting email discussion:

- [POST132][008][6G] UE capability pain points (Xiaomi)

Intended outcome: Identify and capture the pain point issues (including root causes).   Identify what RAN2 can focus on and which ones impact other WGs.   Find a way to provide examples or demonstrate the identified pain issues to other WGs.

Identify what contributes the most to the overhead/complexity

Deadline:  Long

This post meeting email discussion is targeting to have two phases:

Phase 1: Identify pain points and root causes

Comments are welcome to the listed pain points and root causes. Meanwhile, companies are also welcomed to provide examples (e.g., field data, analyzed log, etc) to demonstrate the complexity/overhead. Companies may also add new pain points and root causes into new (sub-)sections if needed. Earlier and interactive inputs are welcomed.

Phase1 Deadline: 19th Dec, 2025

Phase 2: Identify RAN2 focused area and impact to other WGs

Based on inputs during phase 1, phase 2 will categorize pain points and root causes from phase 1 inputs, then identify which one(s) contributes to the overhead/complexity most. RAN2 focused areas and other WGs impacts based on the identified pain points/root causes will also be discussed.

Phase 2 Deadline: 23rd Jan, 2026

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Xiaomi | Ziyi Li | liziyi5@xiaomi.com |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |
| vivo | Xiang Pan | panxiang@vivo.com |
| Ericsson | Lian Araujo | Lian.araujo@ericsson.com |
| CATT | Tangxun | tangxun@catt.cn |
| MTK | Mutai Lin | morton.lin@mediatek.com |

Phase 1 Discussion

This section summarized the pain points and root causes (if any) from contributions submitted to RAN2 #132 meeting.

Problem 1: Capability Signalling Size

Almost all companies acknowledge that the significant signalling size is the key problem/pain point of 5G UE capability. From contributions, several common reasons were mentioned:

- Root cause 1: Significant number of CA and related UE capability parameters; [R2-2508445 (Apple), R2-2508044 (Vivo), R2-2508076 (Xiaomi), R2-2508113 (Oppo), R2-2508616 (Huawei), R2-2508732 (Ericsson), R2-2508868 (Qualcomm), R2-2508876 (Samsung), R2-2508509 (ZTE), R2-2508097 (CATT), R2-2508839 (CMCC), R2-2508209(Sharp)]

- Root cause 2: Multiple band combination list e.g., for regular CA and for UL Tx Switching [R2-2508732 (Ericsson), R2-2508445 (Apple), R2-2508616 (Huawei), R2-2508076 (Xiaomi), R2-2508509 (ZTE)]

- Root cause 3: Same value for capabilities for some/all bands/BCs (e.g., capabilities across all CCs/band/BC); [R2-2508445 (Apple), R2-2508076 (Xiaomi), R2-2508097 (CATT), R2-2508113 (Oppo), R2-2508616 (Huawei), R2-2508732 (Ericsson), R2-2508509 (ZTE)]

- Root cause 4: Overclassified finer granularity [R2-2508616 (Huawei), R2-2508445 (Apple), R2-2508076 (Xiaomi), R2-2508322 (Nokia)]

- Root cause 5: Inefficient BC entry structure [R2-2508732 (Ericsson), R2-2508868 (QC), R2-2508876 (Samsung)]

- Root cause 6: Not well-used FeatureSet and FeatureSetCombination [R2-2508732 (Ericsson), R2-2508445 (Apple), R2-2508044 (vivo), R2-2508868 (QC), R2-2508509 (ZTE), R2-2508668 (NEC)]

- Root cause 7: Pairing between DL and UL cannot be indicated flexibly [R2-2508616 (Huawei), R2-2508509 (ZTE)]

- Root cause 8: Complex/none-forward compatible RF requirements [R2-2508076 (Xiaomi), R2-2508322 (Nokia)]

- Root cause 9: Multiple bandwidth classes & fallback groups lead to more band combinations. Consider e.g. FR1 bandwidth class “B” and “C”. Both means UE can support 2 contiguous CCs. If UE wants to report support for both B and C in a band, it will increase number of BC involving that band significantly (possibly with a factor of 2). (Added by Ericsson during the email discussion)

- Root cause 10: Eliminating multiple subcarrier spacings supported per band will lead to simplifications in capability signaling, (Added by Ericsson during the email discussion)

If this problem is agreeable, please indicate which root cause(s) listed above that can be agreeable, and add new root cause(s) if it is not mentioned above. Please also list the corresponding example(s) for both listed and new root cause(s), rapporteur will consolidate all examples in 2nd phase and facilitate the discussion of identifying which root cause contributes the most. Separate item/row is preferred if multiple root causes are identified:

| Company | Please indicate which root cause(s) above is agreeable and input new root cause(s) if any | Example (e.g., signalling, field data, analysed log, etc) |
| --- | --- | --- |
| OPPO | Root cause 2 |  |
| OPPO | To enhance signaling efficiency, two primary approaches can be considered. First, we can optimize the ASN.1 syntax structure by identifying and eliminating redundant elements (as already been done here in the other root cause). Second, we may apply compression algorithms to the generated capability messages to further reduce their size. Both methods aim to improve overall system performance through more efficient data transmission. | As demonstrated in our previous study (R2-2508113), conventional compression algorithms - including Deflate, LZMA, and PPMD - typically achieve an average compression ratio of approximately 50%, which proves the redundancy in the current capability signaling in the level of ~50%. |
| vivo | Cause 1 | The signaling of UE NR capability is illustrated in the above figure. It can be observed that excessive number of band combinations leads to an almost linear growth in the size of rf-Parameters. We further analyzed the log of commercial UE and observed that even though the network used a filter to allow the UE to only report BCs that include bands 28, 41, and 79, the UE still reported 21 BCs formed by bands 28, 41, and 79. Thus, reducing the number of BC could significantly reduce the signaling overhead of capability information. |
| vivo | Cause 2 | The current relationship between signaling of band combination for CA and ULTxSwitch is shown in the above figure. It can be observed that both BandCombination-UplinkTxSwitch and SupportedBandCombinationList contain the BandCombination IE. Furthermore, in our logs, we observed that certain parameter values under the same band combination, e.g., ca-BandwidthClassUL, are the same between BandCombination-UplinkTxSwitch and SupportedBandCombinationList. |
| vivo | Cause 3/ Cause 4 | Based on the analysed log, we observed that mimo-ParametersPerBand includes a large number of capability parameters, while the values of most of these parameters the UE signalled are the same across all the supported bands, e.g., maxNumberConfiguredTCIstatesPerCC; maxNumberActiveTCI-PerBWP; pusch-TransCoherence; periodicBeamReport; aperiodicBeamReport; … We think Cause 4 is similar to Cause 3, i.e., the same value for capabilities for these parameters is due to unnecessary overclassified finer granularity. |
| vivo | Cause 6 | Based on the analysed log, we observed that the UE reported 21 BCs formed by band 28, 41 and band 79, while no single featureSetCombination was used twice across these combinations. |
| Ericsson | Root cause 1 | Agree. We would like to emphasize that while the capability size (in bytes) is an issue in itself, but the number of BCs has a direct connection to the processing cost that gNB need to spend on validation of capabilities when figuring out how to configure UE. Reduction of number of BCs signalled and also the number of capabilities signalled per BC needs to be a priority. Another observation is that even if UEs do not support capabilities, these capabilities may still consume bits in the ASN1 signalling. The solution is to define fewer capability IEs. The number of reported BCs depends heavily on the number of bands the operator has deployed. There are operators where we today commonly see more than 200 BCs being reported by UE, with some extreme examples going above 500 BCs. |
| Ericsson | Root cause 2 | Agree. Having multiple lists also introduces some ambiguity on fallback rules. E.g., does gNB need to try to validate a single-CC UL MIMO configuration towards the TxSw BCs, or could UE signal support for UL MIMO only in the TxSw BCs? |
| Ericsson | Root cause 3 | Agree |
| Ericsson | Root cause 4 | Agree. We understand there exist needs of conveying RF-related limitations on a quite detailed level. Per-BC and per-FS capabilities should primarily be used to indicate such RF-related limitations, not to signal envelope limitations that are more or less independent of the exact BC. If moving an envelope limitation from per-BC level to per-UE level means that in some cases a somewhat pessimistic capability is used, this might be ok. |
| Ericsson | Root cause 5 | Agree. There are several examples of capabilities defined per-BC in NR, where the intention is to convey a UE envelope limitation, e.g. total number of CSI-RS ports, max aggregated BW etc. These should be defined on a per-UE level instead. Other examples include indication of same value for all BCs of same type, e.g. UL CA FDD+TDD, while for other BC types the capability is irrelevant and hence not included. Envelope limitation examples: maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC,, totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC, simultaneousCSI-ReportsAllCC, codebookParametersPerBC-r16 , codebookParametersAdditionPerBC-r16 , codebookComboParametersAdditionPerBC-r16 , supportedAggBW-FR1-r17 Only relevant for certain BC types and irrelevant otherwise: diffNumerologyWithinPUCCH-GroupSmallerSCS, diffNumerologyWithinPUCCH-GroupLargerSCS |
| Ericsson | Root cause 6 | Agree. Reuse of FSC for multiple BC is limited. The amount of reuse of FSC across BCs varies depending on deployments. Calculating the number of BC divided by number of FSC and averaging this value across UEs, this ratio varies between 1.1 and 2.1 when studying data from seven networks. |
| Ericsson | Root cause 7 | Agree. |
| Ericsson | Root cause 8 | Agree. Complicated for gNB to validate several IEs on the same area, for something simple like figuring out what BW that can be configured. |
| Ericsson | Root cause 9 | RAN4 needs to consider this in their work. |
| Ericsson | Root cause 10 | The 6G SI takes steps in the right direction here. |
| CATT | Root cause 1-9 | Agree. |
| CATT | New Root cause 11 | - Root cause 11: Multiple/Inconsistent capability parameters for features across different branches. For instance, when determining the channel bandwidth supported by a specific band in a band combination, the network must cross-reference three distinct capability parameters from different branches: channelBWs in BandNR, supportedBandwidthCombinationSet in BC and supportedBandwidth in FeatureSet. (Added by CATT during the email discussion) 6G UE capability framework should avoid to acquire a single capability from multiple branches as far as possible, so as to reduce the complexity of processing and retrieval. |
| MTK | Root cause 1, 2, 3, 4, 5, 6, 7, 8 | Root cause 1, 4, 5, 6, 7, 8: The % of the whole container: BC list and extensions 40 ~ 50%; FSC/FS and extensions 35 ~ 45% according to field trial log. Root cause 2: Though we think it is more related to the complexity pain point, the additional ULTxSwitching BC list could contribute more than 5% when the supported BC number goes up to 15 according to field trial log. Root cause 3: Take FG 2-36/2-40/… family as example, though the size reduction gain (contributed to the whole container) is less than 1.5% by switching from R15 to R16 signaling, the size difference in bits can still allow few more BCs to be reported. Furthermore, some other root causes we observed here are: 3.1. Extension IE overhead (defeating the benefit of using R16 IEs) 3.2. Multiple codebook placeholders but some never deployed |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |
| OPPO | Regarding Root Cause 1: While we acknowledge that the BC list (and/or FS/FSC list) contributes significantly to signaling overhead, we don't consider it the fundamental issue. The true root cause lies in the unnecessary reporting of certain information, but it is not the case when UEs do support numerous BCs that are requested for reporting. We suggest R2 monitor R4's progress on the 'band-group' concept as a potential solution. Regarding Root Cause 3: The adoption of per-band capability was primarily driven by the ambiguity created by per-UE capability with XDD/FRX differences. As documented in LS R2-2006367, R2 consequently decided to implement the per-band capability approach. Regarding Root Cause 4: This issue represents a fundamental trade-off between implementation flexibility and signalling overhead. Given this balance, R2 cannot reasonably provide definitive guidance favouring large granularity capabilities. Each case should be evaluated individually based on its specific requirements. Regarding Root Cause 5: As noted in document 8732, a UE supporting multiple bands with various carrier configurations can generate numerous band combination entries. The signaling overhead appears to stem from RF capability requirements for different UL carrier positions and constellation scenarios. However, we have yet to identify specific redundant information that could be reduced, particularly since baseband capability is already compressed using FS/FSC methods. Regarding Root Cause 6: We have observed instances of low FS/FSC reusing ratios. Similar to Root Cause 1, without signaling duplication, no significant gains can be expected from the FS/FSC method. Nevertheless, this doesn't diminish the method's potential to reduce overhead when genuine redundancy exists. This again presents a trade-off between implementation flexibility and signalling efficiency. Regarding Root Cause 7: This issue is closely tied to UL and DL decoupling progress. We believe it's unrealistic to expect a signaling framework that allows completely arbitrary UL/DL band pairing, as each combination presents unique RF challenges. Therefore, achieving truly flexible DL-UL pairing indications appears impractical. Regarding Root Cause 8: While power class and CBW examples are provided - both are known as challenging areas for R4 - we don't yet see how these factors contribute substantially to signaling overhead. At this preliminary stage, it's unclear what feasible actions R4 could take regarding these specific aspects. |
| vivo | The current definition and description of Problem 1 are too high-level. The listed root causes focus solely on RF capabilities and associated Feature Sets/Feature Set Combinations, but these are not the only reasons leading to significant capability signalling size. For instance, Problem 2 and Problem 4 can also contribute to increased capability signalling size. To avoid overlapping, it is recommended to refine Problem 1 as: Capability Signalling Size related to RF parameters and featureSet/featureSetCombination. |

Problem 2: Inefficient network filtering

Several companies [R2-2508044 (Vivo), R2-2508076 (Xiaomi), R2-2508097 (CATT), R2-2508145 (MTK), R2-2508610 (LG)] think that current network filtering mechanism is not efficient.

- Root Cause 1: Improper/coarse filters (only support RAT/frequency band filtering) and UE may still report capabilities that are not supported by network [R2-2508044 (Vivo), R2-2508097 (CATT), R2-2508610 (LG)]

- Root Cause 2: Network requests a large number of bands in the filter defeating the purpose of the filter [R2-2508145 (MTK)]

- Root Cause 3: UE may still further filter reported capabilities due to limited RRC message size [R2-2508076 (Xiaomi), R2-2508145 (MTK)]

Some companies [R2-2508732 (Ericsson)] also mention some rules/criteria needs to consider when introducing new filters.

If this problem is agreeable, please indicate which root cause(s) listed above that can be agreeable, and add new root cause(s) if it is not mentioned above. Please also list the corresponding example(s) for both listed and new root cause(s), rapporteur will consolidate all examples in 2nd phase and facilitate the discussion of identifying which root cause contributes the most. Separate item/row is preferred if multiple root causes are identified:

| Company | Please indicate which root cause(s) above is agreeable and input new root cause(s) if any | Example |
| --- | --- | --- |
| vivo | Cause 1 | The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter, while UE could report more bands in supportedBandListNR that are not included in the appliedFreqBandListFilter. However, the NW may ignore some of the bands in the supportedBandListNR when configuring the UE. |
| Ericsson | Root cause 3 | Agree. That the UE filters capabilities because they cannot fit in the message happens if the capability message size doesn’t fit all band combinations for all bands that the network requested (i.e. included in the band filter). This is a problem in NR since the capability sizes are so big. This was the cause for RAN2 added segmentation for the UE capabilities, but segmentation of course didn’t address the actual problem/root cause which is that UE capabilities in NR are too big and too complex (discussed under Problem 1 above). RAN2 should in 6G focus on reducing the UE capability sizes and their complexity. |
| CATT | Root cause 1 | There are two approaches to feature-based filtering: One is based on the specific features included in the FeatureSet, some of which are structural features. Filtering out the entire structure can effectively reduce the message size. The other approach relies on characteristic features. For example, if the 3CC CA function is disabled, there is no need to report the BandCombinations capability for 3CC. |
| MTK | Root cause 1, 2, 3 | Root cause 1, 2, 3: Some band filter setting from the live networks: {1,257,258,261,28,3,7,77,78,79,80,81,82,83,84,86}; {1,257,258,28,3,38,40,41,66,7,77,78,79,8,96}; {1,28,40,41,5,77,78,79,8,80,81,82,83,84,86}; {18,258,26,28,41,48,5,7,77,78,90}; {2,25,259,26,260,30,40,5,65,66,77} For the Root cause 1, it has some relation to the IODT (commercialization) problem because the UE is under obligation to report all the IODTed features regardless of whether it is supported in the network side. All in all, we think the first priority for study is still the most effective way to control the capability size, which is finer capability request filter. To OPPO and E///: - It’s true that an overly narrow filter specific to a single RAN node would create problems, but that would be naive to implement a finer-grained filter, and normally there will be a region of the network with one (or a limited number of) network vendor(s), within which the nodes will have nearly or entirely homogeneous support. Also, given a reasonable implementation of the signalling, even an incremental series of smaller messages looks better than one gigantic message in many respects. We understood the points E/// raised below and agreed the day-1 simplification (avoiding such as IE extension overhead, enjoying further multiplicity and/or codepoint reduction) is very very very crucial. That’s why we strongly support RAN2 to communicate with other WGs and deliver the informative message. However, quite a few companies didn’t think RAN2 could change or even influence other WGs. What should we do with a mindset of “Hope for the best, prepare for the worst”? |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |
| OPPO | While precise filtering can effectively reduce signaling overhead, it ultimately requires the UE to submit additional reports when transitioning to network nodes with differing capabilities. In essence, filtering simply divides a single large UE capability report into multiple smaller, incremental reports. Therefore, filtering must strike a balance - its primary purpose should be to eliminate capabilities that would never be utilized as the UE moves within the PLMN (or at least across a significant large geographic area). It should not attempt to achieve overly precise filtering tailored to the capabilities of a single RAN node. |
| Ericsson | Root cause 1: We might have misunderstood the described problem. However, the bulk of the UE capability signalling comes from the band combinations. For each band combination the UE indicates which features the UE supports in the band combination. If the UE does not support a certain feature in a certain band combination the UE will omit those fields. If there is a certain feature which the network is not interested in the UE can omit those fields, but the optionality-bits are still there. If there is a binary-feature (i.e. supported or not) there is no gain of "filtering away" that feature since the optionality-bit will still be signalled and set to 0. If there are features with more complex capability indications (i.e. several bits) to filter away such a feature would allow the UE to omit those (several) bits for that band combination. However, this would only bring gains if UE supports features which the network does not support or use. However, most features are supported and used both by UEs and networks. We do not expect a huge gain from this but would be happy to see quantitative examples where in NR this would bring meaningful gains. We think that the main issue RAN2 should focus on is avoiding the huge amount of unused flexibility and reduce band combination signalling. |
| Ericsson | Root cause 2: We think that all gNBs in a PLMN enquire UE capabilities for all bands and features that are used by all the gNBs (even if the enquiring gNB doesn’t support all bands). E.g. a gNB supporting only FR1 should anyway enquire capabilities for the FR2 bands that are used in other gNBs of that PLMN. This ensures that the UE can be handed over to the FR2 gNB without having to re-enquire for the UE capabilities. So we do not recognize Root cause 2 as an issue in real deployments. |
| CATT | Root cause 2. Same view as Ericsson, usually one gNB may include all potential bands used for one PLMN, but not just bands used in current gNB. So the total number of bands in the filters are up to operators’ band resources, but not an issue to resolve in standardization aspect. |
| CATT | Root cause 3. If UE capability segmentation is supported, we don’t think this is still an issue for study. |

Problem 3: Impractical RACS

Several companies [R2-2508044 (vivo), R2-2508076 (Xiaomi), R2-2508616 (Huawei), R2-2508868 (QC), R2-2508876 (Samsung), R2-2509032 (DT), R2-2508422 (DCM), R2-2508610 (LG), R2-2508540 (Sony), R2-2508668 (NEC)] thinks 5GNR RACS design is not practical.

- Root Cause 1: Per UE granularity (e.g., difficult to be reused by other UE(s)) [R2-2508044 (vivo), R2-2508076 (Xiaomi), R2-2508540 (Sony)]

- Root Cause 2: Single UE with multiple RACS IDs without knowledge of current UE situation [R2-2508616 (HW), R2-2508868 (QC)]

- Root Cause 3: Optional feature introduced in later release (R16) [R2-2508422 (DCM), R2-2508668(NEC)]

- Root Cause 4: Maintenance burden [R2-2508610 (LG), R2-2508616 (HW)]

- Root Cause 5: Delay and/or duplicated retransmit of full/wide filtered capability during UE mobility resulting from the network vendor change [R2-2508610 (LG), R2-2508868 (QC), R2-2509032 (DT)]

If this problem is agreeable, please indicate which root cause(s) listed above that can be agreeable, and add new root cause(s) if it is not mentioned above. Please also list the corresponding example(s) for both listed and new root cause(s), rapporteur will consolidate all examples in 2nd phase and facilitate the discussion of identifying which root cause contributes the most. Separate item/row is preferred if multiple root causes are identified:

| Company | Please indicate which root cause(s) above is agreeable and input new root cause(s) if any | Example |
| --- | --- | --- |
| vivo | Cause 1&Cause 4 | We observed that UEs with different versions of one single series, e.g., with/without -pro, have similar but not the same capabilities. As a result, two set of UE capabilities and RACS IDs should be allocated and maintained in NW. We believe the RACS mechanism can effectively reduce the signaling overhead for capability reporting over the air interface. However, the excessive flexibility in NR UE capabilities makes it difficult for a single set of UE radio capabilities to be reused across multiple UEs. As a result, massive capabilities associated with different UEs will increase the maintenance burden from the network’s perspective. In addition, although RACS was mainly specified by other WGs, e.g., SA2, the feature was triggered by RAN2’s SI conclusion in R16. We think it’s still make sense for RAN2 to trigger further discussion on 6G RACS in SA2 if the problem is valid. |
| MTK | Root cause 1, 2, 3, 4, 5 | We cannot provide example because the Capability ID feature was never deployed but sympathies these pain points. We suggest focusing on the root causes 3, 4, 5 first because they’re directly related to the commercialization matters. Then to the root cause 1 is on whether the massive IoT devices could enjoy the benefit. |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |
| OPPO | We concur with the perspective shared by other companies via email that RACS does not constitute an R2 issue. Furthermore, even if RACS were to be ultimately adopted for 6G, its potential impact on the R2 discussions regarding signaling overhead reduction remains unclear to us. This is because, in any case, we must account for scenarios where the network lacks prior knowledge of the RACS identifier. |
| Ericsson | All root causes: We think that the RACS feature can reduce the number of capability enquiries from the UEs via Uu interface. However, capability enquiries via Uu are anyway rare in 5G. It occurs primarily upon initial NAS attach. Thereafter, the AMF stores the UE capabilities and provides them to the RAN upon subsequent RRC connection setup. This may be the reason for the lack of market traction for RACS (we don’t see many implementations). Improving RACS is not on the top of our priority list. A UE chipset ID would be more beneficial in our mind as described in R2-2508732. |
| CATT | No strong view. RACS is still a kind of solution to resolve problem 1 capability signalling size issue, so it could be discussed in later phase compared with other candidate solutions. And anyway UE capability ID is generally a SA2-lead discussion, so maybe we could focus on other RAN2-lead solutions for study, and deprioritize RACS until we have some concrete study tasks from SA2. |

Problem 4: Unnecessary capability signalling

Several companies [R2-2508876 (Samsung), R2-2508076 (Xiaomi), R2-2509032 (DT), R2-2508422 (DCM), R2-2508903 (AT&T, etc), R2-2508209(Sharp)] mentioned that 5GNR defined a lot of unnecessary capability signalling for massive optional features and its components.

- Root Cause 1: massive optional features [R2-2508876 (Samsung), R2-2508076 (Xiaomi), R2-2509032 (DT), R2-2508422 (DCM), R2-2508903 (AT&T, etc), R2-2508209(Sharp)]

If this problem is agreeable, please indicate which root cause(s) listed above that can be agreeable, and add new root cause(s) if it is not mentioned above. Please also list the corresponding example(s) for both listed and new root cause(s), rapporteur will consolidate all examples in 2nd phase and facilitate the discussion of identifying which root cause contributes the most. Separate item/row is preferred if multiple root causes are identified:

| Company | Please indicate which root cause(s) above is agreeable and input new root cause(s) if any | Example |
| --- | --- | --- |
| vivo | Cause 1 | The feature groups and components for a specific feature should balance signaling overhead and flexibility. |
| Ericsson | Root cause 1 | We don’t think that the number of optional features per se is a problem. We think we should make features optional in general. In particular: any feature added after the first release of a G shall be optional. Also, we should not bundle many sub-features such that they share capability bits, e.g. if sub-feature A and sub-feature B have merit on being used on their own, UEs should be allowed to support them independently. However, we do agree that the granularity with which some capabilities are signalled in 5G is beyond reason (too many capabilities are per band per band combination), and this causes the capabilities to be huge and complex. Often the reason why we get the granularity wrong is that the capabilities are defined in the end of a work item which is (naturally) long before anyone has implemented the features. UE vendors fear that if the capability signalling for a feature would become too coarse, they won’t be able to implement and test the feature with the required granularity and therefore they tend to argue for a fine granularity (e.g. per band per band combination), even if we have seen that the flexibility that the specification offers are not used in reality. |
| CATT | Root cause 1 |  |
| MTK | Root cause 1 | This situation practically contributes to the further root causes we shared in the Problem 1 (so example is there). This problem is related to the excessive features so that parameters leading to the considerable extension IE overhead and multiplicity. |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |
| vivo | The current definition of Problem 4 is not aligned with the description, as Optional feature groups/components are not unnecessary capability signaling. To avoid misunderstanding, it is recommended to refine Problem 4 as: unnecessary capability flexibility. |

Problem 5: Commercialization challenges

During RAN2 #131bis/#132 meeting and RAN #110 meeting, several commercialization challenges are brought up from 5G UE capability framework and IODT test point of view. Based on the contributions to both RAN2 and RAN meetings, following root causes are summarized:

- Root Cause 1: No differentiation between non-trivial feature(s) and other feature(s). This further leads to under-reporting/finer granularity UE capability reporting used in 5G for the purpose of addressing individual deployment and infra vendors, but increases signalling overhead (as discussed in Problem 1); [RP-253230 (QC)]

- Root Cause 2: Mandatory feature is only mandating user equipment to implement, but not for the network, and further leads to losing tracking of ecosystem supported features in 3GPP. This makes difficult to guarantee the degree of forward compatibility; [RP-253066 (AT&T, etc), R2-2508903 (AT&T, etc)]

- Root Cause 3: Late deployment to wait for ‘slowest’ network vendor before activating a capability in operator’s network, due to no differentiation treatment of different vendors; [R2-2508868(QC), R2-2506988]

- Root Cause 4: Interoperability issue even after IoDT test is done, due to incompatibility to specification, insufficient tests covering the problematic case(s), lack of IoDT between vendors, etc. [R2-2507607 (ZTE), RP-253048(Oppo)]

If this problem is agreeable, please indicate which root cause(s) listed above that can be agreeable, and add new root cause(s) if it is not mentioned above. Please also list the corresponding example(s) for both listed and new root cause(s), rapporteur will consolidate all examples in 2nd phase and facilitate the discussion of identifying which root cause contributes the most. Separate item/row is preferred if multiple root causes are identified:

| Company | Please indicate which root cause(s) above is agreeable and input new root cause(s) if any | Example |
| --- | --- | --- |
| OPPO | Root cause 2 |  |
| vivo | - Yes for Cause 2 - Cause 1 is ambiguous as the under-reporting seems duplicated with cause 3. - No for Cause 3, the operators are expected to complete comprehensive IODT for all network vendors. - No for Cause 4, avoiding interoperability issues is one of the key motivations of refining the spec and comprehensive IODT tests. In addition, we understand that Cause 3/4 should be discussed in RANP. | Cause 2: Taking RRC_INACTIVE as an example, support for the RRC_INACTIVE state is mandatory for NR UEs, while this feature has not been widely deployed on the network. As a result, the extra implementation complexity for these features is in vain from UE’s perspective. From R2’s perspective, we only focus on the spec impact, e.g., capturing some mandatory features of both UE/NW as stage 2 description, which can a reference for RAN5 to define the test use cases. |
| Ericsson | Root cause 2 | Agree. If 3GPP declares functionality “mandatory without capability bit” even for non-essential features and/or features which are complex. UEs must have an IODT opportunity before releasing their first UE. There could be cases where the UE try to support this feature but finds no network for IODT. Due to market pressure the chipset vendor will then release the device anyway hoping that the feature will work once enabled by a NW. But if that does not succeed later, none of those later NWs knows which UE correctly supports this old mandatory feature. |
| Ericsson | Root cause 3 | Agree. We noticed this problem. 3GPP chipset vendors tends to want to perform IODT with at least two large network vendors before setting the corresponding bits in their chipset’s capabilities. In some cases that leads to the problem that features cannot be rolled out since chipset vendors lack IODT opportunities with a second NW vendor. Even if the risk of identifying problems with the secondary network’s may be low, the risk remains that the chipset is not compliant with another network that implements the feature later but uses it in a different manner than the first network. That second network could base its configuration of the feature on the original capability bit. |
| Ericsson | Root cause 4 | Despite all IODT, some UEs don’t support some features correctly. RAN is then usually required to apply a work-around. It is cumbersome but possible to alleviate the error with an affected UE. Via the OAM system the operators could configure their RAN nodes to disable the feature for all UEs with the IMEI SV that was found in the affected UE. In addition, the operator would need to configure the CN (AMF) so that it passes the IMEI SVs of all UEs to the RAN (usually not the default). However, we notice that such errors are usually not tied to a single UE model but rather to all UEs that use the same chipset. Sometimes these problems affect all devices that use the same hard- and software version of a chipset. But there were also cases where all chipset generations from a chipset vendor required a workaround on the NW side. In some cases, those problems are spanning multiple generations of a chipset model, meaning that that it affects hundreds or thousands of IMEI SVs. And to identify them based on IMEI SVs one need to identify them one-by-one and which all operators would then need list in their OAM systems. Furthermore, we discovered cases where chipset vendors updated their software and where UE vendors pushed the new firmware to their UEs to fix the problem. However, in some cases UE vendors did not increase the IMEI SV. Hence, IMEI SV based filtering would classify this UE anyway as faulty and omit the functionality even though the updated UE supports it correctly now. There is a need for a means for the RAN to identify UEs with a specific chipset hardware and software version to be able to apply necessary workarounds for all impacted UEs. |
| MTK | Root cause 1, 2, 3, 4 | The example for root cause 1 is in the Problem 1. While the root cause 2 points out the importance of uniform and homogeneous feature deployments, the root cause 3 seems to encourage the opposite way (i.e., allow heterogeneous feature deployment within an MNO network) so that we think RAN2 should further study the intent, exercise, and the possible consequence. There was a field issue regarding the root cause 4 so that we think there is still room for improvement on e.g., how to make changes across different specs revisions distinguishable and identifiable. |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |
| OPPO | Regarding RP-253230, our understanding is that the core proposal primarily advocates for establishing basic granularity standards for 6G capability (specifically per-band in P1, per-BC in P2, and per-FS/FSPC in P3). The document also introduces requirements for mandatory network-side capabilities in cases where per-FS or per-FSPC capability proves unfeasible. However, we find the logical connection between these elements somewhat unclear and would appreciate further clarification from the proponents. Additionally, as outlined in RP-253048, we maintain a negative stance toward the IoDT workaround approach. In particular, we oppose the direction of implementing per-vendor punitive measures, which fall under root causes 3 and 4. |
| CATT | All root causes. For root cause 1 and 2: We agree to evaluate carefully whether a feature should be supported mandatorily at UE side. But whether network equipment should support a specific feature depends on the operator's deployment requirements. Mandatory feature that are not utilized in commercial networks may introduce unnecessary costs to equipment without providing operational benefits. In general, we think this flexibility should be kept at network side, i.e., final decision on supported features at network is still up to operators. For root causes 3 and 4: Agree with OPPO's views. |

Problem x: (New problem )

Problem description:

| Company | Pain Point |
| --- | --- |

Please add root cause(s) and the corresponding example(s). Separate item/row is preferred if multiple root causes are identified:

| Company | Root Cause | Example |
| --- | --- | --- |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |

Problem x: (New problem )

Problem description:

| Company | Pain Point |
| --- | --- |

Please add root cause(s) and the corresponding example(s). Separate item/row is preferred if multiple root causes are identified:

| Company | Root Cause | Example |
| --- | --- | --- |

If companies don’t see the pain point as a problem or an area to be considered in 6G UE capability complexity/overhead reduction, please comment in below the reason and justification.

| Company | Comment on Pain Point |
| --- | --- |

Phase 2 Discussion

<to be updated based on phase 1 discussion>

Conclusion

<to be updated>

Reference

- R2-2508044	Discussion on 6GR UE capability framework	vivo

- R2-2508073	Discussion on UE capability framework	Transsion Holdings

- R2-2508076	UE Capability pain points and considerations	Xiaomi

- R2-2508097	Considerations on 6GR UE Capability	CATT

- R2-2508113	Discussion on 6G UE Capability	OPPO

- R2-2508145	UE capability framework considerations for 6GR	MediaTek Inc.

- R2-2508209	6GR UE Capability Framework	SHARP Corporation

- R2-2508322	6GR UE capability framework	Nokia

- R2-2508356	Views on UE capability signaling in 6G	Charter Communications, Inc

- R2-2508422	On UE capability framework for 6G	NTT DOCOMO INC.

- R2-2508445	Considerations on UE capability framework in 6G	Apple

- R2-2508459	Discussion on UE capability aspects in 6G	Fujitsu

- R2-2508509	Consideration on 6G UE Capability	ZTE Corporation, Sanechips

- R2-2508540	Discussion on 6GR UE capability	Sony

- R2-2508610	6G UE Capability Framework	LG Electronics Inc.

- R2-2508616	Discussion approach for 6G UE capability	Huawei, HiSilicon

- R2-2508622	Considerations on UE capability framework for 6GR	Lenovo

- R2-2508624	UE Capability Framework in 6G	Ofinno

- R2-2508668	Discussion on fundamentals of UE capability framework	NEC

- R2-2508732	Improvements for UE capabilities	Ericsson

- R2-2508768	Design of 6GR UE capabilities	InterDigital, Inc.

- R2-2508839	Considerations on UE capability framework for 6GR	CMCC

- R2-2508868	Considerations on UE capability framework in 6G	Qualcomm Incorporated

- R2-2508876	UE capability framework for 6G	Samsung

- R2-2508903	A New UE Capability Framework for 6G	AT&T, Deutsche Telekom, Ericsson, Nokia, Orange, Panasonic, Spark NZ, Telecom Italia, Telstra, T-Mobile USA, Verizon, Viavi

- R2-2508916	Discussion on UE Capability Framework	Futurewei Technologies

- R2-2508937	Consideration on 6GR UE Capability	Spreadtrum, UNISOC

- R2-2508939	Consideration on 6G UE capability framework	ETRI

- R2-2508963	Discussion on UE capability framework	TCL

- R2-2508968	Discussion on IODT problems for 6G UE capability framework	KDDI Corporation

- R2-2509032	Discussion on UE capability framework	Deutsche Telekom AG

- R2-2509073	Discussion on dynamic UE capability update	BT Plc, Ericsson, T-Mobile USA, Deutsche Telekom

- R2-2506988	Considerations on UE capability signalling in 6G	Qualcomm Incorporated

- R2-2507607	Consideration on 6G UE Capability	ZTE Corporation

- RP-253048		Discussion on Handling of IoDT issues	Oppo

- RP-253230		Views on UE capability framework 	Qualcomm

- RP-253066		A New UE Capability Framework for 6G	AT&T, Deutsche Telekom, Ericsson, KT Corp., Nokia, NTT Docomo, Orange, Panasonic, SK Telecom, Spark NZ, Telecom Italia, Telstra, T-Mobile USA, Verizon, Viavi, Vodafone, Xiaomi