3GPP TSG RAN WG2  Meeting #132	R2-25xxxxx

Dallas, USA, 17 - 21 November 2025

Title:	LS on CSI-IM resources/sets in LTM-Candidate

Response to:	-

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN2

To:	RAN1

Cc:	-

Contact person:	David Lecompte (david.lecompte@huawei.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

The mobility tab of the consolidated Rel-19 higher layers parameters list in R1-2508202 includes in LTM-Candidate:

- a new field ltm-CSI-IM-ResourceSetToAddModList-r19, which is a list of 1 to maxNrofCSI-IM-ResourceSets (which is 64) of CSI-IM-ResourceSet;

- a new field ltm-CSI-IM-ResourceToAddModList-r19, which is a list of 1 to maxNrofCSI-IM-Resources (which is 192) of CSI-IM-Resource.

However:

- the ltm-CSI-ReportConfig of an LTM-Candidate seems to be the only field that can refer to a CSI-IM-ResourceSet configured using ltm-CSI-IM-ResourceSetToAddModList-r19 in that LTM-Candidate;

- a CSI-IM-ResourceSet cannot refer to more than 8 CSI-IM-Resource.

Therefore, RAN2 would like to ask RAN1 whether it is the correct understanding that, in one LTM-Candidate, the network needs to configure at most one CSI-IM-ResourceSet and at most 8 CSI-IM-Resource.

# 2	Actions

To: RAN1

ACTION: RAN2 kindly asks RAN1 to answer the above question.

# 3	Dates of next TSG RAN WG 2 meetings

RAN2 #133	9 - 13 February 2026	Stor - Göteborg, Sweden

RAN2 #133bis 	13 - 17 April 2026	San Ġiljan, Malta