3GPP TSG-RAN WG2 Meeting #132		R2-2509442

# Dallas, USA, Nov. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-19 RAN1 UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 agreed in RAN2-132 that RAN2 specify one capability for minimumTimeGap for both OOK and OFDM. Update the capabilities as per the agreement. UE has different minimumTimeGap-r19 values for different SSB periodicities, but doesn’t need to indicate the signalling individually. Capture the below agreement made in RAN2-132: - Indicating the supported band list for LP-WUS and the receiver type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change values of minimumTimeGap-r19 in ‘cap1’, ‘cap2’, and ‘cap3’, for all SSB peridiocities; Define per-UE capability to indicate frequency bands where UE supports LP-WUS operation in IDLE/INACTIVE based on OFDM. Define per-UE capability to indicate frequency bands where UE supports LP-WUS operation in IDLE/INACTIVE based on OOK. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corrections of LPWUS capabilities will not be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.306 draft CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6.3.3	UE capability information elements

START OF CHANGE – POST R-132

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

< Unchanged parts are omitted >

UE-NR-Capability-v1900 ::=               SEQUENCE {

aiml-Parameters-r19                      AIML-Parameters-r19                                          OPTIONAL,

ue-RadioPagingInfo-r19                   OCTET STRING (CONTAINING UE-RadioPagingInfo-r19)             OPTIONAL,

-- R4 61-2: Support of (e)RedCap UE with FR1-NTN

ntn-ERedCap-FR1-r19                      ENUMERATED {supported}                                       OPTIONAL,

onDemandSIB1-r19                         ENUMERATED {supported}                                       OPTIONAL,

onDemandPosSIB-ConnectedCtrlParam-r19    ENUMERATED {supported}                                       OPTIONAL,

ntn-Redirection-r19                      ENUMERATED {supported}                                       OPTIONAL,

drx-PreferenceCellDTX-DRX-r19           ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v19xy                             OPTIONAL

}

UE-NR-Capability-v19xy ::=               SEQUENCE {

lpwus-SupportedBands-IdleInactive-OFDM-r19    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

lpwus-SupportedBands-IdleInactive-OOK-r19    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=          SEQUENCE {

phy-ParametersXDD-Diff                   Phy-ParametersXDD-Diff                                       OPTIONAL,

mac-ParametersXDD-Diff                   MAC-ParametersXDD-Diff                                       OPTIONAL,

measAndMobParametersXDD-Diff             MeasAndMobParametersXDD-Diff                                 OPTIONAL

}

< Unchanged parts are omitted >

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

START OF CHANGE – AT R-132

#### –	UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

UE-RadioPagingInfo-r19 ::=            SEQUENCE {

lpwus-SupportedBandList-r19           SEQUENCE (SIZE (1..maxBands)) OF LPWUS-SupportedBandInfo-r19        OPTIONAL,

pagingAdaptation-r19                  ENUMERATED {supported}                                              OPTIONAL,

pagingAdaptionPEI-SupportBandList-r19 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR                OPTIONAL,

...

}

LPWUS-SupportedBandInfo-r19 ::=      SEQUENCE {

supportedBandIndicator-r19           FreqBandIndicatorNR,

-- R1 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal

lpwus-OOK-r19                        ENUMERATED {supported}                                               OPTIONAL,

-- R1 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence

lpwus-OFDM-r19                       ENUMERATED {supported}                                               OPTIONAL,

-- R1 62-1b: LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured

lpwus-LP-SS-r19                      ENUMERATED {supported}                                               OPTIONAL,

minimumTimeGap-r19

ENUMERATED {cap1, cap2, cap3}

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP

END OF CHANGE