3GPP TSG-RAN WG2 #132	R2-250XXXX

Dallas, USA, Nov. 17th - 21st, 2025

Agenda:	x.x.x

Source:	Ericsson

Title:	Comments on MIMO Running CR for TS 38.331

Document for:	Discussion, Decision

# 1	Introduction

This document collects comments for the following e-mail discussion:

[Post132][213][MIMO_Ph5] CR for TS 38.331(Ericsson)

Intended outcome: Review and agree the CR for TS 38.331

Deadline:  Short

Companies are invited to provide contact details on the table below.

| Company | Name | E-mail |
| --- | --- | --- |
| Ofinno | Hsin-Hsi Tsai | htsai@ofinno.com |
| Nokia | Andrew Lappalainen | andrew.lappalainen@nokia.com |
| CATT | Lei Wang | wanglei24@cictmobile.com |
| Samsung | Shiyang Leng |  |
| OPPO | Yumin Wu | wuyumin@oppo.com |

# 2	Discussion

## 2.1	Topics for further discussion

In RAN2#132, the following was captured for the TDoc R2-2509123:

Further discuss P1, P2 and P3 in the post meeting email discussion for the RRC CR

Proposal 1: To avoid interoperability issues, change startingBitOfFormat2-3-r19 to startingBitOfFormat2-3-r19, defined as INTEGER (32..45).

As pointed out in R2-2509123, the configuration of startingBitOfFormat2-3-r19 and startingBitOfFormat2-3 are defined as independent configurations, but the capability indicating support of startingBitOfFormat2-3-r19 seems to imply that the Rel-19 feature is just an extension of the Rel-15 feature. To remove the contradiction between the two features, we have to align both configuration and UE capability to either make Rel-19 feature dependent on the Rel-15 feature or make Rel-19 feature independent of Rel-15 feature.

[Huawei] The above is a complete misunderstanding of the problem. The problem is that:

- values 1 to 31 have exactly the same meaning regardless whether they are signalled using the Rel-15 or the Rel-19 field

- the Rel-19 UE capability applies to support of values 32 to 45

Accordingly, a network implemented using 38.331 Rel-19 will assume that the Rel-19 UE that does not support the Rel-19 capability still can decode the Rel-19 field set to values 1 to 31, but most likely, such a UE will not understand this field and may ignore it or trigger re-establishment.

To avoid this problem, the network should use the legacy field when it wants to signal a value between 1 and 31, and the new field when it wants to signal a value between 32 and 45, and there is no need for any change to 38.306. The most sensible way to capture that is to remove the values 1 to 31 from the new field.

Option 1: Rel-19 feature depends on support of Rel-15 feature.

With the following changes:

- change in 38.331 startingBitOfFormat2-3-r19 to startingBitOfFormat2-3-v19 and define it as INTEGER (32..45);

- clarify in 38.306 that the Rel-19 feature is only supported if the UE includes the Rel-15 feature;

Option 2: Rel-19 feature depends on support of Rel-15 feature.

- keep 38.331 as it is;

- clarify in 38.306 that the Rel-19 feature shall support the full value range (i.e. 1 to 45);

Option 3:

- change in 38.331 startingBitOfFormat2-3-r19 to startingBitOfFormat2-3-v19 and define it as INTEGER (32..45);

- no change to 38.306

Which option is preferred by companies? Note that the corresponding 38.306 changes do not have ASN1 impact and can be discussed next meeting.

| Company | Option 1/ Option2 | Comments |
| --- | --- | --- |
| Ofinno | Option 2 | TS 38.331 specified that the network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-r19 simultaneously. If the network wants to configure startingBitOfFormat2-3-r19 when the UE is supported, the value from 1 to 31 should also be applicable. The extension normally covers the original range. [Huawei] What does "when the UE is supported" mean? The problem is that now, when the UE does NOT indicate support, it still looks like the network could send the new field to a value from 1 to 31. [Ericsson] Yes, the NW could configure value from 1 to 31 in option 2, and it should be fine if the capability also covers the values from 1 to 31. |
| Nokia | Option 2 | We thought that this was the understanding in RAN1 already. If the R19 capability supports the range 1 to 45, there does not seem to be any interoperability issue here. [Huawei] The question is not whether the UE supports 1 to 45 when it indicates support of this capability, the question is whether the UE understands the new field set to a value from 1 to 31 when it does not indicate support of this capability. Since values 1 to 31 can be signalled with the legacy field, and the values 1 to 31 in the new field have exactly the same meaning, there is not use to signal these values with the new field (apart from creating problems). [Ericsson] The values 1 to 31 in the legacy field are related to the legacy capability. If option 2 is adopted, the UE can only support to be configured with values 1 to 31 in the Rel-19 field if it indicates the Rel-19 capability. |
| Huawei, HiSilicon | Option 3 |  |
| CATT | Option 3 | I sympathize what HW had clarified. For a R19 UE not supporting this capability, if the NW signals the value [1, 31] using the –r19 field, then the UE cannot decode this field and will cause RRC re-establishment due to receiving a configuration the UE does not comply with. |
| Ericsson | Option 2 | Both option 1 and 2 could work, option 3 seems to imply that the UE could support only the extension of the values but not the legacy 1-31 values. If this option is preferred we think this should be confirmed with RAN1. |
| Samsung | Option 2 | Agree with Ericsson’s comment. Option 3 cannot work if we don’t change 38.306. It can work only if we add in 38.306 that UE supporting R19 extension shall support the R15 values, which then becomes Option 1. Also, “startingBitOfFormat2-3-r19” has already been used in the latest RAN1 CR. If we go with Option 1, we have change 331, 306 and RAN1 spec as well. To reduce spec changes, we prefer Option 2. This is also RAN1 understanding as checked with our RAN1. In addition, RAN1 has discussed whether to just signal the extended value, but the final agreement is to signal full value range from 1 to 45. If we change to 32..45, we need to inform RAN1 since it is not aligned with their agreement. Agreement in RAN1#118bis About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise |
| OPPO | Option 2 |  |

Since most companies prefer option 2 and it is raised as the RAN1 understanding already by some companies, we can adopt option 2. But note that this option does not require any change to 38.331, hence nothing will be reflected in the 38.331 CR. The change to 38.306 can be discussed in next meeting since there is no ASN1 impact of such change.

Proposal 2: In the field description of pathlossOffset, change the description from "UL-only TRP" to "UL TRP", or completely remove the phrase "UL-only TRP".

Please provide your preference between changing the description from the term "UL-only TRP" to "UL TRP" or remove the "UL-only TRP" term completely. If any other suggestion please indicate below.

| Company | Change to "UL TRP"/ Remove "UL-only TRP" / Other? | Comments |
| --- | --- | --- |
| Ofinno | Change to "UL TRP" | This parameter is for TCI-UL-State. Remove the whole term makes the purpose of the parameter a bit ambiguous. "UL TRP" is used in TS 38.300, which is ok for TS 38.331. |
| Nokia | Keep (UL-only TRP) or change to UL TRP | We have some preference for UL-only TRP because the point is that it does not apply to a TRP with DL (SSBs), but we also understand that the term “UL-only” is not defined elsewhere, so UL TRP is acceptable if preferred by most companies. We agree with Ofinno that removing UL TRP leaves the description of the field ambiguous. |
| Huawei, HiSilicon | Change to "UL TRP" |  |
| CATT | See comments | I am not sure why we need such following change. pathlossOffset Indicates the pathloss offset applied to the CI tate of an UL-only TRP (see TS 38.213 [13], clause 7). Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on. For unified TCI framework, now we have joint mode and separate mode. In the original field description, the UL only TCI is corresponding to the separate TCI mode, while the joint TCI is corresponding to the joint mode. For both of which, there will be association between the pathloss offset and the UL TCI. The current field description is fine for me. |
| Ericsson |  | We are fine to change it to “UL TRP” |
| Samsung | Change to "UL TRP" | Okay to replace “UL-only TRP” by “UL TRP” to align with stage-2 description, where “UL TRP” is explained: “For asymmetric DL single-TRP and UL multi-TRP operation, the UL TRP may reduce or even disable DL transmission.” |
| OPPO |  | Fine to change it to “UL TRP”. However, in stage-2, it seems that ““UL TRP”” is described in the context of “asymmetric DL single-TRP and UL multi-TRP operation”. In RRC, if there is no further clarification/reference on “UL TRP, it could be applied to any UL TRP. |

Since all companies were fine to change “UL-only TRP” to “UL TRP”, this was taken into account in the CR.

Proposal 3: The field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

Do companies agree with the proposal above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ofinno | No | In the legacy, the value symbols/slots/ms X corresponds to X symbols/slots/ms, etc. Probably we can add “symbol” to align with the legacy, for example: minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting. Value sym0 corresponds to 0 symbol, value sym1 corresponds to 1 symbol, and so on. The legacy parameter measDurationSymbols could be a good reference. |
| Nokia | No | Ofinno’s understanding seems correct. The offset is in total number of symbols, not “first symbol”, “second symbol”, etc. |
| Huawei | See comment | Ok with Offino's suggestion. |
| CATT | No | Same view with ofinno. |
| Samsung | No | Ok with Ofinno’s suggestion. |
| OPPO | No | Fine with Ofinno’s suggestion. |

Offino’s suggestion was taken into account in the CR.

## 2.2	Comments to the MIMO CR

The running CR implements the latest agreements from RAN2#132 and L1 parameters. The additions compared to the previous version are highlighted with bubble comments.

Please do not make changes/comments directly on the running CR – companies are invited to provide suggested changes/comments on the table below. To make it easier to track and reply to the comments, please label each comment i.e. [Issue 1], [Issue 2], and so on.

| Company | Comments | Rapporteur response |
| --- | --- | --- |
| Ofinno | [Issue 1] The full name of UEIRI (UE Initiated Report Indication) does not include CSI. Change “UE initiated CSI reporting indicator” to “UE initiated report indicator” of pucch-Resource as below: pucch-Resource Indicates the periodic PUCCH resource for the UE initiated report indicator for both mode-A and mode-B UE initiated CSI reporting: - to request dynamically scheduled PUSCH to carry UE initiated CSI report for mode-A; - to notify the network of a Type-1 CG PUSCH to carry UE initiated CSI report for mode-B. [Huawei] There is no field called pucch-Resource in CSI-ReportUE-Initiated, there are pucch-ResourceConfig, pucch-ResourceList and resource. Which one is that supposed to be a description of? | Taken into account. (Regarding Huawei’s comment, indeed the field is not called pucch-Resource, but since Huawei in Issue 3 has the suggestion to change the field name, this was taken into account as well). |
| Nokia [Issue 1] | There are redundant spaces (before “-r19”) in these field names that should be deleted: ng-n1-n2-cbsr-r19 CHOICE { two-four-three -r19 BIT STRING (SIZE (192)), two-six-two -r19 BIT STRING (SIZE (192)), two-eight-two -r19 BIT STRING (SIZE (256)), four-four-two -r19 BIT STRING (SIZE (128)), two-four-four -r19 BIT STRING (SIZE (256)), four-four-four -r19 BIT STRING (SIZE (256)), four-eight-two -r19 BIT STRING (SIZE (256)) } | Taken into account. |
| Nokia [Issue 2] | In additionalOneSlotOffsetDoppler each of the choices corresponds to the case where there are X (X∈{2,3,4}) CSI-RS resources per CSI-RS resource group, and the choice configured applies to all KDOPP resource groups simultaneously. Right now the field names sound like “2nd CSI-RS resource group”, “3rd CSI-RS resource group”, “4th CSI-RS resource group”. It would be clearer to use a name like ‘twoResourcesPerGroup’, ‘threeResourcesPerGroup’, ‘fourResourcesPerGroup’. additionalOneSlotOffsetDoppler-r19 CHOICE { resourceGroup2-r19 BIT STRING(SIZE(2)), resourceGroup3-r19 BIT STRING(SIZE(3)), resourceGroup4-r19 BIT STRING(SIZE(4)) } [Huawei] Agree. Samsung: agree, and already implemented. | Field names were updated accordingly. |
| Huawei [Issue 1] | The CR should not include clauses and IEs without change, and the language should not be changed to Portuguese in perhaps more than 100 places. | Irrelevant IEs/clauses were removed and the language fixed. |
| Huawei [Issue 2] | In the text below: - what does the addition "(including codebookType)" mean? - what does "For codebookConfig-r19 ..., cri-Type2 can only be configured when typeII-r16 is configured in codebookConfig-r16" mean, knowing that if codebookConfig-r19, codebookConfig-r16 is not configured? codebookConfig Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network can only configure one of codebookConfig, codebookConfig-r16, codebookConfig-r17, codebookConfig-r18 or codebookConfig-r19 (including codebookType) in a CSI-ReportConfig. The network includes codebookConfig-v1730 only if codebookConfig-r17 is configured. For codebookConfig-r19, cri-TypeI-SinglePanel can only be configured when typeI-SinglePanel is configured in codebookConfig and cri-TypeII can only be configured when typeII-r16 is configured in codebookConfig-r16. Samsung: If understand correctly "(including codebookType)" should mean: Network can only configure one of codebookConfig, codebookConfig-r16, codebookConfig-r17, codebookConfig-r18 or codebookConfig-r19 (if codebookType is configured in codebookConfig-r19) in a CSI-ReportConfig. | As Samsung explains, only when codebookConfig-r19 is set to codebookType then it cannot be configured together with other releases of codebookConfig. The reason is because cri-TypeI-SinglePanel and cri-TypeII (present in codebookConfig-r19) are enhancements to typeI-SinglePanel (configured in codebookConfig) and typeII-r16 (configured in codebookConfig-r16). The change suggested by Samsung is adopted to clarify this. |
| Huawei [Issue 3] | Field names like pucchResourceConfig, pucch-ResourceList and PUCCH-Resource-r19 look like they are, respectively, configuration of, list of, critical extension of PUCCH-Resource (in CSI-ReportConfig) but they are not. To avoid confusion, a different name should be used, e.g. uerir-ResourceConfig, ueiri-ResourceList, ueiri-Resource. | Taken into account. Since the other fields within UE initiated CSI report were already changed from UE IR/IBR/RIR to UE-Initiated, the same was adopted also here e.g. ue-InitiatedResourceConfig. |
| Huawei [Issue 4] | The only extension markers in CSI-ReportUE-Initiated-r19 are inside r19eventTypeUE-Initiated, there should at be extensions markers also (i.e., don't remove the existing ones) at the end of CSI-ReportUE-Initiated-r19, possibly also in reportTransmissionMode-r19 and pucch-ResourceConfig-r19 (to be renamed) | The extensions were added on CSI-ReportUE-Initiated-r19 level and in pucch-ResourceConfig-r19 (now named ue-InitiatedResourceConfig). For reportTransmissionMode-r19, since it could either be extended with new parameters within the existing modes or with new modes as such, maybe better to rely on the extension on CSI-ReportUE-Initiated-r19 level? |
| Huawei [Issue 5] | Wrong English syntax, should be: eventInstanceCount Indicates the minimum number of event instances of the same new beam within a configured time window for the UE to initiate CSI report (see TS 38.214 [19], clause 5.2.1.5.4.1). | Taken into account. |
| Samsung | Do we need Need R at the following places? CodebookConfig-r19 ::= CHOICE { codebookType CHOICE { omitting }, OPTIONAL, --Need R cri-TypeI-SinglePanel SEQUENCE { cri-TypeI-SinglePanelRI-Restriction-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8)) OPTIONAL, -- Need R cri-TypeI-SinglePanelN1-N2-CBSR-r19 CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 OPTIONAL -- Need R }, OPTIONAL, --Need R cri-TypeII SEQUENCE { cri-TypeII-RI-Restriction-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4)) OPTIONAL, -- Need R cri-TypeII-N1-N2-CBSR-r19 CRI-TypeII-N1-N2-CBSR-List-r19 OPTIONAL -- Need R } OPTIONAL, --Need R } | We cannot have optional for the choice structure, and thus we cannot introduce the Need R. |
| Samsung | For CSI-AperiodicTriggerStateList, suggest to refine the wording: “For the mode-A UE-initiated CSI report, upon reception of the value associated with a CSI trigger state for the corresponding CSI-ReportConfig, the UE sends the CSI report as in TS 38.214 [19], clause 5.2.1.5.4.” | Taken into account. |
| Samsung | Based on the agreement and discussion of R2-2508324 The change for additionalOneSlotOffset should be removed, that change is intended for additionalSlotOffset. additionalOneSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. This field is only configured for codebook typeI-SinglePanel-r19, typeI-MultiPanel-r19, etypeII-r19 and typeII-FePortSelection-r19.. additionalSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. The value 0 corresponds to 0 slots, value 1 corresponds to 1 slot and so on. This field is only configured for codebook typeI-SinglePanel and typeII-r16.. | Taken into account. |