3GPP TSG RAN WG2#133	R2-260xxxx

Gothenburg, Sweden, 9th - 13th, February 2026

Title:	draft-LS on AS security aspects

Response to:

Release:	Rel-20

Work Item:	Study on 6G Radio [New RAN1 led SI: FS_6G_Radio]

Source:	[To be] RAN2

To:	SA3

Cc:	RAN

Contact person:	Eswar Vutukuri

eswar.vutukuri@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 had further discussion on the Access Stratum security aspects for 6G and made the following agreements:

Agreements - Currently, in 5G: - MAC layer has no Sequence Number and no in-sequence delivery - MAC CEs can be grouped together with other MAC CEs and/or MAC SDUs - Certain MAC CEs (e.g. BSR, PHR, TA) need to be sent before security establishment - RAN2 will study the impact of MAC CE SA3 security algorithms (with early guidance to SA3) on RAN2 areas like processing, complexity, latency etc. in close collaboration with SA3. - RAN2 will discuss based on new requirements coming from SA3 how to design the MAC

In addition to the above, RAN2 would like to provide SA3 the following information:

- BSR/DSR/PHR are “time critical” MAC CEs.  For example, the content of these MAC CEs—and the decision on whether to include them in a transport block (TB)—may only be finalized late in the TB construction process (e.g. after reception of the uplink UL grant) and there is a strict deadline by which these MAC CEs must be ready for transmission.

- For other MAC CEs (i.e. other than BSR/DSR/PHR), it doesn’t mean there are no other RAN2 concerns/considerations. For instance, overhead and complexity is still a strong consideration and may be an issue for MAC CEs in general

- RAN2 will continue to study further the time criticality of all other MAC CEs in UL and DL considering that there are both TX side and RX side processing requirements with the aim to prepare ourselves with an answer if SA3 ask any questions for these.

RAN2 would like to ask SA3 the following:

For lower layer security:

- From RAN2 point of view, it would be desirable that SA3 provides RAN2 with some understanding of the security algorithm complexity for the lower layer security – Can SA3 kindly provide this information?

Other aspects:

- Is PQC applicable to the air interface?

- Is SA3 considering security (integrity protection) for system information and if yes what is the associated overhead?

# 2	Actions

To SA3

ACTION: RAN2 kindly asks SA3 to take the above information into account, provide any feedback and answer the above questions.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #133-bis 		2026‑04‑13 to 2026‑04‑17		Malta

TSG RAN WG2 Meeting #134 		2026‑05‑18 to 2026‑05‑22		Dalian