3GPP TSG-RAN WG4 Meeting #90bis	R4-1903009

# Xi'an, China, 8 – 12 Apr. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_xBLTE_2BNR_yDL2UL-Core |  |  |  |  |  | Date: |  |  | 2019-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | New EN-DC Inter-band LTE (x bands DL with 1 band UL, x=1,2,3,4) and NR (2 bands DL with 1 band UL) band combinations have been studied and introduced in corresponding TR in rel-16. - LTE (1DL/1UL)+NR(2DL/1UL) DC combinations: 5 DC combos - LTE (2DL/1UL)+NR(2DL/1UL) DC combinations: 4 DC combos - LTE (3DL/1UL)+NR(2DL/1UL) DC combinations: 1 DC combo |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TS38.101-3, include new EN-DC band combinations in rel-16 - In R4-1900248 was endorsed in rel-16. However, the CR on TS38.101-3 of rel-16 is not agreed in last plenary meeting. So, we added these band combos in TS38.101-3 for rel-16. Also follow new band combos are completed in this meeting. - LTE (1DL/1UL)+NR(2DL/1UL) DC combinations including FR1 - EN_DC_1A_n40A-n78A with 2UL_DC_1A_n40A or DC_1A_n78A - EN_DC_3A_n1A-n79A with 2UL_DC_3A_n1A or DC_3A_n79A - EN_DC_3A_n40A-n78A with 2UL_DC_3A_n40A or DC_3A_n78A - LTE (1DL/1UL)+NR(2DL/1UL) DC combinations including both FR1&FR2 - EN_DC_3A_n1A-n257A with 2UL_DC_3A_n1A or DC_3A_n257A - EN_DC_8A_n77A-n257A with 2UL_DC_8A_n77A or DC_8A_n257A - LTE (2DL/1UL)+NR(2DL/1UL) DC combinations including FR1 - EN_DC_1A-3A_n7A-n78A with 2UL_DC_1A_n7A or DC_1A_n78A or DC_3A_n7A or DC_3A_n78A - EN_DC_1A-3C_n28A-n78A with 2UL_DC_1A_n28A or DC_1A_n78A or DC_3C_n28A or DC_3C_n78A - EN_DC_1A-7C_n28A-n78A with 2UL_DC_1A_n28A or DC_1A_n78A or DC_7C_n28A or DC_7C_n78A - EN_DC_3A-7A_n28A-n78A with 2UL_DC_3A_n28A or DC_3A_n78A or DC_7A_n28A or DC_7A_n78A - LTE (3DL/1UL)+NR(2DL/1UL) DC combinations including FR1 - EN_DC_1A-3A-7A_n28A-n78A with 2UL_DC_1A_n28A or DC_1A_n78A or DC_3A_n28A or DC_3A_n78A or DC_7A_n28A or DC_7A_n78A For Some EN-DC band with self-interference problem, MSD exception requirements are defined in section 7.3B.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New EN-DC Inter-band LTE (x bands DL with 1 band UL, x=1,2,3,4) and NR (2 bands DL with 1 band UL) band combinations won’t be supported in Rel-16 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B.4, 5.2B.5, 5.2B.6 5.5B.4, 5.5B.5, 5.5B.6, 6.2B.4.2.3, 7.3B.2.3, 7.3B.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

5.2B.4	Inter-band EN-DC within FR1

5.2B.4.1	EN-DC (two bands)

Table 5.2B.4.1-1: Band combinations for inter-band EN-DC within FR1 (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 | 1 | n28 | No |
| DC_1_n40 | 1 | n40 | No |
| DC_1_n51 | 1 | n51 | No |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_1_n783 | 1 | n78 | No |
| DC_1_n793 | 1 | n79 | No |
| DC_2_n5 | 2 | n5 | No |
| DC_2_n66 | 2 | n66 | DC_2_n66 |
| DC_2_n71 | 2 | n71 | No |
| DC_2_n78 | 2 | n78 | DC_2_n78 |
| DC_3_n7 | 3 | n7 | No |
| DC_3_n28 | 3 | n28 | No |
| DC_3_n40 | 3 | n40 | No |
| DC_3_n51 | 3 | n51 | No |
| DC_3_n773 | 3 | n77 | DC_3_n77 |
| DC_3_n783 | 3 | n78 | DC_3_n78 |
| DC_3_n793 | 3 | n79 | No |
| DC_5_n40 | 5 | n40 | No |
| DC_5_n66 | 5 | n66 | DC_5_n66 |
| DC_5_n783 | 5 | n78 | No |
| DC_7_n28 | 7 | n28 | No |
| DC_7_n51 | 7 | n51 | No |
| DC_7_n783 | 7 | n78 | No |
| DC_7-7_n783 | CA_7-7 | n78 | No |
| DC_8_n40 | 8 | n40 | No |
| DC_8_n773 | 8 | n77 | No |
| DC_8_n783 | 8 | n78 | No |
| DC_8_n793 | 8 | n79 | No |
| DC_11_n773 | 11 | n77 | No |
| DC_11_n783 | 11 | n78 | No |
| DC_11_n793 | 11 | n79 | No |
| DC_12_n5 | 12 | n5 | No |
| DC_12_n66 | 12 | n66 | No |
| DC_18_n773 | 18 | n77 | No |
| DC_18_n783 | 18 | n78 | No |
| DC_18_n793 | 18 | n79 | No |
| DC_19_n773 | 19 | n77 | No |
| DC_19_n783 | 19 | n78 | No |
| DC_19_n793 | 19 | n79 | No |
| DC_20_n8 | 20 | n8 | DC_20_n8 |
| DC_20_n284 | 20 | n28 | No |
| DC_20_n51 | 20 | n51 | No |
| DC_20_n77 | 20 | n77 | No |
| DC_20_n783 | 20 | n78 | No |
| DC_21_n773 | 21 | n77 | No |
| DC_21_n783 | 21 | n78 | No |
| DC_21_n793 | 21 | n79 | No |
| DC_25_n41 | 25 | n41 | No |
| DC_26_n413 | 26 | n41 | No |
| DC_26_n773 | 26 | n77 | No |
| DC_26_n783 | 26 | n78 | No |
| DC_26_n793 | 26 | n79 | No |
| DC_28_n51 | 28 | n51 | No |
| DC_28_n773 | 28 | n77 | No |
| DC_28_n783 | 28 | n78 | No |
| DC_28_n793 | 28 | n79 | No |
| DC_30_n5 | 30 | n5 | No |
| DC_30_n66 | 30 | n66 | No |
| DC_38_n78 | 38 | n78 | No |
| DC_39_n781,3 | 39 | n78 | No |
| DC_39_n793 | 39 | n79 | No |
| DC_40_n77 | 40 | n77 | No |
| DC_41_n77 | 41 | n77 | No |
| DC_41_n78 | 41 | n78 | No |
| DC_41_n792,3 | 41 | n79 | No |
| DC_42_n51 | 42 | n51 | No |
| DC_42_n775 | 42 | n77 | N/A |
| DC_42_n785 | 42 | n78 | N/A |
| DC_42_n795 | 42 | n79 | N/A |
| DC_66_n71 | 66 | n71 | No |
| DC_66_n5 | 66 | n5 | DC_66_n5 |
| DC_66_n78 | 66 | n78 | No |
| NOTE 1: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 2: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 3: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 4: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 5: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

5.2B.4.2	EN-DC (three bands)

Table 5.2B.4.2-1: Band combinations for inter-band EN-DC within FR1 (three bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3_n28 | CA_1-3 | n28 |  |
| DC_1-3_n772 | CA_1-3 | n77 |  |
| DC_1-3_n782 | CA_1-3 | n78 |  |
| DC_1_n3-n782 | 1 | CA_n3-n78 |  |
| DC_1-3_n792 | CA_1-3 | n79 |  |
| DC_1-5_n782 | CA_1-5 | n78 |  |
| DC_1-7_n282 | CA_1-7 | n28 |  |
| DC_1-7_n78 2 | CA_1-7 | n78 |  |
| DC_1-7-7_n782 | CA_1-7-7 | n78 |  |
| DC_1_n7-n782 | 1 | CA_n7-n78 |  |
| DC_1-8_n782 | CA_1-8 | n78 |  |
| DC_1-18_n772 | CA_1-18 | n77 |  |
| DC_1-18_n782 | CA_1-18 | n78 |  |
| DC_1-18_n79 | CA_1-18 | n79 |  |
| DC_1-19_n772 | CA_1-19 | n77 |  |
| DC_1-19_n782 | CA_1-19 | n78 |  |
| DC_1-19_n792 | CA_1-19 | n79 |  |
| DC_1-20_n283 | CA_1-20 | n28 |  |
| DC_1-20_n782 | CA_1-20 | n78 |  |
| DC_1-21_n772 | CA_1-21 | n77 |  |
| DC_1-21_n782 | CA_1-21 | n78 |  |
| DC_1-21_n792 | CA_1-21 | n79 |  |
| DC_1-28_n772 | CA_1-28 | n77 |  |
| DC_1-28_n782 | CA_1-28 | n78 |  |
| DC_1-28_n79 | CA_1-28 | n79 |  |
| DC_1_n28-n782 | 1 | CA_n28-n78 |  |
| DC_1_n77-n79 | 1 | CA_n77-n79 |  |
| DC_1_n78-n79 | 1 | CA_n78-n79 |  |
| DC_1_n40-n78 | 1 | CA_n40-n78 |  |
| DC_1-41_n77 | CA_1-41 | n77 |  |
| DC_1-41_n78 | CA_1-41 | n78 |  |
| DC_1-41_n79 | CA_1-41 | n79 |  |
| DC_1-42_n77 | CA_1-42 | n77 |  |
| DC_1-42_n78 | CA_1-42 | n78 |  |
| DC_1-42_n79 | CA_1-42 | n79 |  |
| DC_1_SUL_n78-n842 | 1 | SUL_n78-n84 |  |
| DC_2-5_n66 | CA_2-5 | n66 |  |
| DC_2-12_n66 | CA_2-12 | n66 |  |
| DC_2-30_n66 | CA_2-30 | n66 |  |
| DC_2-(n)71 | CA_2-71 | n71 |  |
| DC_2-66_n71 | CA_2-66 | n71 |  |
| DC_3_n1-n77 | 3 | CA_n1-n77 |  |
| DC_3_n1-n78 | 3 | CA_n1-n78 |  |
| DC)3_n1-n79 | 3 | CA_n1-n79 |  |
| DC_3_n3-n77 | 3 | CA_n3-n77 |  |
| DC_3_n3-n78 | 3 | CA_n3-n78 |  |
| DC_3-5_n782 | CA_3-5 | n78 |  |
| DC_3-7_n28 | CA_3-7 | n28 |  |
| DC_3-7_n782 | CA_3-7 | n78 |  |
| DC_3-7-7_n782 | CA_3-7-7 | n78 |  |
| DC_3_n7-n782 | 3 | CA_n7-n78 |  |
| DC_3-8_n78 | CA_3-8 | n78 |  |
| DC_3-19_n772 | CA_3-19 | n77 |  |
| DC_3-19_n782 | CA_3-19 | n78 |  |
| DC_3-19_n792 | CA_3-19 | n79 |  |
| DC_3-20_n282,3 | CA_3-20 | n28 |  |
| DC_3-20_n782 | CA_3-20 | n78 |  |
| DC_3_n20-n782 | 3 | CA_n20-n78 |  |
| DC_3-21_n772 | CA_3-21 | n77 |  |
| DC_3-21_n782 | CA_3-21 | n78 |  |
| DC_3-21_n792 | CA_3-21 | n79 |  |
| DC_3-28_n782 | CA_3-28 | n78 |  |
| DC_3_n28-n782 | 3 | CA_n28-n78 |  |
| DC_3-28_n79 | 3 | CA_n28-n79 |  |
| DC_3-38_n78 | CA_3-38 | n78 |  |
| DC_3_n40-n78 | 3 | CA_n40-n78 |  |
| DC_3-41_n78 | CA_3-41 | n78 |  |
| DC_3-42_n77 | CA_3-42 | n77 |  |
| DC_3-42_n78 | CA_3-42 | n78 |  |
| DC_3-42_n79 | CA_3-42 | n79 |  |
| DC_3_n77-n79 | 3 | CA_n77-n79 |  |
| DC_3_n78-n79 | 3 | CA_n78-n79 |  |
| DC_3_SUL_n78-n802 | 3 | SUL_n78-n80 |  |
| DC_3_SUL_n78-n822 | 3 | SUL_n78-n821 |  |
| DC_3_SUL_n79-n802 | 3 | SUL_n79-n80 |  |
| DC_5-7-7_n78 | CA_5-7-7 | n78 |  |
| DC_5-7_n78 | CA_5-7 | n78 |  |
| DC_5-30_n66 | CA_5-30 | n66 |  |
| DC_7-20_n283 | CA_7-20 | n28 |  |
| DC_7-20_n782 | CA_7-20 | n78 |  |
| DC_7-28_n782 | CA_7-28 | n78 |  |
| DC_7_n28-n782 | 7 | CA_n28-n78 |  |
| DC_7-46_n78 | CA_7-46 | n78 |  |
| DC_8_SUL_n78-n812 | 8 | SUL_n78-n81 |  |
| DC_8_SUL_n79-n812 | 8 | SUL_n79-n81 |  |
| DC_12-30_n66 | CA_12-30 | n66 |  |
| DC_18-28_n772 | CA_18-28 | n77 |  |
| DC_18-28_n782 | CA_18-28 | n78 |  |
| DC_18-28_n792 | CA_18-28 | n79 |  |
| DC_19-21_n772 | CA_19-21 | n77 |  |
| DC_19-21_n782 | CA_19-21 | n78 |  |
| DC_19-21_n792 | CA_19-21 | n79 |  |
| DC_19-42_n77 | CA_19-42 | n77 |  |
| DC_19-42_n78 | CA_19-42 | n78 |  |
| DC_19-42_n79 | CA_19-42 | n79 |  |
| DC_19_n77-n79 | 19 | CA_n77-n79 |  |
| DC_19_n78-n79 | 19 | CA_n78-n79 |  |
| DC_20_n8-n75 | 20 | CA_n8-n75 |  |
| DC_20_n28-n753 | 20 | CA_n28-n75 |  |
| DC_20_n28-n782,3 | 20 | CA_n28-n78 |  |
| DC_20_n75-n782 | 20 | CA_n75-n78 |  |
| DC_20_n76-n782 | 20 | CA_n76-n78 |  |
| DC_20_SUL_n78-n822 | 20 | SUL_n78-n82 |  |
| DC_20_SUL_n78-n832 | 20 | SUL_n78-n831 |  |
| DC_21-42_n77 | CA_21-42 | n77 |  |
| DC_21-42_n78 | CA_21-42 | n78 |  |
| DC_21-42_n79 | CA_21-42 | n79 |  |
| DC_21_n77-n79 | 21 | CA_n77-n79 |  |
| DC_21_n78-n79 | 21 | CA_n78-n79 |  |
| DC_28_n8-n78 | 28 | CA_n8-n78 |  |
| DC_28-42_n77 | CA_28-42 | n77 |  |
| DC_28-42_n78 | CA_28-42 | n78 |  |
| DC_28-42_n79 | CA_28-42 | n79 |  |
| DC_41-42_n77 | CA_41-42 | n77 |  |
| DC_41-42_n78 | CA_41-42 | n78 |  |
| DC_41-42_n79 | CA_41-42 | n79 |  |
| DC_28_SUL_n78-n832 | 28 | SUL_n78-n83 |  |
| DC_66_(n)71 | CA_66-71 | n71 |  |
| DC_66_SUL_n78-n862 | 66 | SUL_n78-n86 |  |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

5.2B.4.3	EN-DC (four bands)

Table 5.2B.4.3-1: Band combinations for inter-band EN-DC within FR1 (four bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3-5_n781 | CA_1-3-5 | n78 |  |
| DC_1-3-7_n28 | CA_1-3-7 | n28 |  |
| DC_1-3-7-7_n781 | CA_1-3-7-7 | n78 |  |
| DC_1-3-7_n781 | CA_1-3-7 | n78 |  |
| DC_1-3_n7-n78 | CA_1-3 | CA_n7-n78 |  |
| DC_1-3-8_n781 | CA_1-3-8 | n78 |  |
| DC_1-3-28_n771 | CA_1-3-28 | n77 |  |
| DC_1-3-28_n781 | CA_1-3-28 | n78 |  |
| DC_1-3_n28-n781 | CA_1-3 | CA_n28-n78 |  |
| DC_1-3-28_n791 | CA_1-3-28 | n79 |  |
| DC_1-3-19_n771 | CA_1-3-19 | n77 |  |
| DC_1-3-19_n781 | CA_1-3-19 | n78 |  |
| DC_1-3-19_n791 | CA_1-3-19 | n79 |  |
| DC_1-3-20_n282 | CA_1-3-20 | n28 |  |
| DC_1-3-20_n781 | CA_1-3-20 | n78 |  |
| DC_1-3-21_n771 | CA_1-3-21 | n77 |  |
| DC_1-3-21_n781 | CA_1-3-21 | n78 |  |
| DC_1-3-21_n791 | CA_1-3-21 | n79 |  |
| DC_1-3-42_n77 | CA_1-3-42 | n77 |  |
| DC_1-3-42_n78 | CA_1-3-42 | n78 |  |
| DC_1-3-42_n79 | CA_1-3-42 | n79 |  |
| DC_1-5-7_n78 | CA_1-5-7 | n78 |  |
| DC_1-5-7-7_n78 | CA_1-5-7-7 | n78 |  |
| DC_1-7-20_n282 | CA_1-7-20 | n28 |  |
| DC_1-7-20_n781 | CA_1-7-20 | n78 |  |
| DC_1-7_n28-n781 | CA_1-7 | CA_n28-n78 |  |
| DC_1-18-28_n77 | CA_1-18-28 | n77 |  |
| DC_1-18-28_n78 | CA_1-18-28 | n78 |  |
| DC_1-18-28_n791 | CA_1-18-28 | n79 |  |
| DC_1-19-21_n77 | CA_1-19-21 | n77 |  |
| DC_1-19-21_n78 | CA_1-19-21 | n78 |  |
| DC_1-19-21_n79 | CA_1-19-21 | n79 |  |
| DC_1-19-42_n77 | CA_1-19-42 | n77 |  |
| DC_1-19-42_n78 | CA_1-19-42 | n78 |  |
| DC_1-19-42_n79 | CA_1-19-42 | n79 |  |
| DC_1-20_n28-n781,2 | CA_1-20 | CA_n28-n78 |  |
| DC_1-21-28_n771 | CA_1-21-28 | n77 |  |
| DC_1-21-28_n781 | CA_1-21-28 | n78 |  |
| DC_1-21-28_n791 | CA_1-21-28 | n79 |  |
| DC_1-21-42_n77 | CA_1-21-42 | n77 |  |
| DC_1-21-42_n78 | CA_1-21-42 | n78 |  |
| DC_1-21-42_n79 | CA_1-21-42 | n79 |  |
| DC_1-28-42_n77 | CA_1-28-42 | n77 |  |
| DC_1-28-42_n78 | CA_1-28-42 | n78 |  |
| DC_1-28-42_n79 | CA_1-28-42 | n79 |  |
| DC_1-41-42_n77 | CA_1-41-42 | n77 |  |
| DC_1-41-42_n78 | CA_1-41-42 | n78 |  |
| DC_1-41-42-n79 | CA_1-41-42 | n79 |  |
| DC_2-66-(n)71 | CA_2-66-71 | n71 |  |
| DC_3-5-7_n78 | CA_3-5-7 | n78 |  |
| DC_3-5-7-7_n78 | CA_3-5-7-7 | n78 |  |
| DC_3-7-20_n282 | CA_3-7-20 | n28 |  |
| DC_3-7-20_n781 | CA_3-7-20 | n78 |  |
| DC_3-7-28_n781 | CA_3-7-28 | n78 |  |
| DC_3-7_n28-n781 | CA_3-7 | CA_n28-n78 |  |
| DC_3-19-21_n771 | CA_3-19-21 | n77 |  |
| DC_3-19-21_n781 | CA_3-19-21 | n78 |  |
| DC_3-19-21_n791 | CA_3-19-21 | n79 |  |
| DC_3-19-42_n77 | CA_3-19-42 | n77 |  |
| DC_3-19-42_n78 | CA_3-19-42 | n78 |  |
| DC_3-19-42_n791 | CA_3-19-42 | n79 |  |
| DC_3-20_n28-n781,2 | CA_3-20 | CA_n28-n78 |  |
| DC_3-21-42_n77 | CA_3-21-42 | n77 |  |
| DC_3-21-42_n78 | CA_3-21-42 | n78 |  |
| DC_3-21-42_n79 | CA_3-21-42 | n79 |  |
| DC_3-28-42_n77 | CA_3-28-42 | n77 |  |
| DC_3-28-42_n78 | CA_3-28-42 | n78 |  |
| DC_3-28-42_n79 | CA_3-28-42 | n79 |  |
| DC_7-20_n28-n781,2 | CA_7-20 | CA_n28-n78 |  |
| DC_19-21-42_n77 | CA_19-21-42 | n77 |  |
| DC_19-21-42_n78 | CA_19-21-42 | n78 |  |
| DC_19-21-42_n79 | CA_19-21-42 | n79 |  |
| DC_21-28-42_n77 | CA_21-28-42 | n77 |  |
| DC_21-28-42_n78 | CA_21-28-42 | n78 |  |
| DC_21-28-42_n79 | CA_21-28-42 | n79 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

5.2B.4.4	EN-DC (five bands)

Table 5.2B.4.4-1: Band combinations inter-band EN-DC within FR1 (five bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3-5-7_n78 | CA_1-3-5-7 | n78 |  |
| DC_1-3-5-7-7_n78 | CA_1-3-5-7-7 | n78 |  |
| DC_1-3-7-20_n282 | CA_1-3-7-20 | n28 |  |
| DC_1-3-7-20_n781 | CA_1-3-7-20 | n78 |  |
| DC_1-3-7_n28-n781 | CA_1-3-7 | CA_n28-n78 |  |
| DC_1-3-19-21_n771 | CA_1-3-19-21 | n77 |  |
| DC_1-3-19-21_n781 | CA_1-3-19-21 | n78 |  |
| DC_1-3-19-21_n791 | CA_1-3-19-21 | n79 |  |
| DC_1-3-19-42_n77 | CA_1-3-19-42 | n77 |  |
| DC_1-3-19-42_n78 | CA_1-3-19-42 | n78 |  |
| DC_1-3-19-42_n79 | CA_1-3-19-42 | n79 |  |
| DC_1-3-20_n28-n781,2 | CA_1-3-20 | CA_n28-n78 |  |
| DC_1-3-21-42_n77 | CA_1-3-21-42 | n77 |  |
| DC_1-3-21-42_n78 | CA_1-3-21-42 | n78 |  |
| DC_1-3-21-42_n79 | CA_1-3-21-42 | n79 |  |
| DC_1-7-20_n28-n781,2 | CA_1-7-20 | CA_n28-n78 |  |
| DC_1-19-21-42_n77 | DC_1-19-21-42 | n77 |  |
| DC_1-19-21-42_n78 | DC_1-19-21-42 | n78 |  |
| DC_1-19-21-42_n79 | DC_1-19-21-42 | n79 |  |
| DC_1-3-5-7_n78 | CA_1-3-5-7 | n78 |  |
| DC_1-3-28-42_n77 | CA_1-3-28-42 | n77 |  |
| DC_1-3-28-42_n78 | CA_1-3-28-42 | n78 |  |
| DC_1-3-28-42_n79 | CA_1-3-28-42 | n79 |  |
| DC_1-21-28-42_n77 | CA_1-21-28-42 | n77 |  |
| DC_1-21-28-42_n78 | CA_1-21-28-42 | n78 |  |
| DC_1-21-28-42_n79 | CA_1-21-28-42 | n79 |  |
| DC_3-7-20_n28-n781,2 | CA_3-7-20 | CA_n28-n78 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

5.2B.4.5	EN-DC (six bands)

Table 5.2B.4.5-1: Band combinations for inter-band EN-DC within FR1 (six bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3-7-20_n28-n781,2 | CA_1-3-7-20 | CA_n28-n78 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

5.2B.5	Inter-band EN-DC including FR2

5.2B.5.1	EN-DC (two bands)

Table 5.2B.5.1-1: Band combinations for inter-band EN-DC including FR2 (two bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1_n257 | 1 | n257 |  |
| DC_2-2_n257 | CA_2-2 | n257 |  |
| DC_2_n257 | CA_2 | n257 |  |
| DC_2_n260 | 2 | n260 |  |
| DC_2_n260 | CA_2 | n260 |  |
| DC_2-2_n260 | CA_2-2 | n260 |  |
| DC_3_n257 | 3 | n257 |  |
| DC_3_n258 | 3 | n258 |  |
| DC_5_n257 | 5 | n257 |  |
| DC_5-5_n257 | CA_5-5 | n257 |  |
| DC_5-5_n260 | CA_5-5 | n260 |  |
| DC_5_n260 | 5 | n260 |  |
| DC_5_n261 | 5 | n261 |  |
| DC_7-7_n257 | CA_7-7 | n257 |  |
| DC_7_n257 | 7 | n257 |  |
| DC_7_n258 | 7 | n258 |  |
| DC_8_n257 | 8 | n257 |  |
| DC_8_n258 | 8 | n258 |  |
| DC_11_n257 | 11 | n257 |  |
| DC_12_n260 | 12 | n260 |  |
| DC_13_n257 | 13 | n257 |  |
| DC_13_n260 | 13 | n260 |  |
| DC_18_n257 | 18 | n257 |  |
| DC_19_n257 | 19 | n257 |  |
| DC_20_n258 | 20 | n258 |  |
| DC_21_n257 | 21 | n257 |  |
| DC_26_n257 | 26 | n257 |  |
| DC_28_n257 | 28 | n257 |  |
| DC_28_n258 | 28 | n258 |  |
| DC_30_n260 | 30 | n260 |  |
| DC_39_n258 | 39 | n258 |  |
| DC_41_n257 | 41 | n257 |  |
| DC_41_n258 | 41 | n258 |  |
| DC_42_n257 | 42 | n257 |  |
| DC_48-48_n257 | CA_48-48 | n257 |  |
| DC_48_n257 | CA_48 | n257 |  |
| DC_48-48_n260 | CA_48-48 | n260 |  |
| DC_48_n260 | CA_48 | n260 |  |
| DC_66-66_n257 | CA_66-66 | n257 |  |
| DC_66_n257 | 66 | n257 |  |
| DC_66-66_n260 | CA_66-66 | n260 |  |
| DC_66_n260 | 66 | n260 |  |
| DC_66_n261 | 66 | n261 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |  |  |

5.2B.5.2	EN-DC (three bands)

Table 5.2B.5.2-1: Band combinations inter-band EN-DC including FR2 (three bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3_n2571 | CA_1-3 | n257 |  |
| DC_1-5_n2571 | CA_1-5 | n257 |  |
| DC_1-7_n2571 | CA_1-7 | n257 |  |
| DC_1-7-7_n2571 | CA_1-7-7 | n257 |  |
| DC_1-8_n257 | CA_1-8 | n257 |  |
| DC_1-18_n2571 | CA_1-18 | n257 |  |
| DC_1-19_n2571 | CA_1-19 | n257 |  |
| DC_1-21_n2571 | CA_1-21 | n257 |  |
| DC_1-28_n2571 | CA_1-28 | n257 |  |
| DC_1-41_n257 | CA_1-41 | n257 |  |
| DC_1-42_n257 | CA_1-42 | n257 |  |
| DC_2-5_n2571 | CA_2-5 | n257 |  |
| DC_2-5_n260 | CA_2-5 | n260 |  |
| DC_2-12_n260 | CA_2-12 | n260 |  |
| DC_2-13_n2571 | CA_2-13 | n257 |  |
| DC_2-13_n2601 | CA_2-13 | n260 |  |
| DC_2-30_n260 | CA_2-30 | n260 |  |
| DC_2-66_n2571 | CA_2-66 | n257 |  |
| DC_2-66_n260 | CA_2-66 | n260 |  |
| DC_3-5_n2571 | CA_3-5 | n257 |  |
| DC_3-7_n2571 | CA_3-7 | n257 |  |
| DC_3-7-7_n2571 | CA_3-7-7 | n257 |  |
| DC_3-19_n2571 | CA_3-19 | n257 |  |
| DC_3-21_n2571 | CA_3-21 | n257 |  |
| DC_3-28_n2571 | CA_3-28 | n257 |  |
| DC_3-41_n257 | CA_3-41 | n257 |  |
| DC_3-42_n2571 | CA_3-42 | n257 |  |
| DC_5-7-7_n2571 | CA_5-7-7 | n257 |  |
| DC_5-7_n2571 | CA_5-7 | n257 |  |
| DC_5-30_n260 | CA_5-30 | n260 |  |
| DC_5-66_n260 | CA_5-66 | n260 |  |
| DC_12-30_n260 | CA_12-30 | n260 |  |
| DC_12-66_n260 | CA_12-66 | n260 |  |
| DC_13-66_n2571 | CA_13-66 | n257 |  |
| DC_13-66_n2601 | CA_13-66 | n260 |  |
| DC_18-28_n2571 | CA_18-28 | n257 |  |
| DC_19-21_n2571 | CA_19-21 | n257 |  |
| DC_19-42_n2571 | CA_19-42 | n257 |  |
| DC_21-42_n2571 | CA_21-42 | n257 |  |
| DC_21-28_n2571 | CA_21-28 | n257 |  |
| DC_28-42_n2571 | CA_28-42 | n257 |  |
| DC_30-66_n260 | CA_30-66 | n260 |  |
| DC_41-42_n257 | CA_41-42 | n257 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

5.2B.5.3	EN-DC (four bands)

Table 5.2B.5.3-1: Band combinations inter-band EN-DC including FR2 (four bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3-5_n2571 | CA_1-3-5 | n257 |  |
| DC_1-3-7_n2571 | CA_1-3-7 | n257 |  |
| DC_1-3-7-7_n257 | CA_1-3-7-7 | n257 |  |
| DC_1-3-19_n2571 | CA_1-3-19 | n257 |  |
| DC_1-3-21_n2571 | CA_1-3-21 | n257 |  |
| DC_1-3-28_n2571 | CA_1-3-28 | n257 |  |
| DC_1-3-42_n257 | CA_1-3-42 | n257 |  |
| DC_1-5-7_n2571 | CA_1-5-7 | n257 |  |
| DC_1-5-7-7_n257 | CA_1-5-7-7 | n257 |  |
| DC_1-18-28_n2571 | CA_1-18-28 | n257 |  |
| DC_1-19-21_n257 | CA_1-19-21 | n257 |  |
| DC_1-19-42_n257 | CA_1-19-42 | n257 |  |
| DC_1-21-28_n2571 | CA_1-21-28 | n257 |  |
| DC_1-21-42_n257 | CA_1-21-42 | n257 |  |
| DC_1-28-42_n257 | CA_1-28-42 | n257 |  |
| DC_1-41-42_n257 | CA_1-41-42 | n257 |  |
| DC_3-5-7-7_n257 | CA_3-5-7-7 | n257 |  |
| DC_3-5-7_n2571 | CA_3-5-7 | n257 |  |
| DC_3-19-21_n2571 | CA_3-19-21 | n257 |  |
| DC_3-19-42_n257 | CA_3-19-42 | n257 |  |
| DC_3-21-42_n257 | CA_3-21-42 | n257 |  |
| DC_3-28-42_n257 | CA_3-28-42 | n257 |  |
| DC_19-21-42_n2571 | CA_19-21-42 | n257 |  |
| DC_21-28-42_n2571 | CA_21-28-42 | n257 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

5.2B.5.4	EN-DC (five bands)

Table 5.2B.5.4-1: Band combinations inter-band EN-DC including FR2 (five bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3-5-7_n2571 | CA_1-3-5-7 | n257 |  |
| DC_1-3-5-7-7_n2571 | CA_1-3-5-7-7 | n257 |  |
| DC_1-3-19-21_n2571 | CA_1-3-19-21 | n257 |  |
| DC_1-3-19-42_n257 | CA_1-3-19-42 | n257 |  |
| DC_1-3-21-42_n257 | CA_1-3-21-42 | n257 |  |
| DC_1-3-28-42_n257 | CA_1-3-28-42 | n257 |  |
| DC_1-19-21-42_n257 | DC_1-19-21-42 | n257 |  |
| DC_1-21-28-42_n257 | DC_1-21-28-42 | n257 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

5.2B.6	Inter-band EN-DC including both FR1 and FR2

5.2B.6.1	Void

5.2B.6.2	EN-DC (three bands)

Table 5.2B.6.2-1: Band combinations inter-band EN-DC including both FR1 and FR2 (three bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1_n77-n257 | 1 | CA_n77-n257 |  |
| DC_1_n78-n257 | 1 | CA_n78-n257 |  |
| DC_1_n79-n257 | 1 | CA_n79-n257 |  |
| DC_3_n1-n257 | 3 | CA_n1-n257 |  |
| DC_3_n77-n257 | 3 | CA_n77-n257 |  |
| DC_3_n78-n257 | 3 | CA_n78-n257 |  |
| DC_3_n79-n257 | 3 | CA_n79-n257 |  |
| DC_5_n78-n2571 | 5 | CA_n78-n257 |  |
| DC_7-7_n78-n257 | CA_7-7 | CA_n78-n257 |  |
| DC_7_n78-n257 | 7 | CA_n78-n257 |  |
| DC_8_n77-n257 | 8 | CA_n77-n257 |  |
| DC_19_n77-n257 | 19 | CA_n77-n257 |  |
| DC_19_n78-n257 | 19 | CA_n78-n257 |  |
| DC_19_n79-n257 | 19 | CA_n79-n257 |  |
| DC_21_n77-n257 | 21 | CA_n77-n257 |  |
| DC_21_n78-n257 | 21 | CA_n78-n257 |  |
| DC_21_n79-n257 | 21 | CA_n79-n257 |  |
| DC_28_n8-n258 | 28 | CA_n8-n258 |  |
| DC_66_n5-n260 | 66 | CA_n5-n260 |  |
| DC_66_n71-n260 | 66 | CA_n71-n260 |  |
| DC_66_n71-n261 | 66 | CA_n71-n261 |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

5.2B.6.3	EN-DC (four bands)

Table 5.2B.6.3-1: Band combinations inter-band EN-DC including both FR1 and FR2 (four bands)

| EN-DC Band | E-UTRA Band | NR Band |  |
| --- | --- | --- | --- |
| DC_1-3_n78-n257 | CA_1-3 | CA_n78-n257 |  |
| DC_1-5_n78-n257 | CA_1-5 | CA_n78-n257 |  |
| DC_1-7-7_n78-n257 | CA_1-7-7 | CA_n78-n257 |  |
| DC_1-7_n78-n257 | CA_1-7 | CA_n78-n257 |  |
| DC_3-5_n78-n257 | CA_3-5 | CA_n78-n257 |  |
| DC_3-7-7_n78-n257 | CA_3-7-7 | CA_n78-n257 |  |
| DC_3-7_n78-n257 | CA_3-7 | CA_n78-n257 |  |
| DC_5-7-7_n78-n257 | CA_5-7-7 | CA_n78-n257 |  |
| DC_5-7_n78-n257 | CA_5-7 | CA_n78-n257 |  |

5.2B.6.4	EN-DC (five bands)

Table 5.2B.6.4-1: Band combinations inter-band EN-DC including both FR1 and FR2 (five bands)

| EN-DC Band | E-UTRA Band | NR Band |
| --- | --- | --- |
| DC_1-3-5_n78-n257 | CA_1-3-5 | CA_n78-n257 |
| DC_1-3-7-7_n78-n257 | CA_1-3-7-7 | CA_n78-n257 |
| DC_1-3-7_n78-n257 | CA_1-3-7 | CA_n78-n257 |
| DC_1-5-7-7_n78-n257 | CA_1-5-7-7 | CA_n78-n257 |
| DC_1-5-7_n78-n257 | CA_1-5-7 | CA_n78-n257 |
| DC_3-5-7-7_n78-n257 | CA_3-5-7-7 | CA_n78-n257 |
| DC_3-5-7_n78-n257 | CA_3-5-7 | CA_n78-n257 |

5.2B.6.5	EN-DC (six bands)

Table 5.2B.6.5-1: Band combinations inter-band EN-DC including both FR1 and FR2 (six bands)

| EN-DC Band | E-UTRA Band | NR Band |
| --- | --- | --- |
| DC_1-3-5-7_n78-n257 | CA_1-3-5-7 | CA_n78-n257 |
| DC_1-3-5-7-7_n78-n257 | CA_1-3-5-7-7 | CA_n78-n257 |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |

<Unchanged section are omitted>

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n28A | DC_1A_n28A | 1A | n28A |
| DC_1A_n40A | DC_1A_n40A | 1A | n40A |
| DC_1A_n51A | DC_1A_n51A | 1A | n51A |
| DC_1A_n77A DC_1A_n77C | DC_1A_n77A | 1A | n77A CA_n77C |
| DC_1A_n78A DC_1A_n78C | DC_1A_n78A | 1A | n78A CA_n78C |
| DC_1A_n79A DC_1A_n79C | DC_1A_n79A | 1A | n79A CA_n79C |
| DC_2A_n5A | DC_2A_n5A | 2A | n5A |
| DC_2A_n66A | DC_2A_n66A | 2A | n66A |
| DC_2A_n71A | DC_2A_n71A | 2A | n71A |
| DC_2A_n78A | DC_2A_n78A | 2A | n78A |
| DC_3A_n7A | DC_3A_n7A | 3A | n7A |
| DC_3A_n28A | DC_3A_n28A | 3A | n28A |
| DC_3A_n40A | DC_3A_n40A | 3A | n40A |
| DC_3A_n51A | DC_3A_n51A | 3A | n51A |
| DC_3A_n77A DC_3A_n77C | DC_3A_n77A | 3A | n77A CA_n77C |
| DC_3A_n78A DC_3A_n78C | DC_3A_n78A | 3A | n78A CA_n78C |
| DC_3A_n79A DC_3A_n79C | DC_3A_n79A | 3A | n79A CA_n79C |
| DC_3C_n78A | DC_3A_n78A | CA_3C | n78A |
| DC_5A_n40A | DC_5A_n40A | 5A | n40A |
| DC_5A_n66A | DC_5A_n66A | 5A | n66A |
| DC_5A_n78A | DC_5A_n78A | 5A | n78A |
| DC_7A-7A_n78A | DC_7A_n78A | CA_7A-7A | n78A |
| DC_7A_n28A | DC_7A_n28A | 7A | n28A |
| DC_7A_n51A | DC_7A_n51A | 7A | n51A |
| DC_7A_n78A | DC_7A_n78A | 7A | n78A |
| DC_7C_n78A | DC_7C_n78A | CA_7C | n78A |
| DC_8A_n40A | DC_8A_n40A | 8A | n40A |
| DC_8A_n77A | DC_8A_n77A | 8A | n77A |
| DC_8A_n78A | DC_8A_n78A | 8A | n78A |
| DC_8A_n79A | DC_8A_n79A | 8A | n79A |
| DC_11A_n77A | DC_11A_n77A | 11A | n77A |
| DC_11A_n78A | DC_11A_n78A | 11A | n78A |
| DC_11A_n79A | DC_11A_n79A | 11A | n79A |
| DC_12A_n5A | DC_12A_n5A | 12A | n5A |
| DC_12A_n66A | DC_12A_n66A | 12A | n66A |
| DC_18A_n77A | DC_18A_n77A | 18A | n77A |
| DC_18A_n78A | DC_18A_n78A | 18A | n78A |
| DC_18A_n79A | DC_18A_n79A | 18A | n79A |
| DC_19A_n77A DC_19A_n77C | DC_19A_n77A | 19A | n77A CA_n77C |
| DC_19A_n78A DC_19A_n78C | DC_19A_n78A | 19A | n78A CA_n78C |
| DC_19A_n79A DC_19A_n79C | DC_19A_n79A | 19A | n79A CA_n79C |
| DC_20A_n8A | DC_20A_n8A | 20A | n8A |
| DC_20A_n28A4 | DC_20A_n28A | 20A | n28A |
| DC_20A_n51A | DC_20A_n51A | 20A | n51A |
| DC_20A_n77A | DC_20A_n77A | 20A | n77A |
| DC_20A_n78A | DC_20A_n78A | 20A | n78A |
| DC_21A_n77A DC_21A_n77C | DC_21A_n77A | 21A | n77A CA_n77C |
| DC_21A_n78A DC_21A_n78C | DC_21A_n78A | 21A | n78A CA_n78C |
| DC_21A_n79A DC_21A_n79C | DC_21A_n79A | 21A | n79A CA_n79C |
| DC_25A_n41A | DC_25A_n41A | 25A | n41A |
| DC_26A_n41A | DC_26A_n41A | 26A | n41A |
| DC_26A_n77A | DC_26A_n77A | 26A | n77A |
| DC_26A_n78A | DC_26A_n78A | 26A | n78A |
| DC_26A_n79A | DC_26A_n79A | 26A | n79A |
| DC_28A n51A | DC_28A_n51A | 28A | n51A |
| DC_28A_n77A DC_28A_n77C | DC_28A_n77A | 28A | n77A CA_n77C |
| DC_28A_n78A DC_28A_n78C | DC_28A_n78A | 28A | n78A CA_n78C |
| DC_28A_n79A DC_28A_n79C | DC_28A_n79A | 28A | n79A CA_n79C |
| DC_30A_n5A | DC_30A_n5A | 30A | n5A |
| DC_30A_n66A | DC_30A_n66A | 30A | n66A |
| DC_38A_n78A | N/A | 38A | n78A |
| DC_39A_n78A | DC_39A_n78A | 39A | n78A |
| DC_39A_n79A | DC_39A_n79A | 39A | n79A |
| DC_40A_n77A | N/A | 40A | n77A |
| DC_41A_n77A | DC_41A_n77A | 41A | n77A |
| DC_41A_n78A | DC_41A_n78A | 41A | n78A |
| DC_41A_n79A | DC_41A_n79A | 41A | n79A |
| DC_41C_n77A | DC_41C_n77A | CA_41C | n77A |
| DC_41C_n78A | DC_41C_n78A | CA_41C | n78A |
| DC_41C_n79A | DC_41C_n79A | CA_41C | n79A |
| DC_42A_n51A | DC_42A_n51A | 42A | n51A |
| DC_42A_n77A3,4 DC_42A_n77C3,4 | N/A | 42A | n77A CA_n77C |
| DC_42A_n78A3,4 DC_42A_n78C3,4 | N/A | 42A | n78A CA_n78C |
| DC_42A_n79A DC_42A_n79C | N/A | 42A | n79A CA_n79C |
| DC_42C_n77A3,4 | N/A | CA_42C | n77A |
| DC_42C_n78A3,4 | N/A | CA_42C | n78A |
| DC_42C_n79A | N/A | CA_42C | n79A |
| DC_42C_n77C3,4 | N/A | CA_42C | CA n77C |
| DC_42C_n78C3,4 | N/A | CA_42C | CA n78C |
| DC_42C_n79C | N/A | CA_42C | CA n79C |
| DC_42D_n77A3,4 | N/A | CA_42D | n77A |
| DC_42D_n78A3,4 | N/A | CA_42D | n78A |
| DC_42D_n79A | N/A | CA_42D | n79A |
| DC_42E_n77A3,4 | N/A | CA_42E | n77A |
| DC_42E_n78A3,4 | N/A | CA_42E | n78A |
| DC_42E_n79A | N/A | CA_42E | n79A |
| DC_46A_n78A2 | N/A | 46A | n78A |
| DC_46C_n78A2 | N/A | CA_46C | n78A |
| DC_46D_n78A2 | N/A | CA_46D | n78A |
| DC_46E_n78A2 | N/A | CA_46E | n78A |
| DC_66A_n5A | DC_66A_n5A | 66A | n5A |
| DC_66A_n71A | DC_66A_n71A | 66A | n71A |
| DC_66A_n78A | DC_66A_n78A | 66A | n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC cong\figuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within [6] dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A | CA_1A-3A | n28A |
| DC_1A-3A_n77A DC_1A-3A_n77C | DC_1A_n77A DC_3A_n77A | CA_1A-3A | n77A CA_n77C |
| DC_1A-3A_n78A DC_1A-3A_n78C | DC_1A_n78A DC_3A_n78A | CA_1A-3A | n78A CA_n78C |
| DC_1A-3A_n79A DC_1A-3A_n79C | DC_1A_n79A DC_3A_n79A | CA_1A-3A | n79A CA_n79C |
| DC_1A-3C_n78A | DC_1A_n78A DC_3A_n78A |  | CA_1A-3C |  | n78A |  |
| DC_1A_n3A-n78A | DC_1A_n3A DC_1A_n78A | 1A |  | CA_n3A-n78A |  |
| DC_1A-5A_n78A | DC_1A_n78A DC_5A_n78A | CA_1A-5A | n78A |
| DC_1A-7A_n28A | DC_1A_n28A DC_7A_n28A | CA_1A-7A | n28A |
| DC_1A-7A_n78A | DC_1A_n78A DC_7A_n78A | CA_1A-7A | n78A |
| DC_1A-7A-7A_n78A | DC_1A_n78A DC_7A_n78A | CA_1A-7A-7A | n78A |
| DC_1A_n7A-n78A | DC_1A_n7A DC_1A_n78A | 1A | CA_n7A-n78A |
| DC_1A-8A_n78A | DC_1A_n78A DC_8A_n78A | CA_1A-8A | n78A |
| DC_1A-18A_n77A | DC_1A_n77A DC_18A_n77A | CA_1A-18A | n77A |
| DC_1A-18A_n78A | DC_1A_n78A DC_18A_n78A | CA_1A-18A | n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A | CA_1A-18A | n79A |
| DC_1A-19A_n77A DC_1A-19A_n77C | DC_1A_n77A DC 19A_n77A | CA_1A-19A | n77A CA_n77C |
| DC_1A-19A_n78A DC_1A-19A_n78C | DC_1A_n78A DC_19A_n78A | CA_1A-19A | n78A CA_n78C |
| DC_1A-19A_n79A DC_1A-19A_n79C | DC_1A_n79A DC_19A_n79A | CA_1A-19A | n79A CA_n79C |
| DC_1A-19A_n77A | DC_1A_n77A DC 19A_n77A | CA_1A-19A | n77A |
| DC_1A-19A_n78A | DC_1A_n78A DC_19A_n78A | CA_1A-19A | n78A |
| DC_1A-19A_n79A | DC_1A_n79A DC_19A_n79A | CA_1A-19A | n79A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A | CA_1A-20A | n28A |
| DC_1A-20A_n78A | DC_1A_n78A DC_20A_n78A | CA_1A-20A | n78A |
| DC_1A-21A_n77A DC_1A-21A_n77C | DC_1A_n77A DC_21A_n77A | CA_1A-21A | n77A CA_n77C |
| DC_1A-21A_n78A DC_1A-21A_n78C | DC_1A_n78A DC_21A_n78A | CA_1A-21A | n78A CA_n78C |
| DC_1A-21A_n79A DC_1A-21A_n79C | DC_1A_n79A DC_21A_n79A | CA_1A-21A | n79A CA_n79C |
| DC_1A-21A_n77A | DC_1A_n77A DC_21A_n77A | CA_1A-21A | n77A |
| DC_1A-21A_n78A | DC_1A_n78A DC_21A_n78A | CA_1A-21A | n78A |
| DC_1A-21A_n79A | DC_1A_n79A DC_21A_n79A | CA_1A-21A | n79A |
| DC_1A-28A_n77A DC_1A-28A_n77C | DC_1A_n77A DC_28A_n77A | CA_1A-28A | n77A CA_n77C |
| DC_1A-28A_n78A DC_1A-28A_n78C | DC_1A_n78A DC_28A_n78A | CA_1A-28A | n78A CA_n78C |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A | CA_1A-28A | n79A CA_n79C |
| DC_1A_n28A-n78A | DC_1A_n28A, DC_1A_n78A | 1A | CA_n28A-n78A |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A | 1A | CA_n40A-n78A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A | CA_1A-41A CA_1A-41C | n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A | CA_1A-41A CA_1A-41C | n78A |
| DC_1A-41C_n79A | DC_1A_n79A DC_41C_n79A | CA_1A-41C | n79A |
| DC_1A-42A_n77A DC_1A-42A_n77C | DC_1A_n77A |  | CA_1A-42A |  | n77A CA_n77C |  |
| DC_1A-42A_n78A DC_1A-42A_n78C | DC_1A_n78A |  | CA_1A-42A |  | n78A CA_n78C |  |
| DC_1A-42A_n79A DC_1A-42A_n79C | DC_1A_n79A |  | CA_1A-42A |  | n79A CA_n79C |  |
| DC_1A-42C_n77A DC_1A-42C_n77C | DC_1A_n77A | CA_1A-42C | n77A CA_n77C |
| DC_1A-42C_n78A DC_1A-42C_n78C | DC_1A_n78A | CA_1A-42C | n78A CA_n78C |
| DC_1A-42C_n79A DC_1A-42C_n79C | DC_1A_n79A | CA_1A-42C | n79A CA_n79C |
| DC_1A-42D_n77A | DC_1A_n77A | CA_1A-42D | n77A |
| DC_1A-42D_n78A | DC_1A_n78A | CA_1A-42D | n78A |
| DC_1A-42D_n79A | DC_1A_n79A | CA_1A-42D | n79A |
| DC_1A-42E_n77A | DC_1A_n77A | CA_1A-42E | n77A |
| DC_1A-42E_n78A | DC_1A_n78A | CA_1A-42E | n78A |
| DC_1A-42E_n79A | DC_1A_n79A | CA_1A-42E | n79A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1A | CA_n77A-n79A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1A | CA_n78A-n79A |
| DC_1A_SUL_n78A-n84A | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A, DC_1A_n84A_ULSUP-FDM_n78A | 1A | SUL_n78A-n84A |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5A | n66A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A | CA_2A-12A | n66A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-30A | n66A |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A | CA_2A-66A | n71A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA | CA_2A-71A | n71A |
| DC_3A_n1A-n77A | DC_3A_n1A DC_3A_n77A | 3A | CA_n1A-n77A |
| DC_3A_n1A-n78A | DC_3A_n1A DC_3A_n78A | 3A | CA_n1A-n78A |
| DC_3A_n1A-n79A | DC_3A_n1A DC_3A_n79A | 3A | CA_n1A-n79A |
| DC_3A_n3A-n77A | DC_3A_n77A DC_3A_n3A2 |  | 3A |  | CA_n3A-n77A |  |
| DC_3A_n3A-n78A | DC_3A_n78A DC_3A_n3A2 |  | 3A |  | CA_n3A-n78A |  |
| DC_3A-5A_n78A | DC_3A_n78A DC_5A_n78A | CA_3A-5A | n78A |
| DC_3A-7A-7A_n78A | DC_3A_n78A DC_7A_n78A | CA_3A-7A-7A | n78A |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A | CA_3A-7A | n28A |
| DC_3A-7A_n78A | DC_3A_n78A DC_7A_n78A | CA_3A-7A | n78A |
| DC_3A-7C_n78A | DC_3A_n78A DC_7C_n78A | CA_3A-7C | n78A |
| DC_3C-7C_n78A | DC_3A_n78A DC_7C_n78A | CA_3C-7C | n78A |
| DC_3C-7A_n78A | DC_3A_n78A DC_7A_n78A |  | CA_3C-7A |  | n78A |  |
| DC_3A_n7A-n78A DC_3C_n7A-n78A | DC_3A_n7A DC_3A_n78A | 3A, CA_3C |  | CA_n7A-n78A |  |
| DC_3A-8A_n78A | DC_3A_n78A DC_8A_n78A | CA_3A-8A | n78A |
| DC_3A-19A_n77A DC_3A-19A_n77C | DC_3A_n77A DC_19A_n77A | CA_3A-19A | n77A CA_n77C |
| DC_3A-19A_n78A DC_3A-19A_n78C | DC_3A_n78A DC_19A_n78A | CA_3A-19A | n78A CA_n78C |
| DC_3A-19A_n79A DC_3A-19A_n79C | DC_3A_n79A DC_19A_n79A | CA_3A-19A | n79A CA_n79C |
| DC_3A-20A_n28A | DC_3A_n28A DC_20A_n28A | CA_3A-20A | n28A |
| DC_3A-20A_n78A | DC_3A_n78A DC_20A_n78A | CA_3A-20A | n78A |
| DC_3C-20A_n78A | DC_3A_n78A DC_20A_n78A |  | CA_3C-20A |  | n78A |  |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A | 3A | CA_n20A-n78A |
| DC_3A-21A_n77A DC_3A-21A_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A | n77A CA_n77C |
| DC_3A-21A_n78A DC_3A-21A_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A | n78A CA_n78C |
| DC_3A-21A_n79A DC_3A-21A_n79C | DC_3A_n79A DC_21A_n79A | CA_3A-21A | n79A CA_n79C |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A | CA_3A-28A | n77A CA_n77C |
| DC_3A-28A_n78A DC_3A-28A_n78C | DC_3A_n78A DC_28A_n78A | CA_3A-28A | n78A CA_n78C |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A | CA_3A-28A | n79A CA_n79C |
| DC_3A_n28A-n78A | DC_3A_n28A, DC_3A_n78A | 3A | CA_n28A-n78A |
| DC_3C_n28A-n78A | DC_3A_n28A, DC_3A_n78A | 3A, CA_3C | CA_n28A-n78A |
| DC_3A-38A_n78A | DC_3A_n78A | CA_3A-38A | n78A |
| DC_3A-41A_n78A | DC_3A_n78A DC_41A_n78A | CA_3A-41A | n78A |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A | 3A | CA_n40A-n78A |
| DC_3A-42A_n77A DC_3A-42A_n77C | DC_3A_n77A | CA_3A-42A | n77A CA_n77C |
| DC_3A-42A_n78A DC_3A-42A_n78C | DC_3A_n78A | CA_3A-42A | n78A CA_n78C |
| DC_3A-42A_n79A DC_3A-42A_n79C | DC_3A_n79A | CA_3A-42A | n79A CA_n79C |
| DC_3A-42C_n77A DC_3A-42C_n77C | DC_3A_n77A | CA_3A-42C | n77A CA_n77C |
| DC_3A-42C_n78A DC_3A-42C_n78C | DC_3A_n78A | CA_3A-42C | n78A CA_n78C |
| DC_3A-42C_n79A DC_3A-42C_n79C | DC_3A_n79A | CA_3A-42C | n79A CA_n79C |
| DC_3A-42D_n77A | DC_3A_n77A | CA_3A-42D | n77A |
| DC_3A-42D_n78A | DC_3A_n78A | CA_3A-42D | n78A |
| DC_3A-42D_n79A | DC_3A_n79A | CA_3A-42D | n79A |
| DC_3A-42E_n77A | DC_3A_n77A | CA_3A-42E | n77A |
| DC_3A-42E_n78A | DC_3A_n78A | CA_3A-42E | n78A |
| DC_3A-42E_n79A | DC_3A_n79A | CA_3A-42E | n79A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |  | 3A |  | CA_n77A-n79A |  |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |  | 3A |  | CA_n78A-n79A |  |
| DC_3A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_3A_n80A_ULSUP-FDM_n78A | 3A | SUL_n78A-n80A |
| DC_3A_SUL_n78A-n82A | DC_3A_n78A DC_3A_n82A | 3A | SUL_n78A-n82A |
| DC_3A_SUL_n79A-n80A | DC_3A_n79A, DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A | 3A | SUL_n79A-n80A |
| DC_5A-7A-7A_n78A | DC_5A_n78A DC_7A_n78A | CA_5A-7A-7A | n78A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A | CA_5A-7A | n78A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A | CA_5A-30A | n66A |
| DC_7A-20A_n28A | DC_7A_n28A DC_20A_n28A | CA_7A-20A | n28A |
| DC_7A-20A_n78A | DC_7A_n78A DC_20A_n78A | CA_7A-20A | n78A |
| DC_7A-28A_n78A | DC_7A_n78A DC_28A_n78A | CA_7A-28A | n78A |
| DC_7A_n28A-n78A | DC_7A_n28A, DC_7A_n78A | 7A | CA_n28A-n78A |
| DC_7C_n28A-n78A | DC_7A_n28A, DC_7A_n78A DC_7C_n78A | 7A CA_7C | CA_n28A-n78A |
| DC_7C-28A_n78A | DC_7C_n78A DC_28A_n78A | CA_7C-28A | n78A |
| DC_7A-46A_n78A3 | DC_7A_n78A |  | CA_7A-46A |  | n78A |  |
| DC_7A-46C_n78A3 | DC_7A_n78A | CA_7A-46C | n78A |
| DC_7A-46D_n78A3 | DC_7A_n78A | CA_7A-46D | n78A |
| DC_7A-46E_n78A3 | DC_7A_n78A | CA_7A-46E | n78A |
| DC_8A_SUL_n78A-n81A | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A | 8A | SUL_n78A-n81A |
| DC_8A_SUL_n79A-n81A | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A | 8A | SUL_n79A-n81A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A | CA_12A-30A | n66A |
| DC_18A-28A_n77A | DC_18A_n77A DC_28A_n77A | CA_18A-28A | n77A |
| DC_18A-28A_n78A | DC_18A_n78A DC_28A_n78A | CA_18A-28A | n78A |
| DC_18A-28A_n79A | DC_18A_n79A DC_28A_n79A | CA_18A-28A | n79A |
| DC_19A-21A_n78A DC_19A-21A_n78C | DC_19A_n78A DC_21A_n78A | CA_19A-21A | n78A CA_n78C |
| DC_19A-21A_n79A DC_19A-21A_n79C | DC_19A_n79A DC_21A_n79A | CA_19A-21A | n79A CA_n79C |
| DC_19A-21A_n77A DC_19A-21A_n77C | DC_19A_n77A DC_21A_n77A | CA_19A-21A | n77A CA_n77C |
| DC_19A-42A_n77A DC_19A-42A_n77C | DC_19A_n77A | CA_19A-42A | n77A CA_n77C |
| DC_19A-42A_n78A DC_19A-42A_n78C | DC_19A_n78A | CA_19A-42A | n78A CA_n78C |
| DC_19A-42A_n79A DC_19A-42A_n79C | DC_19A_n79A | CA_19A-42A | n79A CA_n79C |
| DC_19A-42C_n77A DC_19A-42C_n77C | DC_19A_n77A | CA_19A-42C | n77A CA_n77C |
| DC_19A-42C_n78A DC_19A-42C_n78C | DC_19A_n78A | CA_19A-42C | n78A CA_n78C |
| DC_19A-42C_n79A DC_19A-42C_n79C | DC_19A_n79A | CA_19A-42C | n79A CA_n79C |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  | 19A |  | CA_n77A-n79A |  |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  | 19A |  | CA_n78A-n79A |  |
| DC_20A_n8A-n75A | DC_20A_n8A |  | 20A |  | CA_n8A-n75A |  |
| DC_20A_n28A-n75A | DC_20A_n28A |  | 20A |  | CA_n28A-n75A |  |
| DC_20A_n28A-n78A | DC_20A_n28A DC_20A_n78A |  | 20A |  | CA_n28A-n78A |  |
| DC_20A_n75A-n78A | DC_20A_n78A |  | 20A |  | CA_n75A-n78A |  |
| DC_20A_n76A-n78A | DC_20A_n78A |  | 20A |  | CA_n76A-n78A |  |
| DC_20A_SUL_n78A-n82A | DC_20A_n78A, DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A |  | 20A |  | SUL_n78A-n82A |  |
| DC_20A_SUL_n78A-n83A | DC_20A_n78A DC_20A_n83A | 20A | SUL_n78A-n83A |
| DC_21A-28A_n77A DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A | CA_21A-28A | n77A CA_n77C |
| DC_21A-28A_n78A DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A | CA_21A-28A | n78A CA_n78C |
| DC_21A-28A_n79A DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A | CA_21A-28A | n79A CA_n79C |
| DC_21A-42A_n77A DC_21A-42A_n77C | DC_21A_n77A | CA_21A-42A | n77A CA_n77C |
| DC_21A-42A_n78A DC_21A-42A_n78C | DC_21A_n78A | CA_21A-42A | n78A CA_n78C |
| DC_21A-42A_n79A DC_21A-42A_n79C | DC_21A_n79A | CA_21A-42A | n79A CA_n79C |
| DC_21A-42C_n77A DC_21A-42C_n77C | DC_21A_n77A | CA_21A-42C | n77A CA_n77C |
| DC_21A-42C_n78A DC_21A-42C_n77C | DC_21A_n78A | CA_21A-42C | n78A CA_n78C |
| DC_21A-42C_n79A DC_21A-42C_n77C | DC_21A_n79A | CA_21A-42C | n79A CA_n79C |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |  | 21A |  | CA_n77A-n79A |  |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |  | 21A |  | CA_n78A-n79A |  |
| DC_28A_SUL_n78A-n83A | DC_28A_n78A, DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A | 28A | SUL_n78A-n83A |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A | 28A | CA_n8A-n78A |
| DC_28A-42A_n77A DC_28A-42A_n77C | DC_28A_n77A | CA_28A-42A | n77A CA_n77C |
| DC_28A-42A_n78A DC_28A-42A_n78C | DC_28A_n78A | CA_28A-42A | n78A CA_n78C |
| DC_28A-42A_n79A DC_28A-42A_n79C | DC_28A_n79A | CA_28A-42A | n79A CA_n79C |
| DC_28A-42C_n77A | DC_28A_n77A | CA_28A-42C | n77A |
| DC_28A-42C_n78A | DC_28A_n78A | CA_28A-42C | n78A |
| DC_28A-42C_n79A | DC_28A_n79A | CA_28A-42C | n79A |
| DC_41A-42A_n77A | DC_41A_n77A | CA_41A-42A | n77A |
| DC_41C-42C_n77A | DC_41A_n77A | CA_41C-42C | n77A |
| DC_41A-42C_n77A | DC_41A_n77A | CA_41A-42C | n77A |
| DC_41C-42A_n77A | DC_41C_n77A | CA_41C-42A | n77A |
| DC_41A-42A_n78A | DC_41A_n78A | CA_41A-42A | n78A |
| DC_41C-42A_n78A | DC_41C_n78A | CA_41C-42A | n78A |
| DC_41C-42C_n78A | DC_41A_n78A | CA_41C-42C | n78A |
| DC_41A-42C_n78A | DC_41A_n78A | CA_41A-42C | n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A | DC_41A_n79A | CA_41A-42A CA_41A-42C | n79A |
| DC_41C-42C_n79A | DC_41A_n79A | CA_41C-42C | n79A |
| DC_41C-42A_n79A | DC_41C_n79A | CA_41C-42A | n79A |
| DC_66A_(n)71AA | DC_66A_n71A DC_(n)71AA | CA_66A_71A | n71A |
| DC_66A_SUL_n78A-n86A | DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A | 66A | SUL_n78A-n86A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A | CA_1A-3A-5A | n78A |
| DC_1A-3A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A | CA_1A-3A-7A | n28A |
| DC_1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3A-7A | n78A |
| DC_1A-3C-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3C-7A | n78A |
| DC_1A-3A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | CA_1A-3A | CA_n7A-n78A |
| DC_1A-3C_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A | CA_1A-3A CA_1A-3C | CA_n7A-n78A |
| DC_1A-3A-7A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3A-7A-7A | n78A |
| DC_1A-3A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A | CA_1A-3A-8A | n78A |
| DC_1A-3A-19A_n77A DC_1A-3A-19A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A | n77A CA_n77C |
| DC_1A-3A-19A_n78A DC_1A-3A-19A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A | n78A CA_n78C |
| DC_1A-3A-19A_n79A DC_1A-3A-19A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A | n79A CA_n79C |
| DC_1A-3A-20A_n28A | DC_1A_n28A DC_3A_n28A DC_20A_n28A | CA_1A-3A-20A | n28A |
| DC_1A-3A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A | CA_1A-3A-20A | n78A |
| DC_1A-3A-21A_n77A DC_1A-3A-21A_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A | n77A CA_n77C |
| DC_1A-3A-21A_n78A DC_1A-3A-21A_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A | n78A CA_n78C |
| DC_1A-3A-21A_n79A DC_1A-3A-21A_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A | CA_1A-3A-21A | n79A CA_n79C |
| DC_1A-3A-28A_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A | n77A |
| DC_1A-3A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A | n78A |
| DC_1A-3A-28A_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A | n79A |
| DC_1A-3A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A | CA_1A-3A | CA_n28A-n78A |
| DC_1A-3C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A | CA_1A-3A, CA_1A-3C | CA_n28A-n78A |
| DC_1A-3A-42A_n77A DC_1A-3A-42A_n77C | DC_1A_n77A DC_3A_n77A | CA_1A-3A-42A | n77A CA_n77C |
| DC_1A-3A-42A_n78A DC_1A-3A-42A_n78C | DC_1A_n78A DC_3A_n78A | CA_1A-3A-42A | n78A CA_n78C |
| DC_1A-3A-42A_n79A DC_1A-3A-42A_n79C | DC_1A_n79A DC_3A_n79A | CA_1A-3A-42A | n79A CA_n79C |
| DC_1A-3A-42C_n77A | DC_1A_n77A DC_3A_n77A | CA_1A-3A-42C | n77A |
| DC_1A-3A-42C_n78A | DC_1A_n78A DC_3A_n78A | CA_1A-3A-42C | n78A |
| DC_1A-3A-42C_n79A | DC_1A_n79A DC_3A_n79A | CA_1A-3A-42C | n79A |
| DC_1A-3A-42C_n77C | DC_1A_n77A DC_3A_n77A | CA_1A-3A-42C | CA_n77C |
| DC_1A-3A-42C_n78C | DC_1A_n78A DC_3A_n78A | CA_1A-3A-42C | CA_n78C |
| DC_1A-3A-42C_n79C | DC_1A_n79A DC_3A_n79A | CA_1A-3A-42C | CA_n79C |
| DC_1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-5A-7A | n78A |
| DC_1A-5A-7A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-5A-7A-7A | n78A |
| DC_1A-7A-20A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A | CA_1A-7A-20A | n28A |
| DC_1A-7A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A | CA_1A-7A-20A | n78A |
| DC_1A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A | CA_1A-7A | CA_n28A-n78A |
| DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | CA_1A-7A, CA_1A-7C | CA_n28A-n78A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A | CA_1A-18A-28A | n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A | CA_1A-18A-28A | n78A |
| DC_1A-18A-28A_n79A | DC_1A_n79A DC_18A_n79A DC_28A_n79A | CA_1A-18A-28A | n79A |
| DC_1A-19A-21A_n77A DC_1A-19A-21A_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A | CA_n77A CA_n77C |
| DC_1A-19A-21A_n78A DC_1A-19A-21A_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A | CA_n78A CA_n78C |
| DC_1A-19A-21A_n79A DC_1A-19A-21A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A | CA_n79A CA_n79C |
| DC_1A-19A-42A_n77A DC_1A-19A-42A_n77C | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42A | n77A CA_n77C |
| DC_1A-19A-42A_n78A DC_1A-19A-42A_n78C | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42A | n78A CA_n78C |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C | DC_1A_n79A DC_19A_n79A | CA_1A-19A-42A | n79A CA_n79C |
| DC_1A-19A-42C_n77A | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42C | n77A |
| DC_1A-19A-42C_n78A | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42C | n78A |
| DC_1A-19A-42C_n79A | DC_1A_n79A DC_19A_n79A | CA_1A-19A-42C | n79A |
| DC_1A-19A-42C_n77C | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42C | CA_n77C |
| DC_1A-19A-42C_n78C | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42C | CA_n78C |
| DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A | CA_1A-19A-42C | CA_n79C |
| DC_1A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-20A | CA_n28A-n78A |
| DC_1A-21A-28A_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A | n77A |
| DC_1A-21A-28A_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A | n78A |
| DC_1A-21A-28A_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A | n79A |
| DC_1A-21A-42A_n77A DC_1A-21A-42A_n77C | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42A | n77A CA_n77C |
| DC_1A-21A-42A_n78A DC_1A-21A-42A_n78C | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42A | n78A CA_n78C |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C | DC_1A_n79A DC_21A_n79A | CA_1A-21A-42A | n79A CA_n79C |
| DC_1A-21A-42C_n77A | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42C | n77A |
| DC_1A-21A-42C_n78A | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42C | n78A |
| DC_1A-21A-42C_n79A | DC_1A_n79A DC_21A_n79A | CA_1A-21A-42C | n79A |
| DC_1A-21A-42C_n77C | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42C | CA_n77C |
| DC_1A-21A-42C_n78C | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42C | CA_n78C |
| DC_1A-21A-42C_n79C | DC_1A_n79A DC_21A_n79A | CA_1A-21A-42C | CA_n79C |
| DC_1A-28A-42A_n77A | DC_1A_n77A DC_28A_n77A | CA_1A-28A-42A | n77A |
| DC_1A-28A-42A_n78A | DC_1A_n78A DC_28A_n78A | CA_1A-28A-42A | n78A |
| DC_1A-28A-42A_n79A | DC_1A_n79A DC_28A_n79A | CA_1A-28A-42A | n79A |
| DC_1A-28A-42C_n77A | DC_1A_n77A DC_28A_n77A | CA_1A-28A-42C | n77A |
| DC_1A-28A-42C_n78A | DC_1A_n78A DC_28A_n78A | CA_1A-28A-42C | n78A |
| DC_1A-28A-42C_n79A | DC_1A_n79A DC_28A_n79A | CA_1A-28A-42C | n79A |
| DC_1A-41A-42A_n77A | DC_1A_n77A DC_41A_n77A | CA_1A-41A-42A | n77A |
| DC_1A-41A-42C_n77A | DC_1A_n77A DC_41A_n77A | CA_1A-41A-42C | n77A |
| DC_1A-41C-42A_n77A | DC_1A_n77A DC_41A_n77A | CA_1A-41C-42A | n77A |
| DC_1A-41A-42A_n78A | DC_1A_n78A DC_41A_n78A | CA_1A-41A-42A | n78A |
| DC_1A-41A-42C_n78A | DC_1A_n78A DC_41A_n78A | CA_1A-41A-42C | n78A |
| DC_1A-41C-42A_n78A | DC_1A_n78A DC_41A_n78A | CA_1A-41C-42A | n78A |
| DC_1A-41A-42A_n79A | DC_1A_n79A DC_41A_n79A | CA_1A-41A-42A | n79A |
| DC_1A-41A-42C_n79A | DC_1A_n79A DC_41A_n79A | CA_1A-41A-42C | n79A |
| DC_1A-41C-42A_n79A | DC_1A_n79A DC_41A_n79A | CA_1A-41C-42A | n79A |
| DC_1A-41C-42C_n77A | DC_1A_n77A DC_41A_n77A | CA_1A-41C-42C | n77A |
| DC_1A-41C-42C_n78A | DC_1A_n78A DC_41A_n78A | CA_1A-41C-42C | n78A |
| DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A | CA_1A-41C-42C | n79A |
| DC_2A-66A-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA | CA_2A-66A-71A | n71A |
| DC_3A-5A-7A-7A_n78A | DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_3A-5A-7A-7A | n78A |
| DC_3A-5A-7A_n78A | DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_3A-5A-7A | n78A |
| DC_3A-7A-20A_n28A | DC_3A_n28A DC_7A_n28A DC_20A_n28A | CA_3A-7A-20A | n28A |
| DC_3A-7A-20A_n78A | DC_3A_n78A DC_20A_n78A DC_7A_n78A | CA_3A-7A-20A | n78A |
| DC_3A-7A-28A_n78A | DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_3A-7A-28A | n78A |
| DC_3A-7C-28A_n78A | DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_3A-7C-28A | n78A |
| DC_3A-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A | CA_3A-7A | CA_n28A-n78A |
| DC_3A-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | CA_3A-7A CA_3A-7C | CA_n28A-n78A |
| DC_3C-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A | CA_3A-7A CA_3C-7A | CA_n28A-n78A |
| DC_3C-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A DC_7C_n28A DC_7C_n78A | CA_3A-7A CA_3C-7A CA_3A-7C CA_3C-7C | CA_n28A-n78A |
| DC_3A-19A-21A_n77A DC_3A-19A-21A_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A | n77A CA_n77C |
| DC_3A-19A-21A_n78A DC_3A-19A-21A_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A | n78A CA_n78C |
| DC_3A-19A-21A_n79A DC_3A-19A-21A_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A | n79A CA_n79C |
| DC_3A-19A-42A_n77A DC_3A-19A-42A_n77C | DC_3A_n77A DC_19A_n77A | CA_3A-19A-42A | n77A CA_n77C |
| DC_3A-19A-42C_n77A DC_3A-19A-42C_n77C | DC_3A_n77A DC_19A_n77A | CA_3A-19A-42C | n77A CA_n77C |
| DC_3A-19A-42A_n78A DC_3A-19A-42A_n78C | DC_3A_n78A DC_19A_n78A | CA_3A-19A-42A | n78A CA_n78C |
| DC_3A-19A-42C_n78A DC_3A-19A-42C_n78C | DC_3A_n78A DC_19A_n78A | CA_3A-19A-42C | n78A CA_n78C |
| DC_3A-19A-42A_n79A DC_3A-19A-42A_n79C | DC_3A_n79A DC_19A_n79A | CA_3A-19A-42A | n79A CA_n79C |
| DC_3A-19A-42C_n79A DC_3A-19A-42C_n79C | DC_3A_n79A DC_19A_n79A | CA_3A-19A-42C | n79A CA_n79C |
| DC_3A-20A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A | CA_3A-20A | CA_n28A-n78A |
| DC_3A-21A-42A_n77A DC_3A-21A-42A_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42A | n77A CA_n77C |
| DC_3A-21A-42A_n78A DC_3A-21A-42A_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42A | n78A CA_n78C |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42A | n79A CA_n79C |
| DC_3A-21A-42C_n77A | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | n77A |
| DC_3A-21A-42C_n78A | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | n78A |
| DC_3A-21A-42C_n79A | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42C | n79A |
| DC_3A-21A-42C_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | CA_n77C |
| DC_3A-21A-42C_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | CA_n78C |
| DC_3A-21A-42C_n79C | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42C | CA_n79C |
| DC_3A-28A-42A_n77A | DC_3A_n77A DC_28A_n77A | CA_3A-28A-42A | n77A |
| DC_3A-28A-42A_n78A | DC_3A_n78A DC_28A_n78A | CA_3A-28A-42A | n78A |
| DC_3A-28A-42A_n79A | DC_3A_n79A DC_28A_n79A | CA_3A-28A-42A | n79A |
| DC_3A-28A-42C_n77A | DC_3A_n77A DC_28A_n77A | CA_3A-28A-42C | n77A |
| DC_3A-28A-42C_n78A | DC_3A_n78A DC_28A_n78A | CA_3A-28A-42C | n78A |
| DC_3A-28A-42C_n79A | DC_3A_n79A DC_28A_n79A | CA_3A-28A-42C | n79A |
| DC_7A-20A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_7A-20A | CA_n28A-n78A |
| DC_19A-21A-42A_n77A DC_19A-21A-42A_n77C | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42A | n77A CA_n77C |
| DC_19A-21A-42A_n78A DC_19A-21A-42A_n78C | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42A | n78A CA_n78C |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C | DC_19A_n79A DC_21A_n79A | CA_19A-21A-42A | n79A CA_n79C |
| DC_19A-21A-42C_n77A | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42C | n77A |
| DC_19A-21A-42C_n78A | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42C | n78A |
| DC_19A-21A-42C_n79A | DC_19A_n79A DC_21A_n79A | CA_19A-21A-42C | n79A |
| DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42C | CA_n77C |
| DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42C | CA_n78C |
| DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A | CA_19A-21A-42C | CA_n79C |
| DC_21A-28A-42A_n77A | DC_21A_n77A DC_28A_n77A | CA_21A-28A-42A | n77A |
| DC_21A-28A-42A_n78A | DC_21A_n78A DC_28A_n78A | CA_21A-28A-42A | n78A |
| DC_21A-28A-42A_n79A | DC_21A_n79A DC_28A_n79A | CA_21A-28A-42A | n79A |
| DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A | CA_21A-28A-42C | n77A |
| DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A | CA_21A-28A-42C | n78A |
| DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A | CA_21A-28A-42C | n79A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-3A-5A-7A | n78A |
| DC_1A-3A-7A-20A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A | CA_1A-3A-7A-20A | n28A |
| DC_1A-3A-5A-7A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-3A-5A-7A-7A | n78A |
| DC_1A-3A-7A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A | CA_1A-3A-7A-20A | n78A |
| DC_1A-3A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A | CA_1A-3A-7A | CA_n28A-n78A |
| DC_1A-3A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | CA_1A-3A-7A CA_1A-3A-7C | CA_n28A-n78A |
| DC_1A-3C-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A | CA_1A-3A-7A CA_1A-3C-7A | CA_n28A-n78A |
| DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | CA_1A-3A-7A CA_1A-3C-7A CA_1A-3A-7C CA_1A-3C-7C | CA_n28A-n78A |
| DC_1A-3A-19A-21A_n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-3A-19A-21A | n77A |
| DC_1A-3A-19A-21A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-3A-19A-21A | CA_n77C |
| DC_1A-3A-19A-21A_n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-3A-19A-21A | n78A |
| DC_1A-3A-19A-21A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-3A-19A-21A | CA_n78C |
| DC_1A-3A-19A-21A_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-3A-19A-21A | n79A |
| DC_1A-3A-19A-21A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-3A-19A-21A | CA_n79C |
| DC_1A-3A-19A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42A | n77A |
| DC_1A-3A-19A-42A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42A | CA_n77C |
| DC_1A-3A-19A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42C | n77A |
| DC_1A-3A-19A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42C | CA_n77C |
| DC_1A-3A-19A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42A | n78A |
| DC_1A-3A-19A-42A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78 | CA_1A-3A-19A-42A | CA_n78C |
| DC_1A-3A-19A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42C | n78A |
| DC_1A-3A-19A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42C | CA_n78C |
| DC_1A-3A-19A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42A | n79A |
| DC_1A-3A-19A-42A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42A | CA_n79C |
| DC_1A-3A-19A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42C | n79A |
| DC_1A-3A-19A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42C | CA_n79C |
| DC_1A-3A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-3A-20A | CA_n28A-n78A |
| DC_1A-3A-21A-42A_n77A DC_1A-3A-21A-42A_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A-42A | n77A CA_n77C |
| DC_1A-3A-21A-42A_n78A DC_1A-3A-21A-42A_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A-42A | n78A CA_n78C |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-21A-42A | n79A CA_n79C |
| DC_1A-3A-21A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A-42C | n77A |
| DC_1A-3A-21A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A-42C | CA_n77C |
| DC_1A-3A-21A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A-42C | n78A |
| DC_1A-3A-21A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A-42C | CA_n78C |
| DC_1A-3A-21A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_21A_n79A | CA_1A-3A-21A-42C | n79A |
| DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A | CA_1A-3A-21A-42C | CA_n79C |
| DC_1A-3A-28A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A-42A | n77A |
| DC_1A-3A-28A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A-42A | n78A |
| DC_1A-3A-28A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A-42A | n79A |
| DC_1A-3A-28A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A-42C | n77A |
| DC_1A-3A-28A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A-42C | n78A |
| DC_1A-3A-28A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A-42C | n79A |
| DC_1A-7A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-7A-20A | CA_n28A-n78A |
| DC_1A-19A-21A-42A_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42A | n77A |
| DC_1A-19A-21A-42A_n78A | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42A | n78A |
| DC_1A-19A-21A-42A_n79A | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42A | n79A |
| DC_1A-19A-21A-42A_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42A | CA_n77C |
| DC_1A-19A-21A-42A_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42A | CA_n78C |
| DC_1A-19A-21A-42A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42A | CA_n79C |
| DC_1A-19A-21A-42C_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42C | n77A |
| DC_1A-19A-21A-42C_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42C | CA_n77C |
| DC_1A-19A-21A-42C_n78A | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42C | n78A |
| DC_1A-19A-21A-42C_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42C | CA_n78C |
| DC_1A-19A-21A-42C_n79A | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42C | n79A |
| DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42C | CA_n79C |
| DC_1A-21A-28A-42A_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A-42A | n77A |
| DC_1A-21A-28A-42A_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A-42A | n78A |
| DC_1A-21A-28A-42A_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A-42A | n79A |
| DC_1A-21A-28A-42C_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A-42C | n77A |
| DC_1A-21A-28A-42C_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A-42C | n78A |
| DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A-42C | n79A |
| DC_3A-7A-20A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_3A-7A-20A | CA_n28A-n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-3A-7A-20A | CA_n28A-n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.5	Inter-band EN-DC including FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 [4] and for NR operating bands and CA configurations in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3.

5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F | DC_1A_n257A | 1A | n257A CA_n257D CA_n257E CA_n257F |
| DC_2A_n257A DC_2A_n257(2A) | DC_2A_n257A | 2A | n257A CA_n257(2A) |
| DC_2A-2A_n257A | DC_2A_n257A | CA_2A-2A | n257A |
| DC_2A_n257A | DC_2A_n257A | 2A | n257A |
| DC_2C_n257A | DC_2A_n257A | CA_2C | n257A |
| DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260(2A) | DC_2A_n260A | 2A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260(2A) |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A | CA_2A-2A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2C_n260A | DC_2A_n260A | CA_2C | n260A |
| DC_3A_n257A DC_3A_n257D DC_3A_n257E DC_3A_n257F | DC_3A_n257A | 3A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A_n258A | DC_3A_n258A | 3A | n258A |
| DC_5A-5A_n257A | DC_5A_n257A | CA_5A-5A | n257A |
| DC_5A-5A_n260A | DC_5A_n260A | CA_5A-5A | n260A |
| DC_5A_n257A | DC_5A_n257A | 5A | n257A |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_n260(A-I) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) | DC_5A_n260A | 5A | n260A CA_n260B CA_n260C CA_n260D CA_n260E CA_n260F CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260O CA_n260P CA_n260Q CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260(A-I) CA_n260(D-G) CA_n260(D-H) CA_n260(D-I) CA_n260(D-O) CA_n260(D-P) CA_n260(D-Q) CA_n260(E-O) CA_n260(E-P) CA_n260(E-Q) CA_n260(G-I) |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q DC_5A_n261(2A) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) | DC_5A_n261A | 5A | n261A CA_n261B CA_n261C CA_n261D CA_n261E CA_n261F CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M CA_n261O CA_n261P CA_n261Q CA_n261(2A) CA_n261(3A) CA_n261(4A) CA_n261(D-G) CA_n261(D-H) CA_n261(D-I) CA_n261(D-O) CA_n261(D-P) CA_n261(D-Q) CA_n261(E-O) CA_n261(E-P) CA_n261(E-Q) |
| DC_5B_n257A | DC_5B_n257A | CA_5B | n257A |
| DC_5B_n260A | DC_5B_n260A | CA_5B | n260A |
| DC_7A-7A_n257A | DC_7A_n257A | CA_7A-7A | n257A |
| DC_7A_n257A | DC_7A_n257A | 7A | n257A |
| DC_7A_n258A | DC_7A_n258A | 7A | n258A |
| DC_8A_n257A | DC_8A_n257A | 8A | n257A |
| DC_8A_n258A | DC_8A_n258A | 8A | n258A |
| DC_11A_n257A | DC_11A_n257A | 11A | n257A |
| DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A | 12A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260(A-I) CA_n260(G-I) |
| DC_13A_n257A | DC_13A_n257A | 13A | n257A |
| DC_13A_n260A | DC_13A_n260A | 13A | n260A |
| DC_18A_n257A | DC_18A_n257A | 18A | n257A |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F | DC_19A_n257A | 19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_20A_n258A | DC_20A_n258A | 20A | n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F | DC_21A_n257A | 21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_26A_n257A | DC_26A_n257A | 26A | n257A |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F | DC_28A_n257A | 28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_28A_n258A | DC_28A_n258A | 28A | n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A | 30A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260(A-I) CA_n260(G-I) |
| DC_39A_n258A | DC_39A_n258A | 39A | n258A |
| DC_41A_n257A DC_41C_n257A | DC_41A_n257A | 41A CA_41C | n257A |
| DC_41A_n258A | DC_41A_n258A | 41A | n258A |
| DC_41C_n257A | DC_41C_n257A | CA_41C | n257A |
| DC_42A_n257A DC_42C_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F | DC_42A_n257A | 42A CA_42C 42A 42A 42A | n257A n257A CA_n257D CA_n257E CA_n257F |
| DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F | DC_42C_n257A | CA_42C | n257A CA_n257D CA_n257E CA_n257F |
| DC_42D_n257A | DC_42C_n257A | CA_42D | n257A |
| DC_42E_n257A | DC_42A_n257A | CA_42E | n257A |
| DC_48A-48A_n257A | DC_48A_n257A | CA_48A-48A | n257A |
| DC_48A-48A_n260A | DC_48A_n260A | CA_48A-48A | n260A |
| DC_48A_n257A | DC_48A_n257A | 48A | n257A |
| DC_48C_n257A | DC_48C_n257A | CA_48C | n257A |
| DC_48C_n260A | DC_48C_n260A | CA_48C | n260A |
| DC_48A_n260A | DC_48A_n260A | 48A | n260A |
| DC_66A-66A_n257A | DC_66A_n257A | CA_66A-66A | n257A |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A | CA_66A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_66A_n257A DC_66A_n257(2A) DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M | DC_66A_n257A | 66A | n257A CA_n257(2A) CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) | DC_66A_n260A | 66A | n260A CA_n260D CA_n260E CA_n260F CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260O CA_n260P CA_n260Q CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260(A-I) CA_n260(D-G) CA_n260(D-H) CA_n260(D-I) CA_n260(D-O) CA_n260(D-P) CA_n260(D-Q) CA_n260(E-O) CA_n260(E-P) CA_n260(E-Q) CA_n260(G-I) |
| DC_66C_n257A | DC_66C_n257A | CA_66C | n257A |
| DC_66A_n261A DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) | DC_66A_n261A | 66A | n261A CA_n261D CA_n261E CA_n261F CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M CA_n261O CA_n261P CA_n261Q CA_n261(2A) CA_n261(3A) CA_n261(4A) CA_n261(D-G) CA_n261(D-H) CA_n261(D-I) CA_n261(D-O) CA_n261(D-P) CA_n261(D-Q) CA_n261(E-O) CA_n261(E-P) CA_n261(E-Q) |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_n257A DC_1A-3A_n257D DC_1A-3A_n257E DC_1A-3A_n257F | DC_1A_n257A DC_3A_n257A | CA_1A-3A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-5A_n257A | DC_1A_n257A DC_5A_n257A | CA_1A-5A | n257A |
| DC_1A-7A_n257A | DC_1A_n257A DC_7A_n257A | CA_1A-7A | n257A |
| DC_1A-7A-7A_n257A | DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | n257A |
| DC_1A-8A_n257A | DC_1A_n257A DC_8A_n257A | CA_1A-8A | n257A |
| DC_1A-18A_n257A | DC_1A_n257A DC_18A_n257A | CA_1A-18A | n257A |
| DC_1A-19A_n257A DC_1A-19A_n257D DC_1A-19A_n257E DC_1A-19A_n257F | DC_1A_n257A DC_19A_n257A | CA_1A-19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-21A_n257A DC_1A-21A_n257D DC_1A-21A_n257E DC_1A-21A_n257F | DC_1A_n257A DC_21A_n257A | CA_1A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-28A_n257A DC_1A-28A_n257D DC_1A-28A_n257E DC_1A-28A_n257F | DC_1A_n257A DC_28A_n257A | CA_1A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-41A_n257A | DC_1A_n257A DC_41A_n257A | CA_1A-41A | n257A |
| DC_1A-41C_n257A | DC_1A_n257A DC_41C_n257A | CA_1A-41C | n257A |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F | DC_1A_n257A DC_42A_n257A | CA_1A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-42C_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42C | n257A |
| DC_1A-42D_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42D | n257A |
| DC_1A-42E_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42E | n257A |
| DC_2A-13A_n260A | DC_2A_n260A DC_13A_n260A | CA_2A-13A | n260A |
| DC_2A-5A_n257A | DC_2A_n257A DC_5A_n257A | CA_2A-5A | n257A |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260A DC_5A_n260A | CA_2A-5A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-12A_n260A DC_2A-12A_n260G DC_2A-12A_n260H DC_2A-12A_n260I DC_2A-12A_n260J DC_2A-12A_n260K DC_2A-12A_n260L DC_2A-12A_n260M | DC_2A_n260A DC_12A_n260A | CA_2A-12A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-13A_n257A | DC_2A_n257A DC_13A_n257A | CA_2A-13A | n257A |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_30A_n260A | CA_2A-30A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-66A_n257A | DC_2A_n257A DC_66A_n257A | CA_2A-66A | n257A |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M | DC_2A_n260A DC_66A_n260A | CA_2A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_3A-5A_n257A | DC_3A_n257A DC_5A_n257A | CA_3A-5A | n257A |
| DC_3A-7A-7A_n257A | DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | n257A |
| DC_3A-7A_n257A | DC_3A_n257A DC_7A_n257A | CA_3A-7A | n257A |
| DC_3A-19A_n257A DC_3A-19A_n257D DC_3A-19A_n257E DC_3A-19A_n257F | DC_3A_n257A DC_19A_n257A | CA_3A-19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-21A_n257A DC_3A-21A_n257D DC_3A-21A_n257E DC_3A-21A_n257F | DC_3A_n257A DC_21A_n257A | CA_3A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-28A_n257A DC_3A-28A_n257D DC_3A-28A_n257E DC_3A-28A_n257F | DC_3A_n257A DC_28A_n257A | CA_3A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-41A_n257A | DC_3A_n257A DC_41A_n257A | CA_3A-41A | n257A |
| DC_3A-42A_n257A DC_3A-42A_n257D DC_3A-42A_n257E DC_3A-42A_n257F | DC_3A_n257A DC_42A_n257A | CA_3A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-42C_n257A | DC_3A_n257A DC_42A_n257A | CA_3A-42C | n257A |
| DC_3A-42D_n257A | DC_3A_n257A DC_42A_n257A | CA_3A-42D | n257A |
| DC_3A-42E_n257A | DC_3A_n257A DC_42A_n257A | CA_3A-42E | n257A |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_30A_n260A | CA_5A-30A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_5A-66A_n257A | DC_5A_n257A DC_66A_n257A | CA_5A-66A | n257A |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M | DC_5A_n260A DC_66A_n260A | CA_5A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_5A-7A-7A_n257A | DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | n257A |
| DC_5A-7A_n257A | DC_5A_n257A DC_7A_n257A | CA_5A-7A | n257A |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A | CA_12A-30A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M | DC_12A_n260A DC_66A_n260A | CA_12A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_13A-66A_n257A | DC_13A_n257A DC_66A_n257A | CA_13A-66A | n257A |
| DC_13A-66A_n260A | DC_13A_n260A DC_66A_n260A | CA_13A-66A | n260A |
| DC_18A-28A_n257A | DC_18A_n257A DC_28A_n257A | CA_18A-28A | n257A |
| DC_19A-42A_n257A DC_19A-42A_n257D DC_19A-42A_n257E DC_19A-42A_n257F | DC_19A_n257A DC_42A_n257A | CA_19A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_19A-21A_n257A DC_19A-21A_n257D DC_19A-21A_n257E DC_19A-21A_n257F | DC_19A_n257A DC_21A_n257A | CA_19A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_19A-42C_n257A | DC_19A_n257A DC_42A_n257A | CA_19A-42C | n257A |
| DC_21A-28A_n257A DC_21A-28A_n257D DC_21A-28A_n257E DC_21A-28A_n257F | DC_21A_n257A DC_28A_n257A | CA_21A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_21A-42A_n257A DC_21A-42A_n257D DC_21A-42A_n257E DC_21A-42A_n257F | DC_21A_n257A DC_42A_n257A | CA_21A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_21A-42C_n257A | DC_21A_n257A DC_42A_n257A | CA_21A-42C | n257A |
| DC_28A-42C_n257A | DC_28A_n257A DC_42A_n257A | CA_28A-42C | n257A |
| DC_28A-42A_n257A | DC_28A_n257A DC_42A_n257A | CA_28A-42A | n257A |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_66A_n260A | CA_30A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_41A-42A_n257A | DC_41A_n257A DC_42A_n257A | CA_41A-42A | n257A |
| DC_41A-42C_n257A | DC_41A_n257A DC_42C_n257A | CA_41A-42C | n257A |
| DC_41C-42A_n257A | DC_41C_n257A DC_42A_n257A | CA_41C-42A | n257A |
| DC_41C-42C_n257A | DC_41A_n257A DC_42A_n257A | CA_41C-42C | n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)

Table 5.5B.5.3-1: Inter-band EN-DC configurations including FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | n257A |
| DC_1A-3A-7A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_7A_n257A | CA_1A-3A-7A-7A | n257A |
| DC_1A-3A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_7A_n257A | CA_1A-3A-7A | n257A |
| DC_1A-3A-19A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A | CA_1A-3A-19A | n257A |
| DC_1A-3A-21A_n257A | DC_1A_n257A DC_3A_n257A DC_21A_n257A | CA_1A-3A-21A | n257A |
| DC_1A-3A-28A_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A | CA_1A-3A-28A | n257A |
| DC_1A-3A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42A | n257A |
| DC_1A-3A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | n257A |
| DC_1A-3A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | CA_n257D |
| DC_1A-3A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | CA_n257E |
| DC_1A-3A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | CA_n257F |
| DC_1A-5A-7A-7A_n257A | DC_1A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-5A-7A-7A | n257A |
| DC_1A-5A-7A_n257A | DC_1A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-5A-7A | n257A |
| DC_1A-18A-28A_n257A | DC_1A_n257A DC_18A_n257A DC_28A_n257A | CA_1A-18A-28A | n257A |
| DC_1A-19A-21A_n257A DC_1A-19A-21A_n257D DC_1A-19A-21A_n257E DC_1A-19A-21A_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-19A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-19A-42A_n257A | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42A | n257A |
| DC_1A-19A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42C | n257A |
| DC_1A-19A-42C_n257D | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42C | CA_n257D |
| DC_1A-19A-42C_n257E | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42C | CA_n257E |
| DC_1A-19A-42C_n257F | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42C | CA_n257F |
| DC_1A-21A-28A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A | CA_1A-21A-28A | n257A |
| DC_1A-21A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42A | n257A |
| DC_1A-21A-42C_n257A | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | n257A |
| DC_1A-21A-42C_n257D | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | CA_n257D |
| DC_1A-21A-42C_n257E | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | CA_n257E |
| DC_1A-21A-42C_n257F | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | CA_n257F |
| DC_1A-28A-42A_n257A | DC_1A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-28A-42A | n257A |
| DC_1A-28A-42C_n257A | DC_1A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-28A-42C | n257A |
| DC_1A-41A-42A_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41A-42A | n257A |
| DC_1A-41A-42C_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41A-42C | n257A |
| DC_1A-41C-42A_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41C-42A | n257A |
| DC_1A-41C-42C_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41C-42C | n257A |
| DC_3A-5A-7A-7A_n257A | DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_3A-5A-7A-7A | n257A |
| DC_3A-5A-7A_n257A | DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_3A-5A-7A | n257A |
| DC_3A-7A_n78A-n257A DC_3A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A, DC_7A_n78A DC_7A_n257A | CA_3A-7A, CA_3A-7A-7A | CA_n78A-n257A |
| DC_3A-19A-21A_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_3A-19A-21A | n257A |
| DC_3A-19A-42A_n257A | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42A | n257A |
| DC_3A-19A-42C_n257A | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | n257A |
| DC_3A-19A-42C_n257D | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | CA_n257D |
| DC_3A-19A-42C_n257E | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | CA_n257E |
| DC_3A-19A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | CA_n257F |
| DC_3A-21A-42A_n257A | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42A | n257A |
| DC_3A-21A-42C_n257A | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | n257A |
| DC_3A-21A-42C_n257D | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | CA_n257D |
| DC_3A-21A-42C_n257E | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | CA_n257E |
| DC_3A-21A-42C_n257F | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | CA_n257F |
| DC_3A-28A-42A_n257A | DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_3A-28A-42A | n257A |
| DC_3A-28A-42C_n257A | DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_3A-28A-42C | n257A |
| DC_19A-21A-42A_n257A | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42A | n257A |
| DC_19A-21A-42C_n257D | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | CA_n257D |
| DC_19A-21A-42C_n257E | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | CA_n257E |
| DC_19A-21A-42C_n257F | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | CA_n257F |
| DC_19A-21A-42C_n257A | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | n257A |
| DC_21A-28A-42A_n257A | DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_21A-28A-42A | n257A |
| DC_21A-28A-42C_n257A | DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_21A-28A-42C | n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)

Table 5.5B.5.4-1: Inter-band EN-DC configurations including FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A | n257A |
| DC_1A-3A-5A-7A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A-7A | n257A |
| DC_1A-3A-19A-21A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | n257A |
| DC_1A-3A-19A-21A_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | CA_n257D |
| DC_1A-3A-19A-21A_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | CA_n257E |
| DC_1A-3A-19A-21A_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | CA_n257F |
| DC_1A-3A-19A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | n257A |
| DC_1A-3A-19A-42A_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | CA_n257D |
| DC_1A-3A-19A-42A_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | CA_n257E |
| DC_1A-3A-19A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | CA_n257F |
| DC_1A-3A-19A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | n257A |
| DC_1A-3A-19A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257D |
| DC_1A-3A-19A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257E |
| DC_1A-3A-19A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257F |
| DC_1A-3A-21A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-3A-21A-42A | n257A |
| DC_1A-3A-21A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | n257A |
| DC_1A-3A-21A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257D |
| DC_1A-3A-21A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257E |
| DC_1A-3A-21A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257F |
| DC_1A-3A-28A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-3A-28A-42A | n257A |
| DC_1A-3A-28A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-3A-28A-42C | n257A |
| DC_1A-19A-21A-42A_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257D |  | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A |  | CA_1A-19A-21A-42A |  | CA_n257D |
| DC_1A-19A-21A-42A_n257E |  | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A |  | CA_1A-19A-21A-42A |  | CA_n257E |
| DC_1A-19A-21A-42A_n257F |  | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A |  | CA_1A-19A-21A-42A |  | CA_n257F |
| DC_1A-19A-21A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257A |
| DC_1A-19A-21A-42C_n257D | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257D |
| DC_1A-19A-21A-42C_n257E | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257E |
| DC_1A-19A-21A-42C_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257F |
| DC_1A-19A-28A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-19A-28A-42C | n257A |
| DC_1A-21A-28A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-21A-28A-42A | n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.5.5	Inter-band EN-DC configurations including FR2 (six bands)

Table 5.5B.5.5-1: Inter-band EN-DC configurations including FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.6	Inter-band EN-DC including FR1 and FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 [4] and for NR operating bands and CA configurations in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3.

5.5B.6.1	Void

5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257A |
| DC_1A_n77A-n257D | DC_1A_n77A DC_1A-n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257D |
| DC_1A_n77A-n257E | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257E |
| DC_1A_n77A-n257F | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257F |
| DC_1A_n77C-n257A | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257A |
| DC_1A_n77C-n257D | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257D |
| DC_1A_n77C-n257E | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257E |
| DC_1A_n77C-n257F | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257F |
| DC_1A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257A |
| DC_1A_n78A-n257D | DC_1A_n78A DC_1A-n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257D |
| DC_1A_n78A-n257E | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257E |
| DC_1A_n78A-n257F | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257F |
| DC_1A_n78A-n257G | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257G |
| DC_1A_n78A-n257H | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257H |
| DC_1A_n78A-n257I | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257I |
| DC_1A_n78A-n257J | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257J |
| DC_1A_n78A-n257K | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257K |
| DC_1A_n78A-n257L | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257L |
| DC_1A_n78A-n257M | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257M |
| DC_1A_n78C-n257A | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257A |
| DC_1A_n78C-n257D | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257D |
| DC_1A_n78C-n257E | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257E |
| DC_1A_n78C-n257F | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257F |
| DC_1A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79A-n257A |
| DC_1A_n79A-n257D | DC_1A_n79A DC_1A-n257A DC_1A_n79A-n257A | 1A | CA_n79A-n257D |
| DC_1A_n79A-n257E | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79A-n257E |
| DC_1A_n79A-n257F | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79A-n257F |
| DC_1A_n79C-n257A | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79C-n257A |
| DC_1A_n79C-n257D | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79C-n257D |
| DC_1A_n79C-n257E | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79C-n257E |
| DC_1A_n79C-n257F | DC_1A_n79A DC_1A_n257A DC_1A_n79A-n257A | 1A | CA_n79C-n257F |
| DC_3A_n1A-n257A | DC_3A_n1A DC_3A_n257A | 3A | CA_n1A-n257A |
| DC_3A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257A |
| DC_3A_n77A-n257D | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257D |
| DC_3A_n77A-n257E | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257E |
| DC_3A_n77A-n257F | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257F |
| DC_3A_n77C-n257A | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257A |
| DC_3A_n77C-n257D | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257D |
| DC_3A_n77C-n257E | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257E |
| DC_3A_n77C-n257F | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257F |
| DC_3A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78A-n257A |
| DC_3A_n78A-n257D | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78A-n257D |
| DC_3A_n78A-n257E | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78A-n257E |
| DC_3A_n78A-n257F | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78A-n257F |
| DC_3A_n78A-n257G | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257G |
| DC_3A_n78A-n257H | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257H |
| DC_3A_n78A-n257I | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257I |
| DC_3A_n78A-n257J | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257J |
| DC_3A_n78A-n257K | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257K |
| DC_3A_n78A-n257L | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257L |
| DC_3A_n78A-n257M | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257M |
| DC_3A_n78C-n257A | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78C-n257A |
| DC_3A_n78C-n257D | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78C-n257D |
| DC_3A_n78C-n257E | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78C-n257E |
| DC_3A_n78C-n257F | DC_3A_n78A DC_3A_n257A DC_3A_n78A-n257A | 3A | CA_n78C-n257F |
| DC_3A_n79A-n257A | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79A-n257A |
| DC_3A_n79A-n257D | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79A-n257D |
| DC_3A_n79A-n257E | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79A-n257E |
| DC_3A_n79A-n257F | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79A-n257F |
| DC_3A_n79C-n257A | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79C-n257A |
| DC_3A_n79C-n257D | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79C-n257D |
| DC_3A_n79C-n257E | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79C-n257E |
| DC_3A_n79C-n257F | DC_3A_n79A DC_3A_n257A DC_3A_n79A-n257A | 3A | CA_n79C-n257F |
| DC_5A_n78A-n257A | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257A |
| DC_5A_n78A-n257D | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257D |
| DC_5A_n78A-n257E | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257E |
| DC_5A_n78A-n257F | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257F |
| DC_5A_n78A-n257G | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257G |
| DC_5A_n78A-n257H | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257H |
| DC_5A_n78A-n257I | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257I |
| DC_5A_n78A-n257J | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257J |
| DC_5A_n78A-n257K | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257K |
| DC_5A_n78A-n257L | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257L |
| DC_5A_n78A-n257M | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257M |
| DC_7A_n78A-n257A | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257A |
| DC_7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257D |
| DC_7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257E |
| DC_7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257F |
| DC_7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257G |
| DC_7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257H |
| DC_7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257I |
| DC_7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257J |
| DC_7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257K |
| DC_7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257L |
| DC_7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257M |
| DC_7A-7A_n78-n257A | DC_7A_n78A DC_7A_n257A DC_7A_n78A-n257A | CA_7A-7A | CA_n78A-n257A |
| DC_7A-7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257D |
| DC_7A-7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257E |
| DC_7A-7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257F |
| DC_7A-7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257G |
| DC_7A-7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257H |
| DC_7A-7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257I |
| DC_7A-7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257J |
| DC_7A-7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257K |
| DC_7A-7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257L |
| DC_7A-7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | CA_7A-7A | CA_n78A-n257M |
| DC_8A_n77A-n257A | DC_8A_n77A DC_8A_n257A | 8A | CA_n77A-n257A |
| DC_19A_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257A |
| DC_19A_n77A-n257D | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257D |
| DC_19A_n77A-n257E | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257E |
| DC_19A_n77A-n257F | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257F |
| DC_19A_n77C-n257A | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257A |
| DC_19A_n77C-n257D | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257D |
| DC_19A_n77C-n257E | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257E |
| DC_19A_n77C-n257F | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257F |
| DC_19A_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78A-n257A |
| DC_19A_n78A-n257D | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78A-n257D |
| DC_19A_n78A-n257E | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78A-n257E |
| DC_19A_n78A-n257F | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78A-n257F |
| DC_19A_n78C-n257A | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78C-n257A |
| DC_19A_n78C-n257D | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78C-n257D |
| DC_19A_n78C-n257E | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78C-n257E |
| DC_19A_n78C-n257F | DC_19A_n78A DC_19A_n257A DC_19A_n78A-n257A | 19A | CA_n78C-n257F |
| DC_19A_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79A-n257A |
| DC_19A_n79A-n257D | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79A-n257D |
| DC_19A_n79A-n257E | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79A-n257E |
| DC_19A_n79A-n257F | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79A-n257F |
| DC_19A_n79C-n257A | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79C-n257A |
| DC_19A_n79C-n257D | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79C-n257D |
| DC_19A_n79C-n257E | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79C-n257E |
| DC_19A_n79C-n257F | DC_19A_n79A DC_19A_n257A DC_19A_n79A-n257A | 19A | CA_n79C-n257F |
| DC_21A_n77A-n257A | DC_21A_n77A DC_21A_n257A | 21A | CA_n77A-n257A |
| DC_21A_n78A-n257A | DC_21A_n78A DC_21A_n257A | 21A | CA_n78A-n257A |
| DC_21A_n79A-n257A | DC_21A_n79A DC_21A_n257A | 21A | CA_n79A-n257A |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A | 28A | CA_n8A-n258A |
| DC_66A_n5A-n260A | DC_66A_n5A DC_66A_n260A | 66A | CA_n5A-n260A |
| DC_66A_n71A-n260A | DC_66A_n71A DC_66A_n260A | 66A | CA_n71A-n260A |
| DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A | 66A | CA_n71A-n260(2A) |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A | 66A | CA_n71A-n261A |
| DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A | 66A | CA_n71A-n261(2A) |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6.3-1: Inter-band EN-DC configurations including FR1 and FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A | CA_1A-3A | CA_n78A-n257A |
| DC_1A-3A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257D |
| DC_1A-3A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257E |
| DC_1A-3A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257F |
| DC_1A-3A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257G |
| DC_1A-3A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257H |
| DC_1A-3A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257I |
| DC_1A-3A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257J |
| DC_1A-3A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257K |
| DC_1A-3A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257L |
| DC_1A-3A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n78A-n257M |
| DC_1A-5A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A | CA_1A-5A | CA_n78A-n257A |
| DC_1A-5A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257D |
| DC_1A-5A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257E |
| DC_1A-5A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257F |
| DC_1A-5A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257G |
| DC_1A-5A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257H |
| DC_1A-5A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257I |
| DC_1A-5A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257J |
| DC_1A-5A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257K |
| DC_1A-5A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257L |
| DC_1A-5A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n78A-n257M |
| DC_1A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257A |
| DC_1A-7A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257D |
| DC_1A-7A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257E |
| DC_1A-7A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257F |
| DC_1A-7A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257G |
| DC_1A-7A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257H |
| DC_1A-7A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257I |
| DC_1A-7A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257J |
| DC_1A-7A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257K |
| DC_1A-7A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257L |
| DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n78A-n257M |
| DC_1A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-7A | CA_n78A-n257A |
| DC_1A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257D |
| DC_1A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257E |
| DC_1A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257F |
| DC_1A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257G |
| DC_1A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257H |
| DC_1A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257I |
| DC_1A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257J |
| DC_1A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257K |
| DC_1A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257L |
| DC_1A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n78A-n257M |
| DC_3A-5A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A | CA_3A-5A | CA_n78A-n257A |
| DC_3A-5A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257D |
| DC_3A-5A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257E |
| DC_3A-5A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257F |
| DC_3A-5A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257G |
| DC_3A-5A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257H |
| DC_3A-5A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257I |
| DC_3A-5A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257J |
| DC_3A-5A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257K |
| DC_3A-5A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257L |
| DC_3A-5A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n78A-n257M |
| DC_3A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257A |
| DC_3A-7A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257D |
| DC_3A-7A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257E |
| DC_3A-7A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257F |
| DC_3A-7A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257G |
| DC_3A-7A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257H |
| DC_3A-7A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257I |
| DC_3A-7A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257J |
| DC_3A-7A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257K |
| DC_3A-7A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257L |
| DC_3A-7A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257M |
| DC_3A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | CA_3A-7A | CA_n78A-n257A |
| DC_3A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257D |
| DC_3A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257E |
| DC_3A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257F |
| DC_3A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257G |
| DC_3A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257H |
| DC_3A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257I |
| DC_3A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257J |
| DC_3A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257K |
| DC_3A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257L |
| DC_3A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n78A-n257M |
| DC_5A-7A-7A_n78A-n257A | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257A |
| DC_5A-7A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257D |
| DC_5A-7A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257E |
| DC_5A-7A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257F |
| DC_5A-7A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257G |
| DC_5A-7A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257H |
| DC_5A-7A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257I |
| DC_5A-7A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257J |
| DC_5A-7A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257K |
| DC_5A-7A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257L |
| DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257M |
| DC_5A-7A_n78A-n257A | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_5A-7A | CA_n78A-n257A |
| DC_5A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257D |
| DC_5A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257E |
| DC_5A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257F |
| DC_5A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257G |
| DC_5A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257H |
| DC_5A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257I |
| DC_5A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257J |
| DC_5A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257K |
| DC_5A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257L |
| DC_5A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n78A-n257M |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Table 5.5B.6.4-1: Inter-band EN-DC configurations including FR1 and FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257A |
| DC_1A-3A-5A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257D |
| DC_1A-3A-5A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257E |
| DC_1A-3A-5A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257F |
| DC_1A-3A-5A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257G |
| DC_1A-3A-5A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257H |
| DC_1A-3A-5A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257I |
| DC_1A-3A-5A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257J |
| DC_1A-3A-5A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257K |
| DC_1A-3A-5A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257L |
| DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | CA_n78A-n257M |
| DC_1A-3A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-3A-7A-7A | CA_n78A-n257A |
| DC_1A-3A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-3A-7A | CA_n78A-n257A |
| DC_1A-5A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-5A-7A-7A | CA_n78A-n257A |
| DC_1A-5A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-5A-7A | CA_n78A-n257A |
| DC_3A-5A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_3A-5A-7A-7A | CA_n78A-n257A |
| DC_3A-5A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_3A-5A-7A | CA_n78A-n257A |

5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_1A-3A-5A-7A | CA_n78A-n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

<Unchanged sections are omitted>

6.2B.4	Configured output power for EN-DC

6.2B.4.1	Configured output power level

<Unchanged sections are omitted>

6.2B.4.2	ΔTIB,c for EN-DC

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in Tables below applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

6.2B.4.2.1	Intra-band contiguous EN-DC

ΔTIB,c is not applicable for intra-band contiguous EN-DC.

6.2B.4.2.2	Intra-band non-contiguous EN-DC

ΔTIB,c is not applicable for intra-band non-contiguous EN-DC.

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2_n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n40 | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n77 | 0.8 |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n77 | n77 | 0.5 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n7-n78 | 1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-18_n78 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1_n40-n78 | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-28_n77 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n78 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_SUL_n78-n84 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n84 | 0.3 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_2-(n)71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_2-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
| DC_2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2-66_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
| DC_3_n1-n77 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n1-n78 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1-n79 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0.0 |
| DC_3_n3-n77 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n3-n78 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |
| DC_3-7_n78, DC_3-7-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n7-n78 | 3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n79 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-20_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n20-n78 | 3 | 0.5 |
|  | n20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-21_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_3-38_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n40-n78 | 3 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| DC_3-41_n78 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| DC_3-42_n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-42_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-42_n79 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n79 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_3_n78-n79 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_3_SUL_n78-n80 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| DC_3_SUL_n78-n82 | 3 | 0.5 |
|  | n78 | 0.8 |
|  | n82 | 0.3 |
| DC_5-7_n78, DC_5-7-7_n78 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_5-30_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_7-20_n28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_7-20_n78 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_7-28_n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_7-46_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_SUL_n78- n81 | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n81 | 0.6 |
| DC_18-28_n77 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_18-28_n78 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_18-28_n79 | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_19-21_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-21_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
| DC_19-42_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-42_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n79 | 19 | 0.3 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_19_n78-n79 | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_20_n8-n75 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28-n75 | 20 | 0.5 |
|  | n28 | 0.7 |
| DC_20_n28-n78 | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_20_n75-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20_n76-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20_SUL_n78-n82 | 20 | 0.6 |
|  | n78 | 0.8 |
|  | n82 | 0.6 |
| DC_20_SUL_n78-n83 | 20 | 0.8 |
|  | n78 | 0.8 |
|  | n83 | 0.8 |
| DC_21-42_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-42_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n79 | 21 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_21_n78-n79 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_28_n8-n78 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.3 |
| DC_28-42_n77 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_28-42_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_28-42_n79 | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_28_SUL_n78-n83 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n83 | 0.5 |
| DC_41-42_n77 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_41-42_n78 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_41-42_n79 | 41 | 0. |
|  | 42 | 0.8 |
| DC_66_(n)71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 | 0.3 |
| DC_66_SUL_n78-n86 | 66 | 0.6 |
|  | n78 | 0.8 |
|  | n86 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-7_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 DC_1-3_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 or n7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-8_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3-28_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
| DC_1-3-19_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-19_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_1-3-20_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_1-3-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-20_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7-20_n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18-28_n77 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_1-18-28_n78 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_1-18-28_n79 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-20_n28-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-21-28_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| DC_1-21-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-28-42_n77 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-28-42_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-28-42_n79 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-41-42_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-41-42_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-41-42_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| DC_2-66-(n)71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_3-5-7_n78, DC_3-5-7-7_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-20_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n28 | 0.5 |
| DC_3-7-20_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-7-28_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19-21_n77 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-19-21_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-19-21_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_3-19-42_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-19-42_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-19-42_n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_3-20_n28-n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-28-42_n77 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-28-42_n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-28-42_n79 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_3-21-42_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-21-42_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-21-42_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_7-20_n28-n78 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_19-21-42_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-21-42_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-21-42_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21-28-42_n77 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-28-42_n78 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-28-42_n79 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7-20_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.6 |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-19-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_1-3-19-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-19-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-19-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-20_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-21-42_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.6 |
| DC_1-3-21-42_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.6 |
| DC_1-3-21-42_n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n79 | 0 |
| DC_1-3-28-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-28-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-28-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-7-20_n28-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-21-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-21-28-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-28-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-28-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_3-7-20_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

<Unchanged sections are omitted>

7.3B.2	Reference sensitivity for EN-DC

7.3B.2.1	Intra-band contiguous EN-DC

<Unchanged sections are omitted>

7.3B.2.3.5	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In case for the EN-DC in NR FR1 configurations the intermodulation products caused by dual uplink operation do not interfere with the own primary downlink channel bandwidth as defined in Annex-I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-1 and Table 7.3B.2.3.5.3-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-1 and Table 7.3B.2.3.5.3-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in [4] and 7.3.2.1 of [2] for the corresponding channel bandwidths or in clause 7.3.1 of [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-1 and Table 7.3B.2.3.5.3-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in … with parameters specified in Table 7.3B.2.3.5-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.3.5.1	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD23 |
|  |  |  |  |  |  | 32.54 |  |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| DC_1A_n77A, DC_1A_n78A, DC_1A_SUL_n78A-n84A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |
|  |  |  |  |  |  | 10.74 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1940 | 26 | FDD | IMD23 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| DC_2A_n78A | 2 | 1885 | 5 | 25 | 1955 | 8.0 | FDD | IMD43 |
|  |  |  |  |  |  | 10.74 |  |  |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| DC_3A_n77A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3C_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
| DC_3A_n77A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3C_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |
|  |  |  |  |  |  | 10.74 |  |  |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8A-SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| DC_8A_n79A, DC_8A-SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | FDD | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | FDD | IMD3 |
| DC_20A_n77A, DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n77 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | FDD | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | TDD | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | FDD | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | TDD | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | FDD | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | TDD | N/A |
| DC_26A_n77A, DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: Void |  |  |  |  |  |  |  |  |

7.3B.2.3.5.2	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: Reference sensitivity exceptions for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
| DC_1A-3A_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |
| DC_1A-7A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | FDD | IMD2 |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 |  | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 |  | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | FDD | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 |  | IMD2 ¦fn78-fB1¦ |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | FDD | IMD5 ¦2*fn78-3*fB3¦ |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | TDD | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | TDD | IMD2 ¦fB1+fn3¦ |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | FDD | IMD2 ¦fn78-fB1¦ |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 ¦fn78-2*fB5¦ |
|  | 5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | FDD | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
| DC_1A-7A_n78A, DC_1A-7C_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 ¦fn78-3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 ¦fn78 -3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 10.1 | TDD | IMD4 ¦fn7 -3*fB1¦ |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | FDD | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |
| DC_1A-18A_n77A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | FDD | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A |  | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
| DC_1A-18A_n78A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | FDD | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A |  | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | TDD | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | FDD | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | FDD | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | TDD | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | FDD | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | FDD | N/A |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | FDD | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A |  | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 |  | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | FDD | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A |  | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | TDD | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | FDD | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | FDD | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | FDD | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 |  | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | TDD | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A |  | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 |  | IMD5 |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A |  | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 |  | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A |  | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | TDD | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | FDD | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | FDD | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | FDD | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | FDD | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | FDD | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | TDD | N/A |
| DC_1A_n40A-n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | TDD | IMD4 ¦3*fB1-fn40¦ |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | TDD | IMD4 ¦3*fB1 -fn78¦ |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | TDD | N/A |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 |  | TDD |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | TDD | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 |  | TDD |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | TDD | IMD5 |
| DC_1A-41A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | 41 |  | 5 | 25 | 2515 | 12 | TDD | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 |  | TDD |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | TDD | IMD2 |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 |  | TDD |  |
|  | 41 | 2687.5 | 5 | 25 | 2687.5 | 0.0 | TDD | IMD5 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 |  | TDD |  |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | TDD | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | TDD | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 |  | TDD |  |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | FDD | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | TDD | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 |  | TDD |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | FDD | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD5 |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | TDD | IMD2 ¦fB3+fn1¦ |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 | FDD | IMD2 ¦fn77-fB3¦ |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| DC_3A_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | TDD | IMD2 ¦fB3+fn1¦ |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | FDD | IMD5 ¦2*fn78-3*fB3¦ |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| DC_3A_n1A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | TDD | IMD5 ¦4*fB3-fn1¦ |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n1 | 1950 | 40 | 216 | 2140 | 3.6 | FDD | IMD5 ¦4*fB3 -fn79¦ |
|  | n79 | 4860 | 5 | 25 | 4860 | N/A | TDD | N/A |
| DC_3A-7A_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | FDD | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | FDD | IMD2 |
| DC_3C-7C_n78A, DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
| DC_3A_n7A-n78A DC_3C_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 ¦2*fn7-fB3¦ |
| DC_3A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | FDD | IMD4 |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 ¦fB3+2*fn20¦ |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | FDD | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
| DC_3A_n28A-n78A, DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 ¦3*fB3 -2*fn28¦ |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | TDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 4.8 | TDD | IMD5 ¦2*fB3-3*fn40 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | TDD | IMD5 ¦3*fB3 -2*fn78¦ |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | TDD | N/A |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | TDD | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | TDD | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | TDD | IMD5 |
| DC_3A-SUL_n78A-n82A | 3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | FDD |  | IMD4 |  |
|  | n82 |  | 840 |  | 5 |  | 25 |  |  |  | N/A |  | SUL |  | N/A |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
| DC_3A-19A_n79A | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | FDD | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A |  | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | FDD | IMD3 ¦fn78-2*fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | TDD | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | FDD | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 |  | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | FDD | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | FDD | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | FDD | IMD2 ¦fn78-fb5¦ |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | FDD | IMD2 ¦fn78-fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | FDD | IMD5 ¦2*fn78-3fB7¦ |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | FDD | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | FDD | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | TDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | TDD | N/A |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 5.9 | FDD | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | FDD | IMD2 ¦fn78-fB7¦ |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | TDD | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | FDD | IMD5 ¦2*fn78-3*fB7¦ |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | FDD | IMD2 ¦fn78-fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | TDD | N/A |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 |  | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 |  | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | FDD | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | TDD | N/A |
| DC_7A_n28A-n78A, DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A |  | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2 ¦fB7 +fn28¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | FDD | IMD2 ¦fn78 -fB7¦ |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | TDD | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 |  | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | TDD | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | FDD | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A |  | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | TDD | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | FDD | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A |  | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | TDD | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | FDD | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | TDD | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 |  | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | TDD | N/A |
| DC_19A-21A_n79A | 19 | 837.5 | 5 | 25 | 882.2 | N/A | FDD | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 |  | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | FDD | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | TDD | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | FDD | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | FDD | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | FDD | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | TDD | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | TDD | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | TDD | IMD2 |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | FDD | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | FDD, SUL | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | TDD | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | FDD | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | TDD | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | TDD | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | FDD | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | TDD | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | TDD | IMD2 |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3458 | 10 | 52 | 3458 | 9.1 | TDD | IMD4 ¦3*fn8 +fB28¦ |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | FDD | IMD5 ¦fn78 -4*fB28¦ |
|  | n78 | 3787 | 10 | 52 | 3787 | N/A | TDD | N/A |

<Unchanged sections are omitted>

7.3B.3	ΔRIB,c, ΔRIBNC for EN-DC

<Editor’s note: Table number to be updated>

For the UE which supports inter-band EN-DC configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in [4], subclause 7.3.2, 7.3A.2, 7.3C.2 in [2] and subclause 7.3.2, 7.3A.2in [3] shall be increased by the amount given in ΔRIB,c,ΔRIBNC in Tables below where unless otherwise stated, the same ΔRIB,c, ΔRIBNC are applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔRIB,c or ΔRIBNC is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

7.3B.3.1	Intra-band contiguous EN-DC

7.3B.3.2	Intra-band non-contiguous EN-DC

Table 7.3B.3.2-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_3A_n3A | 5MHz+5MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5MHz+10MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5MHz+15MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5MHz+20MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz+25MHz | 25.0 < Wgap ≤ 45.0 | 121 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz+30MHz | 20.0 < Wgap ≤ 40.0 | 121 | 3.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 10MHz+5MHz | 30.0 < Wgap ≤ 60.0 | 125 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10MHz+10MHz | 25.0 < Wgap ≤ 55.0 | 125 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10MHz+15MHz | 20.0 < Wgap ≤ 50.0 | 125 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10MHz+20MHz | 15.0 < Wgap ≤ 45.0 | 125 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz+25MHz | 10.0 < Wgap ≤ 40.0 | 125 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz+30MHz | 5.0 < Wgap ≤ 35.0 | 125 | 2.8 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+5MHz | 25.0 < Wgap ≤ 55.0 | 126 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15MHz+10MHz | 20.0 < Wgap ≤ 50.0 | 126 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15MHz+15MHz | 15.0 < Wgap ≤ 45.0 | 126 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz+20MHz | 10.0 < Wgap ≤ 40.0 | 126 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15MHz+25MHz | 5.0 < Wgap ≤ 35.0 | 126 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+30MHz | 0.0 < Wgap ≤ 30.0 | 126 | 3.3 |  |
|  | 20MHz+5MHz | 15.0 < Wgap ≤ 50.0 | 167 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20MHz+10MHz | 10.0 < Wgap ≤ 45.0 | 167 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz+15MHz | 5.0 < Wgap ≤ 40.0 | 167 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20MHz+20MHz | 0.0 < Wgap ≤ 35.0 | 167 | 4.1 |  |
|  | 20MHz+25MHz | 0.0 < Wgap ≤ 30.0 | 167 | 3.8 |  |
|  | 20MHz+30MHz | 0.0 < Wgap ≤ 25.0 | 167 | 3.6 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. |  |  |  |  |  |

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n51 | n51 | 0.1 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n51 | 3 | 0.2 |
|  | n51 | 0.2 |
| DC_3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n51 | n51 | 0.2 |
| DC_7_n77 | n77 | 0.5 |
| DC_7_n78 | n78 | 0.5 |
| DC_8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12A_n5A | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12A_n66A | 12 | 0.5 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n51 | n51 | 0.2 |
| DC_20_n77 | n77 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_26A_n77A | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28A_n51 | n51 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_39_n78 | n78 | 0.5 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n77 | 40 | 0.4 |
|  | n77 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_42_n51 | n51 | 0.2 |
| DC_66A_n78A | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n78 DC_1-7-7_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n7-n78 | 1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-18_n78 | n78 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 1 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n78 | 1 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1_n40-n78 | n78 | 0.5 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n77-n79 | 1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_1_n78-n79 | 1 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_1-SUL_n78-n84 | n78 | 0.5 |
| DC_2_5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-66_n71B | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3_n1-n77 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n3-n77 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n3-n78 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n78, DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n7-n78 | 3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-19_n79 |  |  |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n20-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n28 | 0 |
|  | n78 | 0.5 |
| DC_3-38_n78 | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_3_n40-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-41_n78 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_3-SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-SUL_n78-n82 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_5_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_7-7_n78 | 7 | 0.0 |
|  | n78 | 0.5 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_7-46_n78 | n78 | 0.5 |
| DC_8A-SUL_n78-n81 | 8 | 0.2 |
|  | n78 | 0.2 |
| DC_18-28_n77 | n77 | 0.5 |
| DC_18-28_n78 | n78 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | 19 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_19_n78-n79 | 19 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_20_n8-n75 | 20 | 0.0 |
|  | n8 | 0.0 |
|  | n75 | 0.0 |
| DC_20_n28-n75 | 20 | 0.0 |
|  | n28 | 0.2 |
|  | n75 | 0.0 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n75-n78 | 20 | 0.0 |
|  | n75 | 0.0 |
|  | n78 | 0.5 |
| DC_20_n76-n78 | 20 | 0.0 |
|  | n76 | 0.0 |
|  | n78 | 0.5 |
| DC_20-SUL_n78-n82 | n78 | 0.5 |
| DC_20-SUL_n78-n83 | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | 21 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_21_n78-n79 | 21 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_28-SUL_n78-n83 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_28_n8-n258 | 28 | 0.1 |
|  | n8 | 0.2 |
| DC_28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_41-42_n79 | 42 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_66-SUL_n78-n86 | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28 | n28 | 0.2 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 DC_1-3_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 or n7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-8_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
| DC_1-3-19_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-7-20_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18-28_n77 | n77 | 0.5 |
| DC_1-18-28_n78 | n78 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-42_n79 | 42 | 0.5 |
| DC_1-20_n28-n78 | 1 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-42_n79 | 42 | 0.5 |
| DC_1-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_2-66-(n)71 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3-5-7_n78, DC_3-5-7-7_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.1 |
| DC_3-7-20_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-28_n78 DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-19-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-42_n78 | 0.2 | 0.2 |
|  | 0.5 | 0.5 |
|  | 0.5 | 0.5 |
| DC_3-19-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-20_n28-n78 | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21-42_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_3-28-42_n77 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-28-42_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-28-42_n79 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_5-7-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_7-20_n28-n78 | 7 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_19-21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-21-42_n79 | 42 | 0.5 |
| DC_21-28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |

7.3B.3.3.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-7-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-19-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-28-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-28-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-28-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21-42_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.2 |
| DC_1-3-21-42_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.2 |
| DC_1-3-21-42_n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_1-7-20_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n79 | 42 | 0.5 |
| DC_1-21-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_3-7-20_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

<End of Changes>