3GPP TSG-RAN WG4 Meeting #92	R4-1910596

# Ljubljana, Slovenia,  26th Aug, 2019 - 30th Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0048 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Background on the EVM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EVM measurement in the Annex F are not explained and documented which are quite important. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the background on the FR1 EVM measurement in the Annex F in TS38.141-1 2.Add the background on the FR2 EVM measurement in Annex F in TS38.141-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background information on the Annex F are missing in the TR38.817-02. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4, 9.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************* BEGIN CHANGE 1 ********************************

6.5.4	Modulation Quality

6.5.4.1	General

The modulation quality requirement is defined by the difference between the measured carrier signal and a reference signal.  Modulation quality can be expressed Error Vector Magnitude (EVM).

The core requirement for the conducted modulation quality for FR1 requirement will be reused from conducted modulation quality requirement from 3GPP TS 37.104 [7] specification.  Although some aspects, such as the requirement level is reused other aspects such as EVM window and equalization algorithms will need to differ to suit NR aspects such as spectrum utilization, reference signal structure, and numerologies. The requirements apply per TAB connector.

A DM-RS pattern will be transmitted, and a standardized receiver will be used to mitigate some linear aspects of the EVM. With the removal of Common Reference Signals (CRS) present in LTE and the new design of demodulation reference signals (DM-RS) the specifics of pattern configurations are needed to be considered as part of the overall requirement conditions.  The overall link performance was studied in a fading channel and also in an AWGN channel for different DM-RS pattern designs.  In fading channels during operation (as opposed to just test operation), the simulations showed that a minimum DM-RS density configuration of comb 2 (every other subcarrier) in symbols 3 and 11 would be required to achieve similar performance with CRS in LTE.

Figure 6.5.4-1: DM-RS patterns for DL Single Layer Front Loaded and Front and Back Loaded (pattern 1 and 2)

| EVM 8% for 64 QAM | EVM 3.5% for 256 QAM |
| --- | --- |

Figure 6.5.4-1: Throughput for PDSCH vs SNR simulation results [R4-1711162, Ericsson]

| EVM impacts on throughput performance, NR DMRS Pattern 1 | EVM impacts on throughput performance, NR DMRS Pattern 2 |
| --- | --- |

Figure 6.5.4-1: Throughput for PDCCH vs SNR simulation results [R4-1713672, NEC]

In the following subclauses, background on the EVM measurement is introduced in detail.

6.5.4.2	Example NRB Values

For NR specification, it is agreed to use 100 MHz channel bandwidth with 30 kHz SCS for FR1 as example case.

6.5.4.3	RSTP and OSTP

The RS TX power (RSTP) is mainly used for testing the DL RS power accuracy requirement as specified for UTRA and E-UTRA system, however for NR specification, DL RS power accuracy requirement is not specified.

The OFDM symbol TX power (OSTP) is required for testing total power dynamic range. In the TS 36.141 [27] specification, the RETP and OSTP is defined as following with considerations on the supported SCS.

RETP=Z't,f2Δf

where Δf is the subcarrier spacing in Hz.

In the TS 36.141 [27] specification, from RETP the OFDM Symbol TX power (OSTP) is derived as follows:

For NR system,the 4th symbol is used to explicitly indicate which OFDM symbols should be used for single RB power measurement and whole RB power measurement.

From RETP the OFDM Symbol TX power (OSTP) is derived as follows:

OSTP=RETP

Where the summation accumulates NRBNscRB RETP values of the 4th OFDM symbol. The 4th (out of 14 OFDM symbols within a slot in case of normal CP length) contains exclusively PDSCH. NscRB=12

6.6.7.4	TDD EVM Measurements

Moving average length 19 from E-UTRA specification is to be used for FR1 in NR.

6.6.7.5	EVM window

For the configurations, the example case for FR1 is 100MHz channel bandwidth with 30 kHz SCS, then sampling rate should be 122.88Msps, therefore is 1 and is 64 for Tc (1/480kHz * 4096).

is on Tf=144 () within  the CP of length 288 ()  (in OFDM symbol    or  )

is on Tf=208 () within  the CP of length 352 ()  (in OFDM symbol   or  ).

9.6.7	OTA modulation quality for FR2

9.6.7.1	General

The EVM requirement for FR2 shall apply also to the range of directions which is described in section 9.6.6.

The intention of the following description is to summarize the parameters used to determine appropriate EVM levels.

A DM-RS pattern will be transmitted, and a standardized receiver will be used to mitigate some linear aspects of the EVM. Additionally, common phase error compensation algorithm is also needed to be applied in addition to the simple equalization algorithm. For this, an additional reference signal is needed, the so-called phase tracking reference signal (PT-RS).

To balance between reference signal overhead and throughput a balance of DM-RS and PT-RS along with achievable EVM levels were studied.

PTRS every 4th OFDM symbol

PDCCH     DM-RS       PT-RS     PDSCH

Figure 9.6.7-1: TYPE-1 Pattern front loaded DM-RS (1 DM-RS symbol)

In the following subclause, background on the EVM measurement are introduced in details.

9.6.7.2	Example NRB Values

For NR specification, it is agreed to use 100 MHz channel bandwidth with 30 kHz SCS for FR1 and 400 MHz channel bandwidth with 120 kHz SCS for FR2 as example cases.

9.6.7.3	RSTP and OSTP

The considerations in section 6.5.4.3 could be applied for FR2.

9.6.7.4	TDD EVM Measurements

Moving average length 19 from E-UTRA specification is to be used for FR2 in NR.

9.6.7.5	EVM window

For the configurations, the example case for FR2 is 400MHz BW with 120 kHz SCS, then sampling rate should be 491.52 Msps, therefore is 3 where is 64 for Tc (1/480kHz * 4096).

is on Tf=144 () within  the CP of length 288 ()  (in OFDM symbol    or  )

is on Tf=400 () within  the CP of length 352 ()  (in OFDM symbol   or  ).

******************************* END OF CHANGE 1 ********************************