SA WG2 Meeting #143E e-meeting	S2-210xxxx

Feb 24 – Mar 09, 2021, Elbonia	(revision of S2-210xxxx)

Source:	CATT

Title:	TS 23.304: Subscription to ProSe

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution provides description on Subscription to ProSe into TS 23.304.

# 1.	Introduction

Based on the conclusions of TR 23.752, this contribution added description on Subscription to ProSe into TS 23.304.

# 2.	Proposal

It is proposed to capture the following changes into TR 23.304.

- * * * * First change * * * *

## 5.8	Subscription to ProSe

The user's profile in the UDM contains the subscription information to give the user permission to use ProSe.

At any time, the operator can remove the ProSe UE subscription rights from user's profile in the UDM, and revoke the user's permission to use ProSe.

a) The following subscription information is defined for ProSe:

-	subscription for open ProSe Direct Discovery for LTE PC5 and for NR PC5:

-	open ProSe Direct Discovery Model A.

-	subscription for restricted ProSe Direct Discovery for LTE PC5 and for NR PC5:

-	restricted ProSe Direct Discovery Model A;

-	restricted ProSe Direct Discovery Model A with application-controlled extension;

-	restricted ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted ProSe Direct Discovery Model B.

-	subscription for Broadcast mode ProSe Direct Communication for NR PC5, and subscription for one-to-many ProSe Direct Communication for LTE PC5.

-	subscription for Groupcast mode ProSe Direct Communication for NR PC5.

-	subscription for Unicast mode ProSe Direct Communication, including for NR PC5, and subscription for one-to-one ProSe Direct Communication for LTE PC5..

-	UE-PC5-AMBR for NR PC5.

b) Additional parameters related to the ProSe Direct service may be stored in the user's profile, such as:

-	the list of the PLMNs where the UE is authorised for open Direct Discovery Model A, i.e. to announce or monitor or both. For each PLMN in the list, the RAT(s) over which the UE is authorised to perform Direct Discovery Model A over PC5 reference point.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model A, i.e. to announce or monitor or both. For each PLMN in the list, the RAT(s) over which the UE is authorised to perform restricted ProSe Direct Discovery Model A over PC5 reference point.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model B , i.e. to perform Discoverer operation or Discoveree operation or both. For each PLMN in the list, the RAT(s) over which the UE is authorised to perform restricted ProSe Direct Discovery Model B over PC5 reference point.

-	the list of the PLMNs where the UE is authorised to perform Broadcast mode ProSe Direct Communication for NR PC5, and the list of the PLMNs where the UE is authorised to perform one-to-many ProSe Direct Communication for LTE PC5.

-	the list of the PLMNs where the UE is authorised to perform Groupcast mode ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to perform Unicast mode ProSe Direct Communication, and the list of the PLMNs where the UE is authorised to perform one-to-one ProSe Direct Communication for LTE PC5.

-	PC5 QoS parameters as defined in clause 5.7.1 used by NG-RAN.

The UDM may retrieve a) from the UDR. a) is provided by the UDM to the AMF during UE Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [xx] using Nudm_SDM service for Subscription data type "ProSe Subscription data" and the AMF provides a) to NG-RAN as part of the UE context information.

b) is provided by the UDR to the PCF during the UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [xx] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [xx] using Nudr service for Data Set "Policy Data" and Data Subset "Policy Set Entry". If the subscription information provided to the PCF from UDR is changed (e.g., PC5 QoS related parameters), the PCF initiates the UE Configuration Update procedure as specified in clause 6.2.2.

- * * * * End of changes * * * *