SA WG2 Meeting #140e	S2-200

- Aug 19th – Sept 2nd, 2020 ; Elbonia         		 	(revision of S2-200)

Source:	Nokia, Nokia Shanghai Bell

Title:	New solution: UAV and networked UAVC identification, authentication/authorization and C2 communication link establishment

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract of the contribution: This paper proposes solution for key Issues #2, #3, #5, #6 and #7

- 1 Introduction

The paper provides simple solutions to KI#2,3,5,6 & 7 that are applicable for both 5GC and EPC. The solution proposed in the paper are designed with two key principles in mind

- Provide a simple solution that is applicable for both 5GC and EPC

- Keep the network and UE impact to minimum that will allow faster implementation and time to market

An USS/UTM may perform various authentication/authorization tasks like (and not limited to) Authentication of UAV HW/SW, Authentication of UAVC HW/SW, Authorization of UAVC to control the UAV, Identification of the pilot operating the UAVC, Flight path authorization, C2 communication authorization between the UAV and UAVC etc.

The USS/UTM may also perform some additional tasks like configuring the UAV/UAVC, remote identification of the UAV, tracking of the UAV and managing the C2 communication between the UAV and UAVC.

The proposal is that the UAV/UAVC establishes separate PDU sessions as mentioned below:

- One PDU session for communication with USS/UTM

- Using this PDU session the UAV/UAVC may register with the USS/UTM and provide necessary information that the USS/UTM may need to allow the flight to operate

- This PDU session is also used for further communication with USS/UTM for:

- Networked Remote Identification (NRID)

- Tracking the UAV

- For configuring necessary C2 communication related information in the UAV/UAVC

- and for any other specific communication between USS/UTM and UAV/UAVC that are not related to command and control (C2) communication.

- A second PDU session only for C2 communication

- During this PDU session establishment the USS/UTM authenticates the PDU session requests for C2 communication by the UAV/UAVC. It further also authorizes the UAVC to take control of the UAV and configures the IP address (and other information) required for C2 communication. The USS/UTM may further perform tasks like flight path authorization etc. during the PDU session establishment.

- Once the PDU session is established, this PDU session is further used only for C2 communication between UAV and mutually exclusively an UAVC or a TPAE or an USS/UTM.

- Another one or more PDU session(s) for internet connectivity, that may be used by the UAV for sending field operation specific data e.g. streaming video etc

# 2 Proposal

It is proposed to update TR 23.748 as follows

* * * * First Change * * * *

- 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 | X | X | X | X | X | X | X |
| 3 |  | X |  |  |  |  |  |
| 4 | X | X | X |  |  |  | X |
| 5 | X | X | X |  |  |  | X |
| 6 | X | X |  |  | X |  |  |
| 7 | X | X |  |  | X |  |  |
| 8 | X |  |  |  |  |  |  |
| 9 | X |  |  | X |  |  |  |
| 10 |  | X |  |  |  |  |  |
| 11 |  | X |  |  |  |  |  |
| 12 |  | X |  |  |  |  |  |
| 13 |  |  |  | X |  |  | X |
| 14 |  |  |  | X |  |  |  |
| 15 |  |  |  | X |  |  |  |
| 16 |  |  |  | X | X |  |  |
| 17 |  | X |  |  | X |  |  |
| 18 |  |  |  |  | X |  |  |
| 19 |  |  |  |  | X |  |  |
| 20 |  |  |  |  |  | X |  |
| 21 |  |  |  |  |  | X | X |
| X |  | X | X |  | X | X | X |

* * * * Next Change * * * *

- 6.X	Solution #X: UAV/UAVC authentication/authorization by USS/UTM and C2 communication establishment

- 6.X.1	Introduction

This solution addresses Key Issue #2, #3, #5, #6 and #7. This solution is applicable to both 5GC and EPC. The solution facilitates below capabilities:

- Authenticate/authorize PDU session establishment request by an UAV for C2 communication

- Authenticate/authorize PDU session establishment request by a networked UAVC for C2 communication

- Obtain UAV and networked UAVC IP address for C2 communication (from 3GPP network) and perform necessary C2 communication pairing configuration.

- Initiate re- authentication/ re-authorization when deemed necessary by the USS/UTM

- Revoke authentication/authorization and tear down the PDU sessions

- 6.X.2	Functional Description

#### 6.X.2.1	Assumptions

The solution is based on below assumptions:

- An UAS consists of one UAV controller (UAVC) and one UAV. A single USS/UTM is serving both the UAV and the UAVC associated within an UAS

- The process for associating the UAV and UAVC to form an UAS is outside the scope of 3GPP and is assumed to be done at the USS/UTM.

- The UAV and UAVC forming the UAS may be served by different MNO. The identification, authentication / authorization of the UAVC and UAV can happen independently.

- The registration of the UAV with aviation authority (CAA) is outside the scope of 3GPP. Once the UAV is registered to the CAA, a CAA-Level UAV ID is assigned to the UAV.

- At 3GPP level the UAV and the networked UAVC are each publicly identified by an MNO assigned GPSI in the form of an External Identifier.

- The UAV and the networked UAVC establishes separate PDU sessions with dedicated DNN/S-NSSAI (or dedicated APNs in EPC) for

-	Communication with USS/UTM (e.g. OAM purpose, Networked Remote Id, Tracking etc)

-	Command and control (C2) communication between UAV and mutually exclusively an UAVC or a TPAE or an USS/UTM

-	Possibly One or more PDU sessions (PDN connection in EPC) for other communication purposes (e.g. for video streaming etc)

- The UAV/UAVC gets via means out of scope of 3GPP a signed USS/UTM address and this is included by the UAV/UAVC in the request for the PDU session establishment for C2 communication. How the signed USS/UTM address is configured in UAV/UAVC is outside the scope of 3GPP.

- Mapping between CAA-Level UAV ID and External Identifier is maintained by the USS/UTM.

- For EPC, the PDN connections used by UAV/UAVC are served by a combo SMF+PGWc regardless of whether the UAV/UAVC support 5G NAS or whether their subscription allows access to 5GC. This allows reusing common procedures between EPC and 5GC; and of course allows UAV/UAVC to freely move between 5GC and EPC coverage.

- The solution re-uses (with necessary adaptation) Secondary authentication/authorization by a DN-AAA, in order for the USS/UTM to be able to control the peers of C2 communications

NOTE 1:	re-using (with necessary adaptation) of Secondary authentication/authorization by a DN-AAA allows to reuse an existing feature (limiting the cost of introduction of the UAS support by 3GPP systems) that can equally apply over EPC and 5GC

NOTE 2: 	APN used by the UAV for contacting the USS/UTM for C2 communication resolves to SMF+PGWc only

#### 6.X.2.2	Overview of the solution

An UAS consists of one UAV controller (UAVC) and one UAV. The UAVC can be a networked meaning the UAVC is connected to a 3GPP network and connected to the UAV via the 3GPP network or the UAVC can be non-networked meaning the UAVC is not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP e.g. internet.

Based on the type of UAVC (networked or non-networked) used in an UAS, the complete solution can be viewed in two different representation from 3GPP point view.

An overview of the solution is first provided for the case of a non-networked UAVC;

NOTE 1: 	Despite the fact that a 5GC wording is used in the figure, it applies also to EPC with e.g. PDU sessions replaced with PDN connections

Figure 6.X.2.1-1: Overall solution for an UAV with non-networked UAVC

The UAV establishes multiple PDU sessions.

In Error: Reference source not found6.X.2.1-1 above, PDU session1 is used for communication with USS/UTM. This PDU session can be used by the UAV to register with the USS/UTM and provide any further information that the USS/UTM may need to register the UAS.  This PDU session is further used for any communication with USS/UTM that are not related to command and control (C2) operations. For example: for Networked Remote ID (NRID) or for tracking purpose or even for USS/UTM to update certain C2 communication related configuration in the UAV.

For C2 communication the UAV establishes a 2nd PDU session. The USS/UTM authenticates/authorizes this PDU session establishment request for C2 communication by the UAV. After the 2nd PDU session establishment, the USS/UTM receives the C2 communication IP address(es) for the UAV from the 3GPP network. The USS/UTM may use this IP address to configure the authorized UAVC to control the UAV, however, how this configuration is done is outside the scope of 3GPP.

When the UAVC IP address for C2 communication is available with the USS/UTM, it configures this IP address in the UAV and may also further do some additional configuration in the UAV to enable C2 communication. PDU Session1 of the UAV may be used for such communication between UAV and USS/UTM.

The USS/UTM also provides the C2 communication IP address of the UAVC to the 3GPP network anchoring the PDU session for C2 communication of the UAV: as part of authorization data, the USS/UTM configures this IP address in an Access Control List (ACL) for the PDU session such that, using the 2nd PDU session the UAV is able to access only the configured IP addresses meant for C2 communication that are authorized by the USS/UTM.

The UAV can now connect on the C2 communication link over the PDU session2

NOTE 2: 	SMF are configured to associate the PDU Session for C2 (corresponding to a specific (DNN, S-NSSAI)) with an empty default ACL; this means that as long as the USS/UTM has not authorized an IP address in the ACLF, no communication is possible on such PDU session.

NOTE 3:	it is possible that by default the USS/UTM puts in the ACL it configures in the network its own IP in order to be able itself to take control of the UAV in case of issue ; this depends on USS/UTM policies that are out of scope of 3GPP.

NOTE 4: The UAV may establish additional one or more other PDU sessions for various other purposes and are not shown in above.

Error: Reference source not found6.X.2.1-2 below shows the solution overview with a networked UAVC. The UAV UE behaviour is independent of whether the UAVC is a networked or non-networked. In the case of a networked UAVC, as can be seen from the Figure 6.X.2.1-2, also establishes 2 different PDU sessions, one for communication with USS/UTM and another one specifically for C2 communication.

The UAV and networked UAVC registers to 3GPP network and establishes PDU sessions independent of each other. It is also possible that the UAV and UAVC are served by different MNOs. However, from UAS operations point of view both are served by a single USS/UTM system.

In the case of a networked UAVC, the C2 communication authentication/authorization and pairing configuration can be facilitated by the 3GPP, similar to how it works for an UAV.

Figure 6.X.2.1-2: Overall solution for an UAV with networked UAVC

#### 6.X.2.3	Usage of secondary DN AAA authentication / authorization

For 5GC the solution makes use of the feature “Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session” provided by 3GPP (TS 23.501, clause 5.6.6 and TS 23.502, clause 4.2.3.2) to facilitate the USS/UTM to authenticate/authorize the UAV and networked UAVC.

For EPC the solution also uses “Secondary authentication/authorization by a DN-AAA server during PDN connection establishment”. The SMF+PGWc will make use of this procedure to facilitate the USS/UTM to authenticate/authorize the UAV and networked UAVC.

USS/UTM will have the functional entity to work as a DN-AAA and perform the EAP based authentication of the UAV/UAVC.

Usage of Secondary authentication/authorization by a DN-AAA server is further defined in clause 6.X.3

The UAV/UAVC provides the necessary identifiers and information needed by the USS/UTM system. These may include (and not limited to and is out of scope of 3GPP, thus not further described):

- UAV SN to identify the UAV HW

- CAA-Level UAV ID

- Other USS/UTM specific Identifiers/information to identify, authenticate/authorize the UAV controller, the pilot etc.

- Additional aviation specific information like Current location, Flight information, flight authorization specific information, location (latitude, longitude, altitude) of the UAV controller etc.

PDU session (PDN connection in EPC) establishment for C2 communication is continued only after the USS/UTM has successfully authenticated/authorized the UAV/UAVC. The authorization data is also provided to the 3GPP network.

The 3GPP system or the USS/UTM may initiate a re-authentication/re-authorization of the UAV/UAVC at any point of time.

The USS/UTM may at any point revoke the authorization for PDU session for C2 communication, forcing 3GPP network to tear down the PDU session for C2 communication or change the ACL associated with the PDU Session for C2 to e.g. remove the UAVC IP address from the list of authorized IP address and e.g. replace it with the IP address of a TPAE.

- 6.X.3	Procedures

#### 6.X.3.1	Authentication/authorization and C2 communication establishment of UAV/UAVC by USS/UTM via 5GC network

The procedure for UAV/UAVC authentication/authorization for C2 communication establishment by the USS/UTM during PDU session establishment in a 5GC network is depicted in Figure 6.x.3.1-1 below

1. The UAV/UAVC registers to the 3GPP network. The UE’s subscription information obtained by AMF from the   UDM has an indication for support for Aerial UE function and the AMF provides the subscription information on Aerial UE authorization to the gNB.

NOTE 1:  	It is assumed that the UAV has been registered to the CAA level authority, a CAA-Level UAV ID is available with the UAV/UAVC and the UAV/UAVC has a USS/UTM address configured.

2. The UAV/UAVC sends PDU session establishment request to the serving AMF.

3. Based on the UAV/UAVC subscription information, the requested DNN and S-NSSAI provided in the request the AMF selects the SMF and sends Nsmf_PDUSession_CreateSMContext Request to the SMF.

4. The SMF obtains subscription data from the UDM for the given SUPI obtained from the AMF. The SMF sends Nsmf_PDUSession_CreateSMContext  Response to AMF and continues with further PDU session establishment procedure.

5. AMF sends PDU session establishment accept to the UE and the UE can now communication with the UTM/USS

6. The UAV/UAVC may use this PDU session for registering to the USS/UTM and for C2 communication specific configuration (outside the scope of 3GPP and happens transparently at the application level). This PDU session is also used for Networked Remote ID (NRID) and tracking related communication with USS/UTM. The UAV/UAVC may also get a signed USS/UTM address using this PDU session, that will be provided by the UAV/UAVC during PDU session establishment for C2 communication, for a secondary authentication/authorization by the USS/UTM acting as a DN-AAA.

7. Once the UAV/UAVC is registered to the USS/UTM and ready for flight operation, it initiates a new PDU session establishment request for C2 communication. The UAV/UAVC shall include the signed USS/UTM address and CAA-Level UAV ID PDU session establishment request.

8. Based on the UAV/UAVC UE’s subscriber profile, DNN and S-NSSAI provided in the request the AMF selects the SMF and sends Nsmf_PDUSession_CreateSMContext Request to the SMF

9. The SMF obtains subscription data from the UDM for the given SUPI obtained from the AMF.

The SMF checks the subscription data whether a secondary authentication is required for the PDU session request and whether the UE request is allowed according to the user subscription and local policies. If the secondary authentication is required and the UE request is allowed, the SMF may further send an EAP request to the UAV/UAVC to fetch the USS/UTM specific identifier (e.g. CAA-Level UAV ID) and a USS/UTM address for secondary authentication if it was not already provided by the UAV/UAVC in step 7 above.

Figure 6.x.3.1-1: Authentication/authorization and C2 communication establishment for UAV/UAVC by USS/UTM via 5GC

The SMF shall verify that the USS/UTM address provided by the UAV/UAVC is signed and trusted. If the USS/UTM address provided by the UAV/UAVC is an FQDN, SMF uses DNS resolution to find out the IP address of the USS/UTM. If there is no existing N4 session to carry the EAP authentication/authorization related message to the USS/UTM, then the SMF selects an UPF and triggers N4 session establishment.

NOTE:	Security solutions for how the signed USS/UTM address is generated/conveyed to the UAV/UAVC and the verification of the signed USS/UTM address by 3GPP network will be covered at SA3

NOTE 4: 	If both the UAV and UAVC are served by the same MNO, the SMF may select the same local UPF for both the UAV and UAVC, to support optimised User Plane C2 communication between the UAV and UAVC

10. The SMF initiates the authentication process with the USS/UTM via the UPF to authenticate the C2 communication request for the CAA-Level UAV ID. The SMF also includes the External Identifier of the UAV/UAVC (as part of GPSI) in the authentication request. The UPF transparently relays the message received from the SMF to the USS/UTM

11. The USS/UTM and the UAV/UAVC exchange multiple EAP messages as required by the EAP method to perform authentication/authorization

12. Upon successful authorization, the USS/UTM notifies the SMF on the authentication/authorization result. The USS/UTM includes authorization data for the established PDU Session to the SMF and also requests to get notified with the IP address(es) allocated to the PDU Session. The authorization data may include an Access Control List (ACL) for the PDU session. The SMF maintains the session with USS/UTM for future communication like sending the IP address(es) allocated for the PDU session or release/update of PDU session or Secondary Re-authentication/Re-authorization or authorization revocation by USS/UTM etc.

The USS/UTM stores a mapping between CAA-Level UAV ID and the External Identifier. This External Identifier can be used at later point by the USS/UTM for accessing various services exposed by 3GPP network e.g. location information retrieval, monitoring event configuration etc.

13. After successful authorization by the USS/UTM, further PDU session establishment procedure is continued. Since the UAV and the UAVC may be served by different 3GPP PLMN, the SMF must ensure that the IP address allocated for PDU session (that will be further used for C2 communication) is routable and not subject to any Network Address Translation (NAT).

14. On completion of the PDU session establishment for C2 communication, the SMF notifies the USS/UTM the IP address(es) allocated to the UAV/UAVC.

15. USS/UTM provides necessary access control list information, so that the PDU session is used only between the paired UAV and UAVC/TPAE. If this pairing information is not available at PDU session establishment, the USS/UTM may provide its own IP address as a UAV controller. And later when the UAVC IP address is available to the USS/UTM, it may update the access control list to allow C2 communication between the UAV and UAVC using this PDU session.

The access control configuration can be updated by the USS/UTM at any point to manage the C2 communication with the UAV. For example, if the USS/UTM may update the access control to remove the UAVC and handover the control to a TPAE or to itself.

16. USS/UTM may now further configure the C2 communication related information on the UAV/UAVC. This is done at the application level, using PDU session that was established in Step 5, and the complete communication is transparent to 3GPP

16a. [Outside the scope of 3GPP]: If the PDU session establishment for C2 communication was from a UAV, and if the USS/UTM has the paired UAVC IP address available, it configures the IP address in the UAV for C2 communication. The USS/UTM may do additional C2 communication related configuration also at this point.

16b. [Outside the scope of 3GPP]: If the PDU session establishment for C2 communication was from a UAVC, and if the USS/UTM has the paired UAV IP address available, it configures the IP address in the UAVC for C2 communication. The USS/UTM may do additional C2 communication related configuration also at this point.

C2 communication channel is now available between the UAV and UAVC (or a TPAE or USS/UTM)

At any time after the PDU Session establishment, the USS/UTM server or SMF may initiate Secondary Re-authentication procedure for the PDU Session.

#### 6.X.3.2	Authentication/authorization and C2 communication establishment of UAV/UAVC by USS/UTM via EPC network

The procedure for UAV/UAVC authentication/authorization for C2 communication establishment by the USS/UTM during PDN connection establishment in a EGC network is depicted in Figure 6.x.3.2-1 below

[1-5] The UAV/UAVC UE attaches to the EPC network. MME performs the primary authentication and downloads subscription data from HSS. The subscription data obtained by MME from the HSS has an indication for support for Aerial UE function and the AMF provides the subscription information on Aerial UE authorization to the eNB. The MME performs default bearer establishment during the attach procedure as per TS 23.401 Figure 5.3.2.1-1

Figure 6.x.3.2-1: Authentication/authorization and C2 communication establishment for UAV/UAVC by USS/UTM via EPC

6. The UAV/UAVC may use this default PDN connection for registering to the USS/UTM and for C2 communication specific configuration (outside the scope of 3GPP and happens transparently at the application level). This PDN connection is also used for Networked Remote ID (NRID) and tracking related communication with the USS/UTM.

Step7 – Step 14 (except 9) are as per TS 23.401 Figure 5.10.2-1

7. Once the UAV/UAVC is registered to the USS/UTM and ready for flight operation, it initiates a new PDN connection request for C2 communication. The UAV/UAVC includes the USS/UTM address and CAA-Level UAV ID PDN connection request in a PCO.

8. The APN used by the UAV/UAVC for this PDN connection request always resolves to SMF+PGWc only. The MME sends a Create Session Request to the selected SMF+PGWc.

[9-10]. The SMF+PGWc checks the subscription data whether a secondary authentication is required for the PDN connection establishment request and whether the UE request is allowed according to the user subscription and local policies and establishes a N4 session with UPF+PGWu (same behaviour as in 5GC case). If the secondary authentication is required, the SMF+PGWc sends back Create Session Response. If the CAA-Level UAV ID and USS/UTM address were not included in step 7, the SMF+PGWc includes a PCO in the Create Session Response requesting for the CAA-Level UAV ID and signed USS/UTM address. The SMF+PGWc also ensures (same behaviour as in 5GC case), with appropriate ACL (Access Control List) configuration at the UPF+PGWu, that the UE is not able to access any data network using this PDN connection until a secondary authentication is performed by the USS/UTM

[11-12].  The MME forwards the PCO containing EAP identity request to the UAV/UAVC in a NAS message as per TS 23.401 Figure 5.4.3-1. The UAV/UVAC sends back a NAS message with PCO containing CAA-Level UAV ID and signed USS/UTM address.

[13-15]. MME sends a Modify Bearer Request including the PCO to the SMF+PGWc. The SMF+PGWc then sends back a Modify Bearer Response to the MME and establishes a DN-AAA session with the signed USS/UTM address provided by the UAV/UAVC for secondary authentication of the PDN connection. The SMF shall verify that the USS/UTM address provided by the UAV/UAVC is signed and trusted. If the USS/UTM address provided by the UAV/UAVC is an FQDN, SMF+PGWc uses DNS resolution to find out the IP address of the USS/UTM (same behaviour as in 5GC case). The SMF+PGWc includes CAA-Level UAV ID, the External Identifier of the UAV/UAVC and the IP address allocated to the UAV/UAVC for the PDN connection in the authentication request towards the USS/UTM (same behaviour as in 5GC case).

NOTE:	Security solutions for how the signed USS/UTM address is generated/conveyed to the UAV/UAVC and the verification of the signed USS/UTM address by 3GPP network will be covered at SA3

16. The USS/UTM further may exchange multiple EAP request/response messages with the UAV/UAVC. This EAP messages are transported via the Modify Bearer request/response between the SMF+PGWc and MME, and using NAS messages between MME and the UAV/UAVC. This step ends with the USS/UTM sending a final EAP Success to the UAV/UAVC

[17-18]. On successful authorization of the UAV/UAVC, the USS/UTM includes authorization data for the established PDU Session to the SMF+PGWc, which includes the IP address(es) that are allowed on the PDN connection. SMF+PGWc now updates the ACL in UPF+PGWu with the IP addresses provided by the USS/UTM in authorization data, such that using this PDN connection the UAV/UAVC is able to access only the configured IP addresses meant for C2 communication that are authorized by the USS/UTM. This behaviour is exactly the same as in 5GC case.

The SMF+PGWc maintains the session with USS/UTM for future communication like any changes in authorization data or release/update of PDN connection or Secondary Re-authentication/Re-authorization or authorization revocation by USS/UTM etc. This behaviour is exactly the same as in 5GC case

The USS/UTM stores a mapping between CAA-Level UAV ID and the External Identifier. This External Identifier can be used at later point by the USS/UTM for accessing various services exposed by 3GPP network e.g. location information retrieval, monitoring event configuration etc. This behaviour is exactly the same as in 5GC case

19. The USS/UTM may now further configure the C2 communication related information on the UAV/UAVC. This is done at the application level, using default PDN connection that was established in Step 5, and the complete communication is transparent to 3GPP

19a. [Outside the scope of 3GPP]: If the PDN connection is from a UAV, and if the USS/UTM has the paired UAVC IP address available, it configures the UAVC IP address in the UAV, for C2 communication. The USS/UTM may do additional C2 communication related configuration also at this point.

19b. [Outside the scope of 3GPP]: If the PDN connection is from a UAVC, and if the USS/UTM has the paired UAV IP address available, it configures the UAV IP address in the UAVC, for C2 communication. The USS/UTM may do additional C2 communication related configuration also at this point.

#### 6.X.3.3		Authentication/authorization revocation by USS/UTM

After the successful DN authentication/authorization, a session is kept between the SMF and the USS/UTM. In an EPC network, the session is maintained between the SMF+PGWc and USS/UTM.

When later on the PDU session gets released the SMF (SMF+PGWc in EPC) notifies the USS/UTM.

The USS/UTM server may revoke the authorization for PDU Session (PDN Connection in EPC) or update DN authorization data for a PDU session (PDN connection in EPC). According to the request from USS/UTM server, the SMF (SMF+PGWc in EPC) may release or update the PDU session (PDN connection in EPC).

- 6.X.4	Impacts on existing entities and interfaces

This solution may have the following impacts to existing entities and interfaces:

For 5GC:

- N1: UE provides the signed USS/UTM address and CAA-Level UAV ID in the PDU session establishment request

- SMF: Retrieves signed USS/UTM address and the CAA-Level UAV ID from the PDU session establishment request. The SMF shall use the USS/UTM address retrieved from PDU session establishment request as the DN-AAA address for secondary authentication. When the DN-AAA provides a success response for the secondary authentication result, use the authorization data to configure the ACL in UPF, such that the UE can access only the configures list of IP addresses using the PDU session.

For EPC:

- SMF+PGWc: Supports “Secondary authorization/authentication by an DN-AAA server during the PDU Session establishment” as explained in TS 23.502, clause 4.3.2.3. Retrieves the USS/UTM address and CAA-Level UAV ID included in the PCO by the UE. Additionally, same changes as described above for SMF in 5GC case.

- S1: Provides USS/UTM address and CAA-Level UAV ID to the SMF+PGWc in a PCO

- 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *