3GPP TSG-SA3 Meeting #121	S3-25xxxx

- Goteborg, Sweden, 7-11 April 2025

- Source:	Lenovo

- Title:	3GPP Cryptographic Inventory for EAP-AKA’/EAP-5G

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

This contribution proposes the inventory of EAP-AKA’ and 5G-AKA.

# 2	References

[1]		3GPP TR 33.938 “Study on 3GPP Cryptographic Inventory”

# 3	Rationale

This document provides content for the inventory of the EAP-AKA’ and 5G-AKA protocols.

# 4	Detailed proposal

Start of Changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[4]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

[a]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[b]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[c]	3GPP TS 35.205: "3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*".

[d]	3GPP TS 35.231: "Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification".

[e]	3GPP TS 35.234: "Specification of the MILENAGE-256 algorithm set; An example set of 256-bit 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5, f5* and f5**; Document 1: General".

Next Changes

## 4.2	3GPP Symmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined onces the detailed protocol list description has been agreed.

Table 4.2-1: 3GPP Symmetric Cryptographic Algorithms (5G System)

| Protocol | 3GPP Cryptographic Profile | Cryptographic Algorithm(s) | Feature(s) |
| --- | --- | --- | --- |
| e.g., PDCP (TS 38.323[]) | TS 33.501 [4] | 128-NxA1 | Confidentiality and Integrity Protection |
|  |  | 128-NxA2 | Confidentiality and Integrity Protection |
| EAP-AKA’ | RFC 5448 [a], TS 33.501 [4] | HMAC-SHA-256, f1, f1*, f2, f3, f4, f5, f5*, f5** | Primary authentication and key agreement |
| 5G-AKA | TS 33.501 [4] | HMAC-SHA-256, f1, f1*, f2, f3, f4, f5, f5*, f5** | Primary authentication and key agreement |

Next Changes

## 4.4	Detailed Protocol List

Editor’s Note: This detailed protocol list is expected to finalize first.

### 4.4.x	EAP-AKA’

EAP-AKA’ enables mutual authentication between the UE and AUSF and provides keying material that can be used between the UE and the serving network in subsequent security procedures.

The root key K and the SUPI are preconfigured in the USIM (in the UE) and in the UDM/ARPF.

EAP-AKA' is specified in RFC 5448 [a].

The 3GPP 5G profile for EAP-AKA' is specified in the normative Annex F of TS 33.501 [4].

KDF for key generation is HMAC-SHA-256 as per Annex B.2.0 of TS 33.220 [b].

EAP-AKA’ requires the following functions as described for 128 Bit MILENAGE in TS 35.205 [c] 128 Bit TUAK in TS 33.231 [d] and in TS 35.234 [e] for 256 Bit MILENAGE:

f0	the random challenge generating function;

f1	the network authentication function;

f1*	the re-synchronisation message authentication function;

f2	the user authentication function;

f3	the cipher key derivation function;

f4	the integrity key derivation function;

f5	the anonymity key derivation function.

f5*	the anonymity key derivation function for the re-synchronisation

f5** 	an alternative to f5* which provides additional protection against subscriber tracing (only TS 35.234).

### 4.4.y	5G-AKA

5G-AKA enables mutual authentication between the UE and AUSF with proof of successful authentication of the UE from the visited network. 5G-AKA provides keying material that can be used between the UE and the serving network in subsequent security procedures.

The root key K and the SUPI are preconfigured in the USIM (in the UE) and in the UDM/ARPF.

5G-AKA is specified in TS 33.501 [4].

KDF for key generation is HMAC-SHA-256 as per Annex B.2.0 of TS 33.220 [b].

5G-AKA requires the following functions as described for 128 Bit MILENAGE in TS 35.205 [c] 128 Bit TUAK in TS 33.231 [d] and in TS 35.234 [e] for 256 Bit MILENAGE:

f0	the random challenge generating function;

f1	the network authentication function;

f1*	the re-synchronisation message authentication function;

f2	the user authentication function;

f3	the cipher key derivation function;

f4	the integrity key derivation function;

f5	the anonymity key derivation function.

f5*	the anonymity key derivation function for the re-synchronisation

f5** 	an alternative to f5* which provides additional protection against subscriber tracing (only TS 35.234).

End of Changes