3GPP TSG-SA3 Meeting #126																				S3-26xxxx

Goa, India, 9 – 13 February 2026

# Source:	Interdigital

# Title:	PCR to 33.801-01 Annex B

# Document for:	Approval

Agenda Item:	5.3.1

# 1	Decision/action requested

Propose to approve the Annexure B skeleton described in this contribution

*** Start of Change ***

### B.2.1	Risk Analysis of MAC-CEs

Editor’s Note: This clause contains risk analysis output in a tabular easy to grasp format.

| MAC-CE | Function | Threat | Risk | Classification |
| --- | --- | --- | --- | --- |
| Timing Advance Command (TA CE) | Adjust UE uplink timing | Timing Advance spoofing | uplink desync, RACH failure | Critical |
| Power Headroom Report (PHR CE) | Report the remaining power margin | Power Headroom spoofing | energy drain or DoS | Major |
| Buffer Status Report (BSR CE) | Indicates UE buffer occupancy | Buffer Status Report spoofing | resource starvation or DoS | Major |
| Delay Status Report (DSR CE) | Reports on the delay status of buffered uplink data | Delay Status Report spoofing | resource starvation or DoS | Major |
| SCell Activation/Deactivation CE | Enables/disables SCells in CA | Spoofing of SCell enablement/disablement | Throughput degradation and DOS (battery drain) | Major |
| Discontinuous Reception Command (DRX Command CE) | Control UE sleep/wake cycles | Spoofing of DRX Command CE | Stealth DOS (Immediate loss of downlink availability) | Major |
| Beam Measurement Report CE | UE beam measurement feedback | Beam measurement spoofing/flooding | gNB overload | Major |
| Beam Failure Recovery CE | Trigger beam recovery | Beam Failure spoofing | Forced handover storms | Critical |
| Antenna Switch/Beam Index CE | Report or switch beam index | Spoofing of Antenna Switch/Beam Index CE | Immediate throughput degradation Packet loss without obvious RF interference | Major |
| LTM Cell Switch Command CE | Instructs a UE to switch to a new serving cell | LTM trigger spoof | Data loss or premature release | Critical |
| Candidate Cell TCI States Activation/Deactivation MAC CE | Enables efficient beam management during mobility in 5G NR | Spoofing of Candidate Cell TCI States Activation/Deactivation MAC CE | Downlink reception failure | Major |

### B.2.2	Risk Prioritization

Editor’s Note: This clause contains agreement on risk prioritization.

The following provides risk severity together with risk prioritization:

Critical Severity (Cat A) – direct impact on network stability or synchronization

- Timing Advance spoofing leading to uplink desync, RACH failure

- Beam Failure spoofing leading to forced handover storms

- LTM trigger spoof leading to data loss or premature release

Major Severity (Cat B) – scheduler/QoS manipulation or traffic misreporting

- Buffer/Delay Status Report spoofing leading to resource starvation or DoS

- Power Headroom spoof leading to energy drain or DoS

- Multi-cell activation replay leading to DU desynchronization

- Beam measurement flood leading to gNB overload

- TCI state activation/deactivation spoofing leading to TCI state desynchronization

Moderate Severity (Cat C) – privacy or efficiency impact

- DRX pattern leakage leading to activity profiling

- Beam index cycling leading to energy drain and DoS

Minor Severity (Cat D) – low impact, handled by upper layers

- Legacy CE replay leading to compatibility error

*** End of Change ***