3GPP TSG-SA3 Meeting #115 	S3-24xxxx

- Athens, Greece, 26 February-1 March 2024

- Source:	Huawei, HiSilicon

- Title:	pCR on a new KI on TNAP mobility

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

approve this new Key issue for inclusion in TR 33.702.

# 2	References

[1]	3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

# 3	Rationale

In TR 33.887[1], KI#4 is agreed and almost be concluded in R-18 phase. This contribution proposes to reuse the content in KI#4 of TR 33.887[1]. The only modification to KI#4 is a new sentence is added to reflect the scenarios that can be addressed by this key issue.

We think the N5CW device case can be covered by this key issue, because the N5CW device accesses to the 5GC by using the trusted non-3GPP access that is also the TNAP. The different is the TNAP will connect to the TWIF in case of N5CW, while in normal UE case it is to the TNGF. In a word, only the gateway is different.

# 4	Detailed proposal

## 5.X	Key issue #X: Security aspect of TNAP mobility without full authentication

### 5.X.1	Key issue details

This Key issue addresses the following use case: a normal UE, a N5CW device described in clause 4.

Mobility between two TNAPs within the same trusted Non-3GPP Access Network Gateway Function (TNGF) is not supported in 3GPP currently. For example, when UE moves between two nearby or overlapping TNAP1 and TNAP2, the connectivity will break. Therefore, UE services will be interrupted. The UE needs to reconnect, go through another authentication procedure to continue the service even though the second non-3GPP access connect to the same 5GC.

There could be some potential security solutions where UE switches the TNAP1 to TNAP2 without breaking the connectivity. However, the security aspects of optimizations of inter-TNAP mobility were never studied in SA3.

### 5.X.2	Threats

Not applicable.

### 5.X.3	Potential security requirements

5GS should support a mechanism for communication between the UE and TNAP/TNGF to establish security with a TNAP without performing full authentication when the UE switches from another TNAP within the same TNGF or TIMF.

While switching from one TNAP to another TNAP within the same TNGF, the interface between UE and the new TNAP shall be confidentiality, integrity, and replay protected.