3GPP TSG-SA3 Meeting #111 	S3-233028

- Berlin, Germany, 22 -26 May 2023													(revision of S3-yyxxxx)

- Source:	NTT DOCOMO, and others

- Title:	pCR to 33.884 - conclusions

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

It is proposed to accept the pCR in order to complete the conclusions

# 2	References

[1]	3GPP TS 33.884

# 3	Rationale

In order to complete the study on SNAAPPY, the concusions need to be completed. This contribution attempts to resolve the editor's notes by taking a minimalist approach of only specifying what is immediately necessary.

# 4	Detailed proposal

It is proposed to accept the following pCR for inclusion in TR33.884

++++++++++++++++ Begin changes ++++++++++++++

## 7.0 	High level conclusions

This conclusions are for enhancing CAPIF regarding resource owner awareness. Existing mechanisms without resource owner awareness are still available.

-	Authorization function is part of CCF

-	https is used as protocol between OAuth client and authorization server on the CCF.

Use case A: AF outside of UE is API invoker

-	For mutual authentication of API invoker AF and API exposing function in this use case, TS 33.122 [5] is reused.

-	For mutual authentication of API invoker AF and CCF in this use case, TS 33.122 [5] is reused.

-	For authorization, the OAuth2.0 Framework will be specified. The API invoker has the role of the OAuth client.

-	If the TLS-PSK-based mechanism and the PKI-based mechanism support the resource owner-aware feature, the CAPIF core function/authorization function and the AEF should contain the authorization information including the resource owner identity.

-	If the TLS-PSK-based mechanism and the PKI-based mechanism does not support the resource owner-aware feature, the AEF should support the following policies. The AEF should reject the request that contains resource owner identity but does not contain the token. The AEF should reject the request including the resource owner identity and the token that does not contain the resource owner identity.

-	PKCE flow and client credential flow provide a different user experience and support different application needs. Thus both flows will be specified in normative work.

-	The claim in the token includes resource owner identity, thus there is no need for additional UE authentication in API invocation.

-	For Authorization Code Flow or PKCE, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AuF.

- 	Based on the operator's policy, mutual authentication between resource owner and authorization function can be performed. [remove following requirement]

-	mutual authentication between resource owner and authorization function has to be performed.

-	the authentication method is out of scope of specification in this release

Use case B: API invoker residing on UE accessing its own resources

Conclusions applicable to both Subcase B.i) and B.ii) below:

-	For mutual authentication of API invoker and CCF in this use case, AKMA is used.

-	For authorization, the OAuth2.0 Framework is used. The API invoker has the role of the OAuth client.

-	mutual authentication between resource owner and authorization function has to be performed

-	the authentication method is out of scope of specification in this release

-	If the TLS-PSK-based mechanism and the PKI-based mechanism support the resource owner-aware feature, the CAPIF core function/authorization function and the AEF should contain the authorization information including the resource owner identity.

-	If the TLS-PSK-based mechanism and the PKI-based mechanism does not support the resource owner-aware feature, the AEF should support the following policies. The AEF should reject the request that contains resource owner identity but does not contain the token. The AEF should reject the request including the resource owner identity and the token that does not contain the resource owner identity.

-	whether API invoker onboarding needs to be specified is left to the normative phase

-	The access token issued by the authorization function contains the identity of the resource owner as a claim.

-	The API exposing function restricts the API requests to resources owned by the resource owner identified in the token claims.

Subcase B.i) API invoker part of third party application (e.g. single page application)

Client credentials flow with the procedure given in solution #6 is selected as a base for the normative work. This solution can be utilized for address both the cases that the resource owner is the user or the UE.

-	PKCE flow will be specified in normative work

Subcase B.ii) API invoker part of UE accessing its own resources

-	PKCE flow will be specified in normative work

Client credentials flow with the procedure given in solution #6 is selected as a base for the normative work. This solution can be utilized for address both the cases that the resource owner is the user or the UE.

General  issues:

-	UE A accessing resource of "UE B" is not specified in this release.

UE A accessing resource of UE B is out of scope of the present document.

Subscriber vs UE as resource owner:

-	Regulatory requirements can affect whether UE user or subscriber is considered resource owner. Therefore, it is left to configuration in the authorization function (i.e. CCF) whose authentication (subscriber or UE) is required for authorizing access to a specific resource

-	Within the token, an identifier is used that the AEF can map to the SUPI/IMSI(s) of the resource owner. How to identify the resources is left for stage 3 work.

Conclusion for revocation:

-	One or multiple revocation mechanisms are required.

-	For revocation, the resource owner interacts with the Authorization Function (i.e. CCF) via a web page.

-	The Authorization Function (i.e. CCF) informs the Application Exposing Function about revocation. The Application Exposing Function then ensures correct handling of the revocation.

-	CAPIF core function/authorization function initiated authorization revocation mechanism, which is defined in clause 8.23.4 of TS 23.222 [12], can be reused.  The AEF needs to receive revocation information, which contains the resource owner identity, from the CCF.

Editor's note: More details on revocation information received by AEF are FFS.

Editor's note: The conclusion on how the CCF/authorization function is triggered to do the revocation is FFS.

-	If the resource owner can ensure API invoker will trigger the revocation procedure, RFC 7009 can be reused.

-	Similar mechanism as giving authorization using Authorization Code Flow or PKCE can be used in order for a resource owner to request revocation to CCF/AuF.

-	Token should include a unique value to identify the token uniquely.

-	After CCF/AuF receives revocation request from a resource owner, CCF/AuF should inform the API exposing function of the revoked token, and the API exposing function should stores the token until the expiration time of the token.

+++++++++++++++ End of changes ++++++++++++++