3GPP TSG-SA3 Meeting #116	S3-24xxxx

Jeju, South Korea,  20th - 24th May 2024

- Source:	Xiaomi

- Title:	Conclusion for KI#1

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is proposed to approve this contribution to conclude KI#1.

# 2	References

[1]	3GPP TR 33.701 Study on mitigations against bidding down attacks

# 3	Rationale

It is proposed to approve this conclusion for key issue#1.

# 4	Detailed proposal

*** Start of Change ***

# X Conlusions

X.Y Conclusion for key issue #1

- The network shall deliver the PLMN ID and the corresponding GERAN/UTRAN decommissioning information to the UE via registration accept message.

Note: It is possible for a FBS to broadcast a PLMN ID (e.g., PLMN 2) different from that of serving network (e.g., PLMN 1). Then the UE may connect to the FBS even if the corresponding PLMN (e.g., PLMN 2) has completed GERAN/UTRAN decommissioning. To mitigate this attack, the decommissioning information of other networks (e.g., PLMN 2), once available in the serving network, also needs to be delivered to the UE. How the serving network obtains the decommissioning information of other networks is out of the scope of 3GPP.

- The network side provided GERAN/UTRAN decommissioning information has a higher priority than the classical PLMN selection information (e.g., PLMN/access technologies combination information configured in the UICC).

- If the home PLMN has completed GERAN/UTRAN decommissioning, such information needs to be made available in the UE, which shall not select a GSM/GERAN/UTRA cell in any serving network (because the HPLMN does not support a successful 2G/3G AKA for the UE).

*** End of Change ***