3GPP TSG-SA3 Meeting #116	S3-24xxxx

Jeju, South Korea,  20th - 24th May 2024

- Source:	Huawei, HiSilicon

- Title:	Preliminary conclusion proposal

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in TR 33.701 [1].

# 2	References

[1]	3GPP TS 33.701: " Study on mitigations against bidding down attacks "

# 3	Rationale

This contribution proposes a preliminary conclusion based on the common traits of the current solutions since they are similar. Therefore, the conclusion is formulated as a list of principles leaving some points open for decision during the normative phase based on other working groups feedback.

# 4	Detailed proposal

Approve the changes below for inclusion in TR 33.701 [1]

**** Start of Changes****

Y.	Conclusion

For the 5GS, it is agreed that the solution for the normative work is to be based on the following principles:

- The registration procedure is enhanced with a new IE so that the network (AMF) can convey to the UE information about RATs that are no longer supported by the current serving PLMN.

NOTE a: Whether this new IE is part of a new standalone mechanism or part of the existing mobility restriction mechanism is to be decided during the normative phase based on feedback from other working groups.

- The information pertains to the current serving PLMN over which the UE is registering or have registered over 3GPP access.

- The UE uses the information accordingly so that it does not select the provisioned RATs when accessing services for the PLMN in question.

- The information is provided after security activation over a protected NAS message, potentially the registration accept message.

- RAN involvement is not required in this new feature.

NOTE b: Whether the UE needs to signal a new capaibility and how is to be decided during the normative phase based on feedback from other working groups.

For the EPS, it is agreed that there is no need for any enhancements.

**** End of Changes****