3GPP TSG-SA3 Meeting #117	S3-24xxxx

Maastricht, Netherland, 19th – 23rd August 2024

- Source:	Xiaomi

- Title:	Key issue on authorizing API invoker on one UE accessing resources owned by another UE

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to approve this key issue for 3GPP TS33.700-22.

# 2	References

- 3GPP TR 33.700-22 Study on security aspects of CAPIF Phase3

# 3	Rationale

It is proposed to approve this key issue for TR 33.700-22 [1] to study potential solutions for CAPIF interconnection security.

# 4	Detailed proposal

*** Start of the first Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[X]	3GPP TR 23.700-22: "Study on CAPIF Phase 3".

[Y]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

*** Start of the second Change ***

## 5.X	Key Issue #X: Authorizing API invoker on one UE accessing resources owned by another UE

### 5.X.1	Key issue details

According to TR 23.700-22 [X], there are cases in which it is needed to support for API invoker(s) which are deployed on the UE accessing resources of other resource owners (users), e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

However, resource owner-aware northbound API access (RNAA) defined in TS 33.122 [Y] only supports authorizing API invoker on one UE to request resources owned by the same UE.

### Therefore, it is proposed to study how to authorize an API invoker on one UE to access resources owned by another UE.

### 5.X.2	Security threats

Malicious API invoker on one UE can access resources owned by another UE without permission.

### 5.X.3	Potential security requirements

The 5G system shall be able to authorize an API invoker on one UE to access resources owned by another UE.

*** End of the Changes ***