3GPP TSG-SA3 Meeting #117																S3-242xxx

Maastricht, Netherlands  19 - 23 August 2024

- Source:	Nokia, QC??

- Title:	TNGF solution

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the proposed solution for TR 33.700-32.

# 2	References

[1]	33.702 - Security for mobility over non-3GPP access to avoid full primary authentication

# 3	Rationale

As KI1 was agreed, proposing the normative text/based on the solution agreed conclusion..

# 4	Detailed proposal

* * * * Start of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[8]	3GPP TS 23.502: "Procedures for the 5G System".

[9]	3GPP TS 33.102: "3G security; Security architecture".

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[12]	IETF RFC 5448: " Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[13]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[14]	3GPP TS 35.215: " Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[26]	Void

[27]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[28]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[29]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[30]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[31]	3GPP TS 38.470: "NG-RAN; F1 General aspects and principles".

[32]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[33] 	3GPP TS 38.474: "NG-RAN; F1 data transport".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

[35]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[36] 	3GPP TS 35.217: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors' test data".

[37] 	3GPP TS 35.223: "Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 3: Implementors' test data".

[38]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[39]	Void

[40]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[41]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[42]	Void.

[43]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[44]	IETF RFC 7519: "JSON Web Token (JWT)".

[45]	IETF RFC 7515: "JSON Web Signature (JWS)".

[46]	IETF RFC 7748: "Elliptic Curves for Security".

[47]	IETF RFC 9113: "HTTP/2".

[48]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[49]	IETF RFC 6960: "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[50]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[51]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[52]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[53]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[54]	3GPP TS28.533: " Management and orchestration; Architecture framework".

[55]	3GPP TS28.531: "Management and orchestration of networks and network slicing; Provisioning".

[56]	Void

[57]	IETF RFC 7542: "The Network Access Identifier".

[58]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[59]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[60]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[61]	IETF RFC 5705,"Keying Material Exporters for Transport Layer Security (TLS)".

[62]	IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[63]	NIST Special Publication 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois Counter Mode (GCM) and GMAC".

[64]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[65]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[66]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[67]	IETF RFC 9048: "Improved Extensible Authentication Protocol Method for 3GPP Mobile Network  Authentication and Key Agreement (EAP-AKA')".

[68]	3GPP TS 29.510: "5G System; Network function repository services".

[69]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[70]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[71]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[72]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[73]	3GPP TS 29.573: " Public Land Mobile Network (PLMN) Interconnection; Stage 3".

[74]	3GP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[75]	IEEE TSN network aspects: see 3GPP TS 23.501 [2] references [95], [96], [97], [98], [104], and [107].

[76]	IETF RFC 9190: "EAP-TLS 1.3: Using the Extensible Authentication Protocol with TLS 1.3".

[77]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[78]	3GPP TS 38.401: "NG-RAN; Architecture description".

[79]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)"

[80]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012) - IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[81]	IETF RFC 2410 "The NULL Encryption Algorithm and Its Use With IPsec".

[82]	Void

[83]	RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[84]	RFC 8310: "Usage Profiles for DNS over TLS and DNS over DTLS".

[85]	RFC 4890: "Recommendations for Filtering ICMPv6 Messages in Firewalls".

[86]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[87]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[88]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[89]	IANA: "Transport Layer Security (TLS) Parameters".

[90]	Void

[91]	3GPP TS 33.535: "Authentication and key management for applications based on 3GPP credentials in the 5G System (5GS)".

[92]	3GP TS 29.573: "5G System; Public Land Mobile Network (PLMN) Interconnection".

[93]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[94]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition".

[95]	3GPP TS 29.502: "5G System; Session Management Services".

[96]	3GPP TS 29.526: "5G System; Network Slice-Specific Authentication and Authorization (NSSAA) services".

[97]	3GPP TS 23.402: "Authentication enhancements for non-3GPP accesses".

[98]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[99]	RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security              Authenticated Protocol Version 0 (EAP-TTLSv0)".

[100]	RFC 6678: "Requirements for a Tunnel-Based Extensible Authentication Protocol (EAP) Method".

[101]	General Data Protection Regulation, https://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=CELEX:02016R0679-20160504&from=EN.

[102]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[103] 	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[104]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[105]	3GPP TS 23.288: "Architecture enhancements for 5G System(5GS) to support network data analytics services".

[106]	3GPP TS 23.554 Application architecture for MSGin5G Service; Stage 2.

[107]	3GPP TS 22.262 Message service with the 5G System (5GS); Stage 1.

[108]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[109]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[110]	NIST Special Publication 800-90A (2015): "Recommendation for Random Number Generation Using Deterministic Random Bit Generators".

[111]	IETF RFC 4555 (2006-06): "RFC IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[112]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[113]	RFC 9110: "HTTP Semantics".

[114]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[I]	IEEE Std 802.11™-2020 Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

* * * * Next Change * * * *

# 7A	Security for trusted non-3GPP access to the 5G core network

## 7A.1	General

Security for trusted non-3GPP access to the 5G Core network is achieved when the UE registers to the 5GC via the TNAN. The UE registers to 5GC and, at the same time, it authenticates with the TNAN by using the EAP-5G procedure, similar to the one used with the registration procedure for untrusted non-3GPP access.

The link between the UE and the TNAN can be any data link (L2) that supports EAP encapsulation. The requirement on the Ta interface between the TNAP and TNGF can be found in clause 4.2.8.3.2 of TS 23.501[2]. The TNGF terminates the EAP-5G signalling andfowards the NAS message to the 5GC when the UE attempts to register to 5GC via the TNAN. The security relies on Layer-2 security between UE and TNAP, which is a trusted entity so that no IPSec encryption would be necessary between UE and TNGF, i.e. NULL encryption is sufficient for the user plane and signalling. However, integrity protection would be provided.

NOTE: The encryption protection over Layer-2 between UE and TNAP is assumed to be enabled.

Separate IPSec SAs may be used for NAS transport and PDU Sessions. At the end of the UE’s registration to 5GC, an IPSec SA (NWt) is established between the UE and TNGF. This is used to protect NAS messages between the UE and TNGF. Later when the UE initiates a PDU session establishment, the TNGF initiates establishment of one or more IPSec child SAs per PDU session. This results in additional IPSec SA’s (NWt) to be setup between the UE and TNGF-UP which are then for user plane transport between the two.

Clause 7A.2.X describes when UE moves from one TNAP to another TNAP, authentication of UE from the previous context without performing the full primary authentication.

Clause 7A.2.4 describes how WLAN UEs that do not support 5GC NAS (N5CW) register via trusted non-3GPP access. Those N5CW devices are able to authenticate to the network with 3GPP credentials and register with the help of an interworking function (TWIF) that provides the 5GC NAS protocol stack towards the AMF.

As defined in clause 7.1, it is the home operator policy decision if a non-3GPP access network is treated as trusted non-3GPP access network. When all of the security domains in clause 4.1 of the present specification related to the non-3GPP access network are considered sufficiently secure by the home operator, the non-3GPP access may be identified as a trusted non-3GPP access for that operator. However, this policy decision may additionally be based on reasons not related to security feature groups.

NOTE: It is specified in clause 7.1a of the current document how the UE gets the operator policy and how it will behave accordingly.

### 7A.2.X	Authentication for UE moving from one TNAP to another TNAP

When UE moves from one TNAP to another TNAP where both TNAPs are nearby or overlapping, the UE connectivity can break while connecting to the new TNAP. Additionally, UE also goes through another full primary authentication procedure, even though the second non-3GPP access connects to the same TNGF. A new full primary authentication may cause additional signaling and latency in the UE connection. This clause provides an optimisation via the Fast BSS Transition protocol as described in 802.11[I], through which primary authentication can be skipped and connection time will improve. The Fast BSS Transition protocol as described in 802.11[I] has certain elements with their properties:

- The R0KH and R1KH are responsible for the derivation of keys in the FT key hierarchy.

- For Fast BSS transition, the functions of the IEEE 802.1X Authenticator are distributed among the R0KH and R1KHs. The R0KH interacts with the IEEE 802.1X Authenticator to receive the key material resulting from an EAP authentication. The R1KH interacts with the IEEE 802.1X Authenticator to open the Controlled Port. Both the R0KH and R1KH interactions with the IEEE 802.1X Authenticator occur within the mobility domain.

- Each R0KH-ID and R1KH-ID is assumed to be expressed as a unique identifier within the mobility domain.

- R0KH and R1KH are collected in TNGF.

Figure 7A.2.X-1 Authentication for UE moving from one TNAP to another TNAP

- The UE wants to connect to the TNAP AP#1 that is advertising FT capability through inserting the MDE into Beacons and ProbeResponses. The MDE informs about that the AP is FT capable, the mobility domain ID (MDID) and the potential support of FT over DS. The UE and AP exchange 802.11 Authentication Request and Response.

If TNAP AP does not support FT, the AP shall not provide FT capability.

NOTE: MDID (mobility domain ID) is used to indicate a group of APs, between which a UE can use fast BSS transition services. Fast BSS transitions are allowed only between APs that have the same MDID. Configuration of MDID in TNAP is up to the implementation.

- UE supporting FT shall send a (Re)association Request to the TNAP AP#1 with a MDE included indicating that the UE wants to perform FT within the indicated mobility domain.

If the UE does not support FT, the UE shall not send (Re)association Request. Therefore, TNAP AP#1 shall perform authentication defined in 7A.2.1 without executing the below steps.

- UE connects to TNGF and EAP authentication  is performed, similar to  figure 7A.2.1-1, steps 2 to 19. Additionally,

- the TNGF shall derive a FT key, KFT, from KTNGF as specified in A.22 of the present document and send the KFT to the R0 key holder (R0KH). If a key hierarchy already exists for this UE belonging to the same mobility domain (i.e., having the same MDID), the R0KH shall delete the existing PMK-R0 security association and PMK-R1 security associations

- the UE shall derive a FT key, KFT, from KTNGF as specified in A.22 of the present document

- The UE and R0KH generate PMK-R0 (e.g., PMK) and PMKR0Name.

- UE and R0KH store the PMK-R0 (e.g. PMK) and PMKR0Name. the UE also stores R0KH-ID and R1KH-ID.

NOTE: Details of R0KH-ID, R1KH-ID and PMKR0Name are defined in 802.11[I].

- The UE finds another AP (called  target AP) advertising FT capability and  the same mobility domain. If the target TNAP does not advertise FT capability, or MDID value does not match with the previously stored MDID value, the UE shall connect to target TNAP following the procedure defined in clause 7A.2.1 and further steps are skipped.

- The UE sends an 802.11 Authentication Request to the Target AP TNAP AP#2) including MDE, R0KH-ID, PMKR0Name and SNonce.

- The TNAP AP#2 queries the R0KH for the required PMK-R1and fetches the PMK-R1 from the R0KH using R0KH-ID and PMKR0Name.

- The target AP sends an 802.11 Authentication Response to the UE including ANonce and R1KH-ID.

- The UE calculates PMKR1Name and initiates its reassociation to the target AP continuing the establishment of a key material for the new association.

- The UE sends a L2-802.11 Reassociation Request including PMKR1Name, ANonce, SNonce and MIC. The target AP checks the MIC and if successful installs the derived key material for the subsequent data exchanges.  The target AP sends a Reassociation Response including ANonce, SNonce and MIC. The UE checks the MIC and, if successful, installs likewise the material for the subsequent data exchanges.

- The UE and AP start securely exchanging data.

### 7A.2.3	Key hierarchy for trusted non-3GPP access

The key hierarchy described in clause 6.2.1 applies, with the following changes:

The key derived for non-3GPP access is called KTNGF in the context of trusted access.

The key KTNGF received from AMF is used for three different purposes; to setup IPSec SAs between the UE and the TNGF and to create WLAN keys between the UE and the TNAP and to create the root key for IEEE 802.11[I] Fast BSS Transition (FT).

To separate the keys for these purposes, the key hierarchy in Figure 7A.2.3-1 shall be used. The KTIPSec key is used to setup IPSec SAs and the KTNAP key is used to setup access security.

The keys KTIPSec, KTNAP, and KFT  are derived as described in Clause A.22.

Figure 7A.2.3-1 Key hierarchy for trusted non-3GPP access

# A.22	KTIPSec,KTNAP and KFTderivation function

When deriving a KTIPSec /KFT  from KTNGF and when deriving a KTNAP from KTWIF or KTNGF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x84

-	P0 = Usage type distinguisher

-	L0 = length of Usage type distinguisher (i.e. 0x00 0x01)

The values for the Usage type distinguisher are defined in table A.22-1. The values 0x00 and 0x04 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

The Usage type distinguisher shall be set to the value for IPSec (0x01) when deriving KTIPSec. The Usage type distinguisher shall be set to the value for TNAP (0x02) when deriving KTNAP. The Usage type distinguisher shall bet set to the value for FT (0x03) when deriving KFT.

The input key KEY shall be the 256-bit KTNGF or KTWIF.

Table A.22-1: Usage type distinguishers

| Usage type distinguisher | Value |
| --- | --- |
| IPSec | 0x01 |
| TNAP | 0x02 |
| FT | 0X03 |

* * * * End of Change * * * *