3GPP TSG-SA WG3 Meeting #123	S3-25xxxx

Goteborg, Sweden, August 25 - 29, 2025	S3-25xxxx

- Source:	OPPO

- Title:	Outstanding Issues After AIoT Call #1

- Document for:	Information

Agenda Item:	X.XX

1. Whether computation of RES for authentication is based on root key (e.g., Kaiot ) or based on a bootstrapping key (e.g., Kaiotf).

NOTE1: If root key is not used directly, this means key bootstrapping is needed to generate Kaiotf that is used for authentation.

2. Whether replay attack needs to be considered if there is only one command request after every inventory procedure.

NOTE2: Replay command request means also replay of the inventory procedure and that the replay of inventory procedure is successful.

3.Whether authentication is explicit or implicit.

NOTE3: Explicit authentication requires MAC (e.g., RES) for authentication and a MAC for command request message.  In implicit authentication, there is only one MAC for both authentication and message verification which means that if the MAC for the message is verified then authentication is also implicitly achieved.

4.Whether a second network generated RAND is needed for generating the session key for communication protection.

NOTE4: AIoT device may not be aware if there is a COMMAND request that follows the inventory procedure and may not “remember” the first RAND sent from the network if session key generation at the AIoT device takes place after the COMMAND request message.

5.Whether device generated RAND is needed.

6. Whether network RAND is generated at the ADM or at the AIoTF.

7. Whether MAC is needed in the paging request to prevent replay attack.

8. Whether ADM or AIoTF performs RES comparison.

NOTE5:  This means either a) ADM performs RES comparison then returns result to AIoTF  or b) ADM returns XRES to AIoTF, AIoTF then compares RES against XRES.