| 3GPP TR 33.703 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Transitioning to Post Quantum Cryptography (PQC) in 3GPP (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	16

3.1	Terms	16

3.2	Symbols	16

3.3	Abbreviations	16

4	Overview	17

4.1	Background Information	17

4.1.1	General	17

4.1.2	Transition Timeline	18

4.1.3	 PQ and PQT Algorithm Standards	18

4.1.4	Summary of cybersecurity organisations’ recommendations	19

4.2	General Assumptions	19

5	 Principles and attributes of PQC to use in 3GPP procedures	19

5.1	PQC security level	19

5.2	Hybrid and standalone schemes	20

5.3	Cryptographic agility	20

5.4	PQC algorithm types and cryptographic diversity	20

6	 Protocols expected to be updated for PQC by other SDOs	20

6.1	General	20

6.2	COSE	21

6.2.1	General	21

6.2.2	Current Work in IETF	21

6.2.2.1	IETF RFCs	21

6.2.2.2	IETF Adopted Drafts	21

6.2.3	 3GPP Considerations	21

6.3	IKEv2	22

6.3.1	General	22

6.3.2	Current Work in IETF	22

6.3.2.1	IETF RFCs	22

6.3.2.1.1 Key Exchange	22

6.3.2.1.2 Authentication and Signature	22

6.3.2.2	IETF Adopted Drafts	22

6.3.2.2.1 Key Exchange	22

6.3.2.2.2 Authentication and Signatures	23

6.3.3	 3GPP Considerations	23

6.4	 JOSE	23

6.4.1	General	23

6.4.2	Current Work in IETF	23

6.4.2.1	IETF RFCs	23

6.4.2.2	IETF Adopted Drafts	23

6.4.3	 3GPP Considerations	23

6.5	PKI certificate	24

6.5.1	General	24

6.5.2	 Current Work in IETF	24

6.5.2.1	IETF RFCs	24

6.5.2.2	IETF Adopted Drafts	24

6.5.3	 3GPP Considerations	24

6.6	TLS 1.2	24

6.6.1	General	24

6.6.2	Current Work in IETF	25

6.6.3	 3GPP Considerations	25

6.7	TLS 1.3	25

6.7.1	General	25

6.7.2	Current Work in IETF	25

6.7.2.1	IETF RFCs	26

6.7.2.2	IETF Adopted Drafts	26

6.7.3	 3GPP Considerations	26

6.8	 3GPP Considerations	26

7	 Protocols expected to be updated for PQC by 3GPP	26

7.1	Threats	27

7.1.1	General	27

7.1.2	SUCI calculation	27

7.1.3	MIKEY-SAKKE key exchange	27

7.2	Solutions	27

7.2.1	Solutions to SUCI calculation	27

7.2.1.1	Solution #1 to SUCI calculation: SUCI calculation with PQC enhancement	27

7.2.1.1.1	Introduction	27

7.2.1.1.2	Solution details	28

7.2.1.1.3	Evaluation	28

7.2.1.2	Solution #2 to SUCI calculation: Solution on PQC for SUCI protection	28

7.2.1.2.1	Introduction	28

7.2.1.2.2	Solution details	29

7.2.1.2.3	Evaluation	31

7.2.1.3	Solution #3 to SUCI calculation: SUCI calculation with hybrid KEMs	31

7.2.1.3.1	Introduction	31

7.2.1.3.2	Solution details	31

7.2.1.3.3	Evaluation	33

7.2.1.4	Solution #4 to SUCI calculation: SUPI Pseudonym	33

7.2.1.4.1	Introduction	33

7.2.1.4.2	Solution details	33

7.2.1.4.3	Evaluation	34

7.2.1.5	Solution #5 to SUCI calculation: Enhancement on SUCI calculations using quantum key	34

7.2.1.5.1	Introduction	34

7.2.1.5.2	Solution details	34

7.2.1.5.2.1	Processing on UE side	34

7.2.1.5.2.2	Processing on home network side	35

7.2.1.5.3	Evaluation	36

7.2.1.6	Solution #6 to SUCI calculation: Enhancement on SUCI calculations using quantum encapsulated key	36

7.2.1.6.1	Introduction	36

7.2.1.6.2	Solution details	36

7.2.1.6.2.1	Processing on UE side	37

7.2.1.6.2.2	Processing on home network side	37

7.2.1.6.3	Evaluation	38

7.2.1.7	Solution #7 to SUCI calculation: SUCI calculations	39

7.2.1.7.1	Introduction	39

7.2.1.7.2	Solution details	39

7.2.1.7.2.1	General	39

7.2.1.7.2.2	ML-KEM is the Most Suitable Option	39

7.2.1.7.2.3	Considerations for Hybrid KEM	39

7.2.1.7.2.4	KDF, MAC, and Encryption	40

7.2.1.7.2.5	New SUCI Profiles	40

7.2.1.7.3	Evaluation	41

7.2.1.8	Solution #8 to SUCI calculation: GSMA-based solution	41

7.2.1.8.1	Introduction	41

7.2.1.8.2	Solution details	41

7.2.1.8.2.1	Profile A’ (update of Profile A to support PQC algorithm)	42

7.2.1.8.2.2	Profile B’ (update of Profile B to support PQC algorithm)	43

7.2.1.8.3	Evaluation	43

7.2.1.9	Solution #9 to SUCI calculation: SUPI Concealment using PQC Shared Key	44

7.2.1.9.1	Introduction	44

7.2.1.9.2	Solution details	44

7.2.1.9.2.1 Processing on UE side	44

7.2.1.9.2.2 Processing on home network side	45

7.2.1.9.2.2	Sample profile for SUCI Calculation	45

7.2.1.9.2.2.1	Profile C (PQC only)	45

7.2.1.9.3	Evaluation	46

7.2.1.10	Solution #10 to SUCI calculation: SUPI Concealment using Hybrid shared Key	46

7.2.1.10.1	Introduction	46

7.2.1.10.2	Solution details	46

7.2.1.10.2.1	Processing on UE side	46

7.2.1.10.2.2	Processing on home network side	48

7.2.1.10.2.3	Sample Profiles for SUCI calculation	48

7.2.1.10.2.3.1	Profile C (Hybrid 1)	49

7.2.1.10.2.3.2	Profile D (Hybrid 2)	49

7.2.1.10.3	Evaluation	50

7.2.1.11	Solution #11 to SUCI calculation: SUPI Concealment using hybrid method	50

7.2.1.11.1	Introduction	50

7.2.1.11.2	Solution details	50

7.2.1.11.2.1 Processing on UE side	50

7.2.1.11.2.2 Processing on home network side	51

7.2.1.11.3	Evaluation	52

7.2.1.12	Solution #12 to SUCI calculation: Hybrid SUCI calculation	52

7.2.1.12.1	 Introduction	52

7.2.1.12.2 	Solution details	52

7.2.1.12.3 	Evaluation	55

7.2.1.13	Solution #13 to SUCI calculation: Symmetric crypto based SUCI	55

7.2.1.13.1	Introduction	55

7.2.1.13.2	Solution details	56

7.2.1.13.3	Evaluation	58

7.2.1.14	Solution #14 to SUCI Calculation: Symmetric solution on SUCI protection	58

7.2.1.14.1	Introduction	58

7.2.1.14.2	Solution details	59

7.2.1.14.3	Evaluation	60

7.2.1.15	Solution #15 to SUCI calculation: SUCI calculation with symmetric key	60

7.2.1.15.1	Introduction	60

7.2.1.15.2	Solution details	61

7.2.1.15.2.1 Processing on UE side	61

7.2.1.15.2.2 Processing on home network side	61

7.2.1.15.3	Evaluation	61

7.2.1.16	Solution #16 to SUCI calculation: Solution for PQC based SUCI Computation	61

7.2.1.16.1	Introduction	61

7.2.1.16.2	Solution details	61

7.2.1.16.3	Evaluation	63

7.2.2	Solutions to MIKEY-SAKKE key exchange	64

7.2.2.1	Solution #1 to MIKEY-SAKKE key exchange: mitigate	64

7.2.2.1.1	Introduction	64

7.2.2.1.2	Solution Details	64

7.2.2.1.3	 Evaluation	64

8	Conclusions	64

Annex A (informative): Change history	65

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the complexities involved with the introduction of standalone and/or hybrid Post Quantum Cryptography (PQC) algorithms in existing security protocols used by 5G specifications. These security protocols and their associated algorithms have been listed in TR 33.938 [2] “3GPP Cryptographic Inventory”. Specifically,

- Studies principles and attributes of PQC relevant to use in 3GPP procedures.

-	Studies the impact of using hybrid and standalone PQC algorithms in 3GPP procedures

-	Impact to 3GPP procedures due to larger length of PQC key, signature, and message compared to the length of those in traditional cryptography.

-	Determines security levels (I-V) required to align with existing 3GPP procedures level of assurance.

-	Studies the suitability of classes of post-quantum signature algorithms (e.g., lattice-based, hash-based) to 3GPP procedures.

- Identifies the protocols with asymmetric cryptography listed in TR 33.938 [2] that are not expected to be updated by other Standards Development Organizations (SDOs) in a near future to use PQC, e.g., MIKEY-SAKKE and SUCI calculation

- Studies security threats and alternative solutions for the 3GPP procedures if they are not updated to use PQC.

- Documents the expected timeline for when security protocols defined by other SDOs will include PQC algorithms and be available for inclusion into 3GPP procedures. The timeline includes the availability of stable protocols.

- Studies solutions to update 3GPP defined security protocols (for example SUCI calculation) to use the appropriate PQC algorithm, if those protocols are not expected to be updated by other SDOs to use PQC algorithms.

The present document is Generation agnostic.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.938: "3GPP Cryptographic Inventory".

[3]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	PQUIP draft-ietf-pquip-pqc-engineers: "Post-Quantum Cryptography for Engineers".

[6]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[7]	IETF RFC 9794: "Terminology for Post-Quantum Traditional Hybrid Schemes".

[8]	NIST IR 8547: "Transition to Post-Quantum Cryptography Standards".

[9]	SECG SEC 1: "Recommended Elliptic Curve Cryptography", Version 2.0, 2009. Available at http://www.secg.org/sec1-v2.pdf.

[10]	SECG SEC 2: "Recommended Elliptic Curve Domain Parameters", Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf.

[11]	EU, Roadmap for the Transition to Post-Quantum Cryptography
https://digital-strategy.ec.europa.eu/en/news/eu-reinforces-its-cybersecurity-post-quantum-cryptography

[12]	UK NCSC, Timelines for migration to post-quantum cryptography
https://www.ncsc.gov.uk/guidance/pqc-migration-timelines

[13]	NSA, The Commercial National Security Algorithm Suite 2.0 and Quantum Computing FAQ
https://media.defense.gov/2022/Sep/07/2003071836/-1/-1/0/CSI_CNSA_2.0_FAQ_.PDF

[14]	ANSSI, Guide des Mécanismes cryptoraphiques
https://cyber.gouv.fr/sites/default/files/2021/03/anssi-guide-mecanismes_crypto-2.04.pdf

[15]	ASD, Guidelines for cryptography
https://www.cyber.gov.au/business-government/asds-cyber-security-frameworks/ism/cybersecurity-guidelines/guidelines-for-cryptography

[16]	Canadian Centre for Cyber Security, Roadmap for the migration to post-quantum cryptography
https://www.cyber.gc.ca/en/guidance/roadmap-migration-post-quantum-cryptography-government-canada-itsm40001

[17]	Swedish NCSC, Kvantsäker kryptografi
https://www.ncsc.se/sv/aktuellt/kvantsaker-kryptografi/

[18]	NSM Cryptographic Recommendations
https://nsm.no/getfile.php/1314334-1742808614/NSM/Filer/Dokumenter/Veiledere/NSM%20Cryptographic%20Recommendations%202025.pdf

[19]	AIVD, The PQC Migration Handbook
https://english.aivd.nl/binaries/aivd-en/documenten/publications/2024/12/3/the-pqc-migration-handbook/The+PQC+Migration+Handbook+.pdf

[20]	3GPP, Release Timeline
https://www.3gpp.org/specifications-technologies/releases/release-20

[21]	NIST FIPS 203: "Module-Lattice-Based Key-Encapsulation Mechanism Standard"
https://doi.org/10.6028/NIST.FIPS.203

[22]	NIST FIPS 204: "Module-Lattice-Based Digital Signature Standard"
https://doi.org/10.6028/NIST.FIPS.204

[23]	NIST FIPS 205: "Stateless Hash-Based Digital Signature Standard"
https://doi.org/10.6028/NIST.FIPS.205

[24]	OpenSSH 10.0 Introduces Default Post-Quantum Key Exchange Algorithm https://quantumcomputingreport.com/openssh-10-0-introduces-default-post-quantum-key-exchange-algorithm

[25]	Cloudflare Radar https://radar.cloudflare.com/adoption-and-usage#post-quantum-encryption-adoption

[26]	A Coordinated Implementation Roadmap for the Transition to Post-Quantum Cryptography https://digital-strategy.ec.europa.eu/en/library/coordinated-implementation-roadmap-transition-post-quantum-cryptography

[27]	NCSC (UK): Next steps in preparing for post-quantum cryptography	https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

[28]	PQC Transition in France ANSSI Views https://cyber.gouv.fr/sites/default/files/document/pqc-transition-in-france.pdf

[29]	ANSSI plan for post-quantum transition https://pkic.org/events/2023/pqc-conference-amsterdam-nl/pkic-pqcc_jerome-plut_anssi_anssi-plan-for-post-quantum-transition.pdf

[30] 	ETSI TS 103 744: "Quantum-safe Hybrid Key Establishment". https://www.etsi.org/deliver/etsi_ts/103700_103799/103744/01.02.01_60/ts_103744v010201p.pdf

[31] 	FIPS 202: "SHA-3 Standard: Permutation-Based Hash and Extendable-Output Functions". https://nvlpubs.nist.gov/nistpubs/fips/nist.fips.202.pdf

[32] 	SP 800-185: "~SHA-3 Derived Functions: cSHAKE, KMAC, TupleHash, and ParallelHash". https://nvlpubs.nist.gov/nistpubs/fips/nist.fips.202.pdf

[33]	GSMA: "Post Quantum Cryptography – Guidelines for Telecom Use Cases - v2.0"

[34]		IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)"

[35]	IETF RFC 7748: "Elliptic Curves for Security".

[36]	FN-DSA:  Falcon is a cryptographic signature algorithm submitted to NIST, Refer to https://falcon-sign.info/falcon.pdf

[37]	NIST: “Submission Requirements and Evaluation Criteria for the Post-Quantum Cryptography Standardization Process “,
https://csrc.nist.gov/CSRC/media/Projects/Post-Quantum-Cryptography/documents/call-for-proposals-final-dec-2016.pdf

[38]	Bernstein, D.J. (2009): "Introduction to post-quantum cryptography ", 2009. Available at https://doi.org/10.1007/978-3-540-88702-7_1

[39]	NIST IR 8545: “Status Report on the Fourth Round of the NIST Post-Quantum Cryptography Standardization Process”, 2025. Available at https://csrc.nist.gov/pubs/ir/8545/final

[40] 	NIST, "Considerations for Achieving Cryptographic Agility: Strategies and Practices," CSWP 39, Jul. 2025. [Online]. Available: https://csrc.nist.gov/pubs/cswp/39/considerations-for-achieving-cryptographic-agility/2pd

[41]	IETF RFC 7696: “Guidelines for Cryptographic Algorithm Agility and Selecting Mandatory-to-Implement Algorithms”.

[42]	IETF: “About RFCs”. Available at https://www.ietf.org/process/rfcs/.

[43]	IETF RFC 9242: "Intermediate Exchange in the Internet Key Exchange Protocol Version 2 (IKEv2) "

[44]	IETF RFC 9370: "Multiple Key Exchanges in the Internet Key Exchange Protocol Version 2 (IKEv2) "

[45]	IETF Draft (Standards Track): "Post-quantum Hybrid Key Exchange with ML-KEM in the Internet Key Exchange Protocol Version 2 (IKEv2) ", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-mlkem/.

[46]	IETF RFC 9593: "Announcing Supported Authentication Methods in the Internet Key Exchange Protocol Version 2 (IKEv2)"

[47]	IETF RFC 8784: "Mixing Preshared Keys in the Internet Key Exchange Protocol Version 2 (IKEv2) for Post-quantum Security"

[48]	IETF Draft (Standards Track): " Signature Authentication in the Internet Key Exchange Version 2 (IKEv2) using PQC ", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-pqc-auth/.

[49]	IETF RFC 7383: "Internet Key Exchange Protocol Version 2 (IKEv2) Message Fragmentation". https://www.rfc-editor.org/rfc/rfc7383

[50]	IETF RFC 9763: "Related Certificates for Use in Multiple Authentications within a Protocol "

[51]	IETF RFC 9802: "Use of the HSS and XMSS Hash-Based Signature Algorithms in Internet X.509 Public Key Infrastructure"

[52]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Key-Encapsulation Mechanism (ML-KEM) ", https://datatracker.ietf.org/doc/draft-ietf-lamps-kyber-certificates/.

[53]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure: Algorithm Identifiers for SLH-DSA", https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[54]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Digital Signature Algorithm (ML-DSA)", https://datatracker.ietf.org/doc/draft-ietf-lamps-dilithium-certificates/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[55]	IETF Draft (Standards Track): "Composite ML-KEM for use in X.509 Public Key Infrastructure", https://datatracker.ietf.org/doc/draft-ietf-lamps-pq-composite-kem/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[56]	IETF Draft (Standards Track): "A Mechanism for X.509 Certificate Discovery", https://datatracker.ietf.org/doc/draft-ietf-lamps-certdiscovery/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[57]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2"

[58]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3"

[59]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[60]	IETF Draft draft-ietf-tls-tls12-frozen-08: "TLS 1.2 is in Feature Freeze "

[61]	https://datatracker.ietf.org/meeting/123/materials/slides-123-tls-wg-status-00

[62]	https://datatracker.ietf.org/liaison/2058/

[63]	IETF Draft draft-ietf-tls-hybrid-design-16: "Hybrid key exchange in TLS 1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-hybrid-design/.

[64]	IETF Draft draft-ietf-tls-mlkem-04: "ML-KEM Post-Quantum Key Agreement for TLS 1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-mlkem/.

[65]	IETF Draft draft-ietf-tls-ecdhe-mlkem-01: "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-ecdhe-mlkem/.

[66]	IETF Draft draft-ietf-tls-mldsa-01: "Use of ML-DSA in TLS 1.3", https://datatracker.ietf.org/doc/draft-ietf-tls-mldsa/

[67]	IETF Draft draft-ietf-jose-pqc-kem-03: "Post-Quantum Key Encapsulation Mechanisms (PQ KEMs) for JOSE and COSE"

[68]	IETF Draft draft-ietf-cose-dilithium-08: "ML-DSA for JOSE and COSE"

[69]	IETF Draft draft-ietf-cose-sphincs-plus-05: "SLH-DSA for JOSE and COSE"

[70]	IETF Draft draft-ietf-cose-falcon-01: "JOSE and COSE Encoding for Falcon"

[71]	IETF Draft (Standards Track): “Use of Hybrid Public Key Encryption (HPKE) with JSON Object Signing and Encryption (JOSE)”, https://datatracker.ietf.org/doc/draft-ietf-jose-hpke-encrypt/.

[72]	IETF Draft (Standards Track): “Use of Hybrid Public-Key Encryption (HPKE) with CBOR Object Signing and Encryption (COSE)”, https://datatracker.ietf.org/doc/draft-ietf-cose-hpke/.

[73]	NIST SP 800-227 Recommendations for Key-Encapsulation Mechanisms, url: https://csrc.nist.gov/pubs/sp/800/227/ipd

[74]	3GPP TS 23.003: "Numbering, addressing and identification".

[75]	NIST.SP.800-56 Recommendation for Pair-Wise Key-Establishment Schemes Using Discrete Logarithm Cryptography. url: https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-56Ar3.pdf

[76]	Galois Counter Mode with Strong Secure Tags (GCM-SST). https://datatracker.ietf.org/doc/html/draft-mattsson-cfrg-aes-gcm-sst

[77] 	Ericssons comments on NIST SP 800-227 (Initial Public Draft). https://csrc.nist.gov/files/pubs/sp/800/227/ipd/docs/sp800-227-ipd-public-comments-received.pdf

[78]	IETF Draft (Standards Track): " Mixing Preshared Keys in the IKE_INTERMEDIATE and in the CREATE_CHILD_SA Exchanges of IKEv2 for Post-quantum Security", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-qr-alt/.

[79] 	BSI: "Cryptographic Mechanisms", https://www.bsi.bund.de/EN/Themen/Unternehmen-und-Organisationen/Standards-und-Zertifizierung/Technische-Richtlinien/TR-nach-Thema-sortiert/tr02102/tr02102_node.html

[80]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[81]		IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[82]	3GPP TS 33.310: " Network Domain Security (NDS); Authentication Framework (AF)".

[83]	IETF RFC 7515: "JSON Web Signature (JWS)".

[84]	IETF RFC 7516: "JSON Web Encryption".

[85]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[86]	NIST SP-800-90A

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AEAD			Authenticated Encryption with Associated Data

ANSSI			Agence Nationale de la Sécurité des Systèmes d'Information

CA			Certification Authority

CBOR			Concise Binary Object Representation

COSE			CBOR Object Signing and Encryption

CRL			Certificate Revocation Lists

CRQC			Cryptographically Relevant Quantum Computer

DSA			Digital Signature Algorithm

ECC			Elliptic Curve Cryptography

ECDH			Elliptic Curve Diffie–Hellman key Exchange

ECIES			Elliptic Curve Integrated Encryption Scheme

FN-DSA			Fast-Fourier Transform over NTRU-Lattice-Based DSA

GCI			Global Cable Identifier

GLI			Global Line Identifier

HBS			Hash-Based Signature

HQC			Hamming Quasi-Cyclic

HSS			Hierarchical Signature System

IKEv2			Internet Key Exchange Protocol Version 2

IMSI			International Mobile Subscriber Identifier

JOSE			Javascript Object Signing and Encryption

JSON			JavaScript Object Notation

JWE			JSON Web Encryption

JWS			JSON Web Signature

KEM			Key Encapsulation Mechanism

MAC			Message Authentication Code

MCC			Mobile Country Code

LTS			Long Term Stable

MIKEY-SAKKE			Multimedia Internet KEYing – Sakai-Kasahara Key Encryption

ML-DSA			Module-Lattice-Based DSA

ML-KEM			Module Lattice-Based Key-Encapsulation Mechanism

MNC			Mobile Network Code

NAI			Network Access Identifier

NCSC 			National Cyber Security Centre

NSA			National Security Agency

NSI			Network Specific Identifier

NSM			National Security Memorandum

NTRU			Nth-degree Truncated Polynomial Ring Units

OCSP			Online Certificate Status Protocol

PKI			Public Key Infrastructure

PKIX			Public Key Infrastructure X.509

PLMN			Public Land Mobile Network

PQC			Post-Quantum Cryptography

PRNG			Pseudo Random Number Generator

SA			Security Association

SDO			Standards Development Organizations

SECG			Security Engineering & Consulting Group

SLH-DSA			Stateless Hash-Based DSA

SUCI			Subscription Concealed Identifier

SUPI			Subscription Permanent Identifier

TLS 1.2			Transport Layer Security Version 1.2

TLS 1.3			Transport Layer Security Version 1.3

UDM			Unified Data Management

XMSS			eXtended Merkle Signature Scheme

# 4	Overview

## 4.1	Background Information

### 4.1.1	General

The security protocols that use symmetric and/or asymmetric cryptography in 3GPP systems are listed in TR 33.938 [2]. Particularly, 3GPP heavily depends on IETF standards for the usages of public-key cryptography. All the security protocols using traditional asymmetric cryptography are vulnerable to attacks using a Cryptographically Relevant Quantum Computer (CRQC).

Given the wide variation in requirements, specifications, technical capabilities, and implementation maturity across protocols, this study is organized by security protocols. Each major protocol (such as COSE, IKEv2, JWE, JWS, MIKEY-SAKKE, SUCI, TLS 1.2, TLS 1.3) is covered in a separate clause.

This study does not focus on any particular generation of mobile networks and analyses various aspects that will be useful for PQC migration.

### 4.1.2	Transition Timeline

Editor’s Note: More timeline information from other organizations is ffs.

Countries and agencies around the world are generally aligned on the need to migrate to Post-Quantum Cryptography (PQC). The common recommendation is to complete migration for high priority systems by around 2030 and for all systems by approximately 2035. Examples of government-issued PQC migration timelines can be found in [8, 11–19].  Whether a system is high priority or not is determined by a variety of factors such as how long the data needs to remain confidentiality protected and what level of risk is the data owner willing to bear. Some parts of telecommunications systems may be assessed by the network operator to be of high priority.

Although the migration of signature-based authentication in protocols such as TLS and IPsec is typically not prioritized for transition until 2035, transitioning Public Key Infrastructures (PKI), which are necessary to support signature-based authentication, often takes a decade or more, making it critical to begin their transition almost immediately.

Furthermore, it is important to note that the above timelines apply to deployments. For full PQC adoption in deployed systems, it is essential that standards are updated, and stable implementations are made available well in advance of those deployment milestones. The timelines for different stakeholders in the ecosystem, such as standards development organizations (SDO), equipment vendors, and operators deploying the systems are inherently different. Standards bodies need to finalize specifications early, vendors need sufficient lead time to implement, test, and certify solutions, and only then can large-scale deployments take place.

3GPP Rel-20 specification is expected to be frozen in the mid-2027 [20]. Rel-21 specification can be expected to be completed in the beginning of 2029 at the earliest. It should be considered that some vendors and operators require to meet the 2030 migration timeline for high priority systems.

### 4.1.3		PQ and PQT Algorithm Standards

There are three principal alternatives to traditional asymmetric cryptographic algorithms which have progressed furthest in relevant standards bodies. These are ML-KEM (FIPS 203) for key encapsulation, and ML-DSA (FIPS 204) and SLH-DSA (FIPS 205) for digital signature [21–23]. These are standards designed by cryptographers from all over the world, and they form the basis for recommendations from a number of agencies. These recommendations vary between organisations and include both standalone and hybrid transition paths.

Most governments require use of standardized PQC algorithms, such as the already standardized ML-KEM (FIPS 203), ML-DSA (FIPS 204), and SLH-DSA (FIPS 205) [21–23]. With the publication of ML-KEM, ML-DSA, and SLH-DSA, Post-Quantum Cryptography (PQC) has quickly moved from research to implementation and deployment. Some agencies recommend standalone ML-KEM and ML-DSA [13, 27], while others recommend that lattice-based algorithms (ML-KEM and ML-DSA) be hybridized [26, 28] with, for example, elliptic curve-based algorithms (ECDHE and ECDSA). The hash-based algorithm (SLH-DSA) doesn’t need to be hybridized as hash algorithms are better understood by the cryptographic research community and have also been cryptanalyzed far longer than lattices, and governments currently do not recommend SLH-DSA to be hybridized [28, 29].

ML-KEM is an algorithm for key encapsulation. It is a replacement for ECDH(E) key exchanges (note that RSA key encipherment has largely been deprecated). Both standalone and hybrid versions have relatively mature implementations available (e.g. OpenSSL 3.5 LTS) and are progressing through other SDOs (e.g. the TLS WG in IETF), with the hybrid version receiving more attention. In TLS, X25519MLKEM has already seen massive implementation support. It has been reported [25] that over 40% of all HTTPS client requests use PQC. OpenSSL 3.5 LTS supports ML-KEM, ML-DSA, and SLH-DSA. OpenSSH is now using mlkem768x25519-sha256 as the default key exchange [24]. Many IKEv2 implementations support ML-KEM. See clause 6 for further details broken down by protocol.

ML-DSA is an algorithm for digital signature. While the IETF and real-world deployments have embraced hybrid KEMs, hybrid signatures have not seen similar adoption. SLH-DSA is a special purpose digital signature algorithm, owing to its significantly large key sizes and slow operation times — making it unsuitable for general use cases like short-lived certificates or high-throughput applications, but excellent for specific tasks such as firmware signing and code signing where long signing times and large signature sizes are not prohibitive. Implementations of standalone versions of both ML-DSA and SLH-DSA are also available (e.g. OpenSSL 3.5 LTS). There is more progress to date integrating standalone ML-DSA into protocols than either hybrid ML-DSA or standalone SLH-DSA. See clause 6 for further details broken down by protocol.

### 4.1.4	Summary of cybersecurity organisations’ recommendations

NOTE:	Details of the meanings of Level 3 and Level 5 are found in clause 5.1.

Advice and recommendations for parameter choices is provided in e.g. NIST [21], NCSC [27], BSI [79], NSA [13], ANSSI [28], and AIVD [19] which is summarised below:

- Level 3 is accepted for general use (i.e. situations where AES-128 is currently used). This is a strict minimum for BSI.

- NIST only provides recommendations on security levels for ML-KEM

- NCSC does not consider SLH-DSA appropriate for general use and makes no recommendations for parameter choices.

- BSI specifically recommends the "hedged" variants of ML-DSA and SLH-DSA to mitigate risks from poor entropy sources.

- Level 5 is required by NSA for National Security Systems (NSS) and recommended for Department of Defense (DoD), Defense Industrial Base (DIB), and those interacting with these systems.

## 4.2	General Assumptions

In the present document, PQC is referred to as cryptographic algorithms that are deemed to be secure against attacks from both classical and quantum computing.

All traditional public key cryptographic algorithms used in 3GPP systems need to be migrated to PQC algorithms. If suitable PQC options are not available, then an alternative path needs to be provided and justified, e.g., deprecation, mitigation, and re-architecting.

The PQC options are to be drawn from well-studied standardised primitives and protocols.

Both hybrid and standalone KEM are in the scope of this study. Standalone and hybrid signatures are also in the scope of this study.

Editor’s Note: Further general assumptions are FFS.

# 5		Principles and attributes of PQC to use in 3GPP procedures

Editor’s Note: This clause contains impact of using hybrid and standalone PQC algorithms in 3GPP procedures, impact to 3GPP procedures due to larger length of PQC key, signature, and message compared to the length of those in traditional cryptography, security levels (I-V) required to align with existing 3GPP procedures level of assurance, suitability of classes of post-quantum signature algorithms (e.g., lattice-based, hash-based) to 3GPP procedures.

## 5.1	PQC security level

The NIST use the concept of security levels/security strength categories to group algorithms, keys, and protocols related to PQC [37]. Security is defined as a function of resources comparable to or greater than those required to break AES and SHA2/SHA3 algorithms, i.e., key search on block cipher for AES and collision search on a 256-bit hash function for SHA2/SHA3. The security strength is broadly grouped into the following 5 levels [8] and to each of the PQ security levels, the corresponding traditional and post-quantum algorithm can be mapped:

Level 1: At least as hard as breaking AES-128 (key search on block cipher) , PQC-Algorithm: ML-KEM-512 [21], FN-DSA-512 [36], SLH-DSA-SHA2/SHAKE-128f/s [23]

Level 2: At least as hard as breaking SHA-256/SHA3-256 (collision search on a 256-bit hash function), PQC-Algorithm: ML-DSA-44 [22]

Level 3: At least as hard as breaking AES-192 (key search on block cipher), PQC-Algorithm: ML-KEM-768 [21], ML-DSA-65 [22], SLH-DSA-SHA2/SHAKE-192f/s [23]

Level 4: At least as hard as breaking SHA-384/SHA3-384 (collision search on a 384-bit hash function), PQC-Algorithm: No algorithm tested at this level

Level 5: At least as hard as breaking AES-256 (key search on block cipher), PQC-Algorithm: ML-KEM-1024 [21], FN-DSA-1024 [36], ML-DSA-87 [22], SLH-DSA-SHA2/SHAKE-256f/s [23]

## 5.2	Hybrid and standalone schemes

Post-Quantum Traditional (PQT) hybrid scheme as defined in RFC 9794 [7] is a multi-algorithm scheme where at least one component algorithm is a post-quantum algorithm and at least one is a traditional algorithm. Both the PQT hybrid scheme and the standalone PQC scheme are considered in the present document.

## 5.3	Cryptographic agility

Cryptographic agility [40, 41] refers to the capabilities needed to replace and adapt cryptographic algorithms while preserving security and ongoing operations. The 3GPP systems need to consider cryptographic agility.

## 5.4	PQC algorithm types and cryptographic diversity

PQC algorithms can be categorized based on different mathematical foundations. The following are a few typical types of PQC algorithms [38, 5]: Lattice-based cryptography, Hash-based cryptography, Multivariate cryptography, Code-based cryptography, and Isogeny-based cryptography.

NOTE: The types for NIST selected algorithms are as follows: ML-KEM for key encapsulation, and ML-DSA and FN-DSA for digital signature are all Lattice-based algorithms; SLH-DSA for digital signature is a Hash-based algorithm; and HQC-KEM for key encapsulation is a Code-based algorithm.

Cryptographic diversity is the practice of having different types of PQC algorithms available. This provides resilience against future attacks in case that a weakness or vulnerability is discovered in one type of algorithm, when other types of algorithms will remain unaffected. For example, NIST has chosen SLH-DSA as a backup algorithm for ML-DSA and HQC algorithm as a backup for ML-KEM [39]. A key enabler for this is cryptographic agility so that if an algorithm is broken it can be removed and replaced with an alternative without undue disruption.

# 6		Protocols expected to be updated for PQC by other SDOs

Editor’s Note: This clause contains the expected timeline for when security protocols defined by other SDOs will include PQC algorithms and be available for inclusion into 3GPP procedures. The timeline includes the availability of stable protocols.

## 6.1	General

According to the inventory in TR 33.938 [2], many security protocols and algorithms used in 3GPP (e.g. (D)TLS, IKEv2, JWE, JWS, etc.) are specified in other standard organizations (e.g. IETF). They are expected to be updated using PQC in the corresponding organizations.

In this clause, the progress of the post-quantum migration of these protocols are reported. Mature specifications developed by related SDOs will be given priority consideration.  In addition, whether the relevant solutions can be directly applied to specific 3GPP scenarios will be evaluated.

The present document discusses several IETF documents that are at different levels of maturity in the overall IETF standardization process [42], and categorizes them as follows:

- IETF Individual Draft: A document that has been submitted to IETF and has not been adopted by one of the working groups in IETF. On the IETF Datatracker website, such documents have type “Active Internet-Draft (individual)”.

- IETF WG Draft: A document that has been reviewed and adopted by one of the working groups in IETF. On the IETF Datatracker website, such documents have type “Active Internet-Draft (xyz WG)”, where xyz is the name of the working group that adopted the document, e.g., tls.

- IETF RFC: A document that has gone through the whole IETF standardization process.

## 6.2	COSE

### 6.2.1	General

### 6.2.2	Current Work in IETF

#### 6.2.2.1	IETF RFCs

No RFCs for the usage of PQC algorithms in COSE are published yet.

#### 6.2.2.2	IETF Adopted Drafts

The IETF is developing support for PQC algorithms in COSE. The following drafts are relevant:

-	IETF Draft draft-ietf-jose-pqc-kem-03, "Post-Quantum Key Encapsulation Mechanisms (PQ KEMs) for JOSE and COSE" [67], describes the conventions for using Post-Quantum Key Encapsulation Mechanisms (PQ-KEMs) within JOSE and COSE.

-	IETF Draft draft-ietf-cose-dilithium-08, "ML-DSA for JOSE and COSE" [68], describes JSON Object Signing and Encryption (JOSE) and CBOR Object Signing and Encryption (COSE) serializations for Module-Lattice-Based Digital Signature Standard (ML-DSA).

-	IETF Draft draft-ietf-cose-sphincs-plus-05: "SLH-DSA for JOSE and COSE" [69], describes JOSE and COSE serializations for SLH-DSA.

-	IETF Draft draft-ietf-cose-falcon-01, "JOSE and COSE Encoding for Falcon" [70], describes JSON and CBOR serializations.

-	IETF Draft draft-ietf-cose-hpke-16, "Use of Hybrid Public-Key Encryption (HPKE) with CBOR Object Signing and Encryption (COSE)" [72] defines a Hybrid Public Key Encryption (HPKE) for use with JOSE utilizing an asymmetric Key Encapsulation Mechanism (KEM), a Key Derivation Function (KDF), and an Authenticated Encryption with Associated Data (AEAD) algorithm.

However, no IETF work on hybrid signature schemes for COSE has been adopted.

### 6.2.3		3GPP Considerations

Editor’s Note: This clause does not include any conclusions.

## 6.3	IKEv2

### 6.3.1	General

IKEv2 specified in IETF RFC 7296 [80] provides mutual authentication and establishes Security Associations (SA) for IPsec tunnels. The IKEv2 is also used by MOBIKE specified in IETF RFC 4555 [81].

The IETF IPSECME group has introduced multiple RFCs and Drafts to enable a smooth PQC transition for the Internet Key Exchange Protocol Version 2 (IKEv2) protocol. They cover both key exchange and authentication.

### 6.3.2	Current Work in IETF

#### 6.3.2.1	IETF RFCs

##### 6.3.2.1.1 Key Exchange

KEM-based Key Exchange

- IETF RFC 9242 [43] introduces a new exchange, called "Intermediate Exchange" for IKEv2 to avoid IP fragmentation of large IKE messages and enable transferring large amounts of data during Security Association (SA) establishment expected for some PQC key exchanges.

- IETF RFC 9370 [44] describes a method to perform multiple successive key exchanges in IKEv2. It allows integration of PQC in IKEv2 and the negotiation of one or more PQC algorithms, in addition to the existing (EC)DH key exchange data that provides backward compatibility.

- IETF RFC 7383, "Internet Key Exchange Protocol Version 2 (IKEv2) Message Fragmentation" [49] describes a way to avoid IP fragmentation of large Internet Key Exchange Protocol version 2 (IKEv2) messages, which is necessary when using ML-KEM-1024, ML-DSA, or SLH-DSA.

PSK-based Key Exchange

-	IETF RFC 8784 [47] describes an extension of IKEv2 resistant to quantum computers using pre-shared keys.

##### 6.3.2.1.2 Authentication and Signature

-	IETF RFC 9593 [46] defines a mechanism that allows implementations of IKEv2 to indicate the list of supported authentication methods to their peers while establishing IKEv2 SAs. This mechanism improves interoperability when IKEv2 partners are configured with multiple credentials of different types (for example, ECC-based certificate and PQC-based certificate) for authenticating each other.

#### 6.3.2.2	IETF Adopted Drafts

##### 6.3.2.2.1 Key Exchange

KEM-based Key Exchange

- IETF Draft draft-ietf-ipsecme-ikev2-mlkem-03, "Post-quantum Hybrid Key Exchange with ML-KEM in the Internet Key Exchange Protocol Version 2 (IKEv2)" [45] proposes to use the ML-KEM [21] as an additional key exchange in IKEv2 along with traditional key exchanges.

- IETF Draft draft-ietf-ipsecme-ikev2-pqc-auth-04, "Signature Authentication in the Internet Key Exchange Version 2 (IKEv2) using PQC" [69], specifies a generic mechanism for integrating post-quantum cryptographic (PQC) digital signature algorithms into the IKEv2 protocol.

PSK-based Key Exchange

-	IETF Draft draft-ietf-ipsecme-ikev2-qr-alt-10, "Mixing Preshared Keys in the IKE_INTERMEDIATE and in the CREATE_CHILD_SA Exchanges of IKEv2 for Post-quantum Security" [78] defines an alternative way to provide protection against quantum computers, which is similar to the solution defined in RFC 8784 [47], but also protects the initial IKEv2 SA.

##### 6.3.2.2.2 Authentication and Signatures

-	IETF Draft draft-ietf-ipsecme-ikev2-pqc-auth-04, "Signature Authentication in the Internet Key Exchange Version 2 (IKEv2) using PQC" [48] outlines how Module-Lattice-Based Digital Signatures (ML-DSA) [22] and Stateless Hash-Based Digital Signatures (SLH-DSA) [23], can be employed as authentication methods within the IKEv2.

### 6.3.3		3GPP Considerations

Editor’s Note: This clause does not include any conclusions.

## 6.4	 JOSE

### 6.4.1	General

The IETF JOSE Working Group has specified the JSON Web Signatures (JWS) [83] and JSON Web Encryption (JWE) [84] that are being used in OAuth 2.0 and other procedures in 3GPP systems. For PQC migration, a few Working Group Adopted Drafts are being developed, as described below.

### 6.4.2	Current Work in IETF

#### 6.4.2.1	IETF RFCs

No RFCs for the usage of PQC algorithms in JWE or JWS are published yet.

#### 6.4.2.2	IETF Adopted Drafts

The IETF is developing support for PQC algorithms in JOSE. The following drafts are relevant:

-	IETF Draft draft-ietf-jose-pqc-kem-03, "Post-Quantum Key Encapsulation Mechanisms (PQ KEMs) for JOSE and COSE" [67], describes the conventions for using Post-Quantum Key Encapsulation Mechanisms (PQ-KEMs) within JOSE and COSE.

-	IETF Draft draft-ietf-cose-dilithium-08, "ML-DSA for JOSE and COSE" [68], describes JSON Object Signing and Encryption (JOSE) and CBOR Object Signing and Encryption (COSE) serializations for Module-Lattice-Based Digital Signature Standard (ML-DSA).

-	IETF Draft draft-ietf-cose-sphincs-plus-05: "SLH-DSA for JOSE and COSE" [69], describes JOSE and COSE serializations for SLH-DSA.

-	IETF Draft draft-ietf-cose-falcon-01, "JOSE and COSE Encoding for Falcon" [70], describes JSON and CBOR serializations.

-	IETF Draft draft-ietf-jose-hpke-encrypt-12, "Use of Hybrid Public Key Encryption (HPKE) with JSON Object Signing and Encryption (JOSE)" [71] defines a Hybrid Public Key Encryption (HPKE) for use with JOSE utilizing an asymmetric Key Encapsulation Mechanism (KEM), a Key Derivation Function (KDF), and an Authenticated Encryption with Associated Data (AEAD) algorithm.

However, no IETF work on hybrid signature schemes for JOSE has been adopted.

### 6.4.3		3GPP Considerations

Editor’s Note: This clause does not include any conclusions.

## 6.5	PKI certificate

### 6.5.1	General

The Internet X.509 (PKIX) Certificate is being used in 3GPP PKI systems [82] and the OCSP protocol listed in the TR 33.938 [2]. The IETF LAMPS Working Group has introduced multiple Drafts to enable a smooth transition to PQC in PKIX to provide quantum-resistant security for PKIX.

### 6.5.2		Current Work in IETF

#### 6.5.2.1	IETF RFCs

- IETF RFC 9802 [51] has specified algorithm identifiers and ASN.1 encoding format for several stateful Hash-Based Signature (HBS) schemes: Hierarchical Signature System (HSS), eXtended Merkle Signature Scheme (XMSS), and a multi-tree variant of XMSS, XMSS^MT. These schemes are applicable to the Internet X.509 Public Key Infrastructure (PKI) when digital signatures are used to sign certificates and certificate revocation lists (CRLs).

-	IETF RFC 9763 [50] defines a method for requesting and issuing two X.509 end-entity certificates for the same entity, in order to perform two authentications using the two certificates where each certificate corresponds to a distinct digital signature.

#### 6.5.2.2	IETF Adopted Drafts

- IETF Draft draft-ietf-lamps-kyber-certificates-11 "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Key-Encapsulation Mechanism (ML-KEM)" [52] specifies the conventions for using the ML-KEM [21] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-x509-slhdsa-09, "Internet X.509 Public Key Infrastructure: Algorithm Identifiers for SLH-DSA" [53] specifies to the conventions for using the SLH-DSA [23] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-dilithium-certificates-13, "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Digital Signature Algorithm (ML-DSA)" [54] specifies the conventions for using the ML-DSA [22] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-pq-composite-kem-08 "Composite ML-KEM for use in X.509 Public Key Infrastructure" [55] defines a specific instantiation of the PQT Hybrid paradigm called "composite" where multiple cryptographic algorithms (i.e. ML-KEM [21] in hybrid with traditional algorithms RSA-OAEP, ECDH, X25519, and X448) are combined to form a single key encapsulation mechanism (KEM) presenting a single public key and ciphertext such that it can be treated as a single atomic algorithm at the protocol level.

-	IETF Draft draft-ietf-lamps-certdiscovery-01, "A Mechanism for X.509 Certificate Discovery" [56] specifies a method to discover a secondary X.509 certificate associated with an X.509 certificate to enable efficient multi-certificate handling in protocols.

### 6.5.3		3GPP Considerations

## 6.6	TLS 1.2

### 6.6.1	General

The TLS 1.2 handshake in IETF RFC 5246 [57] is used in TLS 1.2, DTLS 1.2, EAP-TLS 1.2, EAP-TTLS, and OAuth 2.0. The DTLS handshake is also applied in DTLS over SCTP and can be used in DTLS-SRTP.

The 3GPP TLS profile is defined in clause 6.2 of 3GPP TS 33.210 [59]. Since Release 15, TLS 1.3 has been mandatory for all 3GPP core network nodes, and from Release 16 onward, it is mandatory for all nodes. Because TLS always negotiates the highest mutually supported version, any use of TLS 1.2 in a 3GPP system from Rel-16 onward implies that at least one node is non-compliant with 3GPP specifications.

While a fully updated TLS 1.2 implementation could theoretically provide strong security against classical adversaries in scenarios where identity protection is not required, in practice, TLS 1.2 is only negotiated by outdated implementations. These often suffer from one or more known vulnerabilities.

Therefore, TLS 1.2 is expected to already have been fully phased out in 5G systems.

### 6.6.2	Current Work in IETF

TLS 1.2 has been obsoleted since 2018, as superseded by TLS 1.3 in IETF RFC 8446 [58]. The IETF will no longer approve any additions or updates to TLS 1.2, including PQC support (IETF draft-ietf-tls-tls12-frozen-08 [60]).

### 6.6.3		3GPP Considerations

Since TLS 1.2 will not be updated any further, 3GPP will consider phasing out the use of TLS 1.2.

## 6.7	TLS 1.3

### 6.7.1	General

The TLS 1.3 handshake protocol as defined in clause 4 of IETF RFC 8446 [58] is used in TLS 1.3, EAP-TLS 1.3, EAP-TTLS 1.3, OAuth 2.0, DTLS 1.3, and QUIC, and it can also be used in DTLS-SRTP. Since Release 15, TLS 1.3 has been mandatory to implement for the core network (cf. Annex E in TS 33.310 v15.0.0), and starting in Release 16, it has been mandatory to implement also for the ME (cf. Annex E in TS 33.310 v16.0.0).

IETF is in general recommending hybridization of KEMs and the hybrid KEM X25519MLKEM768 [65] has already received widespread implementation support and is the default in OpenSSL. It has been reported [25] that over 40% of all HTTPS client requests now use X25519MLKEM768. Standalone ML-KEM [64], ML-DSA [66] have seen more limited implementation but are supported in OpenSSL 3.5 LTS.

### 6.7.2	Current Work in IETF

The IETF has prioritized post-quantum migration in TLS as follows [61]:

•	Now (Hybrid + Pure ML-KEM)

•	Later (signatures)

•	Much later (dual certificates/composite signatures)

The IETF TLS Working Group has planned not to adopt work on hybrid signatures until "much later" [61].

The IETF TLS Working Group has introduced multiple drafts to enable a smooth transition to PQC in TLS 1.3. These proposals address both key exchange and authentication. These mechanisms collectively aim to maintain interoperability, minimize latency, and provide quantum-resistant security during and after the PQC transition.

In an LS to GSMA [62], the IETF TLS Working Group stated that they believe the IETF Adopted Draft "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3" [65] is stable enough to be used as normative reference, and that referencing an adopted draft normatively is a practice that other organizations follow as well.

#### 6.7.2.1	IETF RFCs

No RFCs for the usage of PQC algorithms in TLS 1.3 are published yet.

Editor's Note: several of the adopted drafts are in the final stages and may be published before this document is finalised.

#### 6.7.2.2	IETF Adopted Drafts

-	draft-ietf-tls-hybrid-design-16, "Hybrid key exchange in TLS 1.3" [63], specifies combining multiple key exchange algorithms (e.g., classical ECDHE with a PQ KEM) so that session security holds if at least one component remains secure.

-	draft-ietf-tls-mlkem-04, "ML-KEM Post-Quantum Key Agreement for TLS 1.3" [64], proposes to use the NIST specified ML-KEM [21] in TLS 1.3.

-	draft-ietf-tls-mldsa-00, "Use of ML-DSA in TLS 1.3" [66], proposes to use the NIST specified ML-DSA [22] in TLS 1.3.

-	draft-ietf-tls-ecdhe-mlkem-00, "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3" [65], defines three hybrid key agreements for TLS 1.3: X25519MLKEM768, SecP256r1MLKEM768, and SecP384r1MLKEM1024.

### 6.7.3		3GPP Considerations

Editor’s Note: This clause does not include any conclusions.

## 6.8		3GPP Considerations

All the RFCs and adopted drafts mentioned in clauses 6.2, 6.3, 6.4, 6.5, and 6.7 are stable and ready for use in 3GPP systems, except the following two adopted drafts:

- IETF Draft draft-ietf-jose-pqc-kem-03, "Post-Quantum Key Encapsulation Mechanisms (PQ KEMs) for JOSE and COSE" [67]

- IETF Draft draft-ietf-lamps-certdiscovery-01, "A Mechanism for X.509 Certificate Discovery" [56]

3GPP will consider the lifecycle management of long-lived PKIs, especially the lifespan of certificates.

3GPP will consider choosing at least two suitable standardized algorithms, if available, for the same purpose (e.g., key exchange and authentication) with different constructions so that cryptanalytic breakthroughs against one algorithm does not directly apply against the other algorithm(s).

Editor’s Note: Further 3GPP considerations are FFS.

Editor’s Note: This clause does not include any conclusions.

# 7		Protocols expected to be updated for PQC by 3GPP

Editor’s Note: This clause contains identification of the protocols with asymmetric cryptography listed in TR 33.938 that are not expected to be updated by other SDOs in a near future to use PQC, e.g., MIKEY-SAKKE and SUCI calculation, security threats and alternative solutions for the 3GPP procedures if they are not updated to use PQC.

### 7.1	Threats

### 7.1.1	General

Most of security protocols used in 3GPP systems are specified in other standards development organizations (SDOs). In case that these protocols are not updated to use PQC in other SDOs, the 3GPP system may be vulnerable to attacks based on quantum computation. The clauses 7.1.2, 7.1.3, and 7.2 contain all of these protocols identified and potential solutions to address the issues.

### 7.1.2	SUCI calculation

Editor’s Note: If only SUCI calculation is considered, this subclause may be removed. If other protocol, e.g. MIKEY-SAKKE is studied, this subclause is used for each of such protocol identified.

As per TS 33.501 [4] and Table 4.3.2-1 of 3GPP Cryptographic inventory 3GPP TR 33.938 [2], the SUCI calculation is done based on ECIES scheme. The ECIES is specified in the SECG version 2 [9] and [10].

Since ECIES will not be updated by SECG with PQC algorithms, 3GPP should study alternative solutions for SUCI calculation due to post-quantum threats to existing ECIES scheme, e.g. supporting new profiles/algorithms with PQC for SUCI calculations.

### 7.1.3	MIKEY-SAKKE key exchange

MIKEY-SAKKE is a key exchange method specified in the IETF RFC 6509 [6]. As described in TR 33.938 [2], it is used in the 3GPP system to securely transport cryptographic keys for Mission Critical Services [3]. It employs asymmetric cryptography for key distribution.

Assuming MIKEY-SAKKE will not be updated by IETF with PQC algorithms, alternative solutions should be studied for MIKEY-SAKKE due to post-quantum threats to existing signature schemes.

## 7.2	Solutions

Editor’s Note: This clause contains solutions to update 3GPP defined security protocols (for example SUCI calculation) to use the appropriate PQC algorithm, if those protocols are not expected to be updated by other SDOs to use PQC algorithms.

### 7.2.1	Solutions to SUCI calculation

Editor’s Note: If only SUCI calculation is considered, this subclause may be removed. If other protocol, e.g. MIKEY-SAKKE is studied, this subclause is used for each of such protocol identified.

#### 7.2.1.1	Solution #1 to SUCI calculation: SUCI calculation with PQC enhancement

##### 7.2.1.1.1	Introduction

It is proposed to introduce new SUCI calculation mechanism. The solution is applicable for SUCI calculation in ME.

Preassumption:

- ME supports PQC algorithms

- USIM indicates the SUCI calculation is done in ME

- USIM contains new public key for calculating SUCI with PQC

##### 7.2.1.1.2	Solution details

If the indication in USIM is the SUCI calculation should be done in ME, and operator’s decision is to use PQC to calculate the SUCI, then the public key for calculating SUCI using PQC shall be available in USIM. The ME read the SUPI, the SUPI Type, the Routing Indicator, the Home Network Public Key Identifier for PQC, the Home Network Public Key for PQC and the list of protection scheme identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list are obtained from the USIM.

The alternative method is, there is no list of protection scheme identifiers, but only one identifier indicating UE to use PQC algorithm to calculate SUCI. UE will decide with algorithm to be used and attach it to the SUCI output. Network side will choose the same algorithm as the indication in SUCI and decode SUCI.

The candidate new profiles for SUCI may include below:

- ML-KEM [21]

Editor’s Note: Further details are FFS.

##### 7.2.1.1.3	Evaluation

#### 7.2.1.2	Solution #2 to SUCI calculation: Solution on PQC for SUCI protection

##### 7.2.1.2.1	Introduction

The ECIES procedure as depicted by the 5G system architecture [21] is the basis for the development of the PQC solution.

For the transition to PQC the relevant functional blocks will have to replace the existing/corresponding ECIES functional blocks.

The following Figure depicts the Encryption based on ECIES at the UE side.

Figure 7.2.1.2.1-1: Encryption based on ECIES at the UE

The following Figure depicts the Decryption based on ECIES at the home network side.

Figure 7.2.1.2.1-2: Decryption based on ECIES at the Home Network

##### 7.2.1.2.2	Solution details

Editor’s Note: Details on the KDF are FFS

Editor’s Note: Details on how this solution could be used for hybrid PQC are FFS

Editor’s Note: Why is MAC verification after decryption is FFS.

Editor’s Note: Whether and how to support hybrid scheme is FFS.

Editor’s Note: Why relevant functional blocks have to replace existing/corresponding ECIES functional blocks is FFS.

The solution is replacing the ECIES functional blocks with corresponding/related PQC related functional blocks.

The following Figure depicts the PQC concept at the UE side. The functions which must be modified for the support of PQC are with green coloured background.

Figure 7.2.1.2.2-1: SUCI protection based on PQC algorithms at the UE side

At UE: PQC KEM public key of HN is used in Key encapsulation mechanism to generate ciphertext and shared secret. This shared secret is used as an input to Key Derivation Function (KDF) to generate the Encryption key to generate cipher text of SUPI and MAC value.

The following is applicable:

This step 1, as shown by the Figure 7.2.x.y.1-1, is for the transition to PQC not required, i.e., there is no creation of Ephemeral Keys needed in this concept.

2> The Kem Encapsulation Function will get the public key (pk) as input and is providing the cipher text (ct) and the shared secret (ss). The (ct) will be have to be send back to the network, whereas the (ss) will be used as input to the key derivation function.

3> The key derivation function is receiving the shared secret (ss) and is calculating the encryption key. There will be created a single key that is to be used for encryption and integrity protection.

4> The encryption is used for the computation of the encrypted plaintext block, i.e., ciphertext value.

5> The encryption is used for the computation of the MAC-I, i.e., MAC-tag value.

Both the ciphertext and the MAC-tag value will be included into the SUCI framework (see Figure 7.2.x.y.2-3) and will be sent to the Network for further treatment.

At Network side: The received PQC KEM cipher text is used along with the PQC KEM Secret key of HN (corresponding to received PQC KEM public key Id) to decapsulate and generate the shared secret. This shared secret is used as an input to KDF to generate the decryption key to decipher the cipher text and verify the MAC.

The following Figure depicts the PQC concept at the Network side.

Figure 7.2.1.2.2-2: SUCI protection based on PQC algorithms at the Home Network side

The following is applicable:

1> The Network side is retrieving the cipher text (ct) from the SUCI framework. The secret key (sk) is local stored and corresponds to the public key (pk) which has been share with UE. Both, the (ct) and the (sk) will be given as input to the Key Decapsulation function and the outcome is the shared secret (ss). The (ss) will be used as input to the key derivation.

2> The key derivation function is receiving the shared secret (ss) as input and is computing the decryption key. There will be created a single decrypt key that is to be used for decryption and integrity verification.

3> The decryption key is used for the computation of the Plaintext block.

4> The decryption key is used for the computation of the MAC-I verification.

If we apply this solution concept, then the SUCI framework is as depicted by below Figure.

Figure 7.2.x.y.2-3: SUCI framework for PQC

Home Network Public Key Identifier (PQC KEM) represents a public key provisioned by the HPLMN or SNPN and it is used to identify the key used for SUPI protection. Example of the PQC KEM Public key is Kyber (selected by NIST standards).

PQC KEM ciphertext: Post Quantum Cryptography Key encapsulation mechanism uses the PQC KEM public key of Home Network to generate the ciphertext.

##### 7.2.1.2.3	Evaluation

TBD

#### 7.2.1.3	Solution #3 to SUCI calculation: SUCI calculation with hybrid KEMs

##### 7.2.1.3.1	Introduction

This solution proposes a hybrid encryption approach with both PQC and traditional cryptography for SUCI calculation. The proposed solution uses two different KEM algorithms for key derivation.

##### 7.2.1.3.2	Solution details

Editor’s Note: What is the advantage for presenting classical algorithm ECDH-KEM is ffs.

Editor’s Note: it is ffs whether there is no freshness aspect anymore as the stored key will be reused.

Editor’s Note: it is ffs how will the HN identify which key is used if there is no identifier.

Editor’s Note: it is ffs, for the MAC creation, there is no key used, this is just a hashing, not a keyed-hash.

Editor’s Note: SUCI size is ffs since with the c1c2 cipher text, new MAC, still SUCI size will be more than existing SUCI in 5G apart from the PQC addition.

Editor’s note: Details on how the MAC computation is performed are FFS.

Editor’s note: it is ffs the security issue introduced by using a non-keyed hash over part of the message.

Editor’s note: Why MAC on c1 and c2 is required is FFS.

The proposed solution is illustrated below. Figure 7.2.1.Y-1 shows the SUCI calculation at the UE. Figure 7.2.1.Y-2 shows the scheme output that the UE sends to the HN. Figure 7.2.1.Y-3 is the HN decryption of the SUCI from the UE.

Figure 7.2.1.3-1 SUCI calculation using hybrid KEM schemes at UE

1a. UE generates a shared key k1 and the corresponding ciphertext c1 based on the key encapsulation algorithm 1 (KEM1). The KEM1 uses ECDH-KEM with traditional cryptography as specified in NIST.SP.800-227 [73].

1b. UE generates a shared key k2 and the corresponding ciphertext c2 based on the key encapsulation algorithm 2 (KEM2). The KEM2 is PQC secure, and uses the ML-KEM-768 as specified in NIST FIPS 203 [21].

2a. UE generates a hybrid shared key (k) using KDF as specified in TS 33.501 [4], where k1||k2 is one of the inputs of the KDF.

2b. UE generates a MAC value 1 by hashing c1 and c2, e.g., MAC value 1 = SHA256 (c1||c2)

The MAC value 1 is used for the HN to verify correctness of c1 and c2 before performing the computation intensive cryptographic steps (e.g., steps 3-6 in Figure 7.2.1.Y-3).

3-5: UE continue with steps similar to the steps 3 -5 specified in clause C.3.2 in TS 33.501 [4].

The SUCI format generated by UE is as specified in TS 23.003 [74] and the Scheme Output as shown below includes the concatenation of the ciphertext c1||c2, MAC value 1, ciphertext c3 and MAC tag.

Figure 7.2.1.Y-2 The Scheme Output generated at the UE side

The processing of the received packet at the HN is shown in Figure 7.2.1.Y-3 with details as follows:

Figure 7.2.1.3-3 Decryption of SUCI at HN

1. HN verifies the received MAC value 1 of UE. If it succeeds, HN continues to perform the decapsulation of the shared key.

2a. HN decapsulates the shared key k1 based on the key encapsulation algorithm 1 (KEM1), the private key 1 of HN, and the received ciphertext c1 from UE. In the case of KEM1 = ECDH-KEM, ciphertext c1 is used as the ephemeral public key of the UE [75].

2b. HN decapsulates the shared key k2 based on the key encapsulation algorithm 2 (KEM2), the private key 2 of HN, and the received ciphertext c2 from the UE.

3. HN generates the shared key k in the same way at the UE side.

4-6: HN follows the steps 2-4 specified in clause C.3.3 of TS 33.501 [4].

##### 7.2.1.3.3	Evaluation

Editor’s note: Evaluation is FFS.

#### 7.2.1.4	Solution #4 to SUCI calculation: SUPI Pseudonym

##### 7.2.1.4.1	Introduction

This contribution proposes SUPI concealment using pseudonym instead of asymmetric encryption for SUPI.

##### 7.2.1.4.2	Solution details

The Figure 7.2.1.4.2-1 illustrates the procedure:

Figure 7.2.1.4.2-1 procedure of using random number to do SUPI concealment

0. The UE and the UDM are pre-configured with the UE’s SUPI and a pseudonym, i.e., a random value RAND.

1. During registration, the UE uses the preconfigured pseudonym RAND as the UE's SUCI sent over the air interface.

2-3. The UDM/AUSF maps the pseudonym RAND to SUPI and complete the authentication using the SUPI. The RAND can also be reused as the RAND in the primary authentication.

4-5. After authentication, the UDM assigns a new pseudonym RAND' for the SUPI and sends it to the UE.

6. The UE uses the newly assigned pseudonym RAND' in the subsequent procedure.

Editor’s Note: it is ffs that RAND without binding to any UE specific key or encryption or MAC value will result in the attacker is just sending and RAND number blocking the genuine UE.

Editor’s Note: it is ffs that just the RAND can’t be used for routing of the information.

Editor’s Note: How does pre-configured pseudonym prevent traceability is FFS.

##### 7.2.1.4.3	Evaluation

TBD

#### 7.2.1.5	Solution #5 to SUCI calculation: Enhancement on SUCI calculations using quantum key

##### 7.2.1.5.1	Introduction

This solution provides enhancement for SUCI calculations to resolve post-quantum threats to existing ECIES scheme.

##### 7.2.1.5.2	Solution details

This solution describes SUCI calculations using Quantum Channel. The UE can provision Public key of HN and Quantum Public Key. Based on ECIES scheme, the ephemeral public key, cipher text, and MAC tag can be generated as an output. Additionally, using the Quantum Public Key, the cipher text can be encapsulated. The encapsulated cipher text is delivered to the Home Network via Quantum Channel. The Home Network decapsulates it with Quantum private key, then deciphers ciphered text and verifies MAC.

##### 7.2.1.5.2.1	Processing on UE side

The steps shown Figure 7.2.X.Y.2.1 are described as below:

- As a prerequisite, the UE provisions both Public key of HN and Quantum Public key.

- The UE generates Ephemeral key pair consisting of Ephemeral Public Key and Ephemeral Private Key.

- Based on the generated Ephemeral Private Key and the Public key of Home Network, the UE generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Encryption Key, ICB and Ephemeral MAC Key are generated.

- Plaintext is ciphered using the Ephemeral Encryption Key.

- The ciphered text and the Ephemeral MAC key are used to create MAC-tag value.

- The ciphered text value is encapsuled using Quantum Public Key.

Figure 7.2.1.5.2.1: Encryption at UE

The final output shall be the concatenation of the ECC ephemeral public key, the Quantum encapsulated ciphertext value, the MAC tag value, and any other parameters, if applicable.

##### 7.2.1.5.2.2	Processing on home network side

The steps shown Figure 7.2.1.5.2.2 are described as below:

- By decapsulating the encapsulated cipher-text using Quantum Private Key, the Home Network generates the cipher-text.

- Based on the received Ephemeral Public Key, the Home Network generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Decryption Key, ICB and Ephemeral MAC Key are generated.

- The ciphered text is deciphered using the Ephemeral Decryption Key.

- The Home Network verifies received MAC.

Figure 7.2.1.5.2.2: Decryption at Home Network

Editor’s Note: Details on Step 6 at processing on UE side is FFS.

Editor’s Note: Details on Quantum Public key are FFS.

Editor’s Note: What is Quantum Channel is FFS.

Editor’s Note: Whether and how to support hybrid encryption.

Editor’s Note: Why SUCI should require a quantum channel into the HN is FFS.

Editor’s Note: What is a Quantum key and why it is required is FFS.

Editor's Note: The definition and usefulness of these terms for SUCI calculation are FFS: Post-quantum threat, Quantum channel, Quantum public key, Quantum-encapsulated, Quantum-encrypted, Quantum-ciphered, Quantum Private key.

##### 7.2.1.5.3	Evaluation

TBD

#### 7.2.1.6	Solution #6 to SUCI calculation: Enhancement on SUCI calculations using quantum encapsulated key

##### 7.2.1.6.1	Introduction

This solution provides enhancement for SUCI calculations to resolve post-quantum threats to existing ECIES scheme.

##### 7.2.1.6.2	Solution details

This solution describes SUCI calculations using Quantum Channel. The UE can provision Public key of HN and Quantum Public Key. Based on ECIES scheme, the ephemeral public key and MAC tag can be generated as a part of output. To cipher plain text, The Ephemeral Encryption key is encapsulated using Quantum Public Key. Using the Quantum-encapsulated Ephemeral Encryption key, the Plaintext is quantum-encrypted. The cipher text is delivered to the Home Network via Quantum Channel. The Home Network decapsulates the received quantum-ciphered text using HN-generated Ephemeral decryption key. By decrypting it using Quantum Private key, The Home Network obtains plain text. Then verifies received MAC.

##### 7.2.1.6.2.1	Processing on UE side

The steps shown Figure 7.2.1.6.2.1 are described as below:

- As a prerequisite, the UE provisions both Public key of HN and Quantum Public key.

- The UE generates Ephemeral key pair consisting of Ephemeral Public Key and Ephemeral Private Key.

- Based on the generated Ephemeral Private Key and the Public key of Home Network, the UE generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Encryption Key and Ephemeral MAC Key are generated.

- The plain text and the Ephemeral MAC key are used to create MAC-tag value.

- The Ephemeral Encryption Key is encapsulated using Quantum Public Key.

- The Plaintext Block is encrypted using the Quantum Encapsulated Ephemeral Encryption Key.

Figure 7.2.1.6.2.1: Encryption at UE

The final output shall be the concatenation of the ECC ephemeral public key, the Quantum ciphertext value, the MAC tag value, and any other parameters, if applicable.

##### 7.2.1.6.2.2	Processing on home network side

The steps shown Figure 7.2.1.6.2.2 are described as below:

- Based on the received Ephemeral Public Key, the Home Network generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Decryption Key and Ephemeral MAC Key are generated.

- The Home Network decapsulates the received Quantum-ciphered text using the Ephemeral Decryption Key.

- The Home Network decrypts the decapsulated Quantum-ciphered text using the Quantum Private Key. Then the Home network obtains the plain text.

- The Home Network verifies received MAC. For the verification, plaintext and Ephemeral MAC key are utilized.

Figure 7.2.1.6.2.2: Decryption at Home Network

Editor’s Note: Details on Step 5 at processing on UE side is FFS.

Editor’s Note: How to sync of usage of Quantum keys at UE and HN sides is FFS.

Editor’s Note: Details on Quantum Public key are FFS.

Editor’s Note: Details on Step 3 at processing on HN side is FFS.

Editor’s Note: Whether the plaint text is encrypted with quantum public key (the Encryption figure at UE) is FFS.

Editor’s Note: Why SUCI should require a quantum channel into the HN is FFS.

Editor’s Note: What is a Quantum key and why it is required is FFS.

Editor’s Note: How this solution is different from the solution in S3-253475 is FFS.

Editor's Note: The definition and usefulness of these terms for SUCI calculation are FFS: Post-quantum threat, Quantum channel, Quantum public key, Quantum-encapsulated, Quantum-encrypted, Quantum-ciphered, Quantum Private key.

##### 7.2.1.6.3	Evaluation

TBD

#### 7.2.1.7	Solution #7 to SUCI calculation: SUCI calculations

##### 7.2.1.7.1	Introduction

Annex C of TS 33.501 [4] specifies two protection schemes for concealing a SUPI into a SUCI. The protection schemes are called Profile A and Profile B. These two profiles use SECG ECIES [9], which is a so called KEM-DEM scheme — combining a Key Encapsulation Mechanism (KEM) and a Data Encapsulation Mechanism (DEM). SECG is unlikely to update its specifications. PQC migration of SUCI calculations does not require changing any protocols or architectures — it is sufficient to introduce new SUCI profiles.

Editor’s note: It is FFS whether the additional optional inputs to Key Combine which are sent in cleat text over the air can enhance security.

Editor’s note: For easier understanding, further details on how to implement the solution (e.g., the schematic figures as in 33501 and call flows) is FFS.

Editor’s note: For easier understanding, further details on hybrid keys and how hybrid scheme is realized is FFS.

Editor's note: Justification for mixing different security levels, i.e., ML-KEM-768 with AES-256, is FFS.

##### 7.2.1.7.2	Solution details

###### 7.2.1.7.2.1	General

PQC migration for SUCI calculations can be done by introducing new SUCI profiles, and the new SUCI profiles can be created by extending the existing SUCI profiles with simple algorithm updates. Using such extensions is not a new thing to do. It was also the case when 5G was specified — following recommendations from ETSI SAGE, 3GPP not only profiled SECG ECIES, but also extended it to support Montgomery curves like Curve25519, along with HMAC-SHA-256 (with 64-bit long tag).

Adding a PQC KEM (hybrid or standalone) is equally straightforward. Though the “EC” in ECIES gives the impression that it must use an elliptic curve, there are no technical obstacles to replacing the elliptic curve-based KEM in ECIES with either a standalone or a hybrid PQC KEM. It is similar to how TLS 1.3 continues to refer to KEM algorithms as the underlying algebraic groups and KEM encapsulations as KeyShares.

###### 7.2.1.7.2.2	ML-KEM is the Most Suitable Option

ML-KEM is already standardized, and its implementations are widely available. During the specification of SUCI protection in 33.501 [4], SA3 had considered the future need for PQC and therefore specified a maximum SUCI length of 3000 bytes to allow the introduction of quantum-resistant protection schemes. NIST has now standardized the lattice-based ML-KEM in FIPS 203 [21] and, as it was expected, both standalone and hybridized ML-KEM-512, ML-KEM-768, and ML-KEM-1024 fit in 3000 bytes.

Since Rel-15, IETF has specified HPKE — while ECIES is a pure KEM-DEM scheme, parts of HPKE requires Diffie-Hellman and cannot be implemented with a KEM. Besides, HPKE provides no clear benefits for SUCI calculations. In fact, for a fixed tag length, GCM provides worse integrity properties than HMAC-SHA2 and KMAC, which is the reason why ETSI SAGE has specified GCM-SST [76] for use in 6G. Using HPKE would also give up change control to the IETF.

###### 7.2.1.7.2.3	Considerations for Hybrid KEM

When using a hybridized PQC KEM with ML-KEM, it is essential to use a standardized key combiner that preserves the IND-CCA2 security of ML-KEM, hybridization must not weaken the security properties. While ML-KEM is currently the only practical option, the key combiner should be designed in a general way so that the same construction can be reused in future profiles with other KEMs beyond ML-KEM. Additional KEMs may be introduced in proprietary profiles or standardized by 3GPP in the future. Two standardized and compatible IND-CCA2 key combiners are specified in Section 4.6 of SP 800-227 [73] and Section 8.2 of ETSI TS 103 744 [30]. Below is equation (9) from SP 800-227 [73], which focuses on the information elements:

K ← KeyCombine(K1, K2, c1, c2, ek1, ek2, p)

###### 7.2.1.7.2.4	KDF, MAC, and Encryption

Any implementation of ML-KEM [21] already support of SHA3-256, SHA3-512, SHAKE128, and SHAKE256, which ML-KEM uses natively — therefore, using SHA-3 for key derivation and MAC in PQC SUCI is a natural choice. Also, SEC1 standard [9], specifying ECIES, published in 2009, says that future versions of the standard are likely to allow SHA3. Moreover, SHA-3 is theoretically (random oracle and no length extension attacks) and practically (strong side-channels resistance and simplicity) superior to SHA-2 [77]. Considering the ongoing work on 256-bit and AEAD study, all PQC SUCI profiles should use AES-256 for encryption.

###### 7.2.1.7.2.5	New SUCI Profiles

This solution proposes that the 3GPP SUCI profiles in TS 33.501 [4] should be updated to include profiles for both standalone ML-KEM and ML-KEM hybridized with X25519 — both fit into the designed length limit (3000 bytes). These profiles should use algorithms from the SHA-3 family (e.g., SHA3-256, KMAC256) [31, 32], both for the MAC and in the KDF.

Below are two suggested profiles, with the formatting intentionally left out.

Standalone ML-KEM Profile:

The parameters for this profile shall be the following:

-	KEM domain parameters		: ML-KEM-768

-	KEM primitive					: ML-KEM-768

-	point compression				: N/A

-	KDF								: ANSI-X9.63-KDF [9]

-	Hash								: SHA3-256

-	SharedInfo1						: ML-KEM encapsulation (ciphertext)

-	MAC								: KMAC256

-	mackeylen						: 32 octets (256 bits)

-	maclen							: 8 octets (64 bits)

-	SharedInfo2						: the empty string

-	ENC								: AES–256 in CTR mode

-	enckeylen							: 32 octets (256 bits)

-	icblen								: 16 octets (128 bits)

-	backwards compatibility mode	: false

Hybrid ML-KEM Profile:

The parameters for this profile shall be the following:

-	KEM domain parameters		: ML-KEM-768 + X25519

-	KEM primitive					: ML-KEM-768 + X25519

-	point compression				: N/A

-	KDF								: ANSI-X9.63-KDF [9]

-	Hash								: SHA3-256

-	SharedInfo1						: Combine(c1, c2, ek1, ek2, p)

-	MAC								: KMAC256

-	mackeylen						: 32 octets (256 bits)

-	maclen							: 8 octets (64 bits)

-	SharedInfo2						: the empty string

-	ENC								: AES–256 in CTR mode

-	enckeylen							: 32 octets (256 bits)

-	icblen								: 16 octets (128 bits)

-	backwards compatibility mode	: false

##### 7.2.1.7.3	Evaluation

#### 7.2.1.8	Solution #8 to SUCI calculation: GSMA-based solution

##### 7.2.1.8.1	Introduction

GSMA published guidelines "Post Quantum Cryptography – Guidelines for Telecom Use Cases – v2.0" [33] to support the planning, setup and execution of a quantum safe cryptography journey for telco industry. This GSMA report contains a detailed analysis of an initial set of Telcom use cases that are impacted by Post Quantum Cryptography. Concealment of the Subscriber Public Identifier is one of the analysed use cases.

An additional security enhancement is proposed to the solution described in GSMA guidelines [33].

##### 7.2.1.8.2	Solution details

The solution for concealment of the Subscriber Public Identifier is based on the hybridization between ML-KEM (Level 3) and classic ECC based key exchanged algorithms that is described in clause 5.8 of GSMA guidelines [33].

GSMA solution is enhanced thanks to the addition of Post Quantum ciphertext as input to the Key Derivation Function in the Post Quantum Cryptography part, as recommended to obtain IND-CCA (indistinguishability under chosen-ciphertext attack) property for KEM.

Processing on UE side:

Processing on home network side

Profiles

The associated updated profiles are the following ones. In both cases, the Key Derivation Function (KDF) outputs a L-bytes string that must be parsed as Eph Encryption key || ICB || Eph. Mac Key, where Eph Encryption key is of size enkeylen, ICB is of size icblen, and Eph. Mac Key is of size mackeylen.

###### 7.2.1.8.2.1	Profile A’ (update of Profile A to support PQC algorithm)

The ME and SIDF shall implement this profile. The parameters for this profile shall be the following:

-	KEM domain parameters						: ML-KEM-768 [21]

-	EC domain parameters							: Curve25519

-	KEM primitive									: ML-KEM-768 [21]

-	EC Diffie-Hellman primitive					: X25519

-	point compression								: N/A

-	KDF												: HMAC-based KDF RFC 5869 [34] (SHA-256)

-	Hash												: SHA-256

-	KDF inputs (see RFC 5869 [34] terminology):

-salt 											: empty

-IKM (input key material)					: Eph. shared key1 || Eph. shared key 2

-Info											: Post-Quantum Ciphertext || Eph. Public key

-L (output length in octets)					: 80

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 16 octets (128 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES-256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 16 octets (128 bits)

-	backwards compatibility mode					: false

###### 7.2.1.8.2.2	Profile B’ (update of Profile B to support PQC algorithm)

The ME and SIDF shall implement this profile. The parameters for this profile shall be the following:

-	KEM domain parameters						: ML-KEM-768 [21]

-	EC domain parameters							: secp256r1

-	KEM primitive									: ML-KEM-768 [21]

-	EC Diffie-Hellman primitive					: Elliptic Curve Cofactor Diffie-Hellman Primitive

-	point compression								: true

-	KDF												: HMAC-based KDF RFC 5869 [34] (SHA-256)

-	Hash												: SHA-256

-	KDF inputs (see RFC 5869 [34] terminology):

-salt 											: empty

-IKM (input key material)					: Eph. shared key1 || Eph. shared key 2

-Info											: Post-Quantum Ciphertext || Eph. Public key

-L (output length)								: 80

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 16 octets (128 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES-256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 16 octets (128 bits)

-	backwards compatibility mode					: false

Editor’s Note: It is FFS whether the additional inputs to KDF which are sent in cleat text over the air can enhance security.

Editor’s Note: Reasons for using c1c2 as the input for the KDF are FFS.

##### 7.2.1.8.3	Evaluation

TBD

#### 7.2.1.9	Solution #9 to SUCI calculation: SUPI Concealment using PQC Shared Key

##### 7.2.1.9.1	Introduction

To counter the threat of quantum computing to asymmetric cryptography used in ECIES scheme it is necessary to replace existing algorithms with new, quantum-resistant Post Quantum Cryptography (PQC) ML-KEM algorithms proposed by NIST [21].

##### 7.2.1.9.2	Solution details

###### 7.2.1.9.2.1 Processing on UE side

The PQC shared key generation scheme is implemented such that for computing a fresh SUCI, the UE uses the provisioned PQC-based public key of the home network, and PQC-based key encapsulation mechanism (KEM) according to the parameters provisioned by home network. The processing on UE side is done as mentioned below.

1. UE generates an ephemeral shared key and an encrypted PQC shared key based on a PQC-based public key associated with the home network.

2. UE generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and ephemeral shared key.

3,4. UE protects the plaintext block (i.e. SUPI or UE ID), using the encryption key and the MAC key. The final output is the concatenation of encrypted PQC shared key, the ciphertext (i.e., Enc(SUPI)) value, and MAC tag value.

The Figure 7.2.1.9.2-1 illustrates the UE's steps.

Figure 7.2.1.9.2-1: Encryption based on PQC shared key generation at UE

Finally, the proposed solution comprises transmitting the encrypted PQC shared key along with cipher-text value and MAC-tag value associated with the subscriber by the UE to a network entity for authenticating the subscriber. The scheme output as defined in TS 23.003 [74] to be updated to scheme output shown in Figure 7.2.1.9.2-2.

Figure 7.2.1.9.2-2: Scheme output based on SUPI concealment using PQC shared key

NOTE: Ciphertext output from PQC key encapsulation is referred to as encrypted PQC shared key as there is another ciphertext value from step 3 of symmetric encryption, to avoid confusion.

###### 7.2.1.9.2.2 Processing on home network side

The PQC shared key generation scheme is implemented such that for deconcealing a SUCI, the home network  uses the received encrypted PQC shared key, and the PQC-based private key of the home network.

1. Home network (HN) decapsulates the encrypted PQC shared key to derive the ephemeral shared key.

2. HN generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and derived ephemeral shared key.

3,4. HN verifies the MAC and decrypts the ciphertext to derive the plaintext block (i.e. SUPI or UE ID), using the MAC key and encryption key respectively.

Figure 7.2.1.9.2-3 illustrates the home network's steps.

Figure 7.2.1.9.2-3: Decryption based on PQC shared key generation at home network

NOTE: Ciphertext input to PQC key decapsulation is referred to as encrypted PQC shared key as there is another ciphertext value to step 3 of symmetric decryption, to avoid confusion.

###### 7.2.1.9.2.2	Sample profile for SUCI Calculation

Profile C uses ML-KEM as defined in [21] to generate shared key Z1 integrated with AES encryption scheme.

7.2.1.9.2.2.1	Profile C (PQC only)

The ME and SIDF implement this profile. The parameters for this profile are the following:

- 	ML KEM parameters							: Level 3 (k, lattice dimension 3)

-	KDF												: ANSI-X9.63-KDF [9]

-	Hash												: SHA-256

- 	Shared secret key Z1								: Shared secret field from ML-KEM

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 8 octets (64 bits)

-	SharedInfo1										: N/A

-	SharedInfo2										: the empty string

-	ENC												: AES–256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 32 octets (256 bits)

##### 7.2.1.9.3	Evaluation

TBD

#### 7.2.1.10	Solution #10 to SUCI calculation: SUPI Concealment using Hybrid shared Key

Editor’s Note: Details on KDF inputs are FFS.

Editor's Note: The pros and cons (including security, complexity and efficiency) of combining traditional asymmetric cryptographic algorithms with post-quantum cryptographic algorithms for SUCI calculation is FFS.

Editor’s Note: Why to use an ad-hoc KEM combiner instead of adding a standard KEM combiner is FFS.

Editor’s Note:  Detailed profiles needs to update later including other options.

##### 7.2.1.10.1	Introduction

Replacing classical cryptography with PQC algorithms at an early stage carries an inherent risk as a first time widespread deployment and more rigorous testing of PQC algorithms may be needed. So it will be beneficial to have it integrated with classical asymmetric cryptography based security mechanisms via a hybrid approach, where both classical asymmetric algorithms and post-quantum algorithms coexist. The main objective of a hybrid shared key generation mechanism is to enable the creation of a secure shared secret that remains protected as long as at least one of its underlying key exchange components remains uncompromised. In case vulnerabilities are found in either type of algorithm, the presence of both classical and post-quantum algorithms in a hybrid setup reduces the impact of potential breaches, providing additional resilience to the overall cryptography.

##### 7.2.1.10.2	Solution details

##### 7.2.1.10.2.1	Processing on UE side

The Hybrid shared key generation scheme is implemented such that for computing a fresh SUCI, the UE uses the provisioned EC based public key of the home network, provisioned PQC-based public key of the home network, freshly generated ECC (elliptic curve cryptography) ephemeral public/private key pair and PQC-based key encapsulation mechanism (KEM) according to the parameters provisioned by home network. The processing on UE side is done as mentioned below.

- UE generates an ephemeral EC public key and an ephemeral EC private key at UE with Elliptical Curve (EC) key generation function.

- UE generates a first ephemeral shared key (s1) based on the ephemeral EC private key of UE and an EC based home network public key.

- UE generates a second ephemeral PQC shared key (s2) and an encrypted PQC shared key based on a PQC-based public key associated with the home network using ML-KEM [aa].

- UE generates an ephemeral hybrid shared key based on the first ephemeral shared key and the second ephemeral shared key using methods like concatenation.

- UE generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and ephemeral hybrid shared key.

- UE protects the plaintext block (i.e. SUPI or UE ID), using the encryption key and the MAC key. The final output is the concatenation of the ECC ephemeral public key, the encrypted PQC shared key, the ciphertext value, the MAC tag value.

Figure 7.2.1.10.2-1 illustrates the UE's steps.

Figure 7.2.1.10.2-1: Encryption based on Hybrid shared key generation at UE

Finally, the proposed solution comprises transmitting the encrypted PQC shared key along with the ephemeral public key of UE, the encrypted PQC shared key, the cipher-text value, and the MAC-tag value associated with the subscriber by the UE to a network entity for authenticating the subscriber. The scheme output as defined in TS 23.003 [74] to be updated to scheme output shown in Figure 7.2.X.Y.2-2.

Figure 7.2.1.10.2-2: Scheme output based on Hybrid PQC-based SUPI concealment

NOTE: Ciphertext output from PQC key encapsulation is referred to as encrypted PQC shared key as there is another ciphertext value from step 3 of symmetric encryption, to avoid confusion.

##### 7.2.1.10.2.2	Processing on home network side

The Hybrid shared key generation scheme is implemented such that for deconcealing a SUCI, the home network uses the received ECC ephemeral public key of the UE, encrypted PQC shared key, EC based private key of the home network and the PQC-based private key of the home network.

- Home network (HN) generates a first ephemeral shared key (s1) based on the ephemeral EC public key, received from UE, and an EC based home network private key.

- HN decapsulates the encrypted PQC shared key, received from UE, to derive the second ephemeral shared key (s2) using ML-KEM [aa].

- HN generates an ephemeral hybrid shared key based on the first ephemeral shared key (s1) and the second ephemeral shared key (s2) using methods like concatenation.

- HN generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and ephemeral hybrid shared key.

- HN verifies the MAC and decrypts the ciphertext to derive the plaintext block (i.e. SUPI or UE ID), using the MAC key and encryption key respectively.

Figure 7.2.1.10.2-3 illustrates the home network's steps.

Figure 7.2.1.10.2-3: Decryption based on Hybrid shared key generation at home network

NOTE: Ciphertext input to PQC key decapsulation is referred to as encrypted PQC shared key as there is another ciphertext value to step 3 of symmetric decryption, to avoid confusion.

## 7.2.1.10.2.3	Sample Profiles for SUCI calculation

Profile C uses Post-Quantum Traditional (PQ/T) hybrid scheme as defined in RFC 9794 [7] which is a multi-algorithm scheme where at least one component algorithm is a post-quantum algorithm and at least one is a traditional algorithm. The traditional algorithm component uses its own standardized processing for key generation (section 6 of RFC 7748 [35]) and shared secret calculation (section 5 of RFC 7748 [35]). The Diffie-Hellman primitive X25519 (section 5 of RFC 7748 [35]) takes two random octet strings as input, decodes them as scalar and coordinate, performs multiplication, and encodes the result as an octet string. The shared secret output octet string from X25519 is used as the input Z in the ECIES KDF (section 3.6.1 of [9]). The post-quantum algorithm component of PQ/T scheme uses ML-KEM as defined in [aa]. Final shared secret key Z1	is derived from combining Z and shared secret generated from ML-KEM [aa]. Use the key derivation function KDF to generate keying data K of length enckeylen + icblen + mackeylen octets from Z1 and [SharedInfo1]. As the point compression is not applied for profile C, the prefix rule for compression type defined in [9] section 5.1.3 is not be used in profile C, i.e., there is no prefix for the ephemeral public key of Profile C.

Profile D uses Post-Quantum Traditional (PQ/T) hybrid scheme as defined in RFC 9794 [7] which is a multi-algorithm scheme where at least one component algorithm is a post-quantum algorithm and at least one is a traditional algorithm. The traditional algorithm component uses point compression to save overhead and use the Elliptic Curve Cofactor Diffie-Hellman Primitive (section 3.3.2 of [9]) to enable future addition of profiles with cofactor h ≠ 1. For curves with cofactor h = 1 the two primitives (section 3.3.1 and 3.3.2 of [9]) are equal. The post-quantum algorithm component of PQ/T scheme uses ML-KEM as defined in [aa]. Final shared secret key Z1 is derived from combining Z and shared secret generated from ML-KEM [aa]. Use the key derivation function KDF to generate keying data K of length enckeylen + icblen + mackeylen octets from Z1 and [SharedInfo1].

### 7.2.1.10.2.3.1	Profile C (Hybrid 1)

The ME and SIDF implement this profile. The parameters for this profile are the following:

- 	Identifier											: X25519MLKEM768 (Combining X25519 ECDH with ML-KEM-768)

-	EC domain parameters							: Curve25519 [35]

-	EC Diffie-Hellman primitive					: X25519 [35]

-	point compression								: N/A

- 	ML-KEM parameters							: Level 3 (k, lattice dimension 3)

-	KDF												: ANSI-X9.63-KDF [9]

-	Hash												: SHA-256

-	SharedInfo1										:  R(the ephemeral public key octet string – see [9] section 5.1.3)

- 	Shared secret key Z1								:  Z (see [9] section 5.1.3) || Shared secret field from ML-KEM

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 8 octets (64 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES–256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 32 octets (256 bits)

### 7.2.1.10.2.3.2	Profile D (Hybrid 2)

The ME and SIDF implement this profile. The parameters for this profile are the following:

- 	Identifier											: SecP256r1MLKEM768 (Combining secp256r1 ECDH with ML-KEM-768)

-	EC domain parameters							: secp256r1 [10]

-	EC Diffie-Hellman primitive					: Elliptic Curve Cofactor Diffie-Hellman Primitive [9]

-	point compression								: true

- 	ML-KEM parameters							: Level 3 (k, lattice dimension 3)

-	KDF												: ANSI-X9.63-KDF [9]

-	Hash												: SHA-256

-	SharedInfo1										:  R(the ephemeral public key octet string– see [9] section 5.1.3)

- 	Shared secret key Z1								:  Z (see [9] section 5.1.3) || Shared secret field from ML-KEM

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 8 octets (64 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES–256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 32 octets (256 bits)

##### 7.2.1.10.3	Evaluation

TBD

#### 7.2.1.11	Solution #11 to SUCI calculation: SUPI Concealment using hybrid method

Editor’s Note: Performances due to PQC operations performed after ECIES operations are FFS.

Editor’s Note: The pros and cons (including security, complexity and efficiency) of combining traditional asymmetric cryptographic algorithms with post-quantum cryptographic algorithms for SUCI calculation is FFS.

##### 7.2.1.11.1	Introduction

Replacing classical cryptography with PQC algorithms at an early stage carries an inherent risk as a first time widespread deployment and more rigorous testing of PQC algorithms may be needed. So it will be beneficial to have it integrated with classical asymmetric cryptography based security mechanisms via a hybrid approach, where both classical asymmetric algorithms and post-quantum algorithms coexist. In case vulnerabilities are found in either type of algorithm, the presence of both classical and post-quantum algorithms in a hybrid setup reduces the impact of potential breaches, providing additional resilience to the overall cryptography. The hybrid method described here is applying PQC-based key encapsulation mechanism (KEM) to protect final output which is generated via ECIES.

##### 7.2.1.11.2	Solution details

##### 7.2.1.11.2.1 Processing on UE side

The processing on UE side is done as follows.

Figure 7.2.1.11.2.1-1: SUCI generation using hybrid method at UE

1. UE generates a final output_ECC using ECIES as described in Annex C.3.2 in TS 33.501 [4], where the final output_ECC is Eph. EC public key||ciphertext||MAC tag.

2. UE generates an ephemeral shared key (KPQC) and an encrypted PQC shared key based on a PQC-based public key associated with the home network.

3. UE generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and KPQC.

4. UE protects the final output_ECC using the encryption key and the MAC key. The final output is the concatenation of encrypted PQC shared key, ciphertext (i.e., Enc(Eph EC public key||ciphertext||MAC)), and MAC tag value.

Figure 7.2.1.11.2.1-1 defines the scheme output (i.e., the final output in step 4) as a result of the above steps, as defined in TS 23.003 [74].

Figure 7.2.1.11.2.1-2: Scheme output based on hybrid method

NOTE: Ciphertext output from PQC key encapsulation is referred to as encrypted PQC shared key as there is another ciphertext value from step 3 of symmetric encryption, to avoid confusion.

##### 7.2.1.11.2.2 Processing on home network side

The processing on home network (HN) side is done as follows.

Figure 7.2.1.11.2-3: Decryption based on hybrid method at home network

1. Home network (HN) decapsulates the encrypted PQC shared key to derive the ephemeral shared key (KPQC).

2. HN generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and KPQC.

3. HN verifies the MAC and decrypts the ciphertext to derive the final output_ECC, using the MAC key and encryption key respectively.

4. HN obtain the plaintext block (i.e., UE ID) using ECIES as described in Annex C.3.3 in TS 33.501 [4].

NOTE: Ciphertext input to PQC key decapsulation is referred to as encrypted PQC shared key as there is another ciphertext value from step 3 of symmetric decryption, to avoid confusion.

##### 7.2.1.11.3	Evaluation

TBD

#### 7.2.1.12	Solution #12 to SUCI calculation: Hybrid SUCI calculation

##### 7.2.1.12.1	 Introduction

This solution addresses the key issue#1.

##### 7.2.1.12.2 	Solution details

EN#1: The details of the Combiner Function (3>) is FFS.

SUPI and SUCI type

The SUPI is a globally unique 5G Subscription Permanent Identifier allocated to each subscriber in the 5G System. It is defined in clause 5.9.2 of 3GPP TS 23.501 [85].

The SUPI is defined as:

-	a SUPI type: in this release of the specification, it may indicate an IMSI, a Network Specific Identifier (NSI), a Global Line Identifier (GLI) or a Global Cable Identifier (GCI); and

-	dependent on the value of the SUPI type:

-	an IMSI as defined in clause 2.1 of TS 23.003 [74];

-	a Network Specific Identifier (NSI), taking the form of a Network Access Identifier (NAI) as defined in clause 28.7.2 of TS 23.003 [74];

-	a Global Cable Identifier (GCI) taking the form of a NAI as defined in clause 28.15.2 of TS 23.003 [74];

-	a Global Line Identifier (GLI) taking the form of an NAI as defined in clause 28.16.2 of TS 23.003 [74].

NOTE:	Depending on the protocol used to convey the SUPI, the SUPI type can take different formats.

The SUCI is a privacy preserving identifier containing the concealed SUPI. It is defined in clause 6.12.2 of 3GPP TS 33.501 [4].

The SUCI is composed of the following parts:

1)	SUPI Type, consisting in a value in the range 0 to 7. It identifies the type of the SUPI concealed in the SUCI. The following values are defined:

-	0: IMSI

-	1: Network Specific Identifier (NSI)

-	2: Global Line Identifier (GLI)

-	3: Global Cable Identifier (GCI)

-	4 to 7: spare values for future use.

2)	Home Network Identifier, identifying the home network of the subscriber.

When the SUPI Type is an IMSI, the Home Network Identifier is composed of two parts:

-	Mobile Country Code (MCC), consisting of three decimal digits. The MCC identifies uniquely the country of domicile of the mobile subscription.

-	Mobile Network Code (MNC), consisting of two or three decimal digits. The MNC identifies the home PLMN or SNPN of the mobile subscription.

When the SUPI type is a Network Specific Identifier (NSI), a GLI or a GCI, the Home Network Identifier consists of a string of characters with a variable length representing a domain name as specified in clause 2.2 of IETF RFC 7542. For a GLI or a GCI, the domain name shall correspond to the realm part specified in the NAI format for SUPI in clauses 28.15.2 and 28.16.2.

3)	Routing Indicator, consisting of 1 to 4 decimal digits assigned by the home network operator and provisioned in the USIM, that allow together with the Home Network Identifier to route network signalling with SUCI to AUSF and UDM instances capable to serve the subscriber.

Each decimal digit present in the Routing Indicator shall be regarded as meaningful (e.g., value "012" is not the same as value "12"). If no Routing Indicator is configured on the USIM, this data field shall be set to the value 0 (i.e., only consist of one decimal digit of "0").

4)	Protection Scheme Identifier, consisting in a value in the range of 0 to 15 (see Annex C.1 of 3GPP TS 33.501 [4]). It represents the null scheme, or a non-null scheme specified in Annex C of 3GPP TS 33.501 [4], or a protection scheme specified by the HPLMN; the null scheme shall be used if the SUPI type is a GLI or GCI.

5)	Home Network Public Key Identifier (traditional), consisting in a value in the range 0 to 255. It represents a public key provisioned by the HPLMN or SNPN and it is used to identify the key used for SUPI protection. This data field shall be set to the value 0 if and only if null protection scheme is used.

6)	Home Network Public Key Identifier (PQC KEM), consisting in a value in the range 0 to 255. It represents a public key provisioned by the HPLMN or SNPN and it is used to identify the key used for SUPI protection apart from traditional HN public key. Example of the PQC KEM Public key is ML-KEM [21] (selected by NIST standards).

Note: If the above 6 needs to be avoided, then it could be merged with 5 and sent as a bitmap, where the bits are set for those traditional and PQC identifiers (known at USIM and UDM). Example: 01 for traditional alone, 10 for PQC alone and 11 for both traditional and PQC.

So, the bitmap and public keys ID (traditional& PQC) needs to fit into 5).

Moreover this 6) is optional and present only when hybrid key exchange is available.

7)	Scheme Output, consisting of a string of characters with a variable length or hexadecimal digits, dependent on the used protection scheme, as defined below. It represents the output of a public key protection scheme specified in Annex C of 3GPP TS 33.501 [4] or the output of a protection scheme specified by the HPLMN.

PQC KEM ciphertext: Post Quantum Cryptography Key encapsulation mechanism uses the PQC KEM public key of Home Network to generate the ciphertext.

At UE, generate key pair (Ephemeral public key and private key) using key pair generation primitive. Based on the Diffie-Hellman primitive, a shared secret key element is derived (from public key of HN and generated ephemeral private key). PQC KEM ciphertext(ct) is generated using the Key encapsulation mechanism (asymmetric cryptographic scheme) where PK is PQC KEM public key of HN (PQC KEM public key is identified by PQC KEM public key ID and UDM/USIM has list of PQC KEM public keys). A PQC shared secret (ss) is also generated which is used as an input to a Key Derivation Function (KDF) to derive the final PQC shared secret.

Followed by that, key combiner function is used to combine the traditional shared key and newly generated final PQC shared secret (from the KDF function) to generate AEAD key K. With the derived key, symmetric encryption (AEAD) is performed to encrypt the plaintext block (SUPI) to generate the ciphered text and the MAC key.

This hybrid PQC solution is making use of two separate HN identifiers, one for the classic (traditional) and yet another for the hybrid PQC. Consequently, at SIDF, the received UE ephemeral public key and stored private key of home network (Traditional HN public key ID is used to fetch the corresponding HN private key) is used to generate the ephemeral shared key. PQC KEM ciphertext(ct) is used along with the PQC KEM secret key of HN– SK (PQC Public key ID received from UE is used to fetch the Secret Key SK in HN), in Key decapsulation mechanism (asymmetric cryptographic scheme) to generate the shared secret. The newly generated shared secret is used along with the traditional ephemeral shared key as inputs to the key combiner function functions to generate AEAD Key. The generated AEAD Key is used to de-cipher the cipher text using symmetric decryption (AEAD). The expected MAC is compared against the received MAC, and with this comparison the integrity of the SUCI is verified.

##### 7.2.1.12.3 	Evaluation

TBD

Editor’s Note: Further evaluation to be added.

#### 7.2.1.13	Solution #13 to SUCI calculation: Symmetric crypto based SUCI

Overview: The UDM is creating a collection of relevant SUPI values. The UDM is encrypting by using symmetric crypto each of SUPI values from the list and is sending these to the UE. The UE is appending the selected encrypted SUPI and is hashing the encrypted SUPI together with the concatenated hashed Key KSUPI. The UDM can verify the authenticity of the SUPI. The UDM can verify the authenticity of the UE, and in successful case the UDM-UE auth can be processed.

##### 7.2.1.13.1	Introduction

This solution addresses the key issue#1.

##### 7.2.1.13.2	Solution details

Editor’s Note: The description of a resynchronisation procedure is FFS.

Editor’s Note: It is FFS about the first registration procedure.

Editor’s Note: It is FFS How are new encrypted SUPIs provisioned to UEs.

Editor’s Note: It is FFS How does the UDM regenerate the new symmetric keys for Pseudonyms.

Editor’s Note: Resynchronization of desynchronized RANDs is FFS.

Editor’s Note: Impact on fulfilling LI requirements is FFS.

Editor’s note: The elaboration for why the SUCI has to be encrypted is FFS.

Overview of Encrypted SUCI in UDM:

Figure 7.2.1.13.2-1: Encrypted SUCI in UDM (overview)

In the above Figure 7.2.1.13.2-1 the list of Encrypted SUCI is generated in UDM is shown. The UDM will first generate the key KSUPI using hash of SUPI, long term credentials of Subscriber K, RANDSUPI. Also, in parallel there will be list of Random numbers (RANDSUCI#1, RANDSUCI#2, etc) are generated using PRNG (Pseudo Random Number Generator). Use the newly generated key KSUPI to encrypt all the RANDSUCI#1, RANDSUCI#2, etc, to generate Enc(RANDSUCI#1), Enc(RANDSUCI#2), etc.

Overview of Temporary SUCI:

Figure 7.2.1.13.2-2: Temporary SUPI (overview)

The following steps are applicable:

Pre-configuration Phase

1.) The UDM is creating the list of SUCI values and is encrypting these by using symmetric cryptography. (Rationale: Actually, symmetric cryptography is assumed to be quantum safe.) The UDM is creating a Key KSUPI for the SUPI encryption and must store this key, because this key is later used for the decryption.

The SUCI values will be computed as shown by Figure 7.2.1.13.2-1. The SUCI values will be encrypted to provide privacy during transition phase.

The KSUPI is the hashed output value of input parameters/values (i.e., long-term key K, RANDSUPI, and hash value of SUPI). For the KDF, the hash functions of the SHA-3 family are considered quantum-resistant, i.e., digests (hash values) that are 128, 224, 256, 384 or 512 bits, are candidates for use in the KDF. It can be assumed, that the UDM has sufficient processing capacity to run the KDF for KSUPI computation.

2.) The list of encrypted SUCI’s is sent to the UE along with RANDSUPI. This RANDSUPI is used by UDM to generate the Key KSUPI (reference “Overview of Encrypted SUCI in UDM” of this document).

The RANDSUPI is a random value with predefined length and is used for freshness purposes. For random number generation the NIST Special Publication 800-90A [86] is to be used as reference. The RANDSUPI is not encrypted, which is similar to the RAND from the AV in EAP-AKA (refer to TS 33.501 [4]). The privacy of the UE can NOT be compromised by disclosing the RANDSUPI, because the RANDSUPI can NOT be used for identification of the subscriber.

3.) The UE is storing the received list of encrypted SUCI’s. USIM/ ME will also use RANDSUPI to generate Key KSUPI.

The storage place of the SUCI should non-volatile memory. RANDSUCI values have been used should be moved into volatile memory. The KSUPI must be processed, because this is providing a binding to the specific long-term key, basically, this is providing a proof-of-possession, i.e., the encrypted RANDSUCI, which will be sent to the Network includes the long-term key K.

Registration Phase

4.) The UE is now selecting one encrypted SUCI.

5.)  The UE is sending the registration request to the UDM and is added new processed values into this message. The following needs to be processed by the UE: The root key is the Key K which is stored inside the USIM of that UE. The UE is creating a hash of that Key KSUPI. Furthermore, the UE is concatenating the Encrypted SUCI and is hashing both, the Encrypted SUCI together with the hashed Key KSUPI. The rationale for creating this concatenation is the following: The Encrypted SUCI is used by the UDM to verify the authentication of the SUCI value, while the hashed key KSUPI is used by the UDM to verify the authenticity of the UE (could also be called, the legitimacy of the UE for sending these information elements).

6.) The UDM is receiving the Registration Request message and is first using the Encrypted SUCI value for the look-up on which key is needed for the decryption. Now since the UDM knows which key is to be used and since it knows the UE, the UDM is taking the fetching the computed key KSUPI, is hashing this, and is fetching the encrypted SUCI from the local stored memory and is hashing the concatenated encrypted SUCI and the hashed key KSUPI. The outcome of this hashing (refers to the expected hash) will be compared with the received hash value.

7.) This refers to the registration and auth execution and completion. Rationale: The execution steps above refer basically to the auth of the encrypted SUCI and the authentication (legitimacy) of the UE. After this the normal Auth needs to be processed.

8.) After successful encrypted SUCI usage, both UE and UDM deletes this value from the list and same UE can’t use the same for further communications.

9.)10.)11.) This refers to the renewing and deployment of new list of encrypted SUCI values. UDM could use old RANDSUPI and continue to use the KSUPI for encryptions, but also UDM could decide to refresh this key KSUPI by creating new RANDSUPI and pass it to UE.

##### 7.2.1.13.3	Evaluation

TBD

Editor’s Note: Further evaluation to be added.

#### 7.2.1.14	Solution #14 to SUCI Calculation: Symmetric solution on SUCI protection

##### 7.2.1.14.1	Introduction

Editor’s Note: Analysis on the probability of desynchronization of eSUCIs is FFS.

Editor’s Note: Resynchronization of desynchronized eSUCIs is FFS.

Editor’s Note: The benefit of this solution is FFS.

The following are principles of the solution:

-	UE is able to be provisioned with an enhanced SUCI (eSUCI) by UDM, or by pre-configuration, which is calculated with quantum resistant symmetric algorithm, symmetric home network key, and SUPI.

-	If eSUCI is available, UE uses the eSUCI for initial Registration procedure.

-	UDM calculates new eSUCI and updates towards UE after initial Registration procedure, UE does not calculate the eSUCI.

The following figure depicts the Encryption based on quantum resistant symmetric algorithm and symmetric key at the home network side.

Figure 7.2.1.14.1-1: Encryption based on symmetric key and algorithm at the Home Network

The following figure depicts the Decryption based on quantum resistant symmetric algorithm and symmetric key at the home network side.

Figure 7.2.1.14.1-2: Decryption based on symmetric key and algorithm at the Home Network

The Symmetric Key of Home Network, which is not a per UE key, is resident in SIDF/UDM and NOT shared with UE.

##### 7.2.1.14.2	Solution details

The following figure depicts the initial Registration procedure using eSUCI, which is calculated with quantum resistant symmetric algorithm, symmetric home network key, and SUPI.

Figure 7.2.1.14.2-1: Initial Registration with eSUCI

1.	If an enhanced SUCI (eSUCI), which is generated as described in clause 7.2.1.14.1, is provisioned during previous initial Registration procedure or pre-configured in UE (e.g., in NVM of ME or in USIM), the UE sends initial Registration Request (eSUCI) message to AMF/SEAF. If eSUCI is not available in UE, the UE uses asymmetric method (e.g. legacy or enhanced) to calculate a SUCI as an eSUCI for the initial Registration procedure.

Editor’s Note: Format of eSUCI is FFS.

Editor’s Note: Clarification on step 1 is ffs, e.g. proof-of-possession, exception case.

2.	AMF/SEAF invokes Nausf_UEAuthentication_Authenticate Request (eSUCI) towards AUSF.

3.	AUSF invokes Nudm_UEAuthentication_Get Request (eSUCI) towards SIDF/UDM.

4.	SIDF decodes the eSUCI to get SUPI as described in clause 7.2.1.14.1 or using asymmetric method.

5.	The UDM, AUSF, AMF/SEAF, and UE performs authentication procedure based on the SUPI decoded from the eSUCI.

6.	If the authentication succeeds, SIDF/UDM calculates a new eSUCI as described in clause 7.2.1.14.1.

7.	SIDF/UDM returns the calculated eSUCI to AUSF.

8.	AUSF responds to AMF/SEAF with the new eSUCI.

9.	AMF sends Registration Response (new eSUCI) to UE.

10.	UE stores the new eSUCI, e.g. in the NVM of ME or in USIM, which will be used for successive initial Registration procedure.

##### 7.2.1.14.3	Evaluation

TBD

#### 7.2.1.15	Solution #15 to SUCI calculation: SUCI calculation with symmetric key

##### 7.2.1.15.1	Introduction

This solution derives the encryption key EK, ICB and MAC key MK from the root key K. The encryption key length and MAC key length are increased to 256 Bit and AES-256-CTR is used for encrypting the SUPI.

##### 7.2.1.15.2	Solution details

##### 7.2.1.15.2.1 Processing on UE side

The UE generates a 256 Bit Nonce#1 and similar to MILENAGE, the UE creates two additional Nonces by using a 64 Bit rotate operation. The Nonce#2 is created by rotating/shifting 64 Bits to the left of Nonce#1 and Nonce#3 by rotating/shifting 64 Bits to the left of Nonce#2.

The 256 Bit Encryption key EK is derived using a HMAC–SHA-256 with the root key K and the Nonce#1 as input.

The 128 Bit ICB is derived using a HMAC–SHA-256 with the root key K and the Nonce#2 as input with the output hash truncated to the 128 most significant bits.

The 256 Bit MAC key MK is derived using a HMAC–SHA-256 with the root key K and the Nonce#3 as input.

The EK and ICB are input to the AES-256-CTR, the output is a 256 Bit ciphertext of the encrypted SUPI.

The MK is used with the Nonce and the Ciphertext as input to a HMAC-SHA-256 function to generate a 256 Bit long MAC.

Since the computation is different, a 6G SUCI indication is required that the SUCI is differently concealed as in 5G.

The full SUCI has then the format as shown below with SUCI = 6G SUCI Indication  || Nonce#1  || Ciphertext || MAC.

##### 7.2.1.15.2.2 Processing on home network side

The home network detects the new SUCI format based on the 6G SUCI indication.

The home network creates based on the received Nonce#1 the two additional nonces Nonce#2 and Nonce#3.

The home network derives the 256 Bit Encryption key EK, 128 Bit ICB and 256 Bit MAC key MK in the same way as in the UE, using the Nonce#1, Nonce#2 and Nonce#3 respectively.

The home network verifies the MAC and decryptes the SUCI to SUPI.

Editor’s Note: For easier understanding of the solution described, further details on how to implement the solution (e.g., the schematic figures as in TS 33501 and call flows) is FFS.

Editor’s Notes: it is FFS how the network identifies which K to be used to derive the EK.

Editor’s Note: Details about the management of the root key, including generation, agreement, storage, revocation, etc. are FFS.

##### 7.2.1.15.3	Evaluation

TBD

#### 7.2.1.16	Solution #16 to SUCI calculation: Solution for PQC based SUCI Computation

##### 7.2.1.16.1	Introduction

This solution address PQC algorithm based SUCI calculations.

##### 7.2.1.16.2	Solution details

Processing on UE side:

Figure 1a: Encryption based on PQC shared key generation at UE

The UE computes a fresh SUCI, using the provisioned PQC-based public key of the home network (HN), and PQC-based key encapsulation mechanism (KEM) according to the parameters provisioned by home network as follows:

- UE generates an ephemeral shared key and an encrypted PQC shared key based on a PQC-based public key associated with the home network. The PQC-based home network public key is identified using a HN PQC Public key ID or an existing HN Public key ID can indicate the HN PQC Public key with a related value.

2. 	UE generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and ephemeral shared key along with input parameters such as Freshness parameter i.e., timestamp and SUCI Protection Profile ID.

3,4. UE protects the plaintext block (i.e. SUPI or UE ID), using the encryption key and the MAC key. The final output is the concatenation of Freshness Parameter, SUCI Protection Profile ID, encrypted PQC shared key and the ciphertext.

The final output i.e., scheme output coding is upto stage 3 similar to TS 23.003 [74]. The computed SUCI along with scheme output is sent from UE to network for authenticating the subscriber.

Processing on home network side

Figure 1b: Decryption based on PQC shared key generation at home network

For deconcealing the SUCI, the home network uses the received encrypted PQC shared key, and the PQC-based private key of the home network along with other parameters as described in the steps below:

1. Home network (HN) decapsulates the encrypted PQC shared key to derive the ephemeral shared key.

2. HN generates ephemeral symmetric (de)encryption key and ephemeral MAC key using a KDF function and derived ephemeral shared key along with input parameters such as such as Freshness parameter i.e., timestamp and SUCI Protection Profile ID.

3,4. HN verifies the MAC and decrypts the ciphertext to derive the plaintext block (i.e. SUPI or UE ID), using the MAC key and (de)encryption key respectively.

Example profile for SUCI Calculation: Profile C (PQC only): Profile C uses ML-KEM as defined in [21] to generate shared key Z1 integrated with AES encryption scheme.

The ME and SIDF implement this profile. The parameters for this profile are the following:

- 	ML KEM parameters	: Level 3 (k, lattice dimension 3)

-	KDF						: ANSI-X9.63-KDF [9]

-	Hash						: SHA-256

- 	Shared secret key Z1		: Shared secret field from ML-KEM

-	MAC						: HMAC–SHA-256

-	mackeylen				: 32 octets (256 bits)

-	maclen					: 8 octets (64 bits)

-	SharedInfo1				: N/A

-	SharedInfo2				: the empty string

-	ENC						: AES–256 in CTR mode

-	enckeylen					: 32 octets (256 bits)

-	icblen						: 32 octets (256 bits)

##### 7.2.1.16.3	Evaluation

The solution has the following impacts:

New PQC algorithms and related profiles need to be supported by the UE and Network.

The UE generates ephemeral symmetric encryption key and ephemeral MAC key using a KDF function and ephemeral shared key along with input parameters such as Freshness parameter i.e., timestamp and SUCI Protection Profile ID. HN generates ephemeral symmetric (de)encryption key and ephemeral MAC key using a KDF function and derived ephemeral shared key along with input parameters such as such as Freshness parameter i.e., timestamp and SUCI Protection Profile ID. The use of timestamp and profile information as input allows replay protection for the SUCI and binds to the profile being used among multiple profiles respectively.

Editor’s Note 1: How the addition of freshness parameter is useful against an attack using CRQC is FFS.

### 7.2.2	Solutions to MIKEY-SAKKE key exchange

#### 7.2.2.1	Solution #1 to MIKEY-SAKKE key exchange: mitigate

##### 7.2.2.1.1	Introduction

There are a number of existing mitigations built into the Mission Critical system. Pending development of a post-quantum replacement for MIKEY-SAKKE it is possible these offer sufficient mitigation for threats, in particular harvest-now-decrypt-later. This is not proposed as a long term migration plan. That will require either a PQ identity based encryption scheme standard or a re-architecting the Mission Critical system.

##### 7.2.2.1.2	Solution Details

In the on-network case, MIKEY-SAKKE key exchanges are protected by one or more layers of additional cryptographic protections as specified by clauses 5 and 6 in TS 33.180 [3]. Assuming these protocols, e.g. IPsec, are migrated to quantum-safe alternatives, this mitigates the risk of a passive attacker being able to harvest keys from the UE-to-MCX server interface.

There are further built-in protections for on-network access such as secure authentication to the network which further limit what an adversary can do with a forged signature on an I_MESSAGE.

Internal MCX interfaces over which I_MESSAGEs may be transferred may be protected by mTLS. This is currently optional but could be made mandatory.

It is also possible to re-use a security context established on-network when communicating off-network. Deployments could consider prohibiting off-network exchanges as one mitigation without further changes to the protocol.

All of the above is either within the standard already, or a configuration/policy for the UE. Further mitigations could be developed either as part of the standards or as informative text. There is a gap in the off-network case however this is currently out of scope.

Editor’s Note: Whether further mitigations for the off-network case can be considered is FFS.

##### 7.2.2.1.3		Evaluation

Editor’s Note: This clause is FFS.

# 8	Conclusions

Editor’s Note: This clause contains agreed conclusions and any normative work is recommended.

# Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA3#123 | S3-252632 |  |  |  | TR 33.703 skeleton | 0.0.0 |
| 2025-08 | SA3#123 | S3-252975 |  |  |  | Incorporate pCRs from S3-252976, S3-252977, S3-252978, S3-252983, S3-252984, S3-253037 | 0.1.0 |
| 2025-10 | SA3#124 | S3-253686 |  |  |  | Incorporate pCRs from S3-253687, S3-253688, S3-253689, S3-253691, S3-253692, S3-253847, S3-253693, S3-253694, S3-253695, S3-253696, S3-253830, S3-253831, S3-253486, S3-253832, S3-253833, S3-253855, S3-253835, S2-253836, S3-253837, S3-253838, S3-253839, S3-253841, S3-253840, S3-253842, S3-253843, S3-253844, S3-253845 | 0.2.0 |
| 2025-11 | SA3#125 | S3-254534 |  |  |  | Incorporate pCRs from S3-254564, S3-254565, S3-254566, S3-254567, S3-254731, S3-254568, S3-254732, S3-254181, S3-254733, S3-254745, S3-254746, S3-254736, S3-254749, S3-254728 | 0.3.0 |