3GPP TSG-SA3 Meeting #125	S3-254618

Dallas, US, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.518 | CR | Draft CR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living document to TS 33.518 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,BSI (DE), Montsecure |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S3-253026: TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION in 33.117 includes tests for validating the properties in the CCA but it does not consider the timestamp (iat) value because this is NRF specific. S3-254617: TS 33.518 is missing a TC to verify the input in the access token request. An accompanying threat is provided in S3-254099. If the NRF does not verify the input parameters in the access token request against the corresponding ones in the public key certificate of the NF Service Consumer or those in the locally-stored NF profile of the NF Service Consumer, an NF instance can receive an access token that will allow it to access resources that are not permitted. This can lead to impersonation and elevation of privileges, allowing the NF to obtain services of an impersonated NF or an arbitrary set of services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | S3-253026: Add a test case for validating the timestamp (iat) in the CCA value as an addition to TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION in 33.117. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S3-253026: See section TR 33.926 [4], clause 6.3.4.1 for threats that could occur, if the CCA value is not validated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X(new), 4.2.2.X.1(new), 4.2.2.Y(new), 4.2.2.Y.1(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The merger of S3-253026, S3-254617 |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.2.X	Authentication for Indirect Communication

4.2.2.X.1	Correct handling of client credentials assertion validation failure

Requirement Name: Correct handling of client credentials assertion validation failure

Requirement Reference: TS 33.501 [10], clause 13.3.8.3: TS 29.500 [21], clause 6.7.5

Requirement Description:

The verification of the Client credentials assertion is performed by the receiving node, i.e., NRF or NF Service Producer in the following way:

-	It validates the signature of the JWS as described in RFC 7515 [16].

-	It validates the timestamp (iat) and/or the expiration time (exp) as specified in RFC 7519 [17].

-	If the receiving node is the NR	F, the NRF validates the timestamp (iat) and the expiration time (exp).

-	If the receiving node is the NF Service Producer, the NF service Producer validates the expiration time and it may validate the timestamp.

-	It checks that the audience claim in the client credentials assertion matches its own type.

It verifies that the NF instance ID in the client credentials assertion matches the NF instance ID in the public key certificate used for signing the assertion.

Threat References: TR 33.926 [4], clause 6.3.4.1, Incorrect validation of client credentials assertion

NOTE 1:	The following test case only applies if the NF under test implements verification of client credentials assertions.

NOTE 2:	This test case is an extension to TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION in TS 33.117 [2] and only includes the NRF-specific test for the validity of the iat value.

If the verification of the CCA fails at the receiving entity (e.g. NRF or NF service producer), a "403 Forbidden" response is returned with the cause attribute set to "CCA_VERIFICATION_FAILURE", as stated in TS 29.500 [21], clause 6.7.5.

Test Case:

Test Name: TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION_NRF

Purpose:

Verify that the NF under test correctly handles client credentials assertion validation failure. This test case specifically verifies that the NRF under test verifies that the iat value is valid.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a consumer NF and a SCP, which may be simulated. (Potentially simulated) consumer NF and (potentially simulated) SCP can be combined for the testing purpose.

-	The NRF under test is preconfigured with the certificate of the consumer NF.

-	The NRF under test is configured to require assertions for NF consumer authentication for at least one of its services.

-	The NRF under test has implemented the client credentials assertion (CCA) authentication method as specified in TS 33.501 [10], clause 13.3.8.3.

-	The tester has the private key of the consumer NF.

-	The tester has access to the interface between the consumer NF and the NRF under test.

Execution Steps

1.	The tester computes a client credentials assertion correctly, except that the timestamp (iat) value is in the future. The tester should make sure that the expiration time is greater than the timestamp (iat) value.

2.	The tester includes the client credentials assertion in the service request sent from the consumer NF to the NRF under test via the SCP.

3.	The tester captures the response sent back by the NRF under test.

Expected Results:

The NRF under test rejects the consumer NF's service request with a 403 Forbidden HTTP status code and sends back an error message according to the description TS 29.500 [21], clause 6.7.5.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of log messages or a packet trace. A packet trace should at least contain the messages sent between the NRF and the NF service consumer.

********** Next CHANGE **********

##### 4.2.2.Y	NF access token procedure

###### 4.2.2.Y.1	NF access token request verification failure handling within single PLMN

Requirement Name: Access token request verification

Requirement Reference: TS 33.501 [3], clause 13.4.1.1.2

Requirement Description:

According to TS 33.501 [3], clause 13.4.1.1.2, the NRF verifies the input parameters in the access token request as follows:

- It checks that the NF Instance ID and, if available, NF type and PLMN ID(s), in the access token request match the corresponding ones in the public key certificate of the NF service consumer or those in the NF profile of the NF service consumer. If the verification of the access token request fails at the NRF, the NRF shall return "400 Bad Request" status code, and "error" attribute set to "invalid_client", as stated in TS 29.510 [5], clause 5.4.2.2, and the access token request is not further processed.

- Based on the input attributes (e.g., NF Type, PLMN ID(s), scope), it checks that the NF service consumer is authorized to access the requested services from the NF service producer. When certain parameters in the access token request are not supported by the NRF, the NRF shall ignore the unsupported parameters and continue processing the request with the rest of the parameters.

- If available, it verifies the S-NSSAIs of the NF service consumer and checks whether there are restrictions on the NF service consumer to access NF Service Producers' services of a specific NF type depending on the slices for which they offer their services.

If the access token request fails, the NRF shall return an appropriate error status code (e.g., "400 Bad Request", "401 Unauthorized", "403 Forbidden")

Threat References: TR 33.926 [6], clause H.2.2.Y, Elevation of privilege via failure to verify access token request

Test Case:

Test Name: TC_ACCESS_TOKEN_REQUEST_VERIFICATION_NRF

Purpose:

Verify that the NRF under test correctly handles access token request verification.

Procedure and execution steps:

Pre-Conditions:

- The NRF under test is preconfigured with either the public key certificate or NF profile of the NF service consumer.

- The NRF under test is preconfigured with notional access token request authorization policies for an NF service consumer and the tester is provided documentation necessary to generate authorized access token requests.

- Test environment includes an NF service consumer.

- The NF service consumer may be simulated.

Execution Steps

Test Cases A~B are tests on failure handling by the NRF under test when the access token request failed verification

Test Case A: invalid client

1)	The tester constructs an access token request correctly, except that the NF instance ID is different from the NF instance ID of the NF service consumer public key certificate or NF profile stored in the NRF under test. The NF service consumer sends the access token request to the NRF under test.

2)	The NRF under test recognizes the NF instance ID does not match the NF instance ID of the NF service consumer public key certificate or NF profile stored in the NRF under test

3)	The access token request verification fails, and the NRF sends an appropriate response, as specified in TS 29.510 [5] clause 5.4.2.2 (e.g., 400 bad request, 403 forbidden, or 3XX for redirection).

Test Case B: unauthorized request

1)	The tester constructs an access token request correctly, except that the input attributes (e.g., NF Type, PLMN ID(s), scope) are not authorized based on preconfigured authorization policies. The NF service consumer sends the access token request to the NRF under test.

2)	The NRF under test recognizes that at least one of the input attributes implies the NF service consumer is not authorized for the requested services

3)	The access token request verification fails, and the NRF sends an appropriate response, as specified in TS 29.510 [5] clause 5.4.2.2 (e.g., 400 bad request, 403 forbidden, or 3XX for redirection).

Expected Results:

For test cases A~B on verification failure of the access token request parameters, the NRF under test rejects the NF service consumer’s service request.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of log messages or a packet trace. A packet trace should at least contain the messages sent between the NRF and the NF service consumer.

********** END OF CHANGES **********