3GPP TSG-SA3 Meeting #125	S3-254628

# Dallas, US, 17 - 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.513 | CR | Draft CR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | living document to TS 33.513 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT, China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to GSMA NESASG agreements, several corrections to 33.513 are needed. S3-254140 Change requests according to GSMA NESAS feedback, to ensure the SCAS is suitable for use in the NESAS scheme. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Complementing abbreviations not listed in TR 21.905. - Remove H-UPF from the simulated functions. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity for readers. Low quality of SCAS documents. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,3.3, 4.2.1, 4.2.2.3, 4.2.2.7, 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The merger of S3-253029,S3-253325,S3-253326, S3-254140 |  |  |  |  |  |  |  |  |

**************** START of CHANGES****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501: "System Architecture for the 5G System".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	Void.

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	Void.

[6]	3GPP TS 32.255: "Charging Management; 5G Data Connectivity Domain Charging".

[7]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

****************Next Change****************

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [7].

ESP	Encapsulating Security Payload

gNB	gNodeB

H-SMF	Home-Session Management Function

H-UPF	Home-User Plane Function

IKE	Internet Key Exchange

IPUPS	Inter-PLMN User Plane Security

SBA	Service Based Architecture

SBI	Service Based Interfaces

SEG	Security Gateway

SMF	Session Management Function

V-SMF	Visited-Session Management Function

****************Next Change****************

4.2.1	Introduction

The security functional requirements and the related test cases specific for UPF are described in the following clause.

****************Next Change****************

4.2.2.3	Replay protection of user data transported over N3 interface

Requirement Name: Replay protection of user data transported over N3 interface

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: The transported user data between gNB and UPF is replay protected as specified in TS 33.501, clause 9.3.

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data"

TEST CASE:

NOTE 1:	Void.

Test Name: TC_UP_DATA_REPLAY_UPF

Purpose:

Verify that the transported user data between gNB and UPF are replay protected.

Procedure and execution steps:

Pre-Condition:

-	UPF network product is connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the original user data transported via N3 reference point between gNB and UPF.

NOTE 2:	This test case is only applicable to UPF supporting IPSec in N3 interface without the use of a SEG.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported between UE and UPF is replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

****************Next Change****************

4.2.2.7	IPUPS

Requirement Name: IPUPS packeting handling

Requirement Reference: TS 33.501[2], clause 5.9.3.4

Requirement Description:

The IPUPS only forwards GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others as specified in TS 33.501 [2], clause 5.9.3.4.

Threat Reference: TR 33.926 [7], Clause L.2.5, "invalid user plane data forwarding"

TEST CASE:

NOTE 1:	This test case is only applicable to UPF supporting IPUPS.

Test Name: TC_IPUPS_PACKET_HANDLING

Purpose:

Verify that the packets not belonging to an active PDU session are discarded.

Pre-Conditions:

Test environment is set up with a V-SMF, an H-SMF, and a gNB. . The V-SMF, H-SMF and gNB may be simulated.

Execution Steps:

1)	The V-SMF requests the UPF with IPUPS functionality under test to establish an N4 session for a PDU session in home-routing roaming. The UPF with IPUPS functionality under test responds to the SMF with the F-TEID for the N9 tunnel towards the H-UPF, and the F-TEID for the N3 tunnel towards the gNB.

2)	The V-SMF requests the H-SMF to establish a PDU session providing the received F-TEID for the N9 tunnel.

3)	The H-SMF requests the H-UPF to establish an N4 session providing the received F-TEID for the N9 tunnel. H-UPF in the response provides its F-TEID for the N9 tunnel. The H-SMF provides the received F-TEID from the H-UPF to the V-SMF.

4)	The V-SMF requests the gNB to allocate resource for the PDU session providing the F-TEID for the N3 tunnel received at step 1. The gNB replies with its F-TEID for the N3 tunnel to the V-SMF.

5)	The V-SMF provides the UPF with IPUPS functionality under test with the received F-TEID assigned by the gNB for the N3 tunnel and the received F-TEID assigned by the H-UPF for the N9 tunnel.

6)	 The H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel.

7)	The H-UPF is triggered to send GTP-U packets using an F-TEID different than the one assigned by V-UPF for N9 tunnel.

Expected Results:

When the H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel (step 6 in the execution steps), GTP-U packets are witnessed over the N3 tunnel.

When the H-UPF is triggered to send GTP-U packets using an F-TEID different than the one assigned by the V-UPF (step 7 in the execution steps), no GTP-U packets are witnessed over the N3 tunnel.

Expected format of evidence:

Files recording the GTP packets captured (e.g. pcap trace).

****************Next Change****************

4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [3] are applicable to UPF.

The interfaces defined for the UPF are in clause 4.2.3 of TS 23.501 [4].

According to clause 4.4.4 of TS 33.117 [3], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for UPF, the following interfaces and protocols are in the scope of the testing:

-	For N3: the UDP and GTP-U protocols.

-	For N4: the UDP and PFCP protocols.

-	For N9: the UDP and GTP-U protocols.

-	For N19: the UDP and GTP-U protocols.

-	The N6 is the connection with the Data Network (DN). The protocols used in this interface are not defined by the 3GPP and are not under the scope of the present document.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [3].

**************** END of CHANGES****************