3GPP TSG-SA3 Meeting #125	S3-254629

Dallas, US, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.521 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 33.521 based on GSMA NESASG feedbacks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to GSMA NESASG feedbacks, several corrections to 33.521 are needed. S3-254645 The test case “Protecting data and information – data masking on integration analysis” is not complete/unclear. This pCR tries to enhance the pre-conditions and test steps in order to make the test case less ambiguous and automatable. Based on S3-253455 changes from SA3 Meeting #124. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complementing requirement reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not fully comply with 33.916. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,,4.2.1.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Is the merge of S3-254314 and S3-254645 |  |  |  |  |  |  |  |  |

**************** START of CHANGES****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[XX]	3GPP TS 29.552: "Network Data Analytics signalling flows".

**************** THE NEXT CHANGE****************

##### 4.2.1.2.6	Protecting data and information – Data masking on integration analysis

Requirement Name: Data masking on integration analysis about personal data

Requirement Reference: In accordance with industry best practice.

Requirement Description: NWDAF can collect data from OAM, MDAF and/or 5GC NFs (e.g. AMF) etc. for analytics purposes. SincePersonal data of the users could be involved, there is a potential privacy impact. As the NWDAF can expose its service operations with a request for bulked data, anonymization of data fields shall be applied to avoid exposing undesired information, aggregation levels.

Threat References: TR 33.926 [4], clause 5.3.6.7, Personal Identification Information Violation

Test Name: TC_DATA_MASKING

Purpose:

Verify that no privacy-related information of the subscribers  is disclosed to any entity who is not authorizedto access such information.

Pre-Condition:

Privacy information list (contains e.g. PII, location data, network identifiers, session information; should be specified based on local policy, regulation and others).

NOTE:	If user consent check is implemented, user consent for data collection is granted

The following entities are operational, integrated and simulated:

-	NWDAF

-	‘data producer’ (NF- or OAM as source for data collection which generates user data containing privacy info, e.g. AMF)

-	‘analytics consumer’ (NF- or OAM to which the NWDAF exposes analytics)

The data producer is configured to receive and accept subscription requests from the NWDAF for events according to TS 29.552 [XX], clause 5.5.1.1

Execution Steps:

1.	Tester triggers behaviour so that the ‘data producer’ is required to handle privacy information (e.g. for AMF trigger registration request at UE)

2.	The tester sends an Nnwdaf_AnalyticsInfo_Request request message from the ‘analytics consumer’ to NWDAF according to TS 29.552 [XX], clause 5.2.3.1. The request message shall be crafted to capture information from step 1.

3.	The tester retrieves the Nnwdaf_AnalyticsInfo_Request response message from the NWDAF.

Expected Results:

The analytics results do not reveal subscriber permanent identifier nor any other data listed on the Privacy information list.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot, pcap trace, log files containing the results.

**************** END of CHANGES****************