3GPP TSG-SA3 Meeting #103-e 	S3-21xxxx

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Ericsson

- Title:	AMF re-allocation: Analysis of solutions

- Document for:	Discussion

Agenda Item:	5.17

# 1	Decision/action requested

This contribution proposes an analysis of the solutions in the TR.

# 2	References

[1]	TR 33.864

# 3	Rationale

An analysis of all the solutions in the AMF re-allocation study in order to discussion with SA3 group and progress the conclusions.

# 4	Discussion

It would be desirable to solve the AMF problem with a solution that has only network impact in order for the UE not be affected. Our proposal is solution #9.  However in 3GPP consensus needs to be reached for concluding the study.

The following table summarizes the analysis of the solutions with respect to a few evaluation criteria.

|  | UE impact | Serving network impact | Home network impact | Security impact (whether the solution opens up for new security issues) | A common NF for the solution to work | Whether the solution requires that security context is transferred between slices | The complexity of the solution (.e.g in terms of number of authentication runs and number of NAS SMC runs) | PKI needed? | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | Yes | AMF | No | ??? | No | No |  | No |  |
| Sol #2 | ??? | RAN, AMF | No | Yes, RAN node has access to KAMF' | No | Yes, but security context is transferred following backward security principle |  | No |  |
| Sol #3 | Yes | AMF | No | ??? | No | No |  | No |  |
| Sol #4 | Yes | AMF | AUSF | ??? | AUSF | No |  | No |  |
| Sol #5 | Yes | RAN?, AMF | No | ??? | No | No |  | No | Does not cover all connectivity cases between AMFs |

|  | UE impact | Serving network impact | Home network impact | Security impact (whether the solution opens up for new security issues) | A common NF for the solution to work | Whether the solution requires that security context is transferred between slices | The complexity of the solution (.e.g in terms of number of authentication runs and number of NAS SMC runs) | PKI needed? | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #6 | No | AMF | AUSF, UDM | ??? | AUSF | No |  | No |  |
| Sol #7 | No | AMF | AUSF | ??? | AUSF | No |  | No |  |
| Sol #8 | Yes | RAN, AMF | UDM? | ??? | No |  |  | Yes | W.r.t UDM impact how are keys provisioned to the UE? |
| Sol #9 | No if optional horizontal key derivation is not performed, otherwise ? | RAN, AMF | No | If horizontal derivation is not performed there is no backward security. | NSSF | Yes |  | No |  |

The following provides a short evaluation of all the solutions however not all criteria are included and not all the details from the corresponding TR evaluation clauses are included. The notes include potential problematic aspects.

Solution #1: AMF re-allocation via RAN using existing security states

Impacts: UE, AMF

Security impacts: An attacker could send Registration Reject to the UE.

Notes:

-	All unprotected messages are accepted by the UE.

-	The purpose of ngKSI modification is not clear.

Solution #2: Security of AMF re-allocation when 5G NAS security context is rerouted via RAN

Impacts: UE?, RAN, AMF.

Security impacts: A malicious RAN could derive further NAS keys and initiate attacks towards the UE and the Core Network e.g. Registration Reject. However the risk is low if assumed that the RAN node is trusted.

Solution #3: Solving registration failure with AMF re-allocation via RAN

Impacts: UE, AMF

Security impacts: If an attacker initiates unprotected replayed Authentication Request messages towards the UE, then the USIM will protect itself as it can detect a replayed Authentication Request.

Notes:

-	Only AUTHENTICATION REQUEST and IDENTITY REQUEST can be accepted by the UE unprotected.

-	What happens if a UE receives a protected authentication request but verification fails? Does the UE accept the failed request ?

Solution #4: Solution to enable NAS Security for AMF reallocation and reroute via RAN Scenario

Impacts: UE, AMF, AUSF

Security impacts:

Notes:

-	The UE does not send any indication that it supports the new behaviour.

-	It is not clear how the same AUSF instance is selected by the Target AMF.

-	This solution requires the AUSF to maintain the SUPI/KSEAF info for a duration longer than the authentication procedure. Therefore the authentication procedure is coupled with the AMF re-allocation. This may not be desirable.

-	The solution requires the HPLMN and VPLMN to both support the solution. If the HPLMN does not support the solution the registration may fail.

Solution #5: AMF re-allocation by re-directing UE to new AMF

Impacts: UE, RAN?,  AMF

Security impacts:

Notes:

-	The UE does not send any indication that it supports the new behaviour.

-	The UE must release the NAS signalling connection and move to IDLE before it can register with the new 5G-GUTI with GUAMI set to Target AMF in order to be allow to handle unprotected NAS messages.

-	The solution does not support all the connectivity cases between AMFs. The case of handling connectivity to an Old AMF is not analyzed in the solution, i.e. the solution currently assumes that any session state in the Old AMF is discarded.

Solution #6: Solution to provide Security context to AMF capable of serving the UE to ensure system availability

Impacts: AMF, AUSF, UDM

Security impacts:

Notes:

-	It is not clear whether the solution works when the UE sends Requested NSSAI in the Registration Request.

-	This solution requires the AUSF to maintain the SUPI/KSEAF and other state (AMF_AUTN) info for a duration longer than the authentication procedure. Therefore the authentication procedure is coupled with the AMF re-allocation. This may not be desirable.

-	The solution requires the HPLMN and VPLMN to both support the solution. If the HPLMN does not support the solution the registration may fail.

Solution #7 : Solution to enable Reallocated AMF to serve the UE

Impacts: AMF, AUSF

Security impacts:

Notes:

-	It is not clear whether the solution works when the UE sends Requested NSSAI in the Registration Request.

-	This solution requires the AUSF to maintain the SUPI/KSEAF and other state (AMF_AUTN) info for a duration longer than the authentication procedure. Therefore the authentication procedure is coupled with the AMF re-allocation. This may not be desirable.

-	The solution requires the HPLMN and VPLMN to both support the solution. If the HPLMN does not support the solution the registration may fail.

Solution #8 : Solution to enable UE connection directly to the slice AMF

Impacts: UE, RAN, AMF, UDM for provisioning of keys?

Security impacts:

Notes:

-	Requires support of PKI in UE and RAN nodes and provisioning of the public key in the UE and certificates (private key) in the RAN nodes. All RAN nodes need to share the same private key. This may not be desirable.

-	With respect to PKI, there are some unclear questions:

-	Every PLMN have its own certificate and private key/public key pairs. It is not clear how the UE will be provisioned for all the PLMNs in the world assuming it is mobile.

-	The choice of the algorithm has impact on RAN. e.g. With ECIES the ciphertext is longer than the plaintext.

Solution #9: Security of AMF re-allocation when 5G NAS security context is rerouted via RAN

Impacts: UE?, RAN, AMF, NSSF

Security impacts: If horizontal key derivation is not performed then there is no backward security, the Target AMF knows the security context of the Initial AMF.

Notes:

-	There is no UE impact if no horizontal key derivation (HKD) is performed. If HKD is performed the UE may be impacted. This is still under investigation.

-	In this type of solution the 5G NAS security context is transferred protected via RAN.

-	The serving network needs to have a NSSF.