- 3GPP TSG SA WG3 Meeting #104-e                            	S3-21xxxx

- e-meeting, 16 – 27 August 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon

- Title:	Conclusion on How to Handle Revocation of User Consent

- Document for:	Approval

Agenda Item:	5.14

# Decision/action requested

This contribution proposes a new conclusion in TR 33.867

# References

# Rational

The contribution proposes a new conclusion on how to handle revocation of user consent. The highlighted text in yellow is approved in S3-212123 in the last SA3 meeting.

# Detailed proposal

*************** Start of 1st Change ****************

- 8.X	User Consent Revocation

The following generic aspects are concluded for normative work:

- How to handle user consent revocation shall be specified in Annex of 3GPP TS 33.501 [xx] in order to provide guideline if a new use case needs to be notified of revocation of user consent.

- If a service is invoked by NF service consumer directly, NF service provider subscribes Nudm_SDM_Notification service as depicted in 8.Y, and can be notified that user consent is changed, if user consent is revoked, the NF service provider can:

- Delete data subject to the user consent.

- Halt processing of data subject to the user consent.

- If there is an intermediate NF which is involved in service invocation, e.g. NWDAF, NEF, etc., the intermediate NF subscribes Nudm_SDM_Notification service as depicted in 8.Y, and can be notified that user consent is changed, if user consent is revoked, the intermediate NF can:

- Delete data subject to the user consent.

- Halt processing of data subject to the user consent.

- Notify NF service consumers to delete data subject to the user consent (If there is an expired timer for the data subject to the user consent, this procedure may not be needed).

- Notify NF service consumers to halt processing of data subject to the user consent.

The following specific aspects for use case#1 are concluded for normative work:

- Procedure for user consent revocation for use case#1 is specified as pCRs in potential WID of eNA.

- NWDAF performs role of intermediate NF which may subscribe the service for notification of revocation of user consent, and also notify NF service consumers to delete the data subject to the user consent, and to halt processing of data subject to the user consent.

The following specific aspects for use case#2 are concluded for normative work:

- Procedure for user consent revocation for use case#2 is specified as pCRs in WID of MEC.

- NEF performs role of intermediate NF which may subscribe the service for notification of revocation of user consent, and also notify NF service consumers to delete the data subject to the user consent, and to halt processing of data subject to the user consent.

*************** End of 1st Change ****************