3GPP TSG-SA3 Meeting #108e adhoc 	S3-22xxxx

- e-meeting, 10 - 14 August 2022

- Source:	China Mobile

- Title:	Addressing the EN of KI#1

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 32.500 SON Concepts and Requirements

# 3	Rationale

# 4	Detailed proposal

## 5.1	Key Issue #1: Support for AKMA roaming scenario

### 5.1.1	Issue details

AKMA roaming scenarios depend on UE and AF locations. There are different scenarios for AKMA roaming that need to be addressed:

- Case 1: UE in VPLMN and accessing an internal HPLMN AF

Case 2: UE is in VPLMN and accessing an internal VPLMN AF

Case 3: UE is in VPLMN and accessing an AF in the Data Network (Internet)

The AKMA roaming solutions should comply with LI requirements. It’s required either decrypted traffic or the means (e.g. providing keys) for law enforcement to decrypt the traffic should be provided to VPLMN. The LI requirements for access to keys are only for encryption, and in the AKMA case applies when the Ua* protocol is encrypted.  Regarding the means (e.g. providing keys) for law enforcement to decrypt the traffic, for Case 1 and Case 2, the encryption key (not necessarily the KAF unless KAF is used directly as the encryption key) and related information to decrypt the user traffic shall be provided from the AF to the VPLMN. For Case 3,

5.1.2	Security Threats

N/A

### 5.1.3	Potential security requirements

The AKMA architecture shall support the above mentioned AKMA roaming cases.

AKMA service shall be made to comply with the LI requirements.