| 3GPP TR 28.884 V0.1.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Based Management Architecture enhancement phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

Annex <A> (informative): Change history	6

# Foreword

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

New and updated use cases, as well as potential requirements, are emerging for managing 5G networks and services within the Service Based Management Architecture (SBMA). This document captures the corresponding potential enhancements to the SBMA.

# 1	Scope

The present document  describes use cases, potential requirements, and potential solutions aimed at enhancing the Service Based Management Architecture (SBMA) in the context of 5G Advanced (5GA). It also presents conclusions and recommendations regarding the next steps in the standardization process.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and background

## 4.1	Introduction

SBMA adopts a modular, service-oriented approach that enables flexible, scalable, and interoperable management solutions.

The architecture for the present study builds upon the SBMA framework in TS 28.533 [2]. All potential enhancements and solutions are expected to comply with the principles outlined in TS 28.533 [2], including service abstraction, interface standardization, and functional decoupling.

The present study aims to preserve architectural continuity with the SBMA baseline model while expanding its applicability to address emerging requirements in 5G Advanced networks.

## 4.2	Management data streaming based on Message Bus technologies

3GPP TS 28.532 [3] defines Stage 2 and Stage 3 of the streaming data reporting service. Currently, the 3GPP management system supports WebSocket-based data streaming for PM, tracing, and analytics, establishing a point-to-point transport connection between the streaming data reporting MnS consumer and MnS producer. The producer initiates the connection with the consumer and exchanges metadata (informing the consumer about its identity and the nature of the data to be reported, i.e., streamInfo). After this phase, the producer reports streaming data to the consumer via the established connection.

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer, causing the producer to generate and transmit multiple copies of the same data. The WebSocket protocol is standardized by the IETF in RFC 6455 [8]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [9]. Though a server can maintain multiple WebSocket connections simultaneously and broadcast messages to all connected clients. However, this is not a protocol-level feature, it is an application-level point-to-multipoint behaviour.

Editor's note:	It is FFS to investigates the integration and compatibility with the message bus industry solutions by providing a general scope which covers management data types defined in 3GPP TS 28.537 [4] (e.g. performance data, trace data, etc.). This does not imply that the message bus investigated in the present document are intended to replace the currently used WebSocket.

NOTE:	The management data includes performance measurements or KPIs defined in TS 28.552 [5] and TS 28.554 [6], trace reporting data defined in TS 32.423 [7] and analytic reporting data defined in TS 28.532 [3].

# 5	Use cases and potential solutions

## 5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture

### 5.1.1	Description

In a 5G Advanced network deployment, a mobile operator adopts an SBMA that leverages a modular reference model as defined in annex A.11 of TS 28.533 [YY]. This model abstracts Management Functions (MnFs) as logical entities, independent of their physical implementation, and enables them to interact exclusively through Management Services (MnSs).

Each MnF registers its MnSs with a service registry, enabling dynamic discovery and selection of services across domains and vendors.

For example:

- A fault management function in the RAN management domain autonomously detects faults using AI/ML models and triggers a healing workflow.

- This MnF can then orchestrate actions across both RAN and Core domains, invoking MnSs such as performance analytics, configuration updates, and resource scaling.

- The orchestration logic is implementation-agnostic, which means it does not rely on where or how the functions are deployed, only on the standardized MnS interfaces.

- The SBMA framework ensures that each MnF clearly declares its role as a producer or consumer of MnSs, supporting modularity and reusability.

In Release-19, a modular reference model has been defined in annex A.11 of TS 28.533 [YY]. Continue evolving the architecture reference model for management and orchestration, to support new requirements such as AI/ML integration, autonomy, and cross-domain orchestration.

### 5.1.2	Potential requirements

REQ-SBMA-ARM-1:	SBMA should provide a consolidated view of management architecture.

### 5.1.3	Potential solutions

### 5.1.4	Evaluation of potential solutions

## 5.2	Use case #2: Management data streaming based on Message Bus

### 5.2.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers request the same management data for multiple uses. Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer, causing the producer to generate and transmit multiple copies of the same data. The WebSocket protocol is standardized by the IETF in RFC 6455 [f]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [g].

Though a server can maintain multiple WebSocket connections simultaneously and broadcast messages to all connected clients. However, this is not a protocol-level feature, it is an application-level point-to-multipoint behaviour.

Editor's note:	It is FFS to investigates how the producer generates the data and send it to the message bus for the case that multiple consumers request the same management data.

### 5.2.2	Potential requirements

### 5.2.3	Potential solutions

### 5.2.4	Evaluation of potential solutions

## 5.3	Use case #3: Historical alarms

### 5.3.1	Description

The "AlarmList" defined in TS 28.111 [10] contains "AlarmRecords" that represent currently active alarms. Alarms, that are cleared and acknowledged (inactive alarms) are removed from the alarm list and cannot be retrieved any more by MnS consumers.

However, old inactive alarms (historical alarms) are a valuable source of information in many use cases:

-	AI/ML training: Historical alarms provide context information for AI/ML training.

-	Security auditing: Historical security alarms provide information for security monitoring and auditing.

- Fault Repair: By analyzing historical alarms associated with previous faults, operators can identify recurring issues, understand root causes more effectively, and accelerate troubleshooting. This reduces mean time to repair (MTTR) and improves network and service reliability.

- Predictive Maintenance: Historical alarm can be used to forecast potential future failures. For example, repeated alarms from a specific network element may indicate impending hardware degradation. Predictive analytics based on alarm history enables proactive maintenance, minimizing service disruption.

- Enhanced analytics: Multiple MDA capabilities utilize alarm information as enabling data (see TS 28.104, clause 8). Currently the enabling data is defined as “Alarm information and notifications as per TS 28.111”, but the MDA capabilities would be more valuable if historical alarm data is also considered.

### 5.3.2	Potential requirements

Req-1: The 3GPP Management system should have the capability allowing to retrieve historical alarms.

## 5.4	Use case#4 on software management for 5G

### 5.4.1	Description

3GPP TS 32.531 [11], TS 32.532 [12], and TS 32.533 [13] define the concepts, requirements, Information Service, and CORBA solution set for the software management of NEs for 4G, encompassing both automated and non-automated software management approaches.

Software management for 5G enhances 5G network operational efficiency. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [2] does not include support for software management. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs and NFs.

Editor's note:	To enable software management within SBMA, the associated potential requirements and potential solutions are FFS, building upon the legacy of software management of NEs for LTE while adapting to the principles of SBMA.

### 5.4.2	Potential requirements

#### 5.4.2.1	Potential general requirements

REQ-SBMA-SWM-1: The 3GPP management system should support software management.

#### 5.4.2.2	Potential requirements for PNF

REQ-SBMA-PNFSWM-1: The 3GPP management system should support the capability to monitor software download, and activation.

#### 5.4.2.3	Potential requirements for VNF

### 5.4.3	Potential solutions

### 5.4.4	Evaluation of potential solutions

## 5.5	Use case#5 on inventory management for 5G

### 5.5.1	Description

3GPP TS 28.631 [14], TS 28.632 [15], and TS 28.633 [16] define the requirements, Information Service definitions, and XML solution set for inventory management. These specifications cover inventory information of NEs, including hardware, software, licences, Tower Mounted Amplifiers (TMAs) and antennas.

The benefits of inventory management retain for 5G network management:

Inventory management enables operators to track equipment status, plan capacity expansions, schedule upgrades, and manage decommissioning more effectively, which improves consistency across large, geographically distributed networks

Inventory management shows the links/topology among network elements, network functions and physical network resources, which helps operators for faster fault localization and root-cause analysis when problems occur. The insight into these dependencies also help operators minimize risks during maintenance or upgrades.

Currently, TS 28.533 [2] does not include support for Inventory Management (IM). As 5G networks evolve in scale and complexity, it is essential to introduce IM capabilities into SBMA. These capabilities will enable operators to maintain accurate and dynamic records of static network resources, supporting planning, assurance, and operational efficiency.

Editor's note 1:	To enable inventory management in SBMA, the associated potential requirements, and potential solutions are FFS, building upon the legacy of inventory management for LTE while adapting to the principles of SBMA.

Editor's note 2:	For topology or the relationship among different inventory objects, the difference between IM and CM for 5G is FFS.

### 5.5.2	Potential requirements

REQ-SBMA-IM-1: The 3GPP management system should support inventory management.

### 5.5.3	Potential solutions

### 5.5.4	Evaluation of potential solutions

## 5.X	Use case #<X>: <use case title>

### 5.X.1	Description

Editor's note:	This clause provides a description of use case.

### 5.X.2	Potential requirements

Editor's note:	This clause provides potential requirements for the corresponding use case.

### 5.X.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

### 5.X.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

# 6	Conclusions and recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | V0.0.0 |