| 3GPP TR 32.872 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5GA roaming charging reliability enhancement (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Background	8

4.1	General	8

4.2	Home routed Roaming charging	8

4.3	Local Breakout Roaming charging	8

5	Scenarios and key issues	10

5.1	Topic 1: Local Breakout inter CHFs scenario	10

5.1.1	Use cases	10

5.1.1.1	Use case #1.1: Failures detected between the CTF and the V-CHF	10

5.1.1.2	Use case #1.2: Failures detected between the CTF and the V-CHF	10

5.1.2	Potential charging requirements	10

5.1.3	Key issues	10

5.1.3.1	Key issue #1.1: Failure handling enhancement for failures detected between the CTF and the V-CHF	10

5.1.3.2	Key issue #1.2: Failure handling enhancement for failures detected between the V-CHF and the H-CHF	11

5.1.3.3	Key issue #1.3: Trigger conditions of V-CHF	11

5.1.4	Possible solutions	11

5.1.4.1	Solution #1.1: Handling of the charging session between V-CHF and H-CHF when V-CHF detected failure of charging session with CTF	11

5.1.4.2	Solution #1.2: Handling of abnormal messages received by V-CHF	11

5.1.4.3	Solution #1.3: CTF detected failure	12

5.1.4.4	Solution #1.4: V-CHF release the charging session with CTF when V-CHF detected failure of charging session with H-CHF	12

5.1.4.5	Solution #1.5: V-CHF suspend quota management when V-CHF detected failure of charging session with H-CHF	13

5.1.4.6	Solution #1.6: Handling of abnormal messages received by H-CHF	13

5.1.4.7	Solution #1.7: Handling of messages received by H-CHF from alternative V-CHF	13

5.1.4.8	Solution #1.8: Handling of charging data requests stored in V-CHF	13

5.1.4.9	Solution #1.9: Use triggers from V-CTF in V-CHF charging request	13

5.1.4.10	Solution #1.10: Define the trigger conditions for V-CHF	14

5.1.4.11	Solution #1.11: Using service-based architecture for V-CHF detected CTF failure	14

5.1.4.12	Solution #1.12: Using service-based architecture for CTF detected V-CHF failure	14

5.1.4.13	Solution #1.13: Using service-based architecture for H-CHF detected V-CHF failure	14

5.1.4.14	Solution #1.14: Using service-based architecture for V-CHF detected H-CHF failure	14

5.1.5	Evaluation	15

5.1.5.1	Solutions evaluation for Key issue #1.1	15

5.1.5.2	Solutions evaluation for Key issue #1.2	15

5.1.5.3	Solutions evaluation for Key issue #1.3	15

5.1.6	Conclusion	15

5.2	Topic 2: Home Routed charging scenario	15

5.2.1	Use cases	15

5.2.1.1	Use case #2.1: Failure of one charging session	15

5.2.2	Potential charging requirements	16

5.2.3	Key issues	16

5.2.3.1	Key issue #2.1: Failure is detected in one of the charging sessions	16

5.2.4	Possible solutions	16

5.2.4.1	Solution #2.1: Failures detected between the H-CHF and the H-CTF	16

5.2.4.2	Solution #2.2: Failures detected between the V-CHF and the V-CTF	16

5.2.4.3	Solution #2.3: Using service-based architecture for H-SMF (CTF) detected H-CHF failure	17

5.2.5	Evaluation	17

5.2.6	Conclusion	17

5.3	Topic 3: Local Breakout V-SMF to H-CHF scenario	17

5.3.1	Use cases	17

5.3.1.1	Use case #3.1: Failures detected between the CTF and the V-CHF	17

5.3.1.2	Use case #3.2: Failures detected between the CTF and the H-CHF	18

5.3.1.3	Use case #3.3: CDR Generation after CHF-SMF Connection Loss	20

5.3.2	Potential charging requirements	20

5.3.3	Key issues	20

5.3.3.1	Key issue #3.1: Failure handling enhancement for failures detected between the CTF and the V-CHF	20

5.3.3.1	Key issue #3.2: Failure handling enhancement for failures detected between the CTF and the H-CHF	20

5.3.3.3	Key Issue #3.3: Optimization of charging information	21

5.3.4	Possible solutions	21

5.3.4.1	Solution #3.1: Handling of V-CTF when V-CTF detected both V-CHF and H-CHF failure at the same time	21

5.3.4.2	Solution #3.2: Using service-based architecture for V-SMF (CTF) detected V-CHF failure	21

5.3.4.3	Solution #3.3: Using service-based architecture for V-SMF (CTF) detected H-CHF failure	21

5.3.5	Evaluation	21

5.3.6	Conclusion	21

6	Conclusions and recommendations	21

Annex <A> (informative): PlantUML	22

A.1	Figures	22

A.1.1	Figure 5.1.4.1-1	22

A.1.2	Figure 5.1.4.3-1	22

A.1.3	Figure 5.1.4.4-1	22

A.1.4	Figure 5.3.1.1-1	22

A.1.5	Figure 5.3.1.1-2	23

A.1.6	Figure 5.3.1.1-3	23

A.1.7	Figure 5.3.1.2-1	23

A.1.8	Figure 5.3.1.2-2	24

A.1.9	Figure 5.3.1.2-3	24

A.1.10	Figure 5.1.4.2-1	24

Annex <B>: Change history	25

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document focuses on the roaming charging aspects related to the reliability and recoverability enhancement of roaming charging interactions, including Local Breakout roaming and Home Routed roaming.

The following items are studied:

-	Identify the roaming charging reliability enhancement scenarios and requirements.

-	Investigate the potential solutions to support the scenarios and requirements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.255: "5G data connectivity domain charging stage 2".

[3]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

N40	Reference point between SMF and the CHF.

N16	Reference point between two SMFs.

N47	Reference point between V-SMF and H-CHF.

N107	Reference point between V-CHF and H-CHF.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CHF	Charging Function

CTF	Charging Trigger Function

CGF	Charging Gateway Function

NF	Network Function

SMF	Session Management Function

4	Background

4.1	General

5G roaming charging are specified in middle tier charging specifications. Take data connectivity as an example, the home routed roaming charging, LBO roaming charging with N47 and LBO roaming charging with N107 are specified in 3GPP TS 32.255[2]. The common failure handling includes CTF detected failure, CHF detected failure, CHF as NF Consumer detected failure, Retry handling and Response code handling are specified in clause 5.5 of 3GPP TS 32.290[3]. In order to support more failure handling scenarios in roaming, the enhancement of failure handling will be studied.

4.2	Home routed Roaming charging

Figure 4.2-1 illustrates the Home Routed roaming charging architecture depicted in clause 4.2 of 3GPP TS 32.255[2]. The N40 reference point is defined for the interactions between H-SMF and H-CHF and between V-SMF and V-CHF in the reference point representation.

Figure 4.2-1: 5G data connectivity converged charging architecture in roaming Home Routed reference point representation

4.3	Local Breakout Roaming charging

Figure 4.3-1 illustrates the Local Breakout roaming charging with N47 architecture, as specified in clause 4.2.6 of 3GPP TS 32.255[2]. The N47 reference point is defined for the interactions between V-SMF and H-CHF.

Figure 4.3-1: 5G data connectivity converged charging architecture in Local Breakout V-SMF to H-CHF scenario reference point representation

Figure 4.3-2 illustrates the Local Breakout roaming charging with N107 architecture, as specified in clause 4.2.6a of 3GPP TS 32.255[2]. The N107 reference point is defined for the interactions between V-CHF and H-CHF.

Figure 4.3-2: 5G data connectivity converged charging architecture in Local Breakout Inter-CHF scenario reference point representation

5	Scenarios and key issues

5.1	Topic 1: Local Breakout inter CHFs scenario

5.1.1	Use cases

5.1.1.1	Use case #1.1: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 charging architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-SMF(CTF) and V-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.1-1: An example of failure detected between V-SMF(CTF) and V-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-1.

5.1.1.2	Use case #1.2: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-CHF and H-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.2-1: An example of failure detected between V-CHF and H-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-2.

5.1.2	Potential charging requirements

REQ-3GPPCH-LBIC-1: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the V-CHF.

REQ-3GPPCH-LBIC-2: charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the V-CHF and the H-CHF.

5.1.3	Key issues

5.1.3.1	Key issue #1.1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-1. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the CTF and the V-CHF;

5.1.3.2	Key issue #1.2: Failure handling enhancement for failures detected between the V-CHF and the H-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-2. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the V-CHF and the H-CHF;

5.1.3.3	Key issue #1.3: Trigger conditions of V-CHF

This investigation covers the following:

-	Identification of the trigger conditions in V-CHF for the Local Breakout N40+N107 charging scenario;

5.1.4	Possible solutions

5.1.4.1	Solution #1.1: Handling of the charging session between V-CHF and H-CHF when V-CHF detected failure of charging session with CTF

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the V-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the charging session between the V-CHF and CTF may be maintained or released based on local configuration.

-	In the case of the charging session between the V-CHF and CTF is released, the V-CHF sends a Charging Data Request [Termination] to H-CHF to release the charging session between the V-CHF and H-CHF.

Figure 5.1.4.1-1: V-CHF terminates the charging session with H-CHF

-	In the case of the charging session between the V-CHF and CTF is maintained, the V-CHF maintains the charging session between the V-CHF and H-CHF.

5.1.4.2	Solution #1.2: Handling of abnormal messages received by V-CHF

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When a Charging Data Request [Update, or Termination] received by the V-CHF, which cannot be associated to any existing charging session (i.e., resource in V-CHF), should be handled as specified in clause 5.5.1.2 of 3GPP TS 32.290[3], and the V-CHF needs to send the Charging Data Request [Update, or Termination] to the H-CHF.

Figure 5.1.4.2-1: Message flow of abnormal messages received by V-CHF

5.1.4.3	Solution #1.3: CTF detected failure

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF. This solution only applicable on application level failure handling.

When the V-CHF is determined not reachable by V-SMF(CTF), V-SMF(CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) as specified in clause 5.5.1.1 of 3GPP TS 32.290 [3].

When failure handling is set to 'Continue' and a failover procedure is supported in the V-CHF and the V-SMF(CTF), the V-SMF(CTF) re-sends the request with the ‘Remote CHF resource’ to an alternative V-CHF, then the alternative V-CHF replaces the original V-CHF to interact with the H-CHF.

Figure 5.1.4.3-1: V-CHF re-sends the request to an alternative V-CHF

5.1.4.4	Solution #1.4: V-CHF release the charging session with CTF when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement.

In the case of V-CHF decides to release the charging session between the V-CHF and CTF, the V-CHF sends a Charging Data Response or Charging Notify Request to CTF to indicate that the charging session should be released.

Figure 5.1.4.4-1: V-CHF terminates the charging session with CTF

5.1.4.5	Solution #1.5: V-CHF suspend quota management when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF. This solution only applicable for online charging.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement.

In the case of V-CHF decides to continue the charging session between the V-CHF and CTF in online charging scenario, the V-CHF instructs the NF consumer (CTF) to suspend quota management as specified in clause 5.3.2.5 of 3GPP TS 32.290[3], and stores the Charging Data Request(s) or charging information marked with ‘QUOTA_MANAGEMENT_SUSPENDED’ from the NF consumer (CTF).

5.1.4.6	Solution #1.6: Handling of abnormal messages received by H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When a Charging Data Request [Initial] is received by the H-CHF, which can be associated to an existing charging session, the H-CHF handles the Charging Data Request as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When a Charging Data Request [Update or Termination] is received by the H-CHF, which cannot be associated to any existing charging session, the H-CHF handles the Charging Data Request as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

5.1.4.7	Solution #1.7: Handling of messages received by H-CHF from alternative V-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF. This solution only applicable for application level failure handling.

When the V-CHF is determined not reachable by V-SMF(CTF), the V-SMF(CTF) may re-sends the request to an alternative V-CHF when a failover procedure is supported in the V-CHF and the V-SMF(CTF).

In the scenario of switching from V-CHF to an alternative V-CHF, when a Charging Data Request is received by the H-CHF from alternative V-CHF, which can be associated to an existing charging session (i.e., resource in CHF), but the associated NF consumer identification in the Charging Data Request is changed, should be handled as a valid request, with Charging Data Response including the charging session id (i.e., resource id).

5.1.4.8	Solution #1.8: Handling of charging data requests stored in V-CHF

The solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

A pre-set conditions (e.g., stored data size threshold or stored duration threshold) may be pre-configured in the V-CHF according to operator's policy. When failure is detected, the V-CHF stores the charging data requests received from V-SMF(CTF) if failure handling is set to 'Continue'.

Editor's Note: Needs further clarification and addition of a message flow.

5.1.4.9	Solution #1.9: Use triggers from V-CTF in V-CHF charging request

The solution for key issue #1.3, identify the trigger conditions in V-CHF for the Local Breakout N40+N107 charging scenario.

When a Charging Data Request is sent from V-CHF to H-CHF for inter CHFs communication, the trigger condition that triggers the Charging Data Request is from V-SMF(CTF). The value of the Triggers and other parameters are derived from the Charging Data Request previously received from the V-SMF(CTF).

When a Charging Data Response is sent from V-CHF to V-SMF(CTF) for inter CHFs communication, the value of the Triggers and Unit Count Inactivity Timer parameters included in this response are derived from the Charging Data Response previously received from the H-CHF.

5.1.4.10	Solution #1.10: Define the trigger conditions for V-CHF

The solution addresses key issue #1.3.

The Charging Data Requests are issued by the V-CHF towards the H-CHF when certain trigger conditions are met. The applicable triggers in the V-CHF for inter-CHF communication are outlined as follows:

-	Receive Charging Data Request [Initial] from original NF consumer;

-	Receive Charging Data Request [Update] from original NF consumer;

-	Receive Charging Data Request [Termination] from original NF consumer.

The triggers above will not be included in the message of Charging Data Requests sent to the H‑CHF.

5.1.4.11	Solution #1.11: Using service-based architecture for V-CHF detected CTF failure

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the V-CHF detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.2, to restore a connection to the CTF. If the restoring of the connection fails it is operator specific if the connection between V-CHF and H-CHF is to be maintained, and for how long.

5.1.4.12	Solution #1.12: Using service-based architecture for CTF detected V-CHF failure

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the CTF detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.3, to restore a connection V-CHF.

5.1.4.13	Solution #1.13: Using service-based architecture for H-CHF detected V-CHF failure

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the H-CHF detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.2, to restore a connection to the V-CHF. If the restoring of the connection fails it is operator specific if the H-CHF is to report this in the CDR with a specific reason for closing the CDR.

5.1.4.14	Solution #1.14: Using service-based architecture for V-CHF detected H-CHF failure

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.3, to restore a connection to the H-CHF. If the restoring of the connection fails it is operator specific if the connection between V-CHF and CTF is to be maintained and for how long.

5.1.5	Evaluation

5.1.5.1	Solutions evaluation for Key issue #1.1

Solutions #1.1 solves Key issue #1.1, providing the handling method of the charging session between V-CHF and H-CHF when V-CHF detected failure of charging session with CTF in inter CHFs scenario. The benefit of the solution is that the V-CHF synchronously releases or maintains the charging session with the H-CHF and CTF, avoids long-term resource consumption for unilateral inactive charging session.

Solutions #1.2 solves Key issue #1.1, describing in inter CHFs scenario, the V-CHF handles abnormal messages as specified in clause 5.5.1.2 of 3GPP TS 32.290[3] and also necessary to forward the charging date request received from CTF to H-CHF. The benefit of the solution is to ensure consistency and continuity between two charging sessions.

Solutions #1.3 solves Key issue #1.1, describing when the latest failure handling received from the V-CHF is set to 'Continue', CTF re-sends the request with the ‘Remote CHF resource’ to an alternative V-CHF. The benefit of the solution is to increase the service continuity via switching the CHF instance in inter CHFs scenario.

Editor's Note: Further evaluation is needed.

5.1.5.2	Solutions evaluation for Key issue #1.2

Solutions #1.4 solves Key issue #1.2, providing the handling method of the charging session between V-CHF and CTF when V-CHF detected failure of charging session with H-CHF in inter CHFs scenario. The benefit of the solution is that the V-CHF synchronously releases or maintains the charging session with the CTF and H-CHF, avoids long-term resource consumption for unilateral inactive charging session.

Solutions #1.6 solves Key issue #1.2, describing in inter CHFs scenario, the H-CHF handles abnormal messages in the same way as the CHF as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

Solutions #1.7 solves Key issue #1.2, describing the V-CHF handle the Charging Data Request received from alternative V-CHF as a valid request in inter CHFs scenario.

Editor's Note: Further evaluation is needed.

5.1.5.3	Solutions evaluation for Key issue #1.3

Solutions #1.9 solves Key issue #1.3, describing in inter CHFs scenario, the V-CHF use the triggers received from CTF when it sends charging data request to the H-CHF. The solution requires minimal changes to existing specifications and does not affect the interaction between V-CHF and H‑CHF.

Solutions #1.10 solves Key issue #1.3, describing in inter CHFs scenario, the V-CHF use its own trigger when it sends charging data request to the H-CHF.

Editor's Note: Further evaluation is needed.

5.1.6	Conclusion

5.2	Topic 2: Home Routed charging scenario

5.2.1	Use cases

5.2.1.1	Use case #2.1: Failure of one charging session

In the HR roaming charging architecture, for a specific PDU session, there are two charging sessions simultaneously. When a failure is detected in one of the charging sessions, How the charging system handles the failure should be studied.

Figure 5.2.1.1-1: An example of the failure between H-SMF and H-CHF in HR roaming

The potential charging requirements for this UC are: REQ-3GPPCH-LBHR-1.

5.2.2	Potential charging requirements

REQ-3GPPCH-LBHR-1: Charging system shall support failure handling for the Home Routed charging scenario in which a failure is detected in one of the charging sessions.

5.2.3	Key issues

5.2.3.1	Key issue #2.1: Failure is detected in one of the charging sessions

This key issue is for investigating how to support REQ-3GPPCH-LBHR-1. This investigation covers the following:

-	Whether and how to handle when failure is detected in one of the charging sessions;

5.2.4	Possible solutions

5.2.4.1	Solution #2.1: Failures detected between the H-CHF and the H-CTF

A possible solution for key issue #2.1 covering requirements REQ-3GPPCH-LBHR-1, failure is detected in one of the charging sessions. This solution only applicable for application level failure handling.

When the H-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the H-CHF handles as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When the H-CHF is determined not reachable by H-SMF (CTF), the H-SMF (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) and may store Charging Data Request(s) or charging information as specified in clause 5.5.1.1 of 3GPP TS 32.290 [3]. if the failure handling indicates 'Terminate', the service will be terminated, and the associated charging sessions will be terminated accordingly.

5.2.4.2	Solution #2.2: Failures detected between the V-CHF and the V-CTF

A possible solution for key issue #2.1 covering requirements REQ-3GPPCH-LBHR-1, failure is detected in one of the charging sessions. This solution only applicable for application level failure handling.

When the V-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the V-CHF handles as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When the V-CHF is determined not reachable by V-SMF (CTF), the V-SMF (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) and may store Charging Data Request(s) or charging information as specified in clause 5.5.1.1 of 3GPP TS 32.290[3]. if the failure handling indicates 'Terminate', the service will be terminated, and the associated charging sessions will be terminated accordingly.

5.2.4.3	Solution #2.3: Using service-based architecture for H-SMF (CTF) detected H-CHF failure

A possible solution for key issue #2.1 covering requirements REQ-3GPPCH-LBHR-1, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the H-SMF (CTF) detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.3, to restore a connection to the H-CHF. If the restoring of the connection fails it is operator specific if the service (i.e., PDU session) is allowed to continue, and for how long.

5.2.5	Evaluation

Solutions #2.1 and Solutions #2.2 solve Key issue #2.1, describing in HR failure handling scenario, it is handled in the same way as the CHF and CTF as specified in clause 5.5.1.2 and 5.5.1.1 of 3GPP TS 32.290[3]. The key advantage of this solution is that it has minimal impact on current specifications.

Editor's Note: Further evaluation is needed.

5.2.6	Conclusion

5.3	Topic 3: Local Breakout V-SMF to H-CHF scenario

5.3.1	Use cases

5.3.1.1	Use case #3.1: Failures detected between the CTF and the V-CHF

In the LBO N40+N47 charging architecture, for a specific PDU session, a charging session between the V-SMF (CTF) and V-CHF and a charging session between the V-SMF (CTF) and H-CHF is to be initiated, updated or terminated. When a failure detected between V-SMF (CTF) and V-CHF, How the charging system handles the failure should be studied.

Figure 5.3.1.1-1: Failure detected between V-SMF(CTF) and V-CHF

The failure can happen at any time in a session, bellow are two cases described either when the V-SMF cannot reach the V-CHF or when the V-CHF cannot reach V-SMF. If the V-SMF (CTF) trigger the H-CHF in the case of V-CHF failure, should be studied.

Figure 5.3.1.1-2: Failure at request from V-SMF(CTF) to V-CHF

Figure 5.3.1.1-3: Failure at response from V-CHF to V-SMF(CTF)

The potential charging requirements for this UC are: REQ-3GPPCH-LBHC-1.

5.3.1.2	Use case #3.2: Failures detected between the CTF and the H-CHF

In the LBO N40+N47 charging architecture, for a specific PDU session, a charging session between the V-SMF (CTF) and V-CHF and a charging session between the V-SMF (CTF) and H-CHF is to be initiated, updated or terminated. When a failure detected between V-SMF (CTF) and H-CHF. How the charging system handles the failure should be studied.

Figure 5.3.1.2-1: Failure detected between V-SMF(CTF) and H-CHF

The failure can happen at any time in a session, bellow are two cases described either when the V-SMF cannot reach the H-CHF or when the H-CHF cannot reach V-SMF.

Figure 5.3.1.2-2: Failure at request from V-SMF(CTF) to H-CHF

Figure 5.3.1.2-3: Failure at request from V-SMF(CTF) to H-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBHC-2.

5.3.1.3	Use case #3.3: CDR Generation after CHF-SMF Connection Loss

As the Operator, would like to control the size of the charging information stored, on the SMF, in the LBO N40+N47 charging architecture, post CHF-SMF failure, allowing quick recovery with minimal data, as CHF generates CDRs from session essentials, preventing revenue leakage in high-mobility scenarios.

Editor’s Note: Need further clarification.

5.3.2	Potential charging requirements

REQ-3GPPCH-LBHC-1: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the V-CHF.

REQ-3GPPCH-LBHC-2: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the H-CHF.

5.3.3	Key issues

5.3.3.1	Key issue #3.1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBHC-1. This investigation covers the following:

-	Identification of the failure handling enhancement, if any, for the scenario in which a failure is detected between the CTF and the V-CHF;

5.3.3.1	Key issue #3.2: Failure handling enhancement for failures detected between the CTF and the H-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBHC-2. This investigation covers the following:

-	Identification of the failure handling enhancement, if any, for the scenario in which a failure is detected between the CTF and the H-CHF;

5.3.3.3	Key Issue #3.3: Optimization of charging information

This investigation covers the following:

-	Identification of gaps in current mechanisms, leading to inefficiencies during failures between CTF and CHF.

Editor’s Note: need further clarification.

5.3.4	Possible solutions

5.3.4.1	Solution #3.1: Handling of V-CTF when V-CTF detected both V-CHF and H-CHF failure at the same time

The solution for key issue #3.1 and key issue #3.2 covering requirements REQ-3GPPCH-LBHC-1 and REQ-3GPPCH-LBHC-2, enhanced failure handling for scenarios where the CTF detected both V-CHF and H-CHF failure at the same time.

When the V-CHF and H-CHF are determined not reachable at the same time, and the latest application layer failure handling from V-CHF and H-CHF are different, the application layer failure handling priority on CTF is based on operator agreement.

Editor’s Note: Needs further clarification and potential addition of a message flow.

5.3.4.2	Solution #3.2: Using service-based architecture for V-SMF (CTF) detected V-CHF failure

A possible solution for key issue #3.1 covering requirements REQ-3GPPCH-LBHC-1, enhanced failure handling for scenarios where a failure is detected between the V-SMF (CTF) and the V-CHF.

When the V-SMF (CTF) detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.3, to restore a connection to the V-CHF. If the restoring of the connection fails it is operator specific if the connection to the H-CHF is to be maintained, and for how long. In case the connection to the H-CHF is disconnected there is need for a new reason.

5.3.4.3	Solution #3.3: Using service-based architecture for V-SMF (CTF) detected H-CHF failure

A possible solution for key issue #3.2 covering requirements REQ-3GPPCH-LBHC-2, enhanced failure handling for scenarios where a failure is detected between the V-SMF (CTF) and the H-CHF.

When the V-SMF (CTF) detects a failure, it may use the procedure outlined in TS 29.500 [4] clause 6.5.3.3, to restore a connection to the H-CHF. If the restoring of the connection fails it is operator specific if the connection to the V-CHF is to be maintained, and for how long. In case the connection to the V-CHF is disconnected there is need for a new reason.

5.3.5	Evaluation

Solutions #3.1 solves Key issue #3.1 and Key issue #3.2, providing the application layer failure handling priority on CTF when the V-CHF and H-CHF are determined not reachable at the same time.

Editor's Note: Further evaluation is needed.

5.3.6	Conclusion

6	Conclusions and recommendations

Annex <A> (informative):
PlantUML

A.1	Figures

A.1.1	Figure 5.1.4.1-1

@startuml

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "H-CHF" as HCHF

CTF <--> VCHF : 1. Failures detected between the V-SMF(CTF) and the V-CHF

rnote over VCHF #white

2. V-CHF releases the resources between V-SMF(CTF)

endrnote

VCHF -> HCHF : 3. V-CHF sends a Charging Data Request [Termination] to H-CHF

@enduml

A.1.2	Figure 5.1.4.3-1

@startuml

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "alternative V-CHF" as ACHF

participant "H-CHF" as HCHF

CTF --> VCHF : 1. V-CHF is determined not reachable

CTF -> ACHF : 2. NF consumer (CTF) re-sends the request with the ‘Remote CHF resource’ to an alternative V-CHF

ACHF <-> HCHF : 3. alternative V-CHF interact with the H-CHF

@enduml

A.1.3	Figure 5.1.4.4-1

@startuml

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "H-CHF" as HCHF

VCHF <--> HCHF : 1. Failures detected between the V-CHF and the H-CHF

rnote over VCHF #white

2. V-CHF decides to release the charging session with V-SMF(CTF)

endrnote

VCHF -> CTF : 3. V-CHF sends a Charging Notify Request to V-SMF(CTF)

@enduml

A.1.4	Figure 5.3.1.1-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

rectangle "VPLMN" {

rectangle "Converged\nCharging\nSystem" as VCCS {

rectangle  "V-CHF" as VCHF

}

rectangle  "V-SMF\n(CTF)" as CTF

}

rectangle "HPLMN" {

rectangle "Converged\nCharging\nSystem" as HCCS {

rectangle  "H-CHF" as HCHF

}

}

VCHF -- CTF : "N40"

HCHF -- CTF : "N47"

@enduml

A.1.5	Figure 5.3.1.1-2

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant CTF as "V-SMF\n(CTF)"

participant VCHF as "V-CHF"

participant HCHF as "H-CHF"

CTF ->x VCHF : 1. Charging Data request [Initial/Update/Termination]

CTF -> HCHF : 2. Charging Data request [Initial/Update/Termination]

rnote over HCHF: 3. Account, Rating,\nReservation Control

rnote over HCHF: 4. Open/Update/\nClose CDR

CTF <- HCHF : 5. Charging Data response [Initial/Update/Termination]

@enduml

A.1.6	Figure 5.3.1.1-3

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant CTF as "V-SMF\n(CTF)"

participant VCHF as "V-CHF"

participant HCHF as "H-CHF"

CTF -> VCHF : 1. Charging Data request [Initial/Update/Termination]

rnote over VCHF: 2. Open/Update/\nClose CDR

CTF x<- VCHF : 3. Charging Data response [Initial/Update/Termination]

CTF -> HCHF : 4. Charging Data request [Initial/Update/Termination]

rnote over HCHF: 5. Account, Rating,\nReservation Control

rnote over HCHF: 6. Open/Update/\nClose CDR

CTF <- HCHF : 7. Charging Data response [Initial/Update/Termination]

@enduml

A.1.7	Figure 5.3.1.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

rectangle "VPLMN" {

rectangle "Converged\nCharging\nSystem" as VCCS {

rectangle  "V-CHF" as VCHF

}

rectangle  "V-SMF\n(CTF)" as CTF

}

rectangle "HPLMN" {

rectangle "Converged\nCharging\nSystem" as HCCS {

rectangle  "H-CHF" as HCHF

}

}

VCHF -- CTF : "N40"

HCHF -- CTF : "N47"

@enduml

A.1.8	Figure 5.3.1.2-2

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant CTF as "V-SMF\n(CTF)"

participant VCHF as "V-CHF"

participant HCHF as "H-CHF"

CTF -> VCHF : 1. Charging Data request [Initial/Update/Termination]

rnote over VCHF: 3. Open/Update/\nClose CDR

CTF <- VCHF : 4. Charging Data response [Initial/Update/Termination]

CTF ->x HCHF : 5. Charging Data request [Initial/Update/Termination]

@enduml

A.1.9	Figure 5.3.1.2-3

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant CTF as "V-SMF\n(CTF)"

participant VCHF as "V-CHF"

participant HCHF as "H-CHF"

CTF -> VCHF : 1. Charging Data request [Initial/Update/Termination]

rnote over VCHF: 2. Open/Update/\nClose CDR

CTF <- VCHF : 3. Charging Data response [Initial/Update/Termination]

CTF -> HCHF : 4. Charging Data request [Initial/Update/Termination]

rnote over HCHF: 5. Account, Rating,\nReservation Control

rnote over HCHF: 6. Open/Update/\nClose CDR

CTF x<- HCHF : 7. Charging Data response [Initial/Update/Termination]

@enduml

A.1.10	Figure 5.1.4.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "H-CHF" as HCHF

CTF -> VCHF : 1. Charging Data Request [Update, or Termination]

rnote over VCHF: 2. the request cannot be associated \nto any existing charging session \nand is processed as valid

VCHF -> HCHF : 3. the V-CHF sends the Charging Data Request [Update, or Termination] to H-CHF

@enduml

Annex <B>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA5#163 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254381 S5-254488 S5-254490 S5-254489 S5-254491 S5-254812 S5-254813 S5-254815 |  |  |  | Skeleton update Add definitions of terms, symbols and abbreviations Add scope Add reference Introduce the background for roaming charging Introduce the usecase1 on Local Breakout Inter CHFs scenario Introduce the usecase2 on Local Breakout Inter CHFs scenario Introduce the usecase on Home Routed scenario | 0.1.0 |
| 2025-11 | SA5#164 | S5-255178 S5-255434 S5-255435 S5-255436 S5-255437 S5-255438 S5-255439 S5-255440 S5-255441 |  |  |  | Introduce the KI for the UC in HR scenario Failure handling of V-CHF when failures detected between the CTF and the V-CHF Failure handling of CTF when failures detected between the CTF and the V-CHF Failure handling of V-CHF when V-CHF detected failure of charging session with H-CHF Failure handling of H-CHF when H-CHF detected failure of charging session with V-CHF Introduce the solution#1 on HR scenario Key issue #1 Introduce the solution#2 on HR scenario Key issue #1 Use case LBO VSMF to HCHF N40 failure scenario Use case LBO VSMF to HCHF N47 failure scenario | 0.2.0 |
| 2026-02 | SA5#165 | S5-260239 S5-260540 S5-260541 S5-260542 S5-260543 S5-260544 S5-260545 S5-260546 S5-260553 S5-260554 S5-260555 S5-260547 S5-260556 S5-260549 S5-260548 S5-260550 S5-260551 S5-260552 S5-260557 |  |  |  | Rel-20 pCR 32.872 Corrections to TR 32.872 Rel-20 pCR 32.872 update solution #1.2 Rel-20 pCR 32.872 Handling of charging information stored in V-CHF Rel-20 pCR 32.872 Introduce the usecase of the trigger condition of V-CHF Rel-20 pCR 32.872 Use triggers from V-CTF in V-CHF charging request Rel-20 pCR 32.872 Define the trigger conditions for V-CHF Rel-20 pCR 32.872 Solution topic 1 issue 1 using SBA Rel-20 pCR 32.872 Solution topic 1 issue 2 using SBA Rel-20 pCR 32.872 Solutions evaluation for Key issue #1.1 Rel-20 pCR 32.872 Solutions evaluation for Key issue #1.2 Rel-20 pCR 32.872 Solutions evaluation for Key issue #1.3 Rel-20 pCR 32.872 Solution topic 2 issue 1 using SBA Rel-20 pCR 32.872 Solutions evaluation for Key issue #2.1 Rel-20 pCR 32.872 Key Issue for for Topic#3 Rel-20 pCR 32.872 Use Case for CDR Generation after CHF-SMF Connection Loss Rel-20 pCR 32.872 Handling of V-CTF when V-CTF detected both V-CHF and H-CHF failure simultaneously Rel-20 pCR 32.872 Solution topic 3 issue 1 using SBA Rel-20 pCR 32.872 Solution topic 3 issue 2 using SBA Rel-20 pCR 32.872 Solutions evaluation for Key issue #3.1 and 3.2 | 0.3.0 |