3GPP TSG-SA5 Meeting #145-e 	S5-225213

# e-meeting, 15 - 24 August 2022

# Source: China Mobile, HUAWEI

# Title:    TS 28.317 Use case and requirement for RANSC data handling

# Document for:   Approval

Agenda Item:  6.4.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	SP-211431 New WID on Self-Configuration of RAN Nes

[2]           S5-222726  TS 28.317 v0.1.0

3	Rationale

This document is going to provide proposals on usecases and requirements for ARCF data handling.

4	Detailed proposal

This document proposes the following updates for TS 28.317.

1st Change

- Management capabilities

5.1	RANSC data handling

5.1.1	Use cases

The RAN NE initial radio configuration data will be provided to RAN NE by MnS producer during self-configuration processes. MnS producer is the MnF within the capability of RANSC management capability, which can be deployed as a separate entity to provide MnS(s) for several RAN NEs with a specific management scope or embedded in RAN NE to provide MnS(s) for a single RAN NE. Some RAN NE initial data required for successful activation (of e.g. cell, gNB) require coordination between several cells and RAN NEs cannot be generated by MnS Producer. So RANSC data needs to be provided to MnS producer.

Some of the RANSC data (e.g. nRTAC, cellIdentity, gNBId) can be used directly as RAN NE initial configuration data and some of RANSC data (e.g. PCIList) can be used to generate more other RAN NE initial configuration data (e.g. PCI).The goal of the ARCF data handling is to transfer the ARCF data to the MnS producer for RANSC if needed and ensure that it is valid when it is used during self-configuration process.

The MnS producer needs to obtain the RANSC data and generate the completed RAN NE initial configuration data to RAN NE. The authorized MnS consumer will provide the RANSC data to MnS producer directly or indicate it where the RANSC data is available and it can retrieve the data from there.

Before MnS producer obtain the RANSC data, it may indicate need for RANSC data to the authorized MnS consumer.

When MnS producer receive the RANSC data, it may perform RANSC data validation to validate the syntax and semantics of RANSC data.

5.1.2	Requirements

REQ-RANSC_DataHandling-1: MnS producer shall have the capability to obtain the RANSC data or information which indicate where the RANSC data is available for downloading from its authorized consumer.

REQ-RANSC_DataHandling-2: MnS producer shall have the capability to allow its authorized consumer to request to validate the previously downloaded RANSC data.

End of change