3GPP TSG-SA5 Meeting #145e 	S5-22xxxx

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 00xx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedures for Network Slice LCM to spport asynchonous operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNET_SLICEPRO |  |  |  |  |  | Date: |  |  | 2022-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Network Slice and Network Slice Subnet LCM operations are long running and hence need to be asynchronous. The changes in this CR introduces support for asynchronous mode of operations for network slice and network slice subnet LCM operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Network Slice and Network Slice Subnet LCM procedures to support asynchronous or long running operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for asynchronous or long running operations for Network Slice and Network Slice Subnet are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.9, 5.1.12, 6.1, 6.2, 6.5.1, 6.5.2, 6.5.3, 6.5.4, 7.2, 7.3, 7.4, 7.5, 7.6, 7.7, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V2.4.1) (2018-02): "Network Function Virtualisation (NFV); Release 2; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	GSMA NG.116 Generic Network Slice Template v1.0 (2019-05-23) - https://www.gsma.com/newsroom/wp-content/uploads//NG.116-v1.0-4.pdf.

[10]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects;System Architecture for the 5G System;Stage 2".

[11]	3GPP TS 38.300: "Technical Specification Group Radio Access Network;NR; NR and NG-RAN Overall Description;Stage 2".

[12]	ETSI GS NFV-IFA 014 (V4.2.1) (2021-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Network Service Templates Specification".

[x]                      3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

2nd Change

### 5.1.9	Network slice instance modification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To modify an existing network slice instance |  |
| Actors and Roles | Network slice provisioning management service consumer. For example, CSP providing NSaaS plays the role of network slice provisioning management service consumer. |  |
| Telecom resources | Network slice instance Network slice provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A. |  |
| Begins when | The network slice provisioning management service provider receives request from its authorized customer with new set of network slice related requirements and decides to modify the associated network slice instance or associate a different network slice instance. |  |
| Step 1 (M) | If modification of associated network slice instance can satisfy the new network slice related requirements, the network slice provisioning management service provider modifies the associated network slice instance, else if modification of network slice instance can not satisfy the new network slice related requirements, then network slice provisioning management service provider identifies a different network slice instance or creates a new network slice instance to satisfy the new requirements. The network slice provisioning management service provider identifies the NSSI(s) associated with the NSI to be modified and generates network slice subnet related requirements for the NSSI(s). |  |
| Step 2 (M) | The network slice provisioning management service provider sends requests to the network slice subnet provisioning management service provider with new sets of network slice subnet related requirements. The network slice provisioning management service provider receives request and decides whether the NSSI needs to be modified. | Network slice subnet instance modification use case |
| Step 3 (M) | The network slice provisioning management service provider receives the response from the network slice subnet provisioning management service provider. If the NSSI modification request cannot be satisfied by the network slice subnet provisioning management service provider, the network slice provisioning management service provider may re-generate the network slice subnet related requirements for the NSSI and go to step 2, or the network slice provisioning management service provider may decide the modification request cannot be satisfied. |  |
| Step 4 (M) | The network slice provisioning management service provider sends response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSI is modified. |  |
| Traceability | REQ-PRO_NSI-FUN-6 |  |

3rd Change

### 5.1.12	Network slice subnet instance modification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To modify an existing network slice subnet instance |  |
| Actors and Roles | Network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network slice subnet instance Network SliceSubnet slice subnet provisioning management service provider. For example, NSSMF plays the role of NetworkSliceSubnetmanagement service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice subnet provisioning The management service provider receives request from its authorized consumer with new sets of network slice subnet related requirements and decides to modify the associated network slice subnet instance or associate a different network slice subnet instance. |  |
| Step 1 (M) | If modification of associated network slice subnet instance can satisfy the new network slice subnet related requirements, the network slice subnet provisioning management service provider modifies the associated network slice subnet instance, else if modification of network slice subnet instance can not satisfy the new network slice subnet related requirements, then network slice subnet provisioning management service provider identifies a different network slice subnet instance or creates a new network subnet slice instance to satisfy the new requirements. The network slice subnet provisioning management service provider identifies the NSSI constituents as well as the transport network (TN) part within the NSSI that needs to be modified, and generates new sets of requirements for the NSSI constituents and transport network if needed. |  |
| Step 2 (M) | The network slice subnet provisioning management service provider checks whether the requirements for the identified NSSI constituents managed by itself could be satisfied, and then triggers the modification of the corresponding NSSI constituents if needed. |  |
| Step 3 (M) | If the NSSI consists of constituent NSSI managed by other network slice subnet provisioning management service provider, and the constituent NSSI is identified to be modified, the network slice subnet provisioning management service provider sends modification request to other network slice subnet provisioning management service provider which manages the constituent NSSI with new sets of constituent NSSI requirements. | Network slice subnet instance modification use case |
| Step 4 (M) | If the NS instance associated with the NSSI needs to be modified, the network slice subnet provisioning management service provider derives the new sets of NS related requirements and triggers corresponding NS instance request to NFVO with Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2]. | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 5 (M) | If the related TN part of the NSSI is identified to be modified, the network slice subnet provisioning management service provider derives new sets of requirements for the TN part and coordinates with the corresponding TN management system. |  |
| Step 6 (M) | The network slice subnet provisioning management service provider generates the modification result based on the received response and send response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI is modified. |  |
| Traceability | REQ-PRO_NSSI-FUN-11 |  |

4th Change

## 6.1	Management services for network slice provisioning

The management services for network slice provisioning are listed in table 6.1-1.

Table 6.1-1: Management services for network slice provisioning

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| Network Slice Provisioning | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | NetworkSlice and NetworkSliceRequirements information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating, or modifying an an instance of NetworkSlice object with NetworkSliceRequirements as input. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators. |
| Network Slice Provisioning data reporting | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NetworkSlice and NetworkSliceRequirements information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NetworkSlice and NetworkSliceRequirements Information model data. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators |

## 6.2	Management services for network slice subnet provisioning

The management services for network slice subnet provisioning are listed in table 6.2-1.

Table 6.2-1: Management services for NSS provisioning

| MnS Name | MnS Component of type A (Operations and notifications) | MnS Component of type B (information model) |
| --- | --- | --- |
| Network Slice Subnet Provisioning | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - modifyMOIAttributes operation - getMOIAttributes operation - deleteMOI operation | NetworkSliceSubnet and NetworkSliceSubnetRequirements information model defined in clause 6.3 of TS 28.541 [6] |
| Network Slice Subnet Provisioning data reporting | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NetworkSliceSubnet and NetworkSliceSubnetRequirements information model defined in clause 6.3 of TS 28.541 [6] |

5th Change

### 6.5.1	Void

### 6.5.2	Void

### 6.5.3	Void

### 6.5.4	Void

6th Change

## 7.2	Procedure of Network Slice Allocation

The Figure 7.2-1 illustrates the procedure of creating a new instance of NetworkSlice object or using an existing instance of NetworkSlice object to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1a)	Network Slice Management Service Provider (NSMS_Provider) receives a “createMOI” request (defined in TS 28.532 [8]) from Network Slice Management Service Consumer (NSMS_Consumer) with network slice related requirements (see “NetworkSliceRequirements” IOC defined in TS 28.541[6])). The NSMS_Consumer may subscribe for notifications to receive the status of network slice instance allocation.

1b) The NSMS_Provider creates an instance of “NetworkSliceRequirements” object and sends the response to NSMS_Consumer for the “createMOI” request.

2)	Based on the network slice related requirements and the knowledge of the capabilities of existing deployed network slices, the NSMS_Provider shall compare/match the provided requirements against all the candidate “NetworkSlice” instances, and then decides whether to re-use an existing “NetworkSlice” instance or create a new “NetworkSlice” instance. If the network slice related requirements allow the requested “NetworkSlice” instance to be shared and if an existing suitable “NetworkSlice” instance can be reused, the NSMS_Provider may decide to re-use the existing“NetworkSlice” instance.

3a-1) If using an existing instance of “NetworkSlice” object, the NSMS_ Provider identifies the instance of the object to be re-used.

3a-2)NSMS_ Provider may modify the “NetworkSlice” instance to satisfy the network slice related requirements.

3a-3) NSMS_P may derive requirements for network slice subnet and invoke procedure of network slice subnet instance modification (defined in clause 7.7).

3b-1) If creating a new instance of “NetworkSlice” object, the NSMS_Provider creates an instance of “NetworkSlice” IOC.

3b-2) The NSMS_Provider derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_Provider derives the network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

3b-2)	The NSMS_Provider invokes the procedure of network slice subnet instance allocation as described in clause 7.3. Before NSMS_Provider invokes procedure of network slice subnet instance allocation, NSMS_Provider may invoke corresponding network slice subnet feasibility check procedure as described in clause 7.14.

3b-3)	The NSMS_Provider configures the attribute “networkSliceSubnetRef” with the DN of “NetworkSliceSubnet” object instance, other configuration information may be configured for the created “NetworkSlice” object instance.

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

4) If an existing “NetworkSlice” object instance is modified or a new “NetworkSlice” object instance is created successfully to satisfy the network slice related requirements, the NSMS_Provider updates the DN of the instance of “NetworkSlice” object (identified in step 2 or 3b-3) in the attribute “requestedObjectRef” of “NetworkSliceRequirements” object. The NSMS_Provider updates the DN of the instance of “NetworkSliceRequirements” object in the attribute “networkSliceRequirementsRef” of “NetworkSlice” object.

The NSMS_Consumer may check the status and completion of the network slice instance allocation procedure by monitoring the values of “NetworkSliceRequirements” instance attributes “operationalState”, “administrativeState”, “availabilityStatus” and “processMonitor” by querying the values or by subscribing to notifications.

The NSMS_Provider maintains the allocation of the NetworkSlice as long as the instance of NetworkSliceRequirements exists. In other words, the instance of  NetworkSliceRequirements has to exist as long as the MnS producer shall enforce the ServiceProfiles listed in the attribute requirementsList.

## 7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new network slice subnet instance or using an existing network slice subnet instance to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1a)	Network Slice Subnet Management Service Provider (NSSMS_P) receives a “createMOI” request (defined in TS 28.532 [8]) from Network Slice Subnet Management Service Consumer (NSSMS_C) with network slice subnet related requirements (see “NetworkSliceSubnetRequirements” IOC defined in TS 28.541[6])). The NSSMS_C may subscribe for notifications to receive the status of network slice subnet instance allocation.

1b) The NSSMS_P creates an instance of “NetworkSliceSubnetRequirements” object and sends the reponse to NSSMS_C for the “createMOI” request.

2) NSSMS_P checks the feasibility of network slice subnet related requirements. If the network slice subnet related requirements can be satisfied, the following steps shall be executed.

3)	Based on the network slice subnet related requirements and the existing network slice subnet capabilities, NSSMS_P decides whether to re-use an existing “NetworkSliceSubnet” instance or create a new“NetworkSliceSubnet” instance. If the network slice subnet related requirements allow the requested “NetworkSliceSubnet” instance to be shared and if an existing suitable “NetworkSliceSubnet” instance can be reused, the NSSMS_P decides to re-use the existing “NetworkSliceSubnet” instance.

4a-1) If re-using an existing instance of “NetworkSliceSubnet” object, the NSSMS_P identifies the instance of the object to be re-used.

4a-2) NSSMS_P may modify the “NetworkSliceSubnet” instance to to satisfy the network slice subnet related requirements.

4a-3) NSSMS_P may derive requirements for constituent network slice subnet(s) and invoke procedure of network slice subnet instance modification (defined in clause 7.7).

4b-1) If creating a new instance of “NetworkSliceSubnet” object, the NSSMS_P creates an instance of “NetworkSliceSubnet” object.

4b-2) NSSMS_P derives the corresponding network slice subnet constituent (i.e. NF, constituent NSS) related requirements and transport network related requirements (e.g. 3GPP endpoint information, latency requirements, bandwidth requirements and isolation requirements) from the received network slice subnet related requirements. Part of these requirements may be referenced by attribute "epTransportRef" as defined in clause 6.3.2.2 in TS 28.541[6]. Before NSSMS_Provider derives the constituent network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

4b-3)	If the NSSI to be created contains virtualisation part (i.e. VNF or VL), NSSMS_P derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. NSSMS_P determines new VNF instance(s) that need to be deployed and the existing VNF instance(s) that need to be reused according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. NSSMS_P chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. NSSMS_P invokes the NS instantiation procedures to create a NS instance. NSSMS_P configures the NS instance identifier in “nsInfo” attribute of “NetworkSliceSubnet” object.

Note:	NS instantiation procedure is described in TS 28.526 [7].

4b-4) For each required NSSI constituent, the following step 4b-4.a1), 4b-4.a2), 4b-4.b1) and 4b-4.b2) are needed:

4b-4.a1) If the required NSSI constituent is constituent network slice subnet instance, NSSMS_P invokes the procedure of network slice subnet instance allocation (defined in clause 7.3).

4b-4.a2)	NSSMS_P configures the “NetworkSliceSubnet” instance attribute “networkSliceSubnetRef” with the DN of the instance for constituent “NetworkSliceSubnet”.

4b-4.b1) If the required NSSI constituent is NF instance, NSSMS_P invokes NF Creation Procedure as described in clause 7.10 or NF Modification Procedure as described in clause 7.11.

4b-4.b2)	NSSMS_P configures the “NetworkSliceSubnet” instance attribute “managedFunctionRef” with the DN of ManagedFunction MOI.

4b-5)	For each required transport network related requirements, NSSMS_P invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

5)	If an existing instance of “NetworkSliceSubnet” object is re-used or a new instance of “NetworkSliceSubnet” object is created successfully to satisfy the network slice subnet related requirements, the NSSMS_P updates the DN of the instance of “NetworkSliceSubnet” object (identified in step 3 or 4.1b.1) in the attribute “requestedObjectRef” of “NetworkSliceSubnetRequirements” object. The NSSMS_Provider updates the DN of the instance of “NetworkSliceSubnetRequirements” object in the attribute “networkSliceSubnetRequirementsRef” of “NetworkSliceSubnet” object.

The NSSMS_C may check the status and completion of the network slice subnet instance allocation procedure by monitoring the values of “NetworkSliceSubnetRequirements” instance attributes “operationalState”, “administrativeState”, “availabilityStatus” and “processMonitor” by querying the values or by subscribing to notifications.

The NSSMS_P maintains the allocation of the NetworkSliceSubnet as long as the instance of NetworkSliceSubnetRequirements exists. In other words, the instance of  NetworkSliceSubnetRequirements has to exist as long as the MnS producer shall enforce the SliceProfiles listed in the attribute requirementsList.

## 7.4	Procedure of Network Slice Instance Deallocation

Figure 7.4-1 depicts the procedure of deallocating a network slice instance by the network slice management service provider to satisfy the network slice instance deallocation request received from an authorized consumer.

Figure 7.4-1: Network slice instance deallocation procedure

1) The network slice management service provider (NSMS_P) receives a “deleteMOI” request (defined in TS 28.532 [8]) with the DN of the “NetworkSliceRequirements” object instance from network slice management service consumer (NSMS_C) indicating that the given requirements are no longer needed.The NSMS_C may subscribe for notifications to receive the status of network slice instance deallocation.

2)  The NSMS_P identifies the associated “NetworkSlice” object from the attribute “requestedObjectRef” of the “NetworkSliceRequirements” instance to be deleted. NSMS_P unassigns the the DN of the instance of “NetworkSliceRequirements” object in the attribute “networkSliceRequirementsRef” of “NetworkSlice” object. The NSMS_P deletes the instance of “NetworkSliceRequirements” object and sends the reponse to NSMS_C for the “deleteMOI” request.

3) The NSMS_P may decide to delete the “NetworkSlice” object identified in step 2, then it invokes the procedure for network slice subnet deallocation as described in clause 7.5.

4) The NSMS_P may decide not to delete the “NetworkSlice” object identified in step 2, then it may modify the “NetworkSlice” instance to satisfy the request to delete the network slice related requirements.

5) NSMS_P may derive modification of network slice subnet requirements and invoke procedure of network slice subnet instance modification (defined in clause 7.7).

## 7.5	Procedure of network slice subnet instance deallocation

Figure 7.5-1 depicts the procedure of deallocating a network slice subnet instance by the network slice subnet management service provider to satisfy the NSSI deallocation request received from an authorized consumer.

Figure 7.5-1: Network slice subnet instance deallocation procedure

1) The network slice subnet management service provider (NSSMS_P) receives a “deleteMOI” request (defined in TS 28.532 [8]) with the DN of the “NetworkSliceSubnetRequirements” object instance from network slice subnet management service consumer (NSSMS_C) indicating that the given requirements are no longer needed.  The NSSMS_C may subscribe for notifications to receive the status of network slice subnet instance deallocation.

1b) The NSSMS_P identifies the associated “NetworkSliceSubnet” object from the attribute “requestedObjectRef” of the “NetworkSliceSubnetRequirements” instance to be deleted. NSSMS_P unassigns the the DN of the instance of “NetworkSliceSubnetRequirements” object in the attribute “networkSliceSubnetRequirementsRef” of “NetworkSliceSubnet” object. The NSSMS_P deletes the instance of “NetworkSliceSubnetRequirements” object and sends the reponse to NSSMS_Consumer for the “deleteMOI” request.

2a) NSSMS_P may decide to delete the “NetworkSliceSubnet” object identified in step 2, then it invokes procedure of network slice subnet instance deallocation (described in clause 7.5) for the constituent network slice subnets.

2b) NSSMS_P invokes NF deletion procedure as described in clause 7.12 only if the NF is dedicated for this NSSI and not being used by any other NSSI in the network, otherwise, NSSMS_P invokes  NF modification procedure as described in clause 7.11.

2c) NSSMS_P invokes TN related coordination procedure with responsible manager as described in clause 7.9 if the NSSI consists of TN part.

2d) NSSMS_P invokes NS termination procedure if the NSSI contains virtualized part.

Note:	NS termination procedure is described in TS 28.526 [7].

2e) NSSMS_P deletes “NetworkSliceSubnet” MOI.

3a) NSSMS_P may decide not to delete the “NetworkSliceSubnet” object identified in step 2, then it modifies the “NetworkSliceSubnet” instance.

3b) NSSMS_P may derive modification of network slice subnet requirements for its constituent network slice subnets and invoke procedure of network slice subnet instance modification (defined in clause 7.7).

## 7.6	Procedure of Network Slice Instance Modification

The Figure 7.6-1 illustrates the procedure of modifying an existing NSI.

Figure 7.6-1: Network Slice Instance Modification Request procedure

1a)	Network Slice Management Service Provider (NSMS_P) receives a “modifyMOIAttributes” request (defined in TS 28.532 [8]) from Network Slice Management Service Consumer (NSMS_C) with the DN of with “NetworkSliceRequirements” object instance with the new network slice related requirements. The NSMS_C may subscribe for notifications to receive the status of network slice instance modification.

1b) The NSSMS_P modifies the instance of “NetworkSliceRequirements” object and sends the reponse to NSSMS_C for the “modifyMOIAttributes” request.

2)	Based on the new network slice related requirements, NSMS_P invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 3).

3) NSMS_P decomposes the network slice related requirements modification request into network slice subnet related modification request(s), i.e., generating the new network slice subnet related requirements for each NSSI if needed, and determines if the associated “NetworkSlice” instance needs to be modified or to associate a different “NetworkSlice” instance to fulfil the request for modification of network slice related requirements.

4a-1) If NSMS_P determines that the associated NetworkSlice instance needs to be modified to fulfil the request for modification of requirements, then NSMS_P, as the role of Network Slice Subnet Management Service Consumer (NSSMS_C), invokes the procedure for network slice subnet instance modification (described in clause 7.7).

4a-2) NSMS_P may modify the “NetworkSlice” instance to satisfy the request to modify the network slice related requirements.

4b) If NSMS_P determines to associate a different NetworkSlice instance to fulfil the request for modification of network slice related requirements, then based on the network slice related requirements and the knowledge of the capabilities of existing deployed network slices, the NSMS_P shall compare/match the provided requirements against all the candidate “NetworkSlice” instances, and then decides whether to re-use an existing “NetworkSlice” instance or create a new “NetworkSlice” instance. If the network slice related requirements allow the requested “NetworkSlice” instance to be shared and if an existing suitable “NetworkSlice” instance can be reused, the NSMS_Provider may decide to re-use the existing “NetworkSlice” instance. If the previous requirements were fulfilled by shared network slice (shared “NetworkSlice” object instance), and now the NSMS_C modifies the requirements such that the requested requirements do not allow sharing of “NetworkSlice” instance, then NSMS_P creates a new “NetworkSlice” instance to fulfil the request. The procedure to create a new instance of NetworkSlice or use an existing instance of NetworkSlice are described in in the steps from step 3a-1) to step 4) of clause 7.2.

The NSMS_C may check the status and completion of the network slice instance modification procedure by monitoring the values of “NetworkSliceRequirements” instance attributes “operationalState”, “administrativeState”, “availabilityStatus” and “processMonitor” by querying the values or by subscribing to notifications.

## 7.7	Procedure of Network Slice Subnet Instance Modification

The Figure 7.7-1 illustrates the procedure of modifying an existing NSSI.

Figure 7.7-1: Network Slice Subnet Instance Modification Request procedure

1a)	Network Slice Subnet Management Service Provider (NSSMS_P) receives a modifyMOIAttributes operation defined in TS 28.532 [8] from Network Slice Subnet Management Service Consumer (NSSMS_C) with the DN of “NetworkSliceSubnetRequirements” object instance with the new network slice subnet related requirements. The NSSMS_C may subscribe for notifications to receive the status of network slice subnet instance modification.

1b) The NSSMS_P modifies the instance of “NetworkSliceSubnetRequirements” object and sends the reponse to NSSMS_C for the “modifyMOIAttributes” request.

2)	Based on the new network slice subnet related requirements, NSSMS_P invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 3).

3) NSSMS_P decomposes the network slice subnet related requirements modification request into modification requests for each NSSI constituent and determines if the associated “NetworkSliceSubnet” instance needs to be modified or to associate a different “NetworkSliceSubnet” instance to fulfil the request for modification of network slice subnet related requirements.

4a-1) If the requested NSSI constituent is constituent NSSI, NSSMS_P invokes procedure of network slice subnet instance modification as described in clause 7.7.

4a-2) If the requested NSSI constituent is NF instance, NSSMS_P invokes NF creation procedure as described in clause 7.10 or NF modification procedure as described in clause 7.11.

4a-3)	If the NSSI contains the virtualized part, NSSMS_P invokes the NS instance scaling and/or NS instance updating and/or NS instance instantiation procedure as described in TS 28.526 [7].

4a-4) If the NSSI contains the TN part, NSSMS_P invokes the TN related coordination procedure as described in clause 7.9.

4a-5) NSSMS_P may modify the “NetworkSliceSubnet” instance to satisfy the request to modify the network slice related requirements.

4b-1)	Based on the network slice subnet related requirements and the existing network slice subnet capabilities, NSSMS_P decides whether to re-use an existing “NetworkSliceSubnet” instance or create a new “NetworkSliceSubnet” instance. If the network slice subnet related requirements allow the requested “NetworkSliceSubnet” instance to be shared and if an existing suitable “NetworkSliceSubnet” instance can be reused, the NSSMS_P decides to re-use the existing “NetworkSliceSubnet” instance. If the previous requirements were fulfilled by shared network slice (shared “NetworkSlice” object instance), and now the NSMS_C modifies the requirements such that the requested requirements do not allow sharing of “NetworkSlice” instance, then NSMS_P creates a new “NetworkSlice” instance to fulfil the request. The procedure to create a new instance of NetworkSlice or use an existing instance of NetworkSlice are described in the steps from step 4a-1) to step 5) of clause 7.3.

The NSSMS_C may check the status and completion of the network slice instance modification procedure by monitoring the values of “NetworkSliceSubnetRequirements” instance attributes “operationalState”, “administrativeState”, “availabilityStatus” and “processMonitor” by querying the values or by subscribing to notifications.

7th Change

# 9	Void

End of change