3GPP TSG CT WG3 Meeting #136	C3-244107

# Maastricht, NL, 19 - 23 August, 2024										(Revision of C3-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.525 | CR | 0350 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UrspEnforcementPduSession Presence condition and miscellaneous changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 12-8-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In 5.6.2.11, ueReqPduSessionType presence condition is mentioned as optional even thought there is an Condition explained in the description already. - "specifed" typo exists in 4.2.2.2.3.1 - The wrong referenece update in TS 29.518, TS 29.571 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) ueReqPduSessionType presence condition is updated to C. 2) "specifed" typo fix in 4.2.2.2.3.1 3) TS 29.571 reference update from 13 to 11 in clause 5.6.1 for PduSessionType 4) The reference update for TS 29.418 in clause 4.2.4.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The wrong presence condition for ueReqPduSessionType and the wrong reference mentioned for TS 29.518, TS 29.571. - The quality of the document is not up to the mark. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.3.1, 4.2.4.7, 5.6.1, 5.6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

###### 4.2.2.2.3.1	General

The UE Route Selection Policy is used by the UE to determine how to route outgoing traffic.

The UE Route Selection Policy shall consist of one or several URSP rules. The PCF determines whether URSP rule(s) have to be provisioned based on input parameters received from the NF service consumer, the received list of UPSIs from the UE, if available, the UE Policy Sections stored in the UDR, if available, other received UE parameters, if available, the policy subscription and application data retrieved from UDR, if available, analytics information received from NWDAF, if available, and local policies.

URSP rules are encoded as defined in 3GPP TS 24.526 [16].

UE Route Selection Policy may only be provided by a H-PCF or the PCF of the SNPN, but shall not be provided by a V-PCF. However, UE Route Selection Policy determined and provided by the H-PCF may be retrieved by a V-PCF from the H-PCF and forwarded to a UE.

The (H-)PCF shall use the UE policy subscription data stored in UDR as specified in 3GPP TS 29.519 [17] to ensure the values included in the Route Selection Descriptor of the generated URSP rules are always supported by subscription.

For the received list of internal group Ids, the (H-)PCF retrieves the corresponding 5G VN group configuration data stored from the UDR as specified in 3GPP TS 29.504[27] and 3GPP TS 29.505 [26], if available. For each available 5G VN group, the (H-)PCF may use the retrieved 5G VN group configuration values to encode the values for the Route Selection Descriptor and the values for the Traffic Descriptor of the generated URSP rules.

If the "EnhancedBackgroundDataTransfer" feature is supported, the (H-)PCF may retrieve the Background Data Transfer Reference ID(s) by retrieving the UE's Application Data from the UDR as defined in clause 6.2.9 of 3GPP TS 29.519 [17]. In this case, the PCF shall retrieve the transfer policy corresponding to the Background Data Transfer Reference ID(s) as defined in clause 5.2.8 of 3GPP TS 29.519 [17] and then may create the URSP rules including the Route Selection Validation Criteria for the UE as defined in clause 6.6.2.1 of 3GPP TS 23.503 [4]. If the (H-)PCF provisions the URSP rules including the Route Selection Validation Criteria for the UE, it shall use the associated S-NSSAI and DNN to store in the UDR the Background Data Transfer Reference ID(s) in the UE's session management policy data as specified in 3GPP TS 29.519 [17].

If the (H-)PCF retrieves the BDT policy and corresponding related information (e.g. network area information, the volume of data to be transferred per UE, etc.) within the BdtData data type, and the "bdtpStatus" attribute within the BdtData data type is set to value "INVALID", the (H-)PCF shall not provision the URSP rules based on the invalid BDT policy. When the BDT policy re-negotiation is completed the PCF may:

-	if the new BDT Policy is determined, create or update the applicable URSP rules based on the new BDT policy; or

-	if the invalid BDT policy is removed, remove applicable URSP rules.

If the "AfGuideURSP" feature is supported by the Nudr_DataRepository service, the (H-)PCF may receive Service specific parameter information that contains data for AF guidance information on the URSP determination as defined in clause 6.4.2.15 of 3GPP TS 29.519 [17]. In this case, the (H-)PCF may also use this AF guidance information as input to determine the URSP that will be provisioned to the UE. If the received AF guidance information is not consistent with the UE subscription data, or the local operator policy does not allow the specific S-NSSAI and DNN provided by the AF guidance information, the corresponding AF guidance information shall not be used to determine the URSP rules. The PCF may also determine not to use AF guidance based on the analytics info received from the NWDAF.

When the (H-)PCF decides to provide URSP rules based on the AF guidance information, it shall derive the information as follows:

-	Application traffic descriptor within the "trafficDesc" attribute is used to set the Traffic Descriptor of URSP rule (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]).

-	Each route selection parameter set within the "routeSelParamSets" attribute of the UrspRuleRequest data type is used to determine a Route selection descriptor (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]) as follows:

-	DNN (within the "dnn" attribute of the RouteSelectionParameterSet data type) and S-NSSAI (within the "snssai" attribute of the RouteSelectionParameterSet data type) from the route selection parameter set are used to set the Route selection descriptor contents (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]);

-	Route selection precedence (within the "precedence" attribute of the RouteSelectionParameterSet data type) is used to set the Precedence value of route selection descriptor (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]); and

-	the spatial validity condition (within the "spatialValidityTais" attribute of the RouteSelectionParameterSet data type) is used to set the Location criteria of the route selection descriptor (defined in Figure 5.2.5 of 3GPP TS 24.526 [16]).

-	The PCF may use the requested PDU Session type provided within the "pduSessType" attribute of the RouteSelectionParameterSet data structure to derive the PDU Session type of the route selection descriptors of the URSP rule.

-	The precedence for the generated URSP rule is determined by the (H-)PCF. The (H-)PCF may use the "relatPrecedence" attribute within the "UrspRuleRequest" data type to derive the relative precedence of the URSP rule for a request coming from the same AF.

URSP rules based on AF guidance should not be set as the URSP rules with the "match all" application traffic descriptor.

The (H-)PCF may obtain the information about the UE's OS from the UE as described in the Annex D of 3GPP TS 24.501 [15] or it may derive the information about the UE's OS from the PEI provided by the NF service consumer (e.g. AMF).

If the (H-)PCF is required to provide UE policies to the UE that includes application descriptors then:

a)	If the (H-)PCF has been provided with one UE's OS Id by the UE, the (H-)PCF shall use either the traffic descriptor "OS App Id type" or the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16].

NOTE 1:	The (H-)PCF uses the traffic descriptor "OS Id + OS App Id type" when the (H-)PCF does not take the received UE's OS Id into account.

b)	If the (H-)PCF has been provided with more than one UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" for the UE's OS Id provided by the UE as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

c)	If the (H-)PCF has not been provided with the UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

d)	If the (H-)PCF has been provided with the UE's OS Id by the UE and the (H-)PCF has derived the UE's OS Id from the PEI and if there is an inconsistency between the OS Id provided by the UE and the OS Id derived from the PEI, the (H-)PCF shall use the OS Id provided by the UE for providing UE policies to the UE that include application descriptors.

URSP rules may be used to support end to end redundant user plane paths by establishing two redundant PDU sessions. PCF configuration based on e.g. deployment, terminal implementation or policies per group of UE(s) may be used by the PCF to determine whether the URSP Rules shall include PDU Session Pair ID and RSN to indicate that they refer to redundant PDU sessions or whether the UE will determine these values instead.

NOTE 2:	When the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported, the received satellite or non-satellite backhaul category can be used as input to provision or update URSP rules to enable appropriate PDU session capabilities. E.g., when satellite backhaul category is indicated by the AMF, the (H-)PCF can take it into account to determine, based on operator policies, an appropriate Route Selection Descriptor for the URSP rule and the services deployed on the satellite, (e.g., the provisioning or update of URSP rules to indicate the specific DNN for services deployed on-board satellites).

If the AF provided the (H-)PCF with Personal IoT Network identifier (PIN ID) associated with a DNN and S-NSSAI, and the received DNN and S-NSSAI corresponds to a subscribed DNN and S-NSSAI combination in the UE Policy Context as described in 3GPP TS 29.519 [17], the (H-)PCF shall include the PIN ID within the traffic descriptor of the URSP Rule attribute as defined in 3GPP TS 24.526 [16] for UE to choose an appropriate PIN to establish the PDU session.

NOTE 3:	The PCF can provide two distinct URSP rules to support end to end redundant user plane paths using Dual Connectivity for the duplicated traffic of an application. Duplicated traffic from the UE application is differentiated by two distinct traffic descriptors (different DNNs, and for IP traffic, different IP descriptors or non-IP descriptors), each one defined in a different URSP rule, so that the two redundant PDU sessions are matched to the specific Route Selection Descriptors of distinct URSP rules. These Route Selection Descriptors of distinct URSP rules may include corresponding RSNs and PDU Session Pair IDs as defined in 3GPP TS 24.526 [16]. The Route Selection Descriptors share the same PDU Session Pair ID, if included, to denote the two traffic are redundant with each other.

NOTE 4:	For backward compatibility, PCF can provide a Route Selection Descriptor with PDU Session Pair ID and RSN and a Route Selection Descriptor without PDU Session Pair ID and RSN in the URSP rule. In this case, the Route Selection Descriptor with PDU Session Pair ID and RSN has a lower precedence value (i.e. higher prioritised) than the one without PDU Session Pair ID. It allows that if a non-supporting UE receives the Route Selection Descriptor containing PDU Session Pair ID, it ignores this Route Selection Descriptor.

NOTE 5:	PIN ID and other traffic descriptor components are mutually exclusive, i.e., if PIN ID is included in a URSP rule, then no other traffic descriptor components are supported in the same URSP rule.

The PCF may adjust the URSP rules when needed, based on awareness of URSP rule enforcement for an application by using the following mechanisms:

A.	Awareness of URSP rule enforcement with UE assistance:

-	Based on operator policies, and if the UE included in the UE STATE INDICATION message the indication of UE's support of reporting URSP rule enforcement as specified in the Annex D of 3GPP TS 24.501 [15], the PCF may indicate in a URSP rule sent to the UE to send reporting of URSP rule enforcement, as specified in 3GPP TS 24.526 [16]. For this URSP rule, the UE reports URSP rule enforcement information to the SMF if Connection Capabilities are included in the traffic descriptor, as specified in the Annex D of 3GPP TS 24.501 [15] and in 3GPP TS 24.526 [16]. When several URSP rules for multiple applications associated to a PDU session are enforced, several URSP rule enforcement reports are included within the URSP rule enforcement information. The SMF reports URSP rule enforcement information received from the UE and its PDU session parameters (e.g. requested DNN, SSC mode, S-NSSAI of the HPLMN, PDU Session Type) to the PCF for the PDU session as specified in 3GPP TS 29.512 [31].

-	For LBO roaming session case, if the feature "URSPEnforcement" is supported, the H-PCF for the UE may send the "URSP_ENF_INFO" Policy Control Request Trigger to the V-PCF for the UE during the UE Policy Association Establishment or Modification procedures. When the V-PCF receives URSP rule enforcement information and the PDU session parameters as described above, the V-PCF shall invoke the UE Policy Association Update Modification procedure as described in clause 4.2.3.1.

-	If the (V-)(H-)PCF for a UE and the PCF for a PDU session are different, then the (V-)(H-)PCF for a UE may subscribe to the PCF for a PDU session to receive the reporting of URSP rule enforcement information as defined in 3GPP TS 29.514 [37] and the (V-)(H-)PCF for a UE may obtain UE reporting of URSP rule enforcement information and the PDU session parameters from the PCF for a PDU session as defined in 3GPP TS 29.514 [37], where the V-PCF for a UE interacts with the PCF for a PDU session in the VPLMN and the H-PCF for a UE interacts with the PCF for a PDU session in the HPLMN. The (V-)(H-)PCF for the UE discovers the PCF for the PDU session via subscription with the BSF as specified in 3GPP TS 29.521 [22] or, when the feature "URSPEnforcement" is supported, via the request to the AMF to be notified about whether the PCF for the PDU session is available as specified in clauses 4.2.2.1, 4.2.3.1 and 4.2.4.2.

-	Based on the received URSP rule enforcement information and the PDU session parameters, the (H-)PCF may adjust the URSP rules e.g. when the (H-)PCF determines that the UE does not have up-to-date URSP rules. To identify the enforced URSP rule, the PCF may compare the reported Connection Capabilities with the Connection Capabilities of the URSP rules with the indication of "reporting of URSP rule enforcement" and, to identify the Route Selection Descriptor of the enforced URSP rule, the PCF may compare the received PDU session parameters with the parameters in the Route Selection Descriptor components. If there are more than one URSP rule(s) that match the reported Connection Capabilities, the PCF may identify the enforced URSP rule(s) based on implementation specific means. If there are inconsistencies between the received PDU session parameters and the parameters in the Route Selection Descriptor components of the enforced URSP rule, the PCF may perform appropriate actions (e.g. if the S-NSSAI does not match, the PCF may initiate slice replacement procedure).

-	In this release of the specification, the received URSP rule enforcement information shall contain for each URSP rule enforcement report, the one or more Connection Capabilities contained in the traffic descriptor of the concerned URSP rule. If the URSP rule enforcement report does not include connection capabilities, based on local policies, the (H-)PCF for the UE may ignore the received URSP rule enforcement report.

NOTE 6:	A UE supporting the report of URSP rule enforcement reports URSP rule enforcement when the UE (based on the application information matching the traffic descriptor of the URSP rule) associates a newly detected application to a new PDU session or to an existing PDU session and when based on URSP re-evaluation the UE changes the association of an existing application to a PDU session. The UE does not report URSP rule enforcement while the UE is in EPS, delaying the report to when the UE moves from EPS to 5GS.

B.	Awareness of URSP rule enforcement without UE assistance: The PCF may subscribe to or request the PDU Session Traffic analytics statistics using the Nnwdaf_EventsSubscription_Subscribe service operation or Nnwdaf_AnalyticsInfo_Request service operation including the "PDU_SESSION_TRAFFIC" event for traffic monitoring of known traffic according to provisioned PDU Session Traffic requirements of corresponding URSP rule(s) at the NWDAF as defined in 3GPP TS 29.520 [38]. If the PCF is notified or responded with traffic that does not match Traffic Descriptor provided that is the traffic which is not expected according to a URSP rule, the PCF may adjust the URSP rules when unexpected application traffic is detected.

NOTE 7:	The PCF can combine the UE reporting of URSP rule enforcement with the analytics information together to adjust the URSP rules.

# * * * * Next changes * * * *

#### 4.2.4.7	UE policy provisioning for AF-influenced URSP

If the "AfGuideURSP" feature is supported by the Nudr_DataRepository service, after the UE policy association establishment, the (H-)PCF may be informed that service specific parameter information that contains data for AF guidance on the URSP determination has been created, modified or removed via a notification by the UDR for the change or removal of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case, the H-PCF may derive new URSP(s), modify existing URSP(s) or remove existing URSP(s) by using the information received from the UDR (see clause 4.2.2.2.1.1 and 4.2.2.2.3 for the description of how the (H-)PCF may use this information, stored UPSC(s), policy subscription information, analytics information received from NWDAF and local operator policy to determine the URSP that will be provisioned to the UE), and it shall:

-	for the roaming case, provision the derived new UE Policy Sections, and/or update and/or remove existing UE Policy Sections to the V-PCF as defined in clause 4.2.4.2 and then the V-PCF shall invoke the Namf_Communication_N1N2MessageTransfer service operation to provision the received UE Policy Sections  to the UE; or

-	for the non-roaming case, use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to convey the derived new UE Policy Sections and/or to update and/or remove existing UE Policy Sections to the UE via the AMF within "MANAGE UE POLICY COMMAND" message(s).

In the roaming case, when the AMF informs the V-PCF that the UE is temporarily unreachable (see 3GPP TS 29.518 [14]), the V-PCF notifies the H-PCF accordingly (including the "uePolTransFailNotif" attribute within the PolicyAssociationUpdateRequest data structure, as described in clause 4.2.2.2.1.0).

When the (H-)PCF receives the "MANAGE UE POLICY COMPLETE" or the "MANAGE UE POLICY COMMAND REJECT" message and/or the PCF deducts that the UE is temporarily unreachable, and the PCF determines that the received message or the internal deduction indicates a UE Policy Delivery outcome event is matched:

-	if an NF service consumer has subscribed via a request for service specific parameters to the HPLMN, the (H-)PCF shall invoke the Npcf_EventExposure_Notify service operation as defined in clause 4.2.4.2 of 3GPP TS 29.523 [30]; or

-	if a V-PCF has subscribed with the H-PCF as specified in clauses 4.2.2.1 and/or 4.2.3.1 because an AF has subscribed via a request for service parameters to the VPLMN, the H-PCF shall invoke the Npcf_UEPolicyControl_UpdateNotify as specified in this clause to notify about the result of the delivery of UE policies using the "delivReport" attribute. The "delivReport" attribute is a map of "eventNotifs" attributes, where:

a.	the key of the map represents the related AF; and

b.	each "eventNotifs" entry shall contain the reported event(s) ("SUCCESS_UE_POL_DEL_SP" or "UNSUCCESS_UE_POL_DEL_SP") within the "event" attribute and in case of delivery failure, the "delivFailure" attribute with the corresponding failure reason.

When the AMF (non roaming case) or the V-PCF (roaming case) informs the (H-)PCF that the UE is temporarily unreachable (see 3GPP TS 29.518 [14]), the (H-) PCF may subscribe to "CON_STATE_CH" trigger if not done before and reattempt the provisioning of URSP(s) when the UE becomes reachable.

# * * * * Next changes * * * *

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_UEPolicyControl service based interface protocol.

Table 5.6.1-1: Npcf_UEPolicyControl specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A2xCapability |  | 5.6.3.12 |  | Indicates the A2X capabilities |  | A2X |  |
| LboRoamingInformation |  | 5.6.2.10 |  | LBO roaming information for a DNN and S-NSSAI |  | VPLMNSpecificURSP |  |
| N1N2MessTransferErrorReply |  | 5.6.3.8 |  | Error the V-PCF may send to the H-PCF when the V-PCF receives from the AMF an error to the N1N2MessageTransfer request. |  | EnErrorHandling |  |
| Non3gppAccess |  | 5.6.3.7 |  | Represents a Non-3gpp access node. |  | SliceAwareANDSP |  |
| Pc5Capability |  | 5.6.3.5 |  | Indicates the specific PC5 RAT(s) which the UE supports for V2X communications and/or A2X communications over PC5 reference point. |  | V2X |  |
| ProSeCapability |  | 5.6.3.6 |  | Indicates the 5G ProSe capabilities. |  | ProSe |  |
| PolicyAssociation |  | 5.6.2.2 |  | Description of a policy association that is returned by the PCF when a policy Association is created, updated, or read. |  |  |  |
| PolicyAssociationReleaseCause |  | 5.6.3.4 |  | The cause why the PCF requests the termination of the policy association. |  |  |  |
| PolicyAssociationRequest |  | 5.6.2.3 |  | Information that NF service consumer provides when requesting the creation of a policy association. |  |  |  |
| PolicyAssociationUpdateRequest |  | 5.6.2.4 |  | Information that NF service consumer provides when requesting the update of a policy association. |  |  |  |
| PolicyStatus |  | 5.6.3.10 |  | Represents the configuration status of a UE Policy in the UE. |  | SliceAwareANDSP |  |
| PolicyUpdate |  | 5.6.2.5 |  | Updated policies that the PCF provides in a notification or in the reply to an Update Request. |  |  |  |
| RangSLCapability |  | 5.6.3.10 |  | Represents the Ranging/SL capabilities. |  | Ranging_SL |  |
| RequestTrigger |  | 5.6.3.3 |  | Enumeration of possible Request Triggers. |  |  |  |
| TerminationNotification |  | 5.6.2.6 |  | Request to terminate a policy Association that the PCF provides in a notification. |  |  |  |
| UeRequestedValueRep |  | 5.6.2.8 |  | Contains the current applicable values corresponding to the policy control request triggers. |  | ImmediateReport |  |
| UePolicy |  | 5.6.3.2 |  | UE Policies |  |  |  |
| UePolicyDeliveryResult |  | 5.6.3.2 |  | UE Policy delivery Result |  |  |  |
| UePolicyNotification |  | 5.6.2.12 |  | Contains the delivery outcome of VPLMN-Specific URSP rules |  | VPLMNSpecificURSP |  |
| UePolicyParameters |  | 5.6.2.9 |  | Contains the service parameters used to guide the VPLMN-specific URSP rule determination. |  | VPLMNSpecificURSP |  |
| UePolicyRequest |  | 5.6.3.2 |  | Request for UE Policies |  |  |  |
| UePolicyTransferFailureCause |  | 5.6.4.1 |  | UE Policy Transfer Failure Cause |  | EnErrorHandling |  |
| UePolicyTransferFailureNotification |  | 5.6.2.7 |  | Information that the UE policy is failure to be transferred to the UE. |  |  |  |
| UrspEnforcementPduSession |  | 5.6.2.11 |  | Represents URSP rule enforcement information for a PDU session. |  | URSPEnforcement |  |

Table 5.6.1-2 specifies data types re-used by the Npcf_UEPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_UEPolicyControl service based interface.

Table 5.6.1-2: Npcf_UEPolicyControl re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccessType | 3GPP TS 29.571 [11] | Represents an Access Type. |  |
| Bytes | 3GPP TS 29.571 [11] | String with format "byte". |  |
| ChargingInformation | 3GPP TS 29.512 [31] | Contains the addresses, and if available, the instance ID and set ID, of the charging functions. | SLAMUP |
| ConfiguredSnssai | 3GPP TS 29.531 [34] | Contains the configured S-NSSAI and optionally mapped home S-NSSA. | SliceAwareANDSP, NssaiChange |
| CmState | 3GPP TS 29.518 [14] | Connectivity state of UE | ConnectivityStateChange |
| Dnn | 3GPP TS 29.571 [11] | Represents a DNN. | VPLMNSpecificURSP URSPEnforcement |
| Event | 3GPP TS 29.522 [41] | Subscription to notification about delivery of VPLMN specific URSP rule. | VPLMNSpecificURSP |
| Fqdn | 3GPP TS 29.571 [11] | FQDN |  |
| Gpsi | 3GPP TS 29.571 [11] | Generic Public Subscription Identifier |  |
| GroupId | 3GPP TS 29.571 [11] | Represents a UE Group Identifier |  |
| Guami | 3GPP TS 29.571 [11] | Globally Unique AMF Identifier |  |
| Ipv4Addr | 3GPP TS 29.571 [11] | Represents an Ipv4 address. |  |
| Ipv6Addr | 3GPP TS 29.571 [11] | Represents an Ipv6 address. |  |
| N1N2MessageTransferCause | 3GPP TS 29.518 [14] | Contains an error cause for an N1 or N2 message transfer. |  |
| N2InfoContent | 3GPP TS 29.518 [14] | Represents a transparent N2 information content to be relayed by AMF. | V2X, A2X, ProSe, Ranging_SL |
| NfInstanceId | 3GPP TS 29.571 [11] | Represents an NF instance identifier |  |
| PcEventNotification | 3GPP TS 29.523 [30] | Represents notification about UE Policy Delivery outcome | VPLMNSpecificURSP |
| PcfUeCallbackInfo | 3GPP TS 29.571 [11] | Contains the PCF for the UE callback information necessary for the PCF for the PDU session to send Establishment and Termination event. | URSPEnforcement |
| PduSessionInfo | 3GPP TS 29.571 [11] | Contains a DNN and SNSSAI combination | VPLMNSpecificURSP URSPEnforcement |
| PduSessionType | 3GPP TS 29.571 [11] | Contains the PDU Session Type | URSPEnforcement |
| Pei | 3GPP TS 29.571 [11] | Permanent Equipment Identifier |  |
| PlmnId | 3GPP TS 29.571 [11] | Represents a PLMN identifier. |  |
| PlmnIdNid | 3GPP TS 29.571 [11] | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |
| PresenceInfo | 3GPP TS 29.571 [11] | Presence reporting area information |  |
| ProblemDetails | 3GPP TS 29.571 [11] | Contains detailed information about an error response. |  |
| RatType | 3GPP TS 29.571 [11] | Represents a RAT type. |  |
| RedirectResponse | 3GPP TS 29.571 [11] | Contains redirection related information. | ES3XX |
| ServiceName | 3GPP TS 29.510 [13] | Name of the service instance. |  |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [11] | Indicates the satellite backhaul category or non-satellite backhaul. | EnSatBackhaulCategoryChg |
| Snssai | 3GPP TS 29.571 [11] | Represents an S-NSSAI | SliceAwareANDSP |
| SscMode | 3GPP TS 29.571 [11] | Service and session continuity mode. | URSPEnforcement |
| Supi | 3GPP TS 29.571 [11] | Subscription Permanent Identifier |  |
| SupportedFeatures | 3GPP TS 29.571 [11] | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |
| TimeZone | 3GPP TS 29.571 [11] | Represents a time zone. |  |
| Uinteger | 3GPP TS 29.571 [11] | Unsigned integer. |  |
| Uri | 3GPP TS 29.571 [11] | Represents a URI. |  |
| UrspEnforcementInfo | 3GPP TS 29.512 [31] | URSP rule enforcement information as received from the UE. | URSPEnforcement |
| UrspRuleRequest | 3GPP TS 29.522 [41] | URSP rule guidance information | VPLMNSpecificURSP |
| UserLocation | 3GPP TS 29.571 [11] | Contains User Location information. |  |

# * * * * Next changes * * * *

#### 5.6.2.11	Type UrspEnforcementPduSession

Table 5.6.2.11-1: Definition of type UrspEnforcementPduSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| urspEnfInfo | UrspEnforcementInfo | M | 1 | Represents UE provided information about the enforced URSP rule(s) in one PDU session. |  |
| sscMode | SscMode | C | 0..1 | SSC Mode of the PDU session. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| ueReqDnn | Dnn | C | 0..1 | UE requested DNN. It shall be provided when URSP rule enforcement information is provided for the first time, if available and different from the selected DNN. |  |
| ueReqPduSessionType | PduSessionType | C | 0..1 | UE requested PDU session Type. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| dnn | Dnn | C | 0..1 | Selected DNN. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| snssai | Snssai | C | 0..1 | S-NSSAI of the HPLMN. It shall be provided when URSP rule enforcement information is provided for the first time. |  |

# * * * * End of Changes * * * *