# Liaison Statement

| Liaison Statement Title: | CVD-2024-0096 – SNI5GECT: A Practical Approach to Inject aNRchy into 5G NR |
| --- | --- |
| Security Classification: | Non-confidential |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| GSMA CVD PoE |  | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| CVD-2024-0096 | 19/08/25 | Roger Brown |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA CVD PoE | None | GSMALiaisons@gsma.com |

| Action Required by Recipient |  |
| --- | --- |
| External Recipients: | To: 3GPP SA3, 3GPP CT1 |
| Internal Recipients: |  |

Introduction

The GSMA Coordinated Vulnerability Disclosure program received a submission CVD-2024-0096 "SNI5GECT: A Practical Approach to Inject aNRchy into 5G NR" (paper attached), from researchers at Singapore University of Technology and Design. The case was recently presented at USENIX Security Symposium and the research references impacted 3GPP specifications. We would like 3GPP to assess the submission and respond accordingly.

Below is the GSMA CVD assessment and industry recommendations:

Researcher’s Claim

The research highlighted a potential new security concern during the registration process of certain User Equipment (UE). Under specific conditions, an attacker within radio range could replay old authentication messages, which in some tested devices appears to result in a prolonged loss of connectivity to the specific 5G cell or cells targeted by the attacker. If no other legitimate untargeted 5G cells are available to the UE, then the UE may select any available legitimate 4G cells to maintain connectivity. The attack is on the NAS protocol.

The researchers claim that no false base station (FBS) is required to perform the attack. Instead of using a relay function to place the attacker between the UE and the base station, as has been done previously, the researchers implement the message injection/modifications by modifying the signal mid-air. However, as explained in the following, the attack is independent of whether it is implemented using a relay-type of FBS or an FBS of the type proposed by the researchers.

Researcher’s Attack

The attack flow is the following:

Attacker sniffs an AUTHENTICATION REQUEST from the air for the victim

Attacker attracts the victim to its 5G FBS

Attacker sends to the victim the stored, old, invalid AUTHENTICATION REQUEST

Any answer the attacker receives from the victim they keep responding with the old, invalid AUTHENTICATION REQUEST

According to the researchers after some back and forth the UEs they tested completely ignored legitimate 5G cells around them, and even when they toggled airplane mode on-off the connection would not get recovered.

Panel’s assessment of the attack

This appears to be a specific UE deviation from 3GPP standards. The UEs tested seem to keep the cell barred for longer time than specified in 3GPP.

3GPP TS 38.304 – Section 5.3.1 – states that the UE shall exclude the barred cell for cell selection/reselection for 300 seconds (5 mins) while the researchers state they observe from 5 minutes up to a couple of hours of non-5G connectivity:

For a UE that adheres to the standard, the effect would be similar to an attacker preventing MIB / SIB1 acquisition, or MIB / SIB1 overshadowing. Therefore, the standardized behaviour is consistent with the risk assessment of 3GPP.

Summary & Actions

In summary, the researchers have potentially discovered a UE manufacturer deviating from 3GPP standards. To stop the outcome the researchers have found, GSMA CVD recommendation for UE manufacturers is to follow the cell barring section of 3GPP TS 38.304.

GSMA would like to ask 3GPP to take this information into account but has no particular request for actions.