|  | RILTE65_108 LS on multi-device requirements |
| --- | --- |
| Meeting Information |  |
| Meeting Name and Number | RILTE #64 |
| Meeting Date | 2018-04-26 |
| Meeting Location | Nürnberg |
| Document Information |  |
| Document Author | Jörgen Axell (Ericsson) |
| Additional Contributors | [List of contributors] |
| Document Creation Date | 2018-03-28 |
| Document Status | [This document is for] |
| Security Classification | Non-confidential |
| Document Summary |  |
| [Summary] |  |

| Version History |  |  |
| --- | --- | --- |
| Date | Version | Author / Comments |
| 2018-03-28 | 0 | First version |
|  | 1 |  |
|  | 2 |  |

© GSMA © 2018. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

# Liaison Statement

| Liaison Statement Title: | LS on multi-device requirements |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| RILTE #64 | 2018-04-26 | Nuremberg, Germany |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| RILTE #65 Doc 108 | 2018-03-28 | Jorgen Axell |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG RILTE |  | jorgen.axell@ericsson.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | GFRG |
| External Recipients: | 3GPP SA1 |

# Introduction

GSMA NG RILTE has worked on solutions for multi-device support based on requirements from GSMA GFRG (Global Functional Requirements Group).

# Discussion

NG RILTE has concluded that not all requirements from GFRG are supported in 3GPP specifications. Related to the attached Multi Device Voice and Video (MDV2) requirements RILTE would like to highlight the following requirements, which are considered unspecified by 3GPP, but fall within 3GPP responsibility and have an impact on the UNI.

R17-2-8	If a missed call notification has been cleared on one device (e.g. because the user has seen the missed call notification), then the missed call notification shall be immediately cleared on all reachable federated devices.

R17-2-9	If a device is not reachable, then updates on that device are not performed. When a device comes online again, all previous notifications shall be updated.

R17-4-5	Call logs shall provide the following:

- R17-4-5-1	Other party involved in call.

- R17-4-5-2	Incoming or outgoing call.

- R17-4-5-3	Date and timestamp when the call was established.

- R17-4-5-4	Duration.

- R17-4-5-5	Enriched Calling media according to Enriched Calling requirements.

- R17-4-5-6	Some of the call log information may be available in a “Detailed View” only.

- R17-4-5-7	Filter options for display of incoming / outgoing / missed calls / accepted calls / rejected calls.

- R17-4-5-8	Voice call / video call differentiator.

RILTE comment: RILTE believes this can be achieved by using the call log defined in OMA-CPM RESTful API, but a format for the data is needed to be specified in stage 3.

Another set of requirements relate to the possibility for a user to use other identities. GSMA RILTE believes that for identities within one subscription 3GPP provides sufficient support. But it is also required that identities from different subscriptions can be shared. The following requirement seem to have no support in 3GPP:

R1-1-1 It shall be possible to configure the service that any incoming (voice or video) calls either directed to the MSISDN of the SIM in the Mobile Device, i.e., the native MSISDN / phone number / identity, or to any added phone number (fixed or mobile) can be received on the Mobile Device.

RILTE comment: Currently only numbers within same subscription are covered by the 3GPP specifications. The requirement is that also numbers within different subscription and numbers within different networks are covered.

R1-2-1 When making a (voice or video) call, the user shall be able to choose from the available phone numbers the calling party phone number that shall be presented to the called party and in call logs of both calling and called party’s devices.

R1-3-1-1 If a phone number, that belongs to another device, was added to a Mobile Device, it shall not automatically determine the reciprocal addition.

RILTE Comment: This means that if user A is allowed to use identity B, user B is not automatically allowed to use identity A. Current 3GPP specifications do not cover this scenario.

# Action to SA1:

NG RILTE asks SA1 to study the requirements and if there are missing service requirements in 3GPP specifications, NG RILTE asks SA1 to kindly specify the missing requirements.

- Next meetings

RILTE #66	21-22 June, 2018; Location: GSMA, London