- 3GPP TSG-RAN WG4 Meeting #79 AH	                                 R4-79AH-0227

- Hong Kong, China, 28 - 30 June, 2016

Title:	LS on CSI measurement ambiguity in LAA

Release:	Release 13

Work Item:	LTE_LAA-Perf

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

According to RAN1 specification TS 36.213 section 7.2.1 in Rel-13, if a UE receives aperiodic CSI report request triggering Ny CSI reports in a subframe, the UE is required to update CSI for Ny CSI processes from the CSI processes corresponding to all the triggered CSI reports. Ny is given by maxNumberUpdatedCSI-Proc-r13.

1 . In LAA with periodic CSI reporting configuration, when periodic CSI report is configured in LAA carriers and Ny aperiodic CSI reports are triggered in licensed carriers with different report timing, the UE could be required to update more than Ny CSIs in certain subframe since CSI measurement subframe both LAA and licensed carriers could be overlapped in the subframe cause by LAA periodic CSI measurement behaviour. The same issue also arises in case of cross-carrier aperiodic CSI reporting. For this case, a UE may not guarantee valid CSI reporting for some CSI processes.

2. RAN4 would like to know if RAN1 can clarify whether following sentence could be applied in LAA CSI reporting case or not.

“For a non-BL/CE UE, if there is no valid downlink or no valid special subframe for the CSI reference resource in a serving cell, CSI reporting is omitted for the serving cell in uplink subframe n.” (in RAN1 specification TS 36.213 for CSI reporting)

Without clarification for this sentence, RAN4 is not clear about UE behaviour when cross carrier aperiodic CSI reporting is triggered in subframe n and the subframe is not valid subframe.

2. Actions:

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take the above issue into consideration for the description of periodic and cross-carrier aperiodic CSI report for LAA carriers.

3. Date of Next TSG-RAN4 Meetings:

TSG RAN WG4 Meeting #80BIS	October 10-14, 2016		     Ljubljana, Slovenia

TSG RAN WG4 Meeting #81	November 14-18, 2016		Reno, Nevada, US