3GPP TSG-RAN WG4 Meeting # 79AH	R4-79AH-0250

Hong Kong, China, 28 – 30 June, 2016

Agenda Item:	2.3

Source: 		Ericsson

Title: 	List for RRM Tests for NB-IOT

Document for:	Approval

# Introduction

The core work of RRM requrements for NB-IOT was completed at RAN4#79 meeting and all CRs were approved. The performance work is currently on-going. This includes developing of test cases to verify the corresponding core requirements.

This paper contains list of RRM test cases that RAN4 needs to introduce to verify the new NB-IOT core requirements.

# List of test cases

Table 1 RRM Test Cases for Cat-NB1 UEs

| No | Test scenario | Phase | Coverage level: serving cell | Coverage level: target cell | In-band (IB) | Standalone (SA) | Guardband (GB) | Volunteer Company | Issues for investigation | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell Re-selection Tests |  |  |  |  |  |  |  |  |  |  |
| 1 | Intra-frequency cell reselection under NC | I | Normal | Normal | Y | TBD | TBD | Ericsson | Tests in SA and GB TBD until next meeting. Unnecessary test should be avoided. Need to check if UE supports all modes. If UE supports all modes then testing in most stringent case is sufficient. | Need to check accuracy for intra- and inter-frequency and based on accuracy difference decide whether to have test # 2 or not. |
| 2 | Intra-frequency cell reselection under EC | II | Enhanced | Enhanced | Y | TBD | TBD | Ericsson |  |  |
| 3 | Inter-frequency cell reselection under EC | II | Enhanced | Enhanced | Y | TBD | TBD | Ericsson |  |  |
| RRC Re-establishment Tests |  |  |  |  |  |  |  |  |  |  |
| 4 | Intra-frequency RRC Re-establishment under NC | I | Normal | Normal | Y | TBD | TBD | Huawei | Tests in SA and GB TBD until next meeting. Unnecessary test should be avoided. Need to check if UE supports all modes. If UE supports all modes then testing in most stringent case is sufficient. |  |
| 5 | Inter-frequency RRC Re-establishment under EC | II | Enhanced | Enhanced | Y | TBD | TBD | Huawei |  |  |
|  | Random Access Tests |  |  |  |  |  |  |  |  |  |
| 6 | Random access under NC | I | Normal | N/A | Y | TBD | TBD | Nokia | Tests in SA and GB TBD until next meeting. Unnecessary test should be avoided. Need to check if UE supports all modes. If UE supports all modes then testing in most stringent case is sufficient. | UE is signaled 2 NRSRP threshold; but only one threshold is used in the test. Different PRACH configurations in NC and EC |
| 7 | Random access under EC | II | Enhanced | N/A | Y | TBD | TBD |  |  |  |
| Timing Tests |  |  |  |  |  |  |  |  |  |  |
| 8 | UE Transmit timing accuracy | I | Enhanced | N/A | Y | TBD | TBD | DOCOMO | Tests in SA and GB TBD until next meeting. Unnecessary test should be avoided. Need to check if UE supports all modes. If UE supports all modes then testing in most stringent case is sufficient. | Test with repetition in DL and UL. Use NPUSCH for timing. Repetition is FFS. |
| 9 | Timing advance adjustment | I | Enhanced | N/A | N | Y | N | Ericsson |  | Test with repetition in DL and UL. Use NPUSCH for timing. Repetition is FFS. |
| Radio Link Monitoring Tests |  |  |  |  |  |  |  |  |  |  |
| 10 | RLM: insync without DRX under NC | I | Normal | N/A | N | Y | N | Qualcomm |  | Repetition. Rmax is FFS |
| 11 | RLM: insync with DRX under NC | I | Normal | N/A | Y | N | N | Ericsson |  | Repetition. Rmax is FFS. |
| 12 | RLM: insync without DRX under EC | II | Enhanced | N/A | N | N | Y | Qualcomm |  | Repetition. Rmax is FFS |
| 13 | RLM: insync with DRX under EC | II | Enhanced | N/A | Y | N | N | Ericsson | This test is FFS. Need to check difference in requirements | Repetition. Rmax is FFS. |
| 14 | RLM: out of sync without DRX under NC | I | Normal | N/A | N | Y | N | Nokia |  | Repetition. Rmax is FFS |
| 15 | RLM: out of sync with DRX under NC | I | Normal | N/A | Y | N | N | Huawei |  | Repetition. Rmax is FFS. |
| 16 | RLM: out of sync without DRX under EC | II | Enhanced | N/A | N | N | Y | Nokia |  | Repetition. Rmax is FFS |
| 17 | RLM: out of sync with DRX under EC | II | Enhanced | N/A | Y | N | N | Huawei | This test is FFS. Need to check difference in requirements | Repetition. Rmax is FFS. |

Table 2 Basic Test configurations for the different deployment modes

| Test case # | In-band | Standalone | Guardband |
| --- | --- | --- | --- |
| 1-3 | Number of NB-IOT cells = 2 nCell1, nCell2 nCell1 and nCell2 are on the same NB-IoT carrier (nF1) Number of E-UTRA cells = 2 eCell1, eCell2 eCell1 and eCell2 are on the same E-UTRA carrier (eF1) eCell1 BW = 10 MHz eCell2 BW = 10 MHz In-band operation of: nCell1 within eCell1 and nCell2 within eCell2 New PRACH configuration New nRMC for SIBs in anchor carrier of the target cell and nOCNG Anchor carrier is in RB number # 30 Other requirements should be similar to existing test configurations in corresponding UE category M1 test case. | TBD | TBD |
| 4-5 | Number of NB-IOT cells = 2 nCell1, nCell2 nCell1 and nCell2 are on the same NB-IoT carrier (nF1) Number of E-UTRA cells = 2 eCell1, eCell2 eCell1 and eCell2 are on the same E-UTRA carrier (eF1) eCell1 BW = 10 MHz eCell2 BW = 10 MHz In-band operation of: nCell1 within eCell1 and nCell2 within eCell2 DL subframe 0 or 5 per frame; randomly selected by test system. New nRMC and nOCNG Other requirements should be similar to existing test configurations in corresponding UE category M1 test case. | TBD | TBD |
| 6-7 | Number of NB-IOT cells = 1 nCell1 nCell1 is on the same NB-IoT carrier (nF1) Number of E-UTRA cells = 1 eCell1 eCell1 is on the same E-UTRA carrier (eF1) eCell1 BW = 10 MHz In-band operation of: nCell1 within eCell1 DL subframe 0 or 5 per frame; randomly selected by test system. New nRMC and nOCNG Other requirements should be similar to existing test configurations in corresponding UE category M1 test case. | TBD | TBD |
| 8 | Number of NB-IOT cells = 1 nCell1 nCell1 is on the same NB-IoT carrier (nF1) Number of E-UTRA cells = 1 eCell1 eCell1 is on the same E-UTRA carrier (eF1) eCell1 BW = 10 MHz In-band operation of: nCell1 within eCell1 UE needs to be configured with certain number of repetitions in both uplink and downlink DL subframe 0 or 5 per frame; randomly selected by test system. New nRMC and nOCNG Other requirements should be similar to existing test configurations in corresponding UE category M1 test case. | TBD | TBD |
| 9 | N/A | TBD | N/A |
| 10-17 | Tested in deployment modes according to Table 1. Test configurations are as follows: Number of NB-IOT cells = 1 nCell1 nCell1 is on the same NB-IoT carrier (nF1) NB-IoT cell’s DL subframe 5 or 9 per frame is randomly selected by test system. Number of E-UTRA cells = 1 eCell1 eCell1 is on the same E-UTRA carrier (eF1) eCell1 BW = 10 MHz In-band operation of: nCell1 within eCell1 DL subframe 0 or 5 per frame; randomly selected by test system. New nRMC and nOCNG Other requirements should be similar to existing test configurations in corresponding UE category M1 test case. | Tested in deployment modes according to Table 1. Test configurations are TBD. | Tested in deployment modes according to Table 1. Test configurations are TBD. |

# Summary

We have in this contribution provided the list of test cases that is necessary to verify the core requirements of UE category NB1 UEs.

# References

[1]	R4-164457, “Modification on NB-IoT RRC re-establishment requirement”, Huawei, Hisilicon

[2] R4-164481, “Modification on NB-IoT random access requirement”, Huawei, Hisilicon

[3]	R4-164530, “Uplink transmit timing adjustments in HD-FDD operation”, Ericsson

[4]	R4-164901, “CR on Radio Link Monitoring for NB-IoT”, Qualcomm Incorporated