3GPP TSG-RAN WG4 Meeting #79AH															   R4-79AH-0271

Hongkong, P.R.C, 28 – 30 Jun, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev |  | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: RMCs for NPDSCH and NPDCCH for in-band for NB-IoT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-06-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | NB-IoT is introduced in Rel-13. This means that RMCs of NPDCCH and NPDSCH for NB-IoT are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce NPDCCH and NPDSCH RMCs for in-band operation UE category NB1 test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPDCCH and NPDSCH RMCs for in-band operation UE category NB1 RRM test cases are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section: A.3.1.5 and A.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change >>

### A.3.1.5	NPDSCH Reference Channel for UE category NB1

#### A.3.1.5.1	A.3.1.5.1 HD-FDD in-band operation in normal coverage

Table A.3.1.5.1-1: NPDSCH Reference Channel for UE category NB1 for in-band operation in normal coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.14 HD-FDD | R.15 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 2 | Note 2 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 40 | 40 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 | 1 |
| For Sub-Frame 4, 9 |  | Note 4 | Note 4 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 5 | Note 5 |
| NPDCCH start symbol | symbols | 4th | 4th |
| Cell ID | - | Note 6 | Note 6 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: 6 when nfmod 2 = 0, otherwise 8. nf is specified in TS36.211 [16]. Note 3: 0 when nfmod 2 = 0, otherwise 40. Note 4: 0 when nfmod 2 = 0, otherwise 1. Note 5: Maximum number of repetitions shall depend upon the test configuration. Note 6: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.5.2 HD-FDD in-band operation in enhanced coverage

Table A.3.1.5.2-1: NPDSCH Reference Channel for UE category NB1 for in-band operation in enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.16 HD-FDD | R.17 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 2 | Note 2 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 40 | 40 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 | 1 |
| For Sub-Frame 4, 9 |  | Note 4 | Note 4 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 5 | Note 5 |
| NPDCCH start symbol | symbols | 4th | 4th |
| Cell ID | - | Note 6 | Note 6 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: 6 when nfmod 2 = 0, otherwise 8. nf is specified in TS36.211 [16]. Note 3: 0 when nfmod 2 = 0, otherwise 40. Note 4: 0 when nfmod 2 = 0, otherwise 1. Note 5: Maximum number of repetitions shall depend upon the test configuration. Note 6: Cell ID shall depend upon the test configuration. |  |  |  |

## << End of Change >>

## << Start of Change >>

### A.3.1.6	PDCCH Reference Channel for UE category NB1

#### A.3.1.6.1	HD-FDD in-band operation in normal coverage

Table A.3.1.6.1-1: NPDCCH Reference Channel for UE category NB1 for in-band operation in normal coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.26 HD-FDD | R.27 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | N1 | N1 |
| Aggregation level | NCCE | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 |
| NPDCCH start symbol | symbols | 4th | 4th |
| Payload (without CRC) | Bits | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Maximum number of repetitions shall depend upon the test configuration. Note 3: Payload size shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.6.2	HD-FDD in-band operation in enhanced coverage

Table A.3.1.6.2-1: NPDCCH Reference Channel for UE category NB1 for in-band operation in enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.28 HD-FDD | R.29 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | N1 | N1 |
| Aggregation level | NCCE | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 |
| NPDCCH start symbol | symbols | 4th | 4th |
| Payload (without CRC) | Bits | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Maximum number of repetitions shall depend upon the test configuration. Note 3: Payload size shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. |  |  |  |

## << End of Change >>