3GPP TSG-RAN WG4 Meeting #79AH 	R4-79AH-0172

- Hong Kong, P.R.China, 28 - 30 June 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | CRNum | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for introducing LAA PDSCH demodulation performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-6-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | PDSCH demodulation performance requirements for LAA have not been introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the PDSCH demodulation performance requirements for LAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH demodulation performance requirements for LAA will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12(New), 8.12.1(New), 8.12.2(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-1 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

- 8.12	Demodulation of LAA SCell with frame structure Type-3

- 8.12.1	FDD PCell (FDD single carrier)

- The parameters specified in Table 8.12.1-1 are valid for all FDD CCs unless otherwise stated.

Table 8.12.1-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Inter-TTI Distance |  |  | 1 |
| Number of HARQ processes per component carrier | Processes |  | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols |  | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| Cyclic Prefix |  |  | Normal |
| Cell_ID |  |  | 0 |
| dmtc-PeriodOffset |  |  | [ms80] |
| ds-OccasionDuration |  |  | [1] |
| Cross carrier scheduling |  |  | Not configured |

- 8.12.1.1	Minimum Requirement Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (CA with LAA SCell)

For CA with 1 LAA SCell, the requirements are specified in Table 8.12.1.1-4, with the addition of the parameters in Table 8.12.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.12.1.1-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in X.X |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.12.1.1-2: Single carrier performance for licensed PCell and SCell with multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.12.1.1-3: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting only full subframe

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.1-3a: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting initial partial and full subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.1-3b: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting full and ending subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.1-3c: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting initial partial, full and ending partial subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.1-4: Minimum performance (FRC) based on single carrier performance for CA with 1 LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | Licensed CC | CC as LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.12.1.1-2 for licensed CC per CC, and one of Table 8.12.1.1-3, Table 8.12.1.1-3a, Table 8.12.1.1-3b, Table 8.12.1.1-3c per CC according to the UE capability | FFS |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in TBD. Note 2: Apply the per-CC requirements defined in 8.12.1.1-2 for licensed CC and apply the per-CC requirements defined in 8.12.1.1-3 for LAA CC. |  |  |  |  |  |

- 8.12.1.3	Dual-Layer Spatial Multiplexing (CA with LAA SCell)

For CA with 1 LAA SCell, the requirements for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C are specified in Table 8.12.1.3-5, with the addition of the parameters in Table 8.12.1.3-1 and Table 8.12.1.3-2. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power for CA with LAA SCell.

Table 8.12.1.3-1: Test Parameters for Large Delay CDD (FRC) for licensed carrier with 2Rx Antenna ports

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | TM3 |
| Subframe configuration |  |  | Non-MBSFN |
| NOTE 1: . NOTE 2: PUCCH format 3 is used to feedback ACK/NACK. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.12.1.3-2: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) for CA with LAA SCell and multiple CSI-RS configurations

| - Parameter |  | - Unit | - Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 4 |
|  |  | dB | 4 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | [5/2] |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | [3 / [0010000000000000[ |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 9 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in X.X |
| timing error relative of LAA SCell to PCell |  | μs | TBD (Note 3) |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | [200]+Δfi (Note 3) |
| DRS periodicity |  | Ms | 80 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: Δfi is the difference of the nominal centre frequencies between the i-th LAA SCell and PCell. The timing error and frequency offset are applicable only to UE under test which does not support inter-band CA. |  |  |  |

Table 8.12.1.3-3: Single carrier performance Large Delay CDD (FRC) for licensed carrier with 2 RX Antenna Ports, TM3, and multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3MHz | R.11-6 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10 MHz | R.11 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.12.1.3-4: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting only full subframe

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.3-4a: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting initial partial and full subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.3-4b: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting full and ending subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.3-4c: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting initial partial, full and ending partial subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 FDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.1.3-5: Minimum performance (FRC) based on single carrier performance for CA with 1 LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | Licensed CC | CC as LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.12.1.3-3 for licensed CC per CC, and one of Table 8.12.1.3-4, Table 8.12.1.3-4a, Table 8.12.1.3-4b, Table 8.12.1.3-4c per CC according to UE capability | FFS |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in TBD. Note 2: Apply the per-CC requirements defined in Table 8.12.1.3-3 for licensed CC and apply the per-CC requirements defined in Table 8.12.1.3-4 for LAA CC. |  |  |  |  |  |

- 8.12.2	TDD PCell (TDD single carrier)

The parameters specified in Table 8.12.2-1 are valid for all TDD CCs unless otherwise stated.

Table 8.12.2-1: Common Test Parameters (TDD)

| Parameter | Unit |  | Value |  |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |
| Special subframe configuration (Note 2) |  |  |  | 4 |
| Cyclic prefix |  |  |  | Normal |
| Cell ID |  |  |  | 0 |
| Inter-TTI Distance |  |  |  | 1 |
| Number of HARQ processes per component carrier |  | Processes |  | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbols |  | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| dmtc-PeriodOffset |  |  |  | [ms80] |
| ds-OccasionDuration |  |  |  | [1] |
| Cross carrier scheduling |  |  |  | Not configured |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |

- 8.12.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (CA with LAA SCell)

For CA with 1 LAA SCell, the requirements are specified in Table 8.12.2.1-4, with the addition of the parameters in Table 8.12.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.12.2.1-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in X.X |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 5: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 6: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.12.2.1-2: Single carrier performance for licensed PCell and SCell with multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.12.2.1-3: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting only full subframe for UE supporting initial partial full and ending partial subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | R.14-X TDD | OP. 1 TDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.1-3a: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting only full subframe for UE supporting initial partial and full subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | R.14-X TDD | OP. 1 TDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.1-3b: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting only full subframe for UE supporting full and ending subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | R.14-X TDD | OP. 1 TDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.1-3c: Single carrier performance for LAA SCell with multiple CA configurations for UE supporting only full subframe for UE supporting initial partial, full and ending partial subframes

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | R.14-X TDD | OP. 1 TDD | EVA5 | 4x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.1-4: Minimum performance (FRC) based on single carrier performance for CA with 1 LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | Licensed CC | CC as LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.12.2.1-2 for licensed CC per CC, and one of Table 8.12.2.1-3, Table 8.12.2.1-3a, Table 8.12.2.1-3b, Table 8.12.2.1-3c per CC | FFS |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in TBD. Note 2: Apply the per-CC requirements defined in 8.12.2.1-2 for licensed CC and apply the per-CC requirements defined in 8.12.2.1-3 for LAA CC. |  |  |  |  |  |

- 8.12.2.3	Dual-Layer Spatial Multiplexing (CA with LAA SCell)

For CA with 1 LAA SCell, the requirements for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C are specified in Table 8.12.2.3-5, with the addition of the parameters in Table 8.12.2.3-1 and Table 8.12.2.3-2. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power for CA with LAA SCell.

Table 8.12.2.3-1: Test Parameters for Large Delay CDD (FRC) for licensed carrier with 2Rx Antenna ports

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | TM3 |
| Subframe configuration |  |  | Non-MBSFN |
| NOTE 1: . NOTE 2: PUCCH format 3 is used to feedback ACK/NACK. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.12.2.3-2: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) for CA with LAA SCell and multiple CSI-RS configurations

| - Parameter |  | - Unit | - Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 4 |
|  |  | dB | 4 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | [5/4] |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4/ 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 9 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in X.X |
| ming error relative of LAA SCell to PCell |  | μs | TBD (Note 3) |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | [200]+Δfi (Note 3) |
| DRS periodicity |  | Ms | 80 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: Δfi is the difference of the nominal centre frequencies between the i-th LAA SCell and PCell. The timing error and frequency offset are applicable only to UE under test which does not support inter-band CA. |  |  |  |

Table 8.12.2.3-3: Single carrier performance Large Delay CDD (FRC) for licensed carrier with 2 RX Antenna Ports, TM3, and multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3MHz | R.11-6 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10 MHz | R.11-8 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.12.2.3-4: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting only full subframe

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 TDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.3-4a: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting initial partial and full subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 TDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.3-4b: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting full and ending subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 TDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.3-4c: Single carrier performance for CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and for LAA SCell with multiple CA configurations for UE supporting initial partial, full and ending partial subframes

| Bandwidth and MCS | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | TBD | OP. 1 TDD | EVA5 | 2x2 Low | [70] | TBD |
| Note 1: Fraction of nominal maximum throughput is defined per scheduled transmission occasions for PDSCH. |  |  |  |  |  |  |

Table 8.12.2.3-5: Minimum performance (FRC) based on single carrier performance for CA with 1 LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | Licensed CC | CC as LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.12.2.3-3 for licensed CC per CC, and one of Table 8.12.2.3-4, Table 8.12.2.3-4a, Table 8.12.2.3-4b, Table 8.12.2.3-4c for LAA SCell per CC according to UE capability | FFS |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in TBD. Note 2: Apply the per-CC requirements defined in Table 8.12.2.3-3 for licensed CC and apply the per-CC requirements defined in Table 8.12.2.3-4 for LAA CC. |  |  |  |  |  |

<< End of Change >>