3GPP TSG-SA WG3 Meeting #94 Ad-Hoc 	S3-190896

- Stockholm (Sweden), 11-15 March 2019										revision of S3-190775

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to the Test Case of transport layer protection to cover Mutual Authentication between Network Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  |  | Date: |  | 2019-03-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The new version of living document S3-190561 was created at SA3#94 with two new sub-clauses in TS 33.117 for capturing general SBA/SBI aspects, one of which is “4.2.2.1 Security functional requirements derived from 3GPP specifications and related test cases – general SBA/SBI aspects”. This paper proposes some updates to the existing test case “protection at transport layer” in the living document to cover mutual authentication between NFs. The rationale for adding this test case: Interoperability testing on mutual authtication between NFs (by using TLS protocol) can successfully get passed if both NFs use the same TLS profile. However, if the applied TLS profile is not compliant with the requirements defined in TS 33.501, a weak protocol version or cipher suite with known vulnerabilities may be used which cannot provide sufficient security assurance for 5G NFs. The purpose of the test case is to verify that mutual authentication between NFs within the same PLMN does not use TLS profiles restricted or forbidden in TS 33.310 Annex E or RFC 7540. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added updates to “4.2.2.1.2 Protection at the transport layer”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NF-specific SCAS documents will not find reference in TS 33.117 on the general requirement and test case of mutual authentication between network functions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

##### 4.2.2.1.2	Protection at the transport layer

Requirement Name: Protection at the transport layer

Requirement Description:

"NF Service Request and Response procedure shall support mutual authentication between NF consumer and NF producer" as specified in TS 33.501 [5], clause 5.9.2.1;

"All network functions shall support TLS. Network functions shall support both server-side and client-side certificates.

The TLS profile shall follow the profile given in Annex E of TS 33.310 with the restriction that it shall be compliant with the profile given by HTTP/2 as defined in RFC 7540. "

as specified in TS 33.501 [5], clause 13.1.

"Authentication between network functions within one PLMN shall use one of the following methods:

-	If the PLMN uses protection at the transport layer as described in clause 13.1, authentication provided by the transport layer protection solution shall be used for authentication between NFs."

as specified in TS 33.501 [5], clause 13.3.2.

Security Objective references: tba

Editor’s Note: Security objective and threat need to be added with reference to TR 33.926

Test case:

Test Name: TC_PROTECT_TRANSPORT_LAYER

Purpose:

Verify that TLS protocol for NF mutual authentication and NF transport layer protection is implemented in the network products based on the profile required.

Procedure and execution steps:

Pre-Conditions:

Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

A peer implementing the TLS protocol configured by the vendor shall be available.

The tester shall base the tests on the profile defined by 3GPP in Annex E of TS 33.310 with the restriction that it shall be compliant with the profile given by HTTP/2 as defined in RFC 7540.

Execution Steps

1.	The tester shall check that compliance with the TLS profile can be inferred from detailed provisions in the network product documentation.

2.	The tester shall establish a secure connection between the network product under test and the peer and verify that all TLS protocol versions and combinations of cryptographic algorithms that are mandated by the TLS profile are supported by the network product under test.

3.	The tester shall try to establish a secure connection between the network product under test and the peer and verify that this is not possible when the peer only offers a feature, including protocol version and combination of cryptographic algorithms, that is forbidden by the TLS profile.

Expected Results:

- The network product under test and the peer establish TLS if the TLS profiles used by the peer are compliant with the profile requirements in TS 33.310 Annex E and RFC 7540.

- The network product under test and the peer fail to establish TLS if the TLS profiles used by the peer are forbidden in TS 33.310 Annex E or RFC 7540.

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text. Save the logs and the communication flow in a .pcap file.

*************** End of the Change ****************