- 3GPP TSG SA WG3 (Security) Meeting #94ad-hoc	S3-190905

- 11-15 March 2019, Stockholm (Sweden)	revision of S3-190751

- Source:	Ericsson

- Title:	New solution: WLAN measurements from UEs

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is requested that proposed changes to TR 33.814 are approved.

# 2	References

[1]	N/A

# 3	Rationale

Location services (LCS) in 3GPP system sufficiently address privacy of UE's user by providing mechanisms for the user's consent processing, and confidentiality/integrity protection of messages. However, there is no way instruct the UEs to "not" report everything they see in the neighbourhood. Doing so is necessary to respect the privacy of neighbouring UE's users, and to avoid potential issues like "mass surveillance". Nevertheless, it should also be considered that one of the main use cases of LCS is locating UEs during emergency and failure to locate the UEs could prove fatal to the health or life of the UEs' user. Depending upon regulations, privacy of neighbouring WLAN devices may be considered comparatively less important than saving life of the user. Depending upon regulations, it might even be the obligation of neighbouring WLAN devices to assist in locating the concerned UEs. It could also be the case that there are some users who have opted-in so that their WLAN devices could be reported to the network.

To strike a balance between "collect everything" and "collect nothing", this solution proposes a supervision mechanism so that the network can carefully limit the information reported by the UE and tweak the limit according to regulation or network policy.

It could be helpful to look at MDT (minimization of drive tests), i.e., "WLAN-NameList" in 3GPP TS 36.331.

# 4	Detailed proposal

*** BEGIN CHANGES ***

- 6.Y	Solution #Y: Supervised WLAN measurements from UEs

- 6.Y.1	Introduction

This solution addresses following key issues:

(1)	Key issue #3: Security and privacy in WLAN positioning.

Location services (LCS) in 3GPP system include the WLAN positioning method in which the UE position is estimated with the knowledge of geographical coordinates of "reference" WLAN access points in serving network. The 3GPP TS 38.305 [5] is referred for further details. They sufficiently address privacy of UE's user by providing mechanisms for the user's consent processing, and confidentiality/integrity protection of messages. However, there is no way instruct the UE to "not" report everything they see in the neighbourhood. Doing so is necessary to respect the privacy of neighbouring UE's users, and to avoid potential issues like "mass surveillance". Nevertheless, it should also be considered that one of the main use cases of LCS is locating UE during emergency and failure to locate the UE could prove fatal to the health or life of the UE's user. Depending upon regulations, privacy of neighbouring WLAN devices may be considered comparatively less important than saving life of the user. Depending upon regulations, it might even be the obligation of neighbouring WLAN devices to assist in locating the concerned UE. It could also be the case that there are some users who have opted-in so that their WLAN devices could be reported to the network.

To strike a balance between "collect everything" and "collect nothing", this solution proposes a supervision mechanism so that the network can carefully limit the information reported by the UE and tweak the limit according to regulation or network policy. Mind that in MDT (minimization of drive tests) the network can configure the UE with a "whitelist" so that UE only measures the neighbouring WLAN devices whose "names" are included in the whitelist. See "WLAN-NameList" in 3GPP TS 36.331.

There are 3 positioning modes that are supported in WLAN positioning method, namely (a) standalone, (b) UE-based, and (c) UE-assisted. Among those, it is only in the UE-assisted positioning mode (c) that the UE sends measurements to the network and therefore it is the only mode that needs a supervision mechanism. A short summary follows:

-	in the standalone positioning mode (a), the UE itself does the computation of location estimate. WLAN measurements around the UE are not sent from UE to the network.

-	in the UE-based positioning mode (b), it is again the UE that itself does the computation of location estimate. Difference from the standalone positioning mode being that the UE uses assistance data from the network. Nevertheless, WLAN measurements around the UE are not sent from UE to the network.

-	in the UE-assisted positioning mode (c), the UE optionally gets assistance data from network and the network does the computation of location estimate. WLAN measurements around the UE are sent from UE to the network.

- 6.Y.2	Solution details

For the positioning modes requiring the UE to send WLAN measurements to the network (i.e., UE-assisted), the following applies:

(1)	The network should provide "WLAN supervision data" to UE. This WLAN supervision data shall contain a list of WLAN AP data that are to be measured by UE. Each WLAN AP data shall contain the basic service set identifier (BSSID) information and/or the service set identifier (SSID) information. See details below for the values of BSSID and SSID.

(2)	If the UE gets the WLAN supervision data from the network, the UE shall send only those WLAN measurements to the network whose BSSIDs and/or SSIDs match the ones indicated in the provided WLAN supervision data.

(3)	If the UE does not get the WLAN supervision data from the network, the UE shall send only those WLAN measurements to the network whose BSSIDs and/or SSIDs match the ones in "reference WLAN supervision data". This reference WLAN supervision data shall contain "reference BSSID" information and "reference SSID" information. See details below for their values.

The BSSID/SSID information and the reference BSSID/SSID information may be values, e.g., defined by 3GPP, or based on MCC, MNC, realm, or any PLMN specific identifiers. They could also be patterns instead of exact values, e.g., MCC*. They could also be values, e.g., provided by network dynamically to UE as broadcast information or in other protocol messages like RRC, NAS, or LPP. It also means that the reference WLAN supervision data could be static or dynamic.

- 6.Y.3	Evaluation

Editor's Note: The following evaluation is preliminary. It may be updated.

This solution enables the network to control the collection of WLAN measurements from UE. When used, the solution makes it possible for the network to avoid collecting unintended WLAN measurements around the UE.

It should be noted that whilst it is easy to broadcast spoofed WLAN identifiers, it is relatively difficult to block WLAN identifiers being transmitted in the same location. Therefore, any solution that only listens for specific WLAN identifiers may be easier to get spoofed than solutions that listen to all WLAN identifiers in a specific location.

For WLAN based positioning in general, it should be noted that because WLAN identifiers can be spoofed, there is low confidence in collected data.

*** END OF CHANGES ***