- 3GPP TSG SA WG3 (Security) Meeting #94Ad-Hoc	S3-190876

- Stockholm (Sweden), 11-15 March 2019                                    revision of S3-190876 and S3-190737

- Source:	Nokia, Nokia Shanghai Bell, Huawei, Hisilicon

- Title:	New Key Issue: Protection of N9 interface in Inter-PLMN scenario

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

SA3 is kindly asked to approve of the below key issue related to eSBA to TR 33.855.

# 2	References

[1]	3GPP TR 33.855 v1.3.0 "Study on security aspects of the 5G Service Based Architecture (SBA)"

[2]	3GPP TR 23.742	v16.0.0	“Study on Enhancements to the Service-Based Architecture”

# 3	Rationale

In the Roaming 5G System architecture – Home Routed scenario, a GTP_U tunnel is established per PDU session between two User Plane Functions (UPF) in the VPLMN and HPLMN.

Inter-PLMN user traffic on N9 contains home-routed user traffic from the visited network to the home network and vice-versa. This traffic may contain sensitive information, such as privacy-related information and SIP control plane messages for voice calls.

# 4	Detailed proposal

*** BEGIN CHANGES ***

- 4.B.X	Key Issue #X: Protection of N9 interface

- 4.B.X.1	Issue description

In the Roaming 5G System architecture – Home Routed scenario, a GTP_U tunnel is established per PDU session between two User Plane Functions (UPF) in the VPLMN and HPLMN. The GTP_U traffic is sent over the N9 interface between the two UPFs.

Inter-PLMN user traffic on N9 contains any home-routed user traffic from the visited network to the home network. This traffic may contain sensitive information that needs to be protected from external visibility and manipulation.

GTP_U based user traffic on N9 must be confidentiality, integrity and replay protected in Inter-PLMN scenarios.

According the objectives of FS_SBA_Secthis study, improved flexibility for protection of data exchanged on N9 shallneeds to be studied, as different levels of protection may be required for different use cases.including study of protection requirements dependent on use case. Different levels of protection may be needed considering the use cases, e.g., end-to-end delay.

- 4.B.X.2	Threat description

Lack of protection of Inter-PLMN user traffic on N9 may lead to attacks that cause leakage of sensitive information and unauthorized modification of information.

If the protection of N9 data lacks flexibility, is fixed, either all traffic are integrity and confidentiality protected, failing to fulfil the efficiency requirement of some use case, or no protection exist for the traffic, which is not acceptable for other use cases.

- 4.B.X.3	Potential security requirements

Confidentiality, integrity and replay protection shall be supported for Inter-PLMN GTP_U communication on the N9 interface.

Editor’s Note: Flexibile Flexible protection of GTP_U communication on N9 is FFS.

*** END CHANGES ***