- 3GPP TSG SA WG3 (Security) Meeting #94	S3-190967

- Stockholm (Sweden), 11-15 March 2019	revision of S3-190726

- Source:	Deutsche Telekom AG

- Title:	New Key Issue: Protection of SCOP interfaces

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

SA3 is kindly asked to approve of the below key issue related to eSBA.

# 2	References

[1]	3GPP TR 23.742		Study on Enhancements to the Service-Based Architecture

[2]	S3-190727			Secure message transport via the SCP

# 3	Rationale

Since a Rel-16 5GC is intended to support both direct communication as well as indirect communication via an SFSF (c.f. TR 23.742 [1]), both deployment options need to offer the same security with regards to intra-PLMN message transport. In addition to security requirements on message transport within the SCOP itself (i.e. between different decentralized instances), introduced in a companion contribution [2], basic security requirements must also be met at the interface between SCOP and the Network Functions connected to it.

# 4	Detailed proposal

- 4.1.Y	Key Issue #Y: Protection of SCOP interfaces

- 4.1.Y.1	Issue description

If a PLMN operator utilizes an SCOP to enable indirect communication, connections between the SCOP and other 5GC Network Functions must offer the same level of security as direct NF-NF or NF-NRF communication. Therefore, the interfaces between SCOP and all communicating parties connected to it have to be mutually authenticated. With the exception of special cases, such as NRF or SEPP, which allow for service requests without prior authorization, the SCOP may also verify whether a connected NF is allowed to make use of certain SCOP services before accepting any such requests.

The purpose of this key issue is to study the following security aspects of the SCOP:

-	Authentication and authorization of the connecting NFs by the SCOP

-	SCOP interfaces with the NRF to perform service registration/de-registration and service discovery on behalf of another NF

-	Confidentiality protection, integrity protection and replay protection at the SCOP interface towards the Network Functions connected to it

- 4.1.Y.2	Threat description

- Unauthenticated access to the SCOP may lead to the following threats:

-	Spoofing attacks

-	Theft of service

-	Man in the middle attacks between SCOP and N(R)F

A lack of confidentiality protection may lead to leakage of sensitive information.

A lack of integrity protection may lead to unnoticed modification of information in transit.

A lack of replay protection may lead to several negative impacts as a result of replay attacks, such as theft of service, leakage of sensitive information, or loss of control.

- 4.1.Y.3	Potential security requirements

Communication between the SCOP and its communication peers shall support confidentiality protection, integrity protection and replay protection.

The SCOP shall perform mutual authentication with each communication peer before granting access to its services.

The SCOP may further perform authorization of a requesting NF before granting access to certain services.