3GPP TSG-SA WG3 Meeting #94Ad-Hoc	S3-190985

Stockholm (Sweden), 11-15 March 2019	merger of S3-190827, S3-190666, S3-190671

- Source:	Ericsson, Huawei, Hisilicon

- Title:	KI#3 in TR 33.809 – new solution for enriched measurement reports

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested that proposed changes are approved.

# 2	References

[1]	N/A

# 3	Rationale

The present pCR proposes a new solution to enhance the network-based detection of false base stations in order to address the first requirement of the Key Issue #3 in TR 33.809.

# 4	Detailed proposal

*** BEGIN CHANGES ***

- 6.Y	Solution #Y: Enriched measurement reports

- 6.Y.1	Introduction

This solution addresses the first security requirement in the following key issue:

-	Key issue #3: network detection of false base stations (first requirement).

The solution provides a mechanism for enhancing the detection of false base stations by enriching the measurement reports from the UE. The solution is applicable to UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

- 6.Y.2	Solution details

The UE measurement reports specified in 3GPP TR 38.331 [2] already contain several information relevant for the detection of false base stations, e.g., identifier and received-signal strength information of the cell. Additionally, 3GPP TS 38.331 provides support for reporting of Cell Group Info (CGI_info) which contains information broadcasted in MIB and SIB1.

In addition to the existing information, the system shall support the following new information about camped and neighboring cells to be included in the measurement report:

-	mib_info = hash of the MIB, which helps in detection of DoS attempts, e.g., cellBarred=barred;

-	sib_info = list of {SIB number, hash of the SIB}, which helps in detection of DoS, fraud, and subscription identification attempts, e.g., ims-EmergencySupport=false, tampered SI-SchedulingInfo, and useFullResumeID=true;

NOTE 1:	The sib_info could contain at least SIB1 which the UE currently obtains to generate CGI_info.

NOTE 2:	The mib_info and sib_info could contain their corresponding recorded time.

Editor’s Note:	The hash algorithm used by the UE and the network is FFS.

In addition to the information above which can be transmitted by UEs in CONNECTED mode, it is also possible to extend the logged measurements, currently discussed in TS 38.331 for Rel-16, to include the following information that can help in detecting false base stations:

-	reject_info = information about REJECTs that the UE had received earlier, which helps in detection of DoS attempts, e.g., presence of rogue REJECTs;

-	signal_info = information about signal as below, which helps in detection of DoS attempts, e.g., presence of erratic radio signals:

-	just power: signal is not associated with any normal pilots or reference signals.

-	power with just pilots: signal is associated with normal pilots or reference signals, but those signals do not provide any readable system information.

-	power with pilots and system info: signal is associated with normal pilots or reference signals and those signals provide system information, but the system information is wrong (e.g., inconsistent information, not possible to access the network according to the information).

The UE shall send the above information to the network only after a successfull AS security mode command procedure.

The network shall verify and compare the above information with what is expected, e.g., comparing hashes of MIB/SIBs for reported cells with those of genuine cells using the same hash algorithm used in the UE.

Editor’s Note: Detailed procedure for network verification of the hashes of MIB/SIBs reported by the UE is FFS.

If comparision fails, e.g., the hashes do not match, the network may consider it as a factor to detect the presence of the false base station.

During handovers, the serving cell may use the above information to decide whether or not to attempt handover of the UE to the reported neighboring cell.

- 6.Y.3	Evaluation

Editor's Note:	Impacts on UE power consumption is FFS.

Enriched measurement reports certainly help in enhancing the detection of false base stations.

NOTE:	It is not in the merit of SA3 alone to define new information to be included in measurement reports. SA3 could give directions to RAN groups about information helpful for false base station detection. Then, RAN groups would design solutions. Hence, RAN groups must be liaised.

*** END OF CHANGES ***