3GPP TSG-SA WG3 Meeting #94Ad-Hoc	S3-190991

Stockholm (Sweden), 11-15 March 2019	revision of S3-190787

- Source:	Qualcomm Incorporated

- Title:	Proposed solution to the key hierarchy for non-public networks

- Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution proposes a solution to key issue #5.1 on the key hierarchy for non-public networks

# 2	References

[1]	TR 33.819 v0.2.0

[2]	TS 33.501

# 3	Rationale

The text in clauses 6.2.1 and 6.2.2 of TS 33.501 [2] are written given that the authentication method is either 5G AKA or EAP-AKA’. As noted in key issue#5.1 in TR 33.819, it is possible that other EAP methods (that is not EAP-AKA’) are used to authenticate the UE in non-public network and hence the key hierarchy etc given in these clauses cannot directly apply. The below pCR proposes a solution to key issue#5.1 outlining the necessary changes to make these clauses applicable to non-EAP-AKA’ EAP methods.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

**** START OF CHANGES ****

- 6.X	Solution #X: Key hierarchy for authentication using non-AKA EAP methods in non-public network

- 6.X.1	Introduction

This solution addresses key issue #5.1: Key hierarchy for non-public networks.

- 6.X.2	Solution details

#### NOTE 1: The key hierarchy (subclause 6.2.1 of TS 33.501[x]) and the key derivation and distribution (clause 6.2.2 of TS 33.501 [x]) clauses cannot apply to generic non-AKA EAP methods of authentication as they are written on the assumption that an AKA method is used for authentication.

For the purposes of this solution to describe the key hierarchy modifications, it is assumed that the EAP authentication terminates in the AUSF in the network.

The major change needed is on the derivation of KAUSF. It is proposed that this derivation is as follows:

- KAUSF is derived from the EMSK created by the EAP authentication as for EAP-AKA’.

The following figure captures a visual representation of that change:

Figure 6.X.2.1-1: KAUSF derivation for generic non-AKA EAP methods

Other text from clauses 6.2.1 and 6.2.2 cannot be directly used for generic non-AKA EAP methods (e.g. where the text is specific to the key hierarchy and derivation and distribution for the AKA authentication methods).

6.X.3	Evaluation

The proposed solution meets all the requirements of key issue #5.1.

**** END OF CHANGES ****