3GPP TSG-SA WG3 Meeting #94Ad-Hoc	S3-191003

Stockholm (Sweden), 11-15 March 2019	revision of S3-19xabc

- Source:	Orange, Telecom Italia

- Title:	NPN Authentication Way Forward

- Document for:	Endorsement

Agenda Item:	5.15

# 1	Decision/action requested

Endorse the way forward on NPN authentication

# 2	Proposal

It is proposed to endorse to following way forward on NPN authentication:

- The current version of TS 33.501 already covers the SA1 requirements on authentication method and credentials between UE and NPN.

- Therefore, no new authentication method will be defined in R16 and the normative/informative status of the currently inluded authentication methods in TS 33.501 will not be modified.

- For NPN, a new clause or annex eventually referencing other existing parts of TS 33.501 will be provided.

- For NPN UE in isolated scenarios, whether authentication methods are mandatory or optional will not be addressed.

This contribution does not take a stand on potential revision of key hierarchy and key derivation for NPN to be included in R16.

For information the following is stated in TS 33.501, Annex B :

Annex B (informative):
Using additional EAP methods for primary authentication

# B.1 	Introduction

The present annex describes an example of the usage of additional EAP methods for primary authentication in private networks using the 5G system as specified in TS 22.261  [7]. It is provided as an example on how the 5G authentication framework for primary authentication can be applied to EAP methods other than  EAP-AKA' The additional EAP methods are only intended for private networks or with IoT devices in isolated deployment scenarios, i.e. roaming is not considered, as specified in TS 22.261 [7].

When the 5G system is deployed in private networks, the SUPI and SUCI should be encoded using the NAI format as specified in TS 23.501 [2]. UE always includes the realm part in the NAI for routing to the correct UDM.