| Radiocommunication Study Groups |  |
| --- | --- |
|  | Annex 8 to Document 5C/358-E |
|  | 4 November 2014 |
|  | English only |
| Annex 8 to Working Party 5C Chairman’s Report |  |
| Working document towards a Preliminary draft new Report ITU-R F.[FS.IMT/BB] |  |
| Use of fixed service for backhaul for IMT and other terrestrial mobile broadband systems |  |

(Question ITU-R 247/5 and Question ITU-R 253/5)

# 1	Introduction

As traffic demands for mobile broadband communications represented in particular by International Mobile Telecommunications (IMT), including both IMT-2000 and IMT-Advanced as defined in Resolution ITU-R 56 are increasing, fixed service (FS) use in the transport network in the mobile infrastructure is becoming an important application that requires special consideration.

The fixed service transport network supports the connections between the different base stations of the mobile broadband network, as well as the connections of one base station to other network elements of the mobile infrastructure.

Even though ever increasing traffic requirements are generally best addressed by using fiber for backhaul, it may not be technically and economically feasible in every case, for example wireless backhaul is predominantly used in urban areas (for small cells), as well as in semi-rural, rural and along highway corridors where fiber is not readily available. In such cases, fixed wireless systems can be used and serve equivalently as fibers.

# 2	Scope

The aim of this Report is to show how the FS can be used in support of IMT and other mobile broadband systems, in order to ensure the backhaul connections at the base stations level and between base stations and higher-level nodes within the transport network, as well as between nodes in the core network requiring similar connection capabilities.

FS use is necessary to support the operation of IMT and other terrestrial mobile broadband networks in the transport network.  Depending on the evolution of IMT and the required transmission capacities at different levels of the transport network, different FS frequency bands are needed.

This Report gives possible structures of point-to-point (P-P) fixed service transport networks including the outline of the needs of IMT-2000 and IMT-Advanced. The Report also examines the possible use of already allocated FS spectrum. Regardless of the transmission network capacity, the choice of frequency bands depends on the local situation of the various countries (existing deployment of the frequency bands, number of mobile (IMT) operators, traffic, etc.).

# 3	References

The reader will find additional guidance in the references listed below:

| Recommendation ITU-R F.382: | Radio-frequency channel arrangements for fixed wireless systems operating in the 2 and 4 GHz band |
| --- | --- |
| Recommendation ITU-R F.383: | Radio-frequency channel arrangements for high-capacity fixed wireless systems operating in the lower 6 GHz (5 925-6 425 MHz) band |
| Recommendation ITU-R F.384: | Radio-frequency channel arrangements for medium- and high- capacity digital fixed wireless systems operating in the 6 425-7 125 MHz band |
| Recommendation ITU-R F.385: | Radio-frequency channel arrangements for fixed wireless systems operating in the 7 110-7 900 MHz band |
| Recommendation ITU-R F.386: | Radio-frequency channel arrangements for fixed wireless systems operating in the 8 GHz (7 725 to 8 500 MHz) band |
| Recommendation ITU-R F.387: | Radio-frequency channel arrangements for fixed wireless systems operating in the 10.7-11.7 GHz band |
| Recommendation ITU-R F.497: | Radio-frequency arrangements for fixed wireless systems operating in the 13 GHz (12.75-13.25 GHz) frequency band |
| Recommendation ITU-R F.595: | Radio-frequency arrangements for fixed wireless systems operating in the 17.7-19.7 GHz frequency band |
| Recommendation ITU-R F.635: | Radio-frequency arrangements based on a homogeneous pattern for fixed wireless systems operating in the 4 GHz (3 400-4 200 MHz) band |
| Recommendation ITU-R F.636: | Radio-frequency arrangements for fixed wireless systems operating in the 14.4-15.35 GHz band |
| Recommendation ITU-R F.637: | Radio-frequency arrangements for fixed wireless systems operating in the 21.2-23.6 GHz band |
| Recommendation ITU-R F.746: | Radio-frequency arrangements for fixed service systems |
| Recommendation ITU-R F.747: | Radio-frequency arrangements for fixed wireless system operating in the 10.0-10.68 GHz band |
| Recommendation ITU-R F.748: | Radio-frequency arrangements for systems of the fixed service operating in the 25, 26 and 28 GHz bands |
| Recommendation ITU-R F.749: | Radio-frequency arrangements for systems of the fixed service operating in sub-bands in the 36-40.5 GHz band |
| Recommendation ITU-R F.758: | Considerations in the development of criteria for sharing between the terrestrial fixed service and other services |
| Recommendation ITU-R F.1099: | Radio-frequency arrangements for high- and medium-capacity digital fixed wireless systems in the upper 4 GHz (4 400-5 000 MHz) band |
| Recommendation ITU-R F.1245: | Mathematical model of average radiation patterns for line-of-sight point-to-point radio-relay system antennas for use in certain coordination studies and interference assessment in the frequency range from 1 to about 70 GHz |
| Recommendation ITU-R F.1399: | Vocabulary of terms for wireless access |
| Recommendation ITU-R F.1496: | Radio-frequency channel arrangements for fixed wireless systems operating in the band 51.4-52.6 GHz |
| Recommendation ITU-R F.1497: | Radio-frequency channel arrangements for fixed wireless systems operating in the band 55.78-66 GHz |
| Recommendation ITU-R F.1520: | Radio-frequency arrangements for systems in the fixed service operating in the band 31.8-33.4 GHz |
| Recommendation ITU-R F.1568: | Radio-frequency arrangements for fixed wireless access systems in the range 10.15-10.3/10.5-10.65 GHz |
| Recommendation ITU-R F.2004: | Radio-frequency channel arrangements for fixed service systems operating in the 92-95 GHz range |
| Recommendation ITU-R F.2005: | Radio-frequency channel and block arrangements for fixed wireless systems operating in the 42 GHz (40.5 to 43.5 GHz) band |
| Recommendation ITU-R F.2006: | Radio-frequency channel and block arrangements for fixed wireless systems operating in the 71-76 and 81-86 GHz bands |
| Recommendation ITU-R M.1224: | Vocabulary of terms for International Mobile Telecommunications (IMT) |
| Recommendation ITU-R M.1390: | Methodology for the calculation of IMT-2000 terrestrial spectrum requirements |
| Recommendation ITU-R M.1457: | Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000) |
| Recommendation ITU-R M.1645: | Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000 |
| Recommendation ITU-R M.1768: | Methodology for calculation of spectrum requirements for the terrestrial component of International Mobile Telecommunications |
| Recommendation ITU-R P.530: | Propagation data and prediction methods required for the design of terrestrial line-of-sight systems |
| Recommendation ITU-R P.676: | Attenuation by atmospheric gases |
| Recommendation ITU-R P.837: | Characteristics of precipitation for propagation modelling |
| Recommendation ITU-R P.838: | Specific attenuation model for rain for use in prediction methods |
| Report ITU-R F.2107: | Characteristics and applications of fixed wireless systems operating in frequency ranges between 57 GHz and 134 GHz |
| Report ITU-R F.[FS USE-TRENDS] | Fixed service use and future trends (Document 5/yyy) |
| Report ITU-R M.2243: | Assessment of the global mobile broadband deployments and forecasts for International Mobile Telecommunications |
| Report ITU-R M.2292: | Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses |
| Report ITU-R M.[IMT.ARCH] | Architecture and Topology of IMT Networks (Attachment x.x of Chapter 5 in Doc. 5D/836) |
| Report ITU-R M.[IMT.BEYOND2020.TRAFFIC] | IMT Traffic estimates beyond year 2020 (Attachment x.x of Chapter 3 in Doc. 5D/836) |
| Handbook on Deployment of Broadband Wireless Access Systems: | http://www.itu.int/pub/R-HDB-57 |
| Handbook on Deployment of IMT-2000 systems: | http://www.itu.int/pub/T-HDB-MOB.01-2003/en |
| Handbook on Global Trends IMT ITU-R M.[IMT.HANDBOOK] | (Attachment x.x of Chapter 3 in Doc. 5D/836) |
| ECC Report 173: | Fixed Service in Europe, Current use and future trends post 2011 (http://www.erodocdb.dk/Docs/doc98/official/pdf/ECCRep173.PDF ) |

# 4	List of acronyms and definitions

## 4.1	Lits of acronyms

| CPRI FS | Common Public Radio Interface Fixed service |
| --- | --- |
| IMT | International Mobile Telecommunications |
| IMT-2000 | International Mobile Telecommunication-2000 |
| IMT-Advanced | International Mobile Telecommunications-Advanced |
| LOS MME | Line-of-sight Mobile Management Entity |
| NLoS | Non-line-of-sight |
| P-P | Point-to-point |
| QAM | Quadrature amplitude modulation |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| QPSK RRH SWG | Quadrature phase shift keying Remote Radio Head Serving Gateway |

## 4.2	Definitions

Backhaul:	Link between eNodeB and higher level network elements (such as MME and SGW) carrying S1 interface

Fronthaul:	Link between eNodeB and RRH carrying CPRI interface

Midhaul:	Link between eNodeBs and between eNodeB and small cell carrying X2 interface

[Editor’s note: There is a need to develop proper English definition for the above three terms.

If “transport” or “backhauling” is replacing backhaul in a generic way, there is a need to ensure that it is defined correctly in the context of this document.]

# 5	Impact of mobile broadband networks on fixed transport network requirements

In the development of IMT and other terrestrial mobile broadband applications, it is envisioned that different radio access systems will be connected via flexible core networks. IMT-Advanced will influence the evolution of wireless backhaul. Besides the requirement to transport larger quantities of traffic, which will be discussed in section 6, it is expected that IMT-Advanced deployments will use different cell radiuses than IMT-2000. As recent mobile broadband  moves to IMT-Advanced (or even to its beyond) and use of small cells concept, there is a need to have access to wireless short-haul very high capacity links in urban areas close to users where in many cases fiber is not available.

## 5.1	IMT-Advanced network evolution and associated backhaul requirements

To be able to assess the backhaul requirements in IMT networks it is important to understand the evolving radio access network growth strategies and deployments, which will have an impact on the backhaul solutions. The latest information on the architecture, topology and transport requirements of IMT networks could be found in [Working document towards a preliminary draft new] Report ITU-R M.[IMT.ARCH] More specifically, the cell structure for IMT-Advanced network is currently available in Report ITU-R M.2292. The essence of these IMT-related ITU-R texts is provided in Annex 1 to this Report for information.

### 5.1.1	Cell structures for IMT-Advanced network

The deployment of small cells (either additional eNB within an existing macro eNB footprint or extending the macro footprint) and eNB (macro or small cell) radio head remotisation (split radio heads connected to a single eNB data processing and control unit; similar to FWS split mount IDU/ODU technique) result in different requirements on the network interconnecting them.  As a result, it is helpful to subdivide the part of the network referenced:

- Backhaul – traditional eNB to eNB (including macro to small cells) with different footprints and eNB to MME/SGW interconnections typically carrying S1 interface only.

- Midhaul – macro eNB to small cell eNB (within the same footprint)  interconnections typically carrying X2 interface only.

- Fronthaul – connection between one and the other of separated radio transceiver functions within the eNB (e.g. between data processing and control unit to RRH) typically carrying ETSI / ORI / CPRI interface (specifically designed for this application).

This is illustrated in the following Figures 1A and 1B where examples of backhaul (BH), midhaul (MH) and fronthaul (FH) are shown.

Figure 1A

Backhaul, Midhaul & Fronthaul in transport networks

Figure 1B

Backhaul, Midhaul & Fronthaul in transport networks

In IMT-Advanced network topology, it is required to deploy backhaul for connections between “eNodeB” (base stations) or for connections between “eNodeB” and the upper level node as shown in Figure 2 below.

FIGURE 2

E-UTRAN architecture
(Reproduced from Figure 4-2 in Report ITU-R M/[IMT ARCH])

Note: S1 interface is sometimes referred as “backhaul” and the X2 interface is sometimes referred as “midhaul”.

Figure 3 shows the case where an eNodeB coverage area is extended by deploying additional antennas away from the eNodeB site. In this case, there is also the need to deploy backhaul for connection between the eNodeB site and these additional antennas.

FIGURE 3

Example of small cell extensions from eNodeB in IMT-Advanced network

Note: The extension link (or CPRI interface) is sometimes referred as “fronthaul”

In many cases of the above mentioned scenarios, it will not be technically and/or financially viable to deploy wireline based backhaul (fiber optic, cable, etc.) to connect small cells together and with upper level nodes. Therefore, wireless backhaul may become the only effective alternative to provide this important link. It will be however subject to the same constraints as the small cells in terms of location, space and power.

Another important aspect, also covered in more details in Annex 1, is latency. The endless race to maximize the use of spectrum, by including deployment of small cells, requires tight coordination with macro cells and upper level nodes. This creates the need to account for delay variations and synchronization.  Wireless backhaul must not become the bottleneck and should avoid impacting the quality delivery of services to users.

[Editor’s note: The following paragraph is added for a provisional basis. The text needs to be reviewed after receiving a response from WP 5D on the capacity requirements of the current and future mobile base stations and higher-level nodes within the transport network.]

In considering the capacity aspects of future backhaul links, information on IMT traffic increase estimate is needed. In this regard, Report ITU-R M.2243 discusses increase tendency of mobile traffic, which concludes that global mobile traffic in the world would represent more than 30 times increase during this decade (2010-2020). Furthermore, in recent studies, this global mobile traffic will continue to increase more than 70 times toward the year 2030 according to Report ITU-R M.[IMT.BEYOND2020.TRAFFIC], being supported by expansion of video and M2M applications. Such traffic growth would certainly impact capacity requirements of mobile backhaul networks.

[Editor’s note: It is needed to discuss whether detailed description on IMT network performance  or cell density increase is within the scope of this Report. Main interest in the fixed service group will be what kind of “challenge” is required in applying FWS to mobile broadband/IMT backhaul networks.]

## 5.1.2	New backhaul requirements

As shown in the previous section, changes foreseen in IMT-Advanced are driving required modifications in backhaul networks. However, a number of backhaul challenges must be overcome in order to support mobile broadband networks.

- Backhaul must be able to transport more traffic to accommodate the increases in data throughput required by users.

- Backhaul must also transport this traffic with low latency, in order to prevent negative impact on the users’ quality of experience (QoE).

Specifically for small cell and small cell extensions,

- Backhaul facilities should be cost effective, easy to install, and have a small footprint, as large number of new small cells are expected to address the demand for mobile broadband growth.

- To adapt to a challenging environment (lamp post, traffic light, etc.), the use of a new form factor antenna may be necessary.

Section 6 will explore the various existing FS frequency bands that can support these requirements.

## 5.2	Other mobile broadband network evolution and associated backhaul requirements

For terrestrial mobile broadband networks other than IMT, topology/configurations of backhaul networks are described in the Handbook on Land mobile (including wireless access) Volume 5 – Deployment of broadband wireless access systems to address the significant increase in user traffic, which has put substantial demands on the backhaul network,

# 6	FS transport network applications

In this section, the fixed service frequency bands and their appropriateness/usability for use with IMT and other terrestrial mobile broadband are reviewed. Included in this section are: technically and physically related band characteristics, []requirements for current and future systems, as well as other factors which influence the appropriateness and usability of certain bands. In addition, a comparison of the topology and band specifics is provided.

## 6.1	Characteristics of FS bands

In general, all frequency bands available for the fixed service could be used in the transport networks. In the following sections, technical characteristics of certain fixed service bands such as appropriate transmission capacities, channel spacing, channel size availability and typical link lengths are examined.

First and foremost, the increase in traffic requirements for IMT and other terrestrial mobile broadband systems requires a minimal transmission capacity. Backhaul links with too small capacity would become a bottleneck, impacting the operations of the mobile broadband system. Therefore, links with narrow RF channels are not suitable, as it will not support the large basic capacity of IMT-2000 and IMT-Advanced.

The transmission capacity of the fixed service link should be appropriate to fulfil the requirement of the mobile system i-e the number and capacity of the base stations that the fixed link needs to  backhaul (e.g. in the NGMN Report (http://ngmn.org/uploads/media/NGMN_Whitepaper_Guideline_for_LTE_Backhaul_Traffic_Estimation.pdf).

With this capacity requirement, many fixed service bands are capable of supporting deployment of IMT and other terrestrial mobile broadband systems. These cover short hops, in the range of less than a kilometer up to tens of kilometers. Fixed service bands can be divided in three broad categories, each fulfilling specific traffic requirements: Low frequency bands, mid-range frequency bands and high frequency bands.

### 6.1.1 	Low range frequency bands (below 11 GHz)

Due to their good propagation characteristics, the main application of these bands for backhaul is to support long-haul hops (typically [20] kilometres or more). These bands are very important for mobile broadband services deployment in communities outside urban areas, as well as along corridors between population centers, where wired transport mediums (such as fiber) are not technically or economically feasible. It should also be recognized that the long-hop lengths achievable by these fixed services’ bands allow minimizing the number of sites; this is an important aspect in providing economical access to mobile broadband services in remote areas. Another application could be for shorter hops without line-of-sight. In such cases, the low frequency bands with their better propagation characteristics can compensate for loses due to obstacles between two fixed stations, which is not possible with higher frequency bands. The following bands could be considered for fulfilling the above-mentioned applications:

Table 1

| Band(a) (GHz) | Frequency range (GHz) | Recommendations ITU-R F Series | Channel separation (MHz) |
| --- | --- | --- | --- |
| 2(b) | 1.7-2.1; 1.9-2.3 2.3-2.5 | 382 746, Annex 1 | 29 Up to 28 |
| 4 | 3.8-4.2 3.6-4.2 | 382 635 | Up to 29 Up to 80 |
| Upper 4 | 4.4-5.0 | 1099 | Up to 80 |
| Lower 6 | 5.925-6.425 | 383 | Up to 40 |
| Upper 6 | 6.425-7.11 | 384 | Up to 80 |
| 7 | 7.11 to 7.90 | 385 | Up to 28 |
| 8 | 7.725 to 8.5 | 386 | Up to 40 |
| 10 | 10.0 to 10.68 | 747 1568 | Up to 28 Up to 30 |
| Notes: a Although part of Recommendation ITU-R F.746, all frequency bands may not be designated for fixed service in some countries. b Frequency bands below 2 GHz are not recommended, as it is anticipated it would not support the transmission capacity required for IMT and similar mobile broadband systems. |  |  |  |

### 6.1.2 	Mid-range frequency bands (11 to 23 GHz)

Fixed service frequency bands in this range provide wireless backhaul applications supporting medium-haul links (typically between about [8] and [20] km). In this range, larger RF channel bandwidths are possible, allowing traffic requirements for IMT and other terrestrial [mobile] broadband systems in populated areas outside dense city centres, such as suburbs and industrial parks, to be better addressed.

Table 2

| Band(a) (GHz) | Frequency range (GHz) | Recommendations ITU-R F Series | Channel separation (MHz) |
| --- | --- | --- | --- |
| 11 | 10.7-11.7 | 387 | Up to 80 |
| 13 | 12.7-13.25 | 497 | Up to 28 |
| 14 | 14.25-14.5 | 746 | Up to 28 |
| 15 | 14.4-15.35 | 636 | Up to 56 |
| 18 | 17.7-19.7 | 595 | Up to 220 |
| 23 | 21.2-23.6 | 637 | Up to 112 |
| Note: a Although part of Recommendation ITU-R F.746, all frequency bands may not be designated for fixed service in some countries. |  |  |  |

6.1.3 	High range frequency bands (above 23 GHz)

Wireless backhaul applications in these fixed service frequency bands are used for short-haul links (typically less than about [8] kilometres). However, they offer very large transmission capacity, which is well suited to fulfill the high traffic requirements and small distance between cells in IMT and similar mobile broadband systems deployed in dense urban areas. Another increasingly important application is to backhaul traffic from small cells, as the location of small cells often could make fiber use impractical. In the design of mobile backhaul networks, this could imply use of fixed service systems using new form factor antennas to adapt to challenging environment for traditional parabolic antennas (lamp post, traffic light, etc.). The use of a low profile waveguide slot array antenna for fixed wireless systems for high capacity links and active antennas are being developed in millimeter-wave bands (for these technologies, see [preliminary draft new] Report ITU-R F.[FS-USE TRENDS] It is noted that some of these bands are also suitable for point-to-multipoint systems.

Table 3

| Band(a) (GHz) | Frequency range (GHz) | Recommendations ITU-R F Series | Channel separation (MHz) |
| --- | --- | --- | --- |
| 27 | 24.25-29.5 | 748 | Up to 112 |
| 31 | 31.0-31.3 | 746 | Up to 50 |
| 32 | 31.8-33.4 | 1520 | Up to 112 |
| 38 | 36.0-40.5 | 749 | Up to 112 |
| 42 | 40.5-43.5 | 2005 | Up to 112 |
| 52 | 51.4-52.6 | 1496 | Up to 56 |
| 57 | 55.78-66 | 1497 | Up to 56 |
| 70/80 | 71-76/81-86 | 2006 | Up to 5000 |
| 94 | 92.0-94 / 94.1-95 | 2004 | Up to100, N × 100 |
| Note: a Although part of Recommendation ITU-R F.746, all frequency bands may not be designated for fixed service in some countries |  |  |  |

## 6.2	Enhancement of link capacity

In the strategy to scale the link capacity up, two main directions have been taken: consideration of the use of increasingly higher modulation schemes (up to 4096QAM) in conjunction with adaptive modulation and the usage of wider channel spacing. The former point is important as low modulation may not support the required large capacity for IMT-2000 and IMT-Advanced. This second point is supported by the current trend in allowing the aggregation of two adjacent channels into one (e.g. Recommendations ITU-R F.384 ITU-R F.387 and ITU-R F.497) resulting in  a channel spacing up to about 60 MHz or 80 MHz, in frequency bands up to 13 GHz and by the extension of traditional channel spacing from 56 MHz up to 112 MHz in frequency bands above 15 GHz (e.g. Recommendations ITU-R F.1520 and ITU-R F.749), and channel aggregation or spacing of up to 224 MHz in frequency bands above 40.5 GHz by CEPT. Wider channel spacing, up to 5 GHz are today available in the 71-76/81-86 GHz bands, (the E band), allowing transport capacity in the order of several Gigabits per second. Also, channel capacity up to 10 Gbps with multilevel modulation schemes is under study.  For this reason it is expected that the frequency bands 71-76/81-86 GHz will be used primarily for applications requiring gigabit interconnections rather than for applications that require lower capacity which can be effectively deployed  in lower frequency bands (24 to 66 GHz) or in the forthcoming 90 GHz. The channel availability in various bands is presented in Table 4.

Table 4

Channel size availability according ITU-R channel arrangement recommendations

| Reference Recommendations |  |  |  | High capacity Channel spacing (MHz) (NOTE) |
| --- | --- | --- | --- | --- |
| Below 11 GHz | 11 to 23 GHz | 23 to 43 GHz | Above 43 GHz |  |
| F.382, F.635, F.383, F.384, F.385, F.386, F.747, F.1099 | F.387, F.497, F.595, F.636, F.637 | F.748, F.749, F.1520, F.2205 | F.1496, F.1497 | 28/30 (2 x 28 possible) |
| F.635, F.383, F.384, F.386, F.635, F.1099, | F.387, F.595, F.636, F.637 | F.748 |  | 40 (2 x 40 possible) |
| F.635, F.1099 | F.387 |  |  | 80 |
|  | F.636, F.595, F.637, | F.748, F.1520, F.749, F.2005 | F.1496, F.1497 | 55/56 |
|  | F.595, F.637, | F.748, F.1520, F.749, F.2005 |  | 110/112 |
|  | F.595 |  |  | 220 |
|  |  |  | F.2006 | 250 and N x 250 (up to 4750) |
| NOTE: some channel size, referred in different recommendation annexes, may not be available in some countries. |  |  |  |  |

Another solution, enabled by the adoption of packet technologies in microwave, is a technique called channel bonding. It can increase a link capacity, overcoming the limitation derived by the modulation schemes and the limited channel spacing available today.

Radio link bonding is a technique allowing the bonding together of two or more data streams from radio transceivers operating with radio channels, that although not necessarily having the same size or operating the same frequency bands, used in conjunction with adaptive modulation, to provide a seamless single link interconnection carrying an increased capacity.  It is worth noting that radio link bonding would provide traffic protection in case of severe propagation conditions, acting as a smart hitless 1:1 or N:1 protection system. When applied to a long haul application, the benefit of radio link bonding over N channels working in conjunction with adaptive modulation becomes more evident when affected by well-known selective phenomena. It allows for more link capacity using fewer channels since the protection is obtained through the extra capacity provided by adaptive modulation, available periodically in the unaffected channels.

## 6.3	Impact of rain and gaseous attenuation

Guidance is needed on how rain and gases in the air could affect the hop length of the FWS for the choice of frequency bands for backhaul links supporting the deployment of IMT and other terrestrial mobile broadband systems.

As for the rain attenuation, it is obvious that the choice of frequency band will be highly dependent on the meteorological condition, i.e. a rainfall rate, of the location where the backhaul links are deployed. For the gaseous attenuation, it is clear that this choice will be affected by oxygen absorption which is particularly significant near 60 GHz.

Example calculation on these effects using P-Series Recommendations are provided in Annex 2.

## 6.4	Non-line-of-sight microwave backhaul for small cells

As described in section 5.1 the evolution to denser radio-access networks with small cells in cluttered urban environments has introduced new challenges for microwave backhaul. A direct line of sight does not always exist between nodes, and this creates a need for near- and non-line-of-sight microwave backhaul.

Using non-line-of-sight (NLoS) propagation is a proven approach when it comes to building radio access networks. However, deploying high-performance microwave backhaul in places where there is no direct line of sight brings new challenges for network architects. Hence, bands below 6 GHz are often implemented to ensure performance for such environments. However, studies and trials have demonstrated that microwave backhaul using frequency bands above 20 GHz can, under certain conditions, perform similar to those using bands below 6 GHz – even in locations with no direct line of sight. Indeed, in traditional LoS solutions, high system gain is used to support targeted link distance and to mitigate fading factor, such as rain. For short-distance solutions, this gain may be used to compensate for NLoS propagation losses instead The key system parameter enabling the use of high-frequency bands is the much higher antenna gain for the same antenna size. With just a few simple engineering guidelines, it is possible to plan NLoS backhaul deployments that provide high network performance. And so, in the vast amount of dedicated spectrum available above 20 GHz, microwave backhaul is not only capable of providing fiber-like multi-gigabit capacity, but also supporting high performance backhaul for small cells, even in locations where there is no direct line of sight.

## 6.5	Infrastructure sharing between operators

Where allowed or encouraged by the frequency management authorities, sharing of infrastructure by mobile operators could be advantageous. Information on infrastructure sharing can be found in the Handbook on “Deployment of IMT-2000 systems.”

## 6.6	Regulatory considerations

[Editor’s note: Need to highlight the importance of a country’s regulator to support use of bands with very high capacity in short-haul, medium-haul and long-haul situations.]

[Editor’s note: it is desirable to more describe what kind of information is specifically requested in this section.]

# 7	Summary

To be developed.

Annex 1

Information on IMT network architecture and cell structure

[Editor’s note: The substance of this Annex is for the moment provisional and needs to be further updated as a new Report ITU-R M.[IMT ARCH] is being developed by Working Party 5D.]

# 1	Cell structures for different areas for IMT-Advanced network

According to Report ITU-R M.2292, IMT-advanced network topology adopts different cell sizes for the different areas (urban/suburban/rural) depending on the frequency bands as shown in the Tables A-1, A-2 and A-3.

Therefore, it is required to deploy backhaul links to meet the cell structure requirements in these Tables for connections between “eNodeBs” (base stations) or for connections between “eNodeB” and the upper level node.

Table A-1

Deployment-related parameters for IMT bands below 1 GHz
(according to Report ITU-R M.2292)

|  | Cell radius of Macro cells |  |
| --- | --- | --- |
| Frequency | Macro rural | Macro urban/suburban |
| > 1 GHz | > 5 km | 0.5-5 km |

Table A-2

Deployment-related parameters for IMT bands between 1 and 3 GHz
(according to Report ITU-R M.2292)

|  | Cell radius of Macro cells |  |  | Deployment density of Small cells within a macro cell |  |
| --- | --- | --- | --- | --- | --- |
| Frequency | Macro rural | Macro suburban | Macro urban | Small cell outdoor | Small cell indoor |
| 1 to 2 GHz | > 3 km | 0.5-3 km | 0.25-1 km | 1-3 per Macro urban ≤ 1 per Macro suburban | Depending on indoor coverage capacity demand |
| 2 to 3GHz | > 2 km | 0.4-2.5 km | 0.2-0.8 km | 1-3 per Macro urban ≤ 1 per Macro suburban | depending on indoor coverage capacity demand |

TABLE A-3

Deployment-related parameters for IMT bands between 3 and 6 GHz
(according to Report ITU-R M.2292)

|  | Cell radius of Macro cells |  | Deployment density of Small cells within a macro cell |  |
| --- | --- | --- | --- | --- |
| Frequency | Macro suburban | Macro urban | Small cell outdoor | Small cell indoor |
| 3 to 6 GHz | 0.3-2 km | 0.15-6 km | 1-3 per Macro urban ≤ 1 per Macro suburban | Depending on indoor coverage capacity demand |

# 2	General overview on the recent IMT network architecture and the cell structure

Although macro cells have been proven to be a cost effective solution in most scenarios, meeting the demand for mobile broadband is now increasingly challenging in certain situations; such as:

–	large outdoor hotspots, such as town squares and commercial streets with high traffic demand and an already dense macro network (i.e. areas where interference is high);

–	large, isolated indoor hotspots, such as businesses and hotels that may be difficult to reach from an outdoor macro network;

–	large indoor hotspots, such as shopping malls, airports and subway stations, where mobility demands and interference are high; and

–	localized, indoor hotspots or minor coverage holes, such as small offices, restaurants and retail outlets that challenge the deployment and cost structure of conventional cellular networks.

In each of these scenarios, heterogeneous networks in which small cells complement macro cells can address the growing demand for mobile broadband. However, to make this happen, a number of challenges need to be overcome, in particular the backhaul challenge.

As the number of radio nodes increases, backhaul becomes more important. Backhaul performance not only affects the data throughput available to users, but also the overall performance of the radio-access network; high performance backhaul with low latency enables tighter coordination between nodes, which in turn uses available spectrum more efficiently. Networks with large numbers of (small) cell sites require backhaul solutions that use a combination  of physical transmission media, including microwave, fiber, copper lines and wireless connectivity. For operators, there is a clear trade-off between using existing infrastructure resources, but perhaps not at exactly the needed location, and investing in dedicated backhaul transmission that results in higher overall network performance. This decision also balances installation costs and the time needed for site acquisition and deployment .

Overall, backhaul should not limit the radio access network and should have sufficient end-to-end performance to meet the desired user quality of experience (QoE). However, ongoing development of radio networks to maximize the use of available spectrum puts greater demands on delay, delay variation and synchronization – particularly between the macrocells and small cells. Such performance requirements as well as technical performance, immediacy of deployment, capacity, cost and accessibility impact the choice of backhaul solution for any given scenario. The best solution will result from a holistic view of the network.

The main driver for heterogeneous networks is the users’ need for high-quality mobile broadband services everywhere – a requirement that can be met by boosting capacity at hotspots and improving performance both at the cell edges and in buildings. Operators balance supporting users’ rising quality demands and minimizing the cost of deploying, operating and maintaining the network.  As a result, backhaul solutions should not limit the performance of heterogeneous networks, and must be cost-efficient, easy to install and rapid to deploy. Network performance should be uniform across the entire network, so that users do not experience a drop in performance when covered by small cells – resulting in a negative experience  of the whole network and operator brand.

As IMT networks evolve and grow there are three ways to boost performance. Designing a heterogeneous network in the most effective way means improving, densifying and adding to the mobile broadband infrastructure as follows:

–	improving existing macrocell sites: Enhancing macrocells with access to more spectrum, advanced antennas, increased order of diversity on the receiver and/or the transmitter, and greater baseband processing capacity within and between nodes;

–	densifying the macro network: Targeted addition of strategically located small cells can improve capacity. This approach keeps the total number of sites relatively low, while network performance becomes less sensitive to traffic location. A simple way to densify a network could be a cell-split, which enables a site to transition, for example, from a three-sector site to a six-sector site. These strategic cells could use macro equipment or even micro equipment;

–	adding small cells: Complementing macrocells with small cells and dedicated indoor solutions based on the same or complementary standards. This approach can include the use of microcells, picocells or low-power remote radio units, as well as Wi-Fi.

The most effective way to increase overall network performance is to improve existing radio sites in the macro layer through, for example, carrier aggregation. With carrier aggregation, the main requirement for backhaul is the capability to deliver more capacity on existing links.

The second way to increase overall capacity and throughput is to deploy additional macro sites. Site densification is an attractive solution as it keeps the total number of sites low, while network performance is less sensitive to traffic location. To simplify site acquisition in this case, the backhaul solution should provide low-footprint options.

When and where it is not feasible to improve or densify the macro network within the time and cost constraints dictated by the market, the macro layer can be complemented with small low-power cells and dedicated indoor solutions. The performance advantage delivered by these low-power cells (the amount of traffic they are able to handle) will depend on each small cell’s performance, how effectively each is integrated with the macro layer, and the selected backhaul option .

The deployment of small cells as part of a heterogeneous network solution could involve the installation of thousands of new sites. Managing the deployment of such a large number of radio and backhaul sites will be a key challenge as operators work to deliver additional capacity and coverage quickly and on budget.

Such deployment highlights the need for cost-efficient solutions that support the overall business case. The chosen backhaul solution must be cost effective to plan for, install, operate and maintain; it must address  the anticipated volume of data traffic that the site will handle, as well as support the required network performance. Developing backhaul options to support mobile networks that can serve any site, regardless of location, will maximize return on investment for existing sites and active equipment.  This is particularly important for small cells that have limited reach and may serve a small percentage of an operator’s subscriber base.

Different sites will pose different challenges. Having the flexibility to choose between dedicated high-performance backhaul and the reuse of existing resources on site will be essential to supporting short time-to-market at the right cost. This may be increasingly difficult to achieve without a holistic approach to radio, backhaul and installation services.

While fibre tends to be the primary mode of backhaul employed for high traffic cell sites given its large capacity and high reliability, point-to-point microwave is a cost-efficient technology for flexible and rapid backhaul deployment in most rural and remote locations, as well as metropolitan markets. The wireless microwave solution is the dominant backhaul medium for mobile networks, and is expected to maintain this position as mobile broadband evolves; with microwave technology that is capable of providing backhaul capacity in the order of several gigabits-per-second.

To obtain the maximum performance from the radio network, it is important to have an end-to-end backhaul solution that does not limit the user’s QoE. The challenge is how to strike the best balance of performance, cost, market timing and location.

Annex 2

Example calculation on the effects of rain and gaseous attenuation

Figure A-1 shows rain attenuation for the long-term statistics based on Recommendations ITU-R P.530-15 and ITU-R P.838-3. The rain attenuation increases as frequency becomes higher depending on the rain rate R0.01 X mm/h, which means that rainfall rate of X mm/hour is exceeded for 0.01% of the time. It is noted that the curve for 23 GHz in Figure A-1 (b) of R0.01 = 60 mm/h almost shows the same performance (possible distance) as the curve for 38 GHz in Figure A-1 (a) of R0.01 = 30 mm/h, taking into account the different impact of rain attenuation.

Figure A-1

Rain attenuation versus distance

(a) Rain rate R0.01 = 30 mm/h

(b) Rain rate R0.01 = 60 mm/h

Figure A-2 depicts gaseous attenuation consisting of dry air and water vapor based on Annex 2 to Recommendation ITU-R P.676-10. In this calculation, the following parameters are used for ground-level standard according to ITU-R P.835-5:

–	temperature: 288.15 K;

–	pressure: 1013.25 hPa (hectopascal);

–	water-vapor density: 7.5 g/cm3.

Figure A-2

Gaseous attenuation versus distance

It is observed that there is sharp attenuation near 60 GHz due to oxygen absorption.

Moreover comparisons are made for the 18, 23, 38, 57 and 80 GHz bands with respect to total path losses consisting of the rain attenuation, the gaseous attenuation and the free-space propagation loss. Figure A-3 reveals the total path losses.

In general, the total path losses increase as frequency becomes higher.  However, care should be taken to the fact that, due to the oxygen absorption near 60 GHz, the 57 GHz curve increases with the steepest incline among other curves and shows larger path loss than that of 80 GHz.

Figure A-3

Total path losses versus distance

(a) Rain rate R0.01 = 30 mm/h

(b) Rain rate R0.01 = 60 mm/h

______________

____________________

Resolution ITU-R 56 (2012), “Naming for International Mobile Telecommunications.”

This value should be considered as an average and is not necessarily applicable in all environments where a microwave hop is being implemented. The reader is encouraged to refer to Recommendation ITU-R P.530 and other ITU-R P Series documents for planning wireless backhaul hops.

Draft revision of ECC Recommendation (01)04 - Recommended guidelines for the accommodation and assignment of multimedia wireless systems (MWS) and point-to-point (P-P) fixed wireless systems in the frequency band 40.5-43.5 GHz.  http://www.cept.org/Documents/wg-se/15908/SE(14)038A13_Annex-13-Draft-revision-of-ECC-REC(01)04-for-public-consultation. and Proposals on WI SE19_32 (revision of channel arrangements for 50 and 52 GHz bands.  http://www.cept.org/DocumentRevisions/se-19---fixed-service/5364/SE19(14)28_WI%2032%20-%20revision%20of%20channel%20arrangements%20for%2050%20and%2052%20GHz%20bands.

http://www.itu.int/rec/R-REC-F.2006/en.

7th Framework Programme for Research European project: E3NETWORK “Energy efficient 
E-band transceiver for backhaul of the future networks” available at: http://www.ict-e3network.eu/.

Gianluca Boiocchi, Paolo DI Prisco, Ahmed Lahrech, Pierre Lopez, Maurizio Moretto and 
Paolo Volpato, “Next-generation microwave packet radio: Characteristics and evolution areas to support new scenarios in wireless backhauling” Bell Labs Technical Journal 18(2),September 2013, available at: http://onlinelibrary.wiley.com/doi/10.1002/bltj.21610/pdf.

Jonas Hansryd, Jonas Edstam, Bengt-Erik Olsson and Christina Larsson, “Non-line-of-sight microwave backhaul for small cells”, Ericsson Review, 22 February 2013; available at: http://www.ericsson.com/res/thecompany/docs/publications/ericsson_review/2013/er-nlos-microwave-backhaul.pdf

Jonas Hansryd and Jonas Edstam, “Microwave capacity evolution”, Ericsson Review, 2011; available at: http://www.ericsson.com/res/docs/review/Microwave-Capacity-Evolution.pdf.